3GPP TSG RAN WG1 #112-bis-e		R1-2302885

e-Meeting, April 17th – 26th, 2023

Source:	Panasonic

Title: 	Discussion on PRACH coverage enhancement

Agenda Item:		9.12.1

Document for:	Discussion

# Introduction

PRACH transmission is important for many procedures, e.g., initial access and beam failure recovery. One of the objectives of the Rel.18 WID of further NR coverage enhancements is to enhance the coverage performance of PRACH as follows [1]

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure

  - Note 1: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable.

This document provides our view on PRACH coverage enhancements.

# Discussion on multi-PRACH transmission

## PRACH resource for multi-PRACH transmission with the same beam

### Resource configuration

Multi-PRACH resource

Multi-PRACH resource, which denotes available RO(s) and/or preamble(s), used for a multi-PRACH transmission with the same Tx beam has been discussed. In RAN1#112, it has been agreed two working assumptions to differentiate at least between a multi-PRACH transmission and single PRACH transmission in the following.

Working Assumption #1 For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption #2 For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Each of the two working assumptions has different advantage and disadvantage as follows

- For working assumption #1, it might have limited specification impact. However, there are many existing UE features which are distinguished by partitioning preambles, such as CBRA/CFRA, 2-step/4-step RACH, Msg3 repetition, SI request, etc. Therefore, the space of separate preamble resource for the multi-PRACH transmission would be limited when they are used together.

- For working assumption #2, there is no need to further partition the preambles and to consider the coexistence of legacy UE without capable of multi-PRACH transmission. It requires more specification impact than working assumption #1, e.g., it requires to introduce frequency-domain and/or time-domain offset(s) to define separate ROs, and SSB-to-RO mapping.

Depending on the RO resource usage related to the number of UEs in a serving cell and the combinational usage of PRACH resource partitioning, one of the working assumption #1 and the working assumption #2 can be selected to be configured in the serving cell. Therefore, we support to confirm both of the working assumptions.

Proposal 1: Support to confirm two working assumptions to differentiate the multiple PRACH transmissions with single PRACH transmission, i.e., multiple PRACH are transmitted on separate ROs, or multiple PRACH are transmitted with separate preamble on shared ROs.

Proposal 2: Support to select one of the two working assumptions to configure in a serving cell.

RO group for a multi-PRACH transmission

In RAN1#112, it has been agreed to support a RO group consisting of RO(s) for a specific number of multiple PRACH transmissions, wherein the RO(s) in the RO group are either shared RO(s) only or separate RO(s) only, in the following.

Agreement For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. Note 1: All ROs in one RO group is associated with the same SSB(s). Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Note 3: whether/how to define “RO group” in specification will be discussed separately Note 4: Valid RO(s) refers to what is defined in existing specification FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH. FFS: the time span of RO group. FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions. FFS: other details

Regarding “FFS: the time span of RO group”, our view is shown in the following:

- A RO group should span over multiple periods of SSB-to-RO mapping because the RO group is associated with the same SSB for a multi-PRACH transmission with same Tx beam. In this manner, there could be two possible RO counting methods for a multi-PRACH transmission.

  - “Valid RO only counting”: For a given number of multiple PRACH transmissions, the counting is based on valid RO(s) only in a RO group, where valid RO(s) are used for actual PRACH transmissions.

  - “Any RO counting”: For a given number of multiple PRACH transmissions, the counting is regardless of valid RO(s) or invalid RO(s) in a RO group, where a RO is identified as invalid RO due to an RO dropping event and is not used for an actual PRACH transmission, while valid RO(s) are used for actual PRACH transmissions.

- The RO counting method for multi-PRACH transmission should be common among UEs in a cell. If the RO counting method is different among UEs in a cell, the first PRACH and the last PRACH transmission can be different among UEs, which makes impossible to have a combined detection over the multi-PRACH transmission at gNB.

- The number of actual PRACH transmissions using only valid ROs can be different between UEs. Especially, for a HD UE in paired band, a determination of valid RO from the shared RO can be based on its implementation (as highlighted in yellow part in below box) according to Clause 17.2 of TS 38.213. In order to have a common RO counting method among UEs in a cell, the “any RO counting method” would be more reasonable as it is simple. The number of actual PRACH transmissions can be less than the given number of multiple PRACH transmission such that needs to be managed by the network configuration.

If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or SRS and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols. If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks by ssbPositionsInBurst in SIB1 or in ServingCellConfigCommon in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks. If a HD-UE would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS based on a configuration by higher layers or is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, and the HD-UE would transmit PRACH or MsgA PUSCH triggered by higher layers starting or ending at a symbol that is earlier or later than Rx-Tx ⋅ c or Tx-Rx ⋅ c, respectively, from the last or first symbol in the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the DL PRS, or the PDCCH, or the SS/PBCH blocks

Proposal 3: Support a common RO counting method for multi-PRACH transmission among UEs in a cell.

Proposal 4: Support “any RO counting method” to define a RO group for a number of multiple PRACH transmissions

- The “any RO counting method” is regardless of valid RO(s) or invalid RO(s) in a RO group, where a RO is identified as invalid RO due to a RO dropping/cancellation event and is not used for an actual PRACH transmission, while valid RO(s) are used for actual PRACH transmissions.

Regarding “FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH”, we think it is directly related to a determination of a valid RO due to a RO dropping/cancellation event. There could be some differences depending on whether a RO is a shared RO or a separate RO.

- For shared RO

  - For paired spectrum or supplementary uplink band (except a HD-UE), all shared RO are valid.

  - For paired spectrum of a HD-UE, a shared RO can be validated as a valid RO based on the existing validation rule according to Clause 17.2 of TS 38.213 (it is shown above box).

  - For unpaired spectrum a shared RO can be validated as a valid RO based on the existing validation rule according to Clause 8.1, TS 38.213 (It is copied in Appendix).

- For separate RO

  - For paired spectrum or supplementary uplink band (except a HD-UE), a separate RO can be created as the same way as the shared RO such that all separate ROs are valid ROs.

  - For paired spectrum of a HD-UE, a separate RO can be validated as a valid RO based on the existing validation rule according to Clause 17.2 of TS 38.213

  - For unpaired spectrum, there can be 2 alternatives for a separate RO.

    - Alt. 1: A separate RO is allocated in UL slots as specified in Rel. 15/16 for the shared RO.

      - Different frequency-domain resource is used for the separate RO, compared to a shared RO.

      - A separate RO can be validated as a valid RO based on the existing validation rule according to Clause 8.1, TS 38.213

    - Alt. 2: A separate RO is allocated in a basis of available UL slots as specified for Rel. 17 PUSCH repetition type A

      - Same or different frequency-domain resources can be used for the separate RO, compared to a shared RO.

      - A separate RO can be validated as a valid RO based on Rel. 17 cancellation rule in addition to the existing validation rule according to Clause 8.1, TS 38.213.

We think that Alt. 1 can be supported because Alt. 2 might be required to have more time to discussion in RAN1. Therefore, we propose the following.

Proposal 5: For a separate RO, support to determine it as a valid RO as follows

- For paired spectrum or supplementary uplink band (except a HD-UE), a separate RO can be created as the same way as the shared RO such that all separate ROs are valid ROs.

- For paired spectrum of a HD-UE, a separate RO can be validated as a valid RO based on the existing validation rule according to Clause 17.2 of TS 38.213.

- For unpaired spectrum a separate RO is allocated in UL slots as specified in Rel. 15/16 for the shared RO.

  - Different frequency-domain resource is used for the separate RO, compared to a shared RO.

  - A separate RO can be validated as a valid RO based on the existing validation rule according to Clause 8.1, TS 38.213.

Regarding “FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions”, it might be related to an assignment between a multi-PRACH resource and a specific number of the PRACH transmission.

- There are two design choices for the assignment as follows.

  - Dedicated n-th PRACH transmission resource (RO or preamble): To assign a dedicated multi-PRACH resource for a specific number of the PRACH transmissions, e.g., a specific n-th multi-PRACH transmission is transmitted only by using a specific n-th RO in a dedicated RO group.

  - Shared n-th PRACH transmission resource: To commonly use multi-PRACH resource for all possible numbers of the PRACH transmissions, i.e., the first PRACH transmission from multi-PRACH transmission can start any of the index of multi-PRACH resource.

- If/when a combined detection over the multi-PRACH transmission is supported, a dedicated n-th PRACH transmission resource would be useful as gNB can combine from the first multi-PRACH to the last multi-PRACH transmission. However, the required PRACH resources could be too large, especially narrower system bandwidth, common to sub 2 GHz operation. Although the dedicated n-th PRACH transmission resource would provide better performance gain, our concern is that it requires many preamble resources for multi-PRACH transmissions. If a sufficient number of multi-PRACH transmission resource is not configured, it increases the latency due to waiting for the corresponding index of preamble resource. Our view is that the reduced PRACH resource usage and reduced latency can be more important than to have a combined detection of multi-PRACH combining gain depending on PRACH resource availability. Therefore, we propose to support both of the dedicated and shared n-th PRACH transmission resources for CBRA.

- In case of CFRA, the preamble resource is dedicated for the specific UE. gNB could have sufficient knowledge of UE channel condition, compared to the CBRA and no contention happen. In this case, the design that specific PRACH resource is dedicated for the specific number of the PRACH transmission can have the merit for the combined detection over the multiple PRACH transmissions. In order to save PRACH resource, the second design, i.e., a preamble resource can be used for any number of PRACH repetition transmission, can be also supported. These two designs can be semi-statically configured.

- Moreover, in both cases of CBRA and CRRA, we support that one preamble is sent in each of N ROs of a RO group.

Proposal 6: For CBRA and CFRA, support the following resource configuration for the multi-PRACH transmission

- Dedicated n-th PRACH transmission resource: A specific PRACH resource is used for a specific number of the PRACH transmissions.

- Shared n-th PRACH transmission resource: A PRACH resource can be used for all possible numbers of PRACH transmissions.

Proposal 7: For CBRA and CFRA, support to configure the dedicated or shared n-th PRACH transmission resource to a UE in a semi-static manner.

Proposal 8: Support that one preamble is sent in each of N ROs of a RO group in one PRACH attempt.

SSB-to-RO mapping

In RAN1#111, the SSB-to-RO mapping was discussed and shown in the following FL’s proposal [2].

#### FL’s proposal 10 For the separate ROs configured for multiple PRACH transmissions if any, consider the following options for SSB-to-RO/RO group mapping: - Option 1: SSB-to-RO mapping follows the legacy mechanism. - Option 2: SSB-to-RO mapping follows a new defined mechanism.

Our preference is to support that SSB-to-RO mapping follows the legacy mechanism as we don't identify any specific need of a new defined mechanism.

Proposal 9: Support to reuse the legacy mechanism for SSB-to-RO mapping for the multi-PRACH transmission.

### Number of multiple PRACH transmissions

Regarding candidate values for the number of multi-PRACH transmissions, it has been agreed a set of {2, 4, 8} to compensate the performance shortage of PRACH coverage, and gNB can configure one or multiple values for the number of multiple PRACH transmissions. If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is also supported. In this case, a UE can determine the number of multiple PRACH transmissions based on SSB-RSRP measurement and SSB-RSRP threshold(s) comparison. Moreover, for a case of RA with PDCCH order, a number of multi-PRACH transmission can be indicated in a DCI. We are open to discuss a detailed indication in the DCI. Therefore, we propose the following.

Proposal 10: Support a determination of a number of multiple PRACH transmissions as follows

- For RA without PDCCH order, a UE can determine a number of multiple PRACH transmissions based on a comparison between SSB-RSRP measurement and SSB-RSRP threshold.

- For RA with PDCCH order, a number of multiple PRACH transmissions can be indicated in a DCI.

  - FFS on detailed indication.

Note that if RAN1 is able to agree "any RO counting method" in section 2.1.1, the configuration value of a number of multiple PRACH transmissions can be larger than {2, 4, 8} as the actual PRACH transmission can be less than a configured number of PRACH transmission. For the consideration that some ROs can be invalid, to support {16} in addition to {2, 4, 8} could be sufficient. On the other hand, the range can be determined after the discussion of "any RO counting method" or "valid RO only counting method".

### RAR window and RA-RNTI calculation

RAR window

For the single PRACH transmission, the UE attempts to detect a DCI format 1_0 with CRC scrambled by a RA-RNTI candidate during a window that starts at least one symbol after the last symbol of PRACH occasion corresponding to the PRACH transmission. For the multi-PRACH transmission, it has been agreed to support only one RAR window for RAR monitoring for one RACH attempt. In this case, the starting point of a RAR window have following alternatives

- Alt. A1: The RAR window starts after the first RO used for multiple PRACH transmissions.

- Alt. A2: The RAR window starts after the last RO used for multiple PRACH transmissions.

Alt. A1 allows early termination of multi-PRACH transmissions to save UE transmission power consumption and RACH resource usage as UE can identify the successful reception of PRACH by RAR in the RAW window. For Alt. A2, the UE does not need to monitor the RAR until it completes the multi-PRACH transmission. This is useful for a combined detection of the multi-PRACH transmission and useful to save UE reception power for RAR. We propose to support both Alternatives and configure one of them semi-statically.

RA-RNTI calculation

The RA-RNTI candidate is calculated based on a RO of the single PRACH transmission according to the following equation [TS 38.321]

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id,

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for PRACH preamble transmission (0 for NUL carrier, and 1 for SUL carrier). The design of RA-RNTI is related to the corresponding RAR window.

When only one RAR window for all of the multiple PRACH transmissions is applied based on the agreement, a single RA-RNTI candidate for the multi-PRACH transmission can be calculated based on the following alternatives

- Alt. B1: The indexes of time and frequency resources of the first RO are used for RA-RNTI calculation.

- Alt. B2: The indexes of time and frequency resources of the last RO are used for RA-RNTI calculation.

A selection of above alternatives is related to the corresponding alternative for the starting point of RAR window as the window starting point and selected RO should be aligned. It should be either of the pair of {A1, B1} or {A2, B2}. It means both RNTI calculation and starting point of the RAR are based on either the first RO or the last RO of the multi-PRACH transmission. We propose to support both alternatives for calculating RA-RNTI candidate for the multi-PRACH transmission depending on the usage of early termination or combined detection.

In summary, we propose to support the following two schemes for RAR window and RA-RNTI calculation

- Scheme 1: For enabling early termination of a multi-PRACH transmission,

  - A RAR window for PRACH transmissions can start after the first RO in the RO UE group,

  - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the first RO in the RO UE group.

- Scheme 2: For achieving a combined detection of a multi-PRACH transmission,

  - A RAR window for PRACH transmissions can start after the last RO in the RO UE group,

  - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the last RO in the RO UE group.

Scheme 1 can be applied for both scenarios of using the same beam or using different beams for the multi-PRACH transmission, and it might not provide a combined detection gain. On the other hand, the purpose of scheme 2 is to achieve a combined detection gain, hence it can be applied to a scenario of using the same beam for the multi-PRACH transmission. gNB can configure one of these schemes to a UE in a semi-static manner depending on the usage of the early termination or the combined detection. Moreover, scheme 1 is suitable for the limited number of ROs in a cell, i.e., narrower system bandwidth, and scheme 2 is suitable for the sufficient number of ROs in a cell, i.e., wider system bandwidth.

Proposal 11: Support the following two schemes for RAR window and RA-RNTI calculation

- Scheme 1: For enabling early termination of a multi-PRACH transmission,

  - A RAR window for PRACH transmissions can start after the first RO in the RO UE group,

  - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the first RO in the RO UE group.

- Scheme 2: For achieving a combined detection of a multi-PRACH transmission,

  - A RAR window for PRACH transmissions can start after the last RO in the RO UE group,

  - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the last RO in the RO UE group.

Proposal 12: Support to configure one of two schemes (Schemes 1 and 2) for RAR window and RA-RNTI calculation in a semi-static manner.

### Interaction between multi-PRACH transmission and Msg3 repetition

In Rel.17, Msg.3 PUSCH repetition is supported. For requesting Msg3 repetition, the separate PRACH resources are used, and a UE selects one of them based on the RSRP of the downlink pathloss reference (i.e., RSRP is less than rsrp-ThresholdMsg3 or not). Since Msg.3 is more coverage bottleneck channel than PRACH, when multi-PRACH transmission is triggered for the CBRA, the typical subsequent operation is that Msg3 repetition would also be required to be applied. The case of having PRACH repetition and not having Msg3 repetition would not be required to be supported, but the case of not having PRACH repetition and having Msg3 repetition needs to be supported. Such design can make PRACH resource usage more efficient. Note that a multi-PRACH transmission may also be referred to PRACH repetition and they may be used interchangeably in this paper.

Proposal 13: The PRACH resource for requesting Msg3 repetition with not having PRACH repetition and PRACH resource for requesting both Msg3 and PRACH repetitions should be supported. PRACH repetition for requesting PRACH repetition with not having Msg 3 repetition is not required to be supported.

Similar to Msg3 repetition, RSRP threshold could be defined to trigger multi-PRACH transmission. When RSRP is lower than the RSRP threshold, UE may trigger multi-PRACH transmission. The multiple RSRP thresholds are required when the number of PRACH repetition is determined by UE. As mentioned above, when the multi-PRACH transmission is triggered, it is reasonable approach that Msg3 repetition is also requested.

One of the issues related to subsequent operation of Msg3 would be further lower coding rate (i.e., more repetitions and/or lower MCS index) in multi-PRACH transmission scenario. As multi-PRACH transmission scenario intends to extend the coverage more than that of just Msg.3 repetition only scenario, an achievable Msg3 performance improvement based on multi-PRACH transmission scenario is necessary. Therefore, the support of further lower coding rate of Msg3 is necessary. In Rel.17, when UE requests Msg3 repetition, MCS information field is interpreted based on the following manner.

- 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values

- 2 LSB bits of the MCS information field are used for selecting one MCS index from a SIB1 configured set with 4 candidate MCS indices.

When the number of PRACH repetition is determined by UE, the proper number of repetitions and/or MCS for Msg3 PUSCH can be different depending on the number of PRACH repetition. Since 4 repetition factors and 4 MCS indices can be configured in Rel.17 specification, the configuration of Msg3 repetitions and/or MCS can be adjusted by these 4 configured values corresponding to the multi-PRACH transmission case. However, for the channel corresponding to not having PRACH repetition and but having Msg3 repetition, these four configured can be insufficient. Therefore, to adjust the number Msg3 repetitions and/or MCS depending on PRACH repetition would be necessary. It can be realized by additional SIB1 configured set or applying scaling factor to Rel.17 configured set depending on PRACH repetition.

Proposal 14: When the multi-PRACH transmission is triggered by UE, the mechanism to enable more repetitions and/or lower MCS index than the Rel.17 configured set for Msg3 repetition should be supported.

In the agenda item 9.14.3 of Dynamic waveform switching, we identified that using DFT-s-OFDM for cell-edge UEs is desirable even when the cell is mainly operated with CP-OFDM. Therefore, we propose that a multi-PRACH transmission capable UE does not follow the configuration of msg3-transformPrecoder for Msg.3 repetition, i.e., UE transmits Msg.3 with DFT-s-OFDM even if CP-OFDM is configured via msg3-transformPrecoder [3].

## Multi-PRACH transmission with different beams

Regarding multi-PRACH transmission with different beams, in RAN1#111, the following agreement has been reached.

Agreement - Study at least the following case for multiple PRACH transmissions with different Tx beams. - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO. - Note: not related to decision on CFRA - Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered

Our preference is to prioritize to complete the basic design concept for multi-PRACH transmission with the same beam as shown in above sub-sections. On the other hand, our position of the same beam case is not always to use a combined detection of the multi-PRACH transmission. Then, there would be no need to mandate to use the same beam for other than combined detection of the multi-PRACH transmission, but it is better to select the best beam by UE. The choice of beam can be up to UE implementation. Therefore, our current position is no need of the differentiation between same beam and different beams.

Proposal 15: The design other than a combined detection of the multi-PRACH transmission, i.e., Scheme 1 in proposal 11, can be used for multi-PRACH transmission with different beams. The choice of beam can be up to UE implementation.

# Conclusions

In this contribution, we provide our view on PRACH coverage enhancements. We made the following proposals.

Proposal 1: Support to confirm two working assumptions to differentiate the multiple PRACH transmissions with single PRACH transmission, i.e., multiple PRACH are transmitted on separate ROs, or multiple PRACH are transmitted with separate preamble on shared ROs.

Proposal 2: Support to select one of the two working assumptions to configure in a serving cell.

Proposal 3: Support a common RO counting method for multi-PRACH transmission among UEs in a cell.

Proposal 4: Support “any RO counting method” to define a RO group for a number of multiple PRACH transmissions

- The “any RO counting method” is regardless of valid RO(s) or invalid RO(s) in a RO group, where a RO is identified as invalid RO due to a RO dropping/cancellation event and is not used for an actual PRACH transmissions, while valid RO(s) are used for actual PRACH transmissions.

Proposal 5: For a separate RO, support to determine it as a valid RO as follows

- For paired spectrum or supplementary uplink band (except a HD-UE), a separate RO can be created as the same way as the shared RO such that all separate ROs are valid ROs.

- For paired spectrum of a HD-UE, a separate RO can be validated as a valid RO based on the existing validation rule according to Clause 17.2 of TS 38.213.

- For unpaired spectrum a separate RO is allocated in UL slots as specified in Rel. 15/16 for the shared RO.

  - Different frequency-domain resource is used for the separate RO, compared to a shared RO.

  - A separate RO can be validated as a valid RO based on the existing validation rule according to Clause 8.1, TS 38.213.

Proposal 6: For CBRA and CFRA, support the following resource configuration for the multi-PRACH transmission

- Dedicated n-th PRACH transmission resource: A specific PRACH resource is used for a specific number of the PRACH transmissions.

- Shared n-th PRACH transmission resource: A PRACH resource can be used for all possible numbers of PRACH transmissions.

Proposal 7: For CBRA and CFRA, support to configure the dedicated or shared n-th PRACH transmission resource to a UE in a semi-static manner.

Proposal 8: Support that one preamble is sent in each of N ROs of a RO group in one PRACH attempt.

Proposal 9: Support to reuse the legacy mechanism for SSB-to-RO mapping for the multi-PRACH transmission.

Proposal 10: Support a determination of a number of multiple PRACH transmissions as follows

- For RA without PDCCH order, a UE can determine a number of multiple PRACH transmissions based on a comparison between SSB-RSRP measurement and SSB-RSRP threshold.

- For RA with PDCCH order, a number of multiple PRACH transmissions can be indicated in a DCI.

  - FFS on detailed indication.

Proposal 11: Support the following two schemes for RAR window and RA-RNTI calculation

- Scheme 1: For enabling early termination of a multi-PRACH transmission,

  - A RAR window for PRACH transmissions can start after the first RO in the RO UE group,

  - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the first RO in the RO UE group.

- Scheme 2: For achieving a combined detection of a multi-PRACH transmission,

  - A RAR window for PRACH transmissions can start after the last RO in the RO UE group,

  - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the last RO in the RO UE group.

Proposal 12: Support to configure one of two schemes (Schemes 1 and 2) for RAR window and RA-RNTI calculation in a semi-static manner.

Proposal 13: The PRACH resource for requesting Msg3 repetition with not having PRACH repetition and PRACH resource for requesting both Msg3 and PRACH repetitions should be supported. PRACH repetition for requesting PRACH repetition with not having Msg 3 repetition is not required to be supported.

Proposal 14: When the multi-PRACH transmission is triggered by UE, the mechanism to enable more repetitions and/or lower MCS index than the Rel.17 configured set for Msg3 repetition should be supported.

Proposal 15: The design other than a combined detection of the multi-PRACH transmission, i.e., Scheme 1 in proposal 11, can be used for multi-PRACH transmission with different beams. The choice of beam can be up to UE implementation.

# Appendix

An existing validation rule is copied in the following [Clause 8.1, TS 38.213].

For paired spectrum or supplementary uplink band all PRACH occasions are valid. For unpaired spectrum, - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213]. - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in clause 4.1 - If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in clause 4.1.

# Reference

[1]. RP-221858, “Revised WID on Further NR coverage enhancements,” China Telecom, RAN#96-e, June 2022.

[2]. R1-2212568, “FL Summary#3 of PRACH coverage enhancements”, China Telecom, RAN1#111, Nov. 2022.

[3]. R1-2303039, " Discussion on dynamic waveform switching", Panasonic, RAN1#112-bis-e, April 2023.