3GPP TSG RAN WG1 #112-bis-e		R1-2302886

e-Meeting, April 17th – 26th, 2023

Source:	Panasonic

Title: 	Discussion on power domain enhancement to reduce MPR/PAR

Agenda Item:		9.12.2

Document for:	Discussion

# Introduction

Within a WID for Rel. 18 CovEnh [1], power domain enhancements are described as follows

- Study and if necessary, specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

This document provides our view on the enhancements to reduce MPR/PAR.

# Enhancements to realize increasing UE power high limit

In RAN1#112, the following agreement was reached to realize increasing UE power high limit.

Agreement Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input). - FFS: details.

Regarding information exchange needed between the UE and gNB to improve scheduling and network performance when using higher power CA/DC, RAN4 has discussed several solutions on PHR reporting enhancement in RAN4#106 meeting. The solutions, on which RAN4 focuses, are described in the LS [2] as shown in below box.

## Issue 5: Whether and how PHR reporting enhancement should be considered for FR1 carriers <Recommended WF> RAN4 discussion will focus on the following solutions that have been proposed in this meeting: - Power class fallback ΔPPowerClass with aperiodic PHR. - Report power-class fallback ΔPPowerClass in the PHR per serving cell, any power-class change, fallback or return to declared power class, should trigger an aperiodic PHR. This also includes FDD PC2. - Report power-class fallback ΔPPowerClass,CA in the multi-entry PHR for the BC; any BC power-class change, fallback or return to advertised BC power class, should also trigger an aperiodic PHR. - For EN-DC report power-class fallback ΔPPowerClass,EN-DC in the multi-entry PHR for the BC. - Power class being used by the UE. Because reporting ΔPPowerClass must be a huge burden for both UE and network. - For single band HPUE operation, PC being used by a UE must be able to be reported per serving cell. - For UL inter band CA HPUE operation, PC being used by a UE must be able to be reported per serving cell per band within a band combination as well as CA PC being used CA for the band combination itself. - The sustainable duty cycle over a certain duration that would prevent triggering a power class fallback at the UE, as well as period of applicability of the ΔPPowerClass report. - Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing. - Enhance the current power headroom reporting framework to enable P-MPR reporting (via MPE field) for FR1 carriers.

With such information from RAN4 [2], we are open to the following options for PHR reporting enhancement.

- Option 1: RAN1 can discuss whether there is any signalling impact for the above solutions.

- Option 2: RAN1 can wait for additional guideline after RAN4 would reach a further progress.

# Enhancements to reduce MPR/PAR

In RAN1#110-bis-e, the following agreement has been reached.

Agreement At least the following candidate solutions for MPR/PAR reduction will be studied in RAN1. - Frequency domain spectrum shaping w/ spectrum extension - Frequency domain spectrum shaping w/o spectrum extension - Tone reservation (which can only be w/ spectrum extension)

In Rel. 15, NR supports frequency domain spectrum shaping (FDSS) without spectrum extension for pi/2 BPSK. In Rel. 16, the FDSS work has continued to design lowered-PAPR DMRS for the case of PUSCH with pi/2 BPSK in order to reduce the PAPR to the same level as that of data symbols. With such understanding, we think that there is no need to further study FDSS without SE in Rel. 18 from RAN1 perspective, but it is rather reference to other schemes. Therefore, we focus on discussing the design aspect of FDSS with SE and the design aspect of tone reservation in the following sections.

## Design aspect of FDSS-SE

It has been observed that in the power-domain, with a reduction of MPR/PAR, larger output power can be obtained from a given power amplifier (PA) system under given emission constraints and transmit signal passband quality requirements. This directly improves coverage area. In Rel. 15/16, NR FDSS without SE does not specify an exact shaping function, however, certain requirements are defined which yield boundary conditions for shaping filter implementations, i.e., gNB does not have knowledge of the exact FDSS filter or shaping function used in a UE. This approach allows UE vendors to pursue their own implementation and performance optimizations, while the system performance is guaranteed through the minimum RF requirements specified, such as transmit signal spectral flatness, in-band / output back-off (OOB) emissions, and EVM [3]. We think this approach can be reused for Rel. 18 CovEnh.

Moreover, during SI phase, it has been shown that by applying FDSS with SE for QPSK, the achievable reduction of PAR can reach the lowered-PAR level of pi/2 BPSK FDSS at the PRB allocations of interest for coverage, as shown in the following observation [4].

At the PRB allocations of interest for coverage, the OBO difference between pi/2 BPSK FDSS and QPSK FDSS with SE is reported as 0-0.3 dB.

Regarding general design aspect, the FDSS with SE includes SE part and non-SE part. In RAN1#111, it has been agreed that at least the symmetric SE option for FDSS with SE is considered for studying MPR/PAR reduction enhancements in Rel-18.

The SE part and non-SE part can be determined by using an extension factor α. For instance, Fig. 1 shows an example of diagram of NR UL transmitter supporting FDSS and SE, wherein the whole bandwidth of the SE part and non-SE part is expressed as 1+αW, where W denotes the bandwidth of non-SE part, named as possibility A. Another possibility B is that when the SE part and non-SE part are allocated within W, then the SE part and non-SE part can be determined as αW and 1-αW, respectively, wherein α is given by spectrum extension size / total allocation size such that a size of SE part is expressed in integer units of RBs. Both possibilities can work. For simplicity, we can select the possibility B such that the current values (W) of bandwidth configuration of the FDSS in the existing specifications can be reused to the FDSS with SE by just using the calculations of αW and  1-αW when a feature of the FDSS with SE is configured/triggered.

Proposal 1: If FDSS with SE is supported, determine SE size based on an extensions factor α, where it is given by spectrum extension size / total allocation size.

- SE size is expressed in integer units of RBs

Fig. 1. An example of diagram of NR UL transmitter supporting FDSS with SE

The non-SE part can be used for both data and DMRS, while SE part can be used for DMRS. Hence, DMRSs in both SE part and non-SE part are used to estimate channel for achieving diversity gain. Inter-user interference due to overlapping of SEs among different UEs can be handled by gNB’s receiver depending on spatial orthogonality (or close to orthogonality).

Regarding the extension factor α, we observe that for a given modulation order, a greater value of α is used, more PRBs or sub-carriers can be used for SE part, so a lower MPR/PAR can be achieved, larger output power can be obtained while still meeting the minimum RF requirements. However, when the greater value of α is used, the achievable spectral efficiency is reduced. Therefore, the extension factor α can be considered as a tradeoff between the achievable reduction of MPR/PAR and the achievable spectral efficiency. It implies that an optimal value of α could be determined by analyzing the tradeoff. For an example, it has been shown in [5] that α=0.75 provides lowest PAPR for QPSK modulation.

Observation 1: The extension factor α can be considered as a tradeoff between the achievable reduction of MPR/PAR and the achievable spectral efficiency.

Furthermore, when the extension factor α is a constant value, there is no need of signalling of α. Due to the PRB allocations of interest for coverage and a consideration of the achievable spectral efficiency, α is only used less than a certain PRB allocation, where the condition to apply the extension factor α is commonly known between UE and gNB.

## Design aspect of tone reservation

A common design aspect between FDSS with SE and tone reservation is to require the specific frequency-domain resource allocations for a purpose of reducing MPR/PAR. Particularly, the principle of tone reservation method is to reserve tones (sub-carriers, PRBs, etc.) for a UE to generate a compensating signal that is added to the original signal in order to achieve a lowered-MPR/PAR signal. The size of the reserved tones (aka sideband tone reservation size) can impact on MPR/PAR reduction. There would be no overlapping between the the reserved tones and the subcarriers used for transmitting the data.

Proposal 2: If tone reservation is supported, to determine sideband tone reservation size based on an extensions factor α, where it is given by spectrum extension size / total allocation size, is necessary.

- Sideband tone reservation size is expressed in integer units of RBs.

If the tone reservation is supported, there could be two available methods that can be configured to a UE, i.e., a method of Rel. 16 FDSS without SE and a method of tone reservation in Rel. 18. If both methods are independently configured to work at the same time, where each component works separately on its own. There may be following potential issues.

- Potential issue 1: Radio resource in frequency-domain are not utilized which results in low spectral efficiency.

- For example, in Fig. 2, only a few tones from legacy resource allocations in frequency-domain in Rel. 15/16 FDSS are used for data

Fig. 2. An example of radio resource allocations in frequency-domain creates issue 1

- Potential issue 2: If the resource allocations in frequency-domain are not properly configured, the achievable reduction of MPR/PAR may be decreased.

- For example, in Fig. 3, Rel. 15/16 FDSS and tone reservation do not work well because a part of the reserved tones for UE#1 is overlapped with a part of data tones for UE#2 in a not proper configuration of the resource allocations.

Fig. 3. An example of radio resource allocations in frequency-domain creates issue 2

Observation 2: In Alt. 2, when the reserved tones is not properly configured, the operation of tone reservation can conflict with the operation of Rel. 15/16 FDSS without SE.

In order to avoid the potential issues 1 and 2, the resource allocations in frequency-domain are required to be properly configured for the two methods. In this case, it requires the discussion on integration of FDSS with SE and tone reservation

## Performance evaluation of potential designs of DMRS in FDSS-SE

In RAN1#112, the following agreements and working assumption were made on DMRS enhancements in FDSS-SE.

Agreement: - If FDSS-SE is supported in Rel.18, RAN1 to further study the following approaches for DMRS, when the DMRS sequence length before extension of the sequence, if any, is larger than or equal to 30. - Approach A: The DMRS sequence is extended. A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered. - A.1: The sequence is a Type 1 DMRS sequence. - A.2: The sequence is a Type 2 DMRS sequence. - FFS: How the sequence is extended. - Approach B: The DMRS sequence is not extended. A DMRS sequence based on Type 1 or Type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension. - Note: If Type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband + extension must be valid DFT sizes as per NR specification. - Performance metrics considered for the study are PAPR, CM, [and OBO] for DMRS and 10 % BLER SNR for data (to measure channel estimation accuracy). Agreement: - If FDSS-SE is supported in Rel.18, and RB allocation resulting in DMRS sequence length smaller than 30 before extension of the sequence, if any, are supported, RAN1 to study at least the following approaches. - Approach A: The DMRS sequence is extended. A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered. - A.1: The sequence is obtained by DFT transformation of an existing DMRS sequence, e.g., Type 1 DMRS sequence. - A.2: The sequence is a Type 1 or Type 2 DMRS sequence. - FFS: How the sequence is extended. - Approach B: The DMRS sequence is not extended. A DMRS sequence based on Type 1 or Type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension. - Note: Other sequences are not precluded for Approach A and Approach B. - Performance metrics considered for the study are PAPR, CM, [and OBO] for DMRS and 10 % BLER SNR for data (to measure channel estimation accuracy). Working assumption: - The following set of configurations is for companies’ consideration for the comparison of the performance of DMRS with FDSS-SE. No spectrum extension; With spectrum extension. #PRBs; MCS; #PRBs before extension; #PRBs after extension; MCS; Spectrum extension factor. 8; 0 [only QPSK]; 6; 8; 1 [only QPSK]; 1/4. 8; 6; 6; 8; 8; 1/4. 40; 2; 30; 40; 3; 1/4. 40; 6; 30; 40; 8; 1/4. [6; 3; 4; 6; 5; 1/3]. [36; 7; 32; 36; 8; 1/9]. - FR1 4 GHz Urban scenario is prioritized. - The following filters are for companies’ consideration for the calibration of the performance of DMRS with FDSS-SE. - 3-tap (0.28 1 0.28) - [Truncated RRC (0.5 0.1667) or 2-tap (1 0.28)] - Note 1: Considered metrics are PAPR / CM, 10 % BLER SNR of data for the considered DMRS configuration (for measuring impact of channel estimation accuracy) [, and OBO] - Note 2: Companies are encouraged to consider a receiver which at least makes use of the extension of the decoding (e.g., MRC). - Note 3: The values above serve as a common basis, but any other configuration can be studied by companies.

Based on the above agreements and working assumption, we evaluate PAPR/CM performance of potential designs of DMRS in FDSS-SE. Table 1 shows the evaluation assumptions.

Table 1: Evaluation assumptions

| Carrier frequency | 4 GHz |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SCS | 30 kHz |
| Number of PRBs | For no spectrum extension: 8, 40 For spectrum extension: 6, 30 (before extension) 8, 40 (after extension) |
| Modulation scheme | QPSK |
| Spectrum extension factor | 1/4 |
| Spectrum sharing filter | 3-tap (0.28 1 0.28) |

We evaluate the following approaches of DMRS sequence generation for the potential designs. The examples of the approaches of DMRS sequence generation are illustrated in Fig. 4.

- For DMRS sequence Type 1 (i.e., ZC-sequence based DMRS is used)

  - Approach A: DMRS sequence is extended. A DMRS sequence is generated by considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband.

    - Approach A-1: The sequence is cyclically extended to span the PRBs in the extension.

    - Approach A-2: DMRS extension is applied similar to data to span the PRBs in the extension.

  - Approach B: The DMRS sequence is not extended. A DMRS sequence based on Type 1 or Type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension.

- For DMRS sequence Type 2 (i.e., pi/2-BPSK based DMRS is used)

  - Approach A: DMRS sequence is extended. A DMRS sequence is generated by considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband.

    - Approach A-2: DMRS extension is applied similar to data to span the PRBs in the extension.

  - Approach B: The DMRS sequence is not extended. A DMRS sequence based on Type 1 or Type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension.

- Approach A-1

- Approach A-2

- Approach B

Fig. 4. DMRS sequence generation approaches (6 PRBs => 8 PRBs)

Tables 2 and 3 show the PAPR and CM of DMRS sequence at CCDF = 10-1. For a comparison, PAPR and CM of data are also shown. Note that CCDF of PAPR and CM are shown in the figures in Appendix.

It can be observed from Table 2 that when DMRS sequence Type 1 is used, Approach A-1 (the sequence is cyclically extended to span the PRBs in the extension) provides the best PAPR/CM performance. It can be observed from Table 3 that when DMRS sequence Type 2 is used, Approach B (a DMRS sequence is generated considering the number of PRBs in the inband + extension) provides the slightly better PAPR/CM performance than Approach A-2.

Table 2: PAPR/CM of DMRS sequence Type 1 at CCDF = 10-1

|  | PAPR at CCDF = 10-1 |  | CM at CCDF = 10-1 |  |
| --- | --- | --- | --- | --- |
| Number of PRBs after extension | 8 PRBs | 40 PRBs | 8 PRBs | 40 PRBs |
| DMRS w/o FDSS-SE | 4.4 dB | 6.2 dB | 2.00 dB | 3.05 dB |
| DMRS w/ FDSS-SE, Approach A-1 | 2.3 dB | 3.0 dB | 0.85 dB | 1.05 dB |
| DMRS w/ FDSS-SE, Approach A-2 | 4.9 dB | 6.2 dB | 2.02 dB | 2.04 dB |
| DMRS w/ FDSS-SE, Approach B | 3.4 dB | 4.0 dB | 1.62 dB | 2.40 dB |
| Data w/o FDSS-SE | 6.4 dB | 6.9 dB | 2.34 dB | 2.15 dB |
| Data w/ FDSS-SE | 3.3 dB | 3.4 dB | 1.22 dB | 1.12 dB |

Table 3: PAPR/CM of DMRS sequence Type 2 at CCDF = 10-1

|  | PAPR at CCDF = 10-1 |  | CM at CCDF = 10-1 |  |
| --- | --- | --- | --- | --- |
| Number of PRBs after extension | 8 PRBs | 40 PRBs | 8 PRBs | 40 PRBs |
| DMRS w/o FDSS-SE | 4.8 dB | 5.3 dB | 1.26 dB | 2.12 dB |
| DMRS w/ FDSS-SE, Approach A-2 | 2.1 dB | 2.1 dB | 0.12 dB | 0.04 dB |
| DMRS w/ FDSS-SE, Approach B | 1.9 dB | 2.0 dB | 0.10 dB | 0.01 dB |
| Data w/o FDSS-SE | 6.4 dB | 6.9 dB | 2.34 dB | 2.15 dB |
| Data w/ FDSS-SE | 3.3 dB | 3.4 dB | 1.22 dB | 1.12 dB |

Based on the above PAPR/CM evaluation, Approach A-1 is the best candidate design for DMRS sequence Type 1. For DMRS sequence Type 2, Approach B provides slightly better PAPR/CM performance than Approach A-2, but since there is not much performance difference between Approach A-2 and Approach B, either Approach A-2 or Approach B can be the candidate design for DMRS sequence Type 2.

Observation 3: For DMRS enhancement in FDSS-SE, for DMRS sequence Type 1, Approach A-1 (the sequence is cyclically extended to span the PRBs in the extension) provides the best PAPR/CM performance.

Observation 4: For DMRS enhancement in FDSS-SE, for DMRS sequence Type 2, Approach B (a DMRS sequence is generated by considering the number of PRBs in the inband + extension) provides the slightly better PAPR/CM performance than Approach A-2 (DMRS extension is applied similar to data to span the PRBs in the extension).

# Conclusions

In this contribution, we provide our view on power domain enhancements. We made the following observations and proposals.

Observations:

Observation 1: The extension factor α can be considered as a tradeoff between the achievable reduction of MPR/PAR and the achievable spectral efficiency.

Observation 2: In Alt. 2, when the reserved tones is not properly configured, the operation of tone reservation can conflict with the operation of Rel. 15/16 FDSS without SE.

Observation 3: For DMRS enhancement in FDSS-SE, for DMRS sequence Type 1, Approach A-1 (the sequence is cyclically extended to span the PRBs in the extension) provides the best PAPR/CM performance.

Observation 4: For DMRS enhancement in FDSS-SE, for DMRS sequence Type 2, Approach B (a DMRS sequence is generated by considering the number of PRBs in the inband + extension) provides the slightly better PAPR/CM performance than Approach A-2 (DMRS extension is applied similar to data to span the PRBs in the extension).

Proposals:

Proposal 1: If FDSS with SE is supported, determine SE size based on an extensions factor α, where it is given by spectrum extension size / total allocation size.

- SE size is expressed in integer units of RBs

Proposal 2: If tone reservation is supported, to determine sideband tone reservation size based on an extensions factor α, where it is given by spectrum extension size / total allocation size, is necessary.

- Sideband tone reservation size is expressed in integer units of RBs.

# Reference

[1]. RP-221858, “Revised WID on Further NR coverage enhancements,” China Telecom, RAN#96-e, June 2022.

[2]. R1-2302270(R4-2303701), “Reply LS on enhancements to realize increasing UE power high limit for CA and DC”, Athens, Greece, March 2023.

[3]. TS 38.101-2, “UE Radio Transmission and Reception; Part 2: Range 2 Standalone, Technical Specification Group Radio Access Network Rel-16”, 3GPP.

[4]. TR 38.830, “Study on NR coverage enhancements TR v1.0.0”, 3GPP.

[5]. A. Boonkajay, T. Obara, T. Yamamoto, and F. Adachi, “Excess-Bandwidth Transmit Filtering Based on Minimization of Variance of Instantaneous Transmit Power for Low-PAPR SC-FDE”, IEICE Trans. on Commun., April, 2015.

# Appendix: CCDF of PAPR / CM

- PAPR

- CM

Fig. 5. CCDF of PAPR/CM (DMRS sequence Type 1, Number of PRBs after extension = 8)

- PAPR

- CM

Fig. 6. CCDF of PAPR/CM (DMRS sequence Type 1, Number of PRBs after extension = 40)

- PAPR

- CM

Fig. 7. CCDF of PAPR/CM (DMRS sequence Type 2, Number of PRBs after extension = 8)

- PAPR

- CM

Fig. 8. CCDF of PAPR/CM (DMRS sequence Type 2, Number of PRBs after extension =408)