3GPP TSG RAN WG1 #112bis-e	R1-2302894

e-Meeting, April 17th – April 26th 2023

Source:	Panasonic

Title: 	Discussion on Network-verified UE location for NR NTN

Agenda Item:		9.9.2

Document for:	Discussion and decision

# Introduction

The Rel.18 work item on Non-Terrestrial Network (NTN) [1] has been started with the following scope:

- Coverage enhancement

- NR-NTN deployment in above 10 GHz bands

- Network verified UE location

- NTN-TN and NTN-NTN mobility and service continuity enhancements

On Network-verified UE location, after the study of RAN captured in TR 38.882 [2] and the RAN1 study, it was determined that RAN is to specify necessary enhancements for Multi-RTT and DL-TDOA [1]. Multi-RTT has higher priority than DL-TDOA.

In this document, we discuss open issues on Network-verified UE location for NR NTN based on Multi-RTT positioning, i.e., aspects on reference signal design, the definitions of Rx-Tx time difference measurements for NTN, the reference point for TgNB-RX and TgNB-TX used for the gNB Rx – Tx time difference measurement in NTN, accuracy of RTT measurement based on Rx-Tx time difference, the triggering time of multi-RTT positioning and multiple RTT positioning using only Timing Advance.

# Reference signal for network verified UE location

In NTN, a PRS resource set contains PRS resources originating from the same satellite. When configuring the UE to measure on a certain PRS resource within a PRS resource set, it may be needed to indicate the associated Satellite ID. More importantly, in the case of a single satellite, PRS measurements may be restricted to the same serving cell (e.g., Earth fixed cell), thereby, the same DL PRS resource/DL PRS Resource Set will be measured but at different occasions/instants in time. It was argued in RAN1#112, that to uniquely identify a DL PRS Resource, the Satellite position and/or EpochTime/NTN timestamp may need to be added to the parameters describing a DL PRS Resource/DL PRS Resource Set.

Observation 1: When configuring PRS resource, time resources will be indicated to UE. So, UE can know which PRS is configured to be measured. And LMF can likewise distinguish the PRS based on receiving time of measurement result, i.e, when UE carried out the measurement, which can already be signalled back with NR-TimeStamp. Other parameters to describe PRS resource may beneeded only if the Network-verified UE location is extended with the multi-satellite use case.

# Definition of Rx-Tx time difference measurements in NTN

For RAN1#112, many different definitions for gNB/UE Rx-Tx Time Difference were proposed. These ranged from repurposing existing legacy R17 methods to new definitions which diverge from the legacy methods. Below there are four options for each of gNB/UE Rx-Tx Time Difference, which were agreed for down-selection. gNB Rx-Tx Time Difference and UE Rx-Tx Time Difference must match, irrespectively of which options will be selected.

UE Rx-Tx Time Difference (Agreement from RAN1#112)

Agreement Select one (or more) of the following options for enhancing UE Rx-Tx time difference in NTN Option 1: The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: - UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP. For a Transmission Point - TUE-RX is the UE received timing of downlink subframe #i from this Transmission Point (TP), defined by the first detected path in time. - TUE-TX is the UE transmit timing of the uplink subframe corresponding to subframe #i received from the TP - One or multiple DL RS for positioning, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP) Option 2: - For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. - FFS: details of report and the definition of UE Rx-Tx time difference Option 3: The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on one of the following options: - Option 3-1: This offset is reported as the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of uplink subframe #i and receive timing of downlink subframe#i - Option 3-2: UE report the index of the subframe j that is closest in time to the subframe #i received from the TP and LMF can derive the offset - Option 3-3: TA report which corresponds to the time difference of received timing of downlink subframe #i and transmit timing of uplink subframe#i rounding up to slot granularity. Option 4: - UE Rx – Tx time difference TUE-RX – TUE-TX can be directly derived from timing advance TTA - FFS: the granularity and the reporting range of TA. - Note: This implies that the existing framework for Multi-RTT positioning report can be used without need to specify a new TA report. Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

gNB Rx-Tx Time Difference (Agreement from RAN1#112)

Agreement Select one (or more) of the following options for the enhancement of gNB Rx-Tx time difference in NTN Option 1: - The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: For a Transmission Point - TgNB-RX is the Transmission and Reception Point (TRP) received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. - TgNB-TX is the TRP transmit timing of the downlink subframe corresponding to uplink subframe #i received from the UE - Multiple SRS resources can be used to determine the start of one subframe containing SRS. FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP) Option 2: - For RTT measurement in NTN, support gNB report of gNB Rx-Tx time as defined in 38.215 with the following change: - Only the SRS resource starting within a subframe can be used to determine the start of the subframe. Option 3: - Keep the current gNB Rx-Tx definition and report an offset which can covers the time duration corresponds to kmac if needed. Option 4: - For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS. FFS: details of report. Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

As guiding principle, RAN noted at the start of the specification phase that the enhancements of Network-verified UE location assume the reuse of the RAT dependent positioning framework which are reflected in Option 3 for each Rx-Tx Time Difference.

Proposal 1: RAN1 prioritizes adopting the legacy R17 definitions of Rx-Tx time difference (Option 3 each) for Network-verified UE location and falls back to other solutions only if problems are identified.

In our understanding, for all above options under Option 3 (i.e. 3-1, 3-2 and 3-3) UE reports information corresponding to the integer part of the RTT in addition to UE Rx-Tx time difference which corresponds to the fractional part of the RTT. All information in option 3-1, 3-2 and 3-3 can be faked by the UE, but we think TA report (as specified in Rel.17) is most difficult to be faked because it is used for the scheduling (e.g. determination of Koffset) by gNB. Note that although there was an argument that TA report is not reliable because GNSS module is used, all options utilize TA value, which is derived from GNSS location information, to derive the information to be reported.

Proposal 2: Option 3-3 should be adopted because TA report (as specified in Rel.17) is most difficult to be faked.

Proposal 3: (Option 3) Keep the current gNB Rx-Tx definition and report an offset which can covers the time duration corresponds to kmac if needed.

We don’t see a good reason to introduce new definitions for NR-NTN. During RAN1#112, some companies conjectured that the achievable measurement accuracy of the legacy methods is not viable. We address that point under Section 5 “Rx-Tx time difference measurements accuracy”.

# The reference point for TgNB-RX and TgNB-TX used for the gNB Rx – Tx time difference measurement in NTN

Agreement In NTN, for the position of the reference point for definition of gNB Rx – Tx time difference measurement, consider the following options: - Option 1: Onboard the satellite - Option 2: The uplink time synchronization reference point - Option 3: on the gNB

Initial proposal 3-2: For FR1, the reference point for TgNB-RX and TgNB-TX used for the gNB Rx – Tx time difference measurement in NTN shall be: - For TgNB-RX : - for SAN type 1-O: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna) - for SAN type 1-H: the Rx Transceiver Array Boundary connector. - For TgNB-TX: - for SAN type 1-O: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for SAN type 1-H: the Tx Transceiver Array Boundary connector Note: The supported SAN types are defined in TS 38.108 FFS: for FR2

From our point of view, technically all options of the reference point allow to determine the gNB Rx-Tx Time Difference and ultimately the RTT.

The figure above shows the timing relationships in NR NTN among UE, satellite, UL-synchronisation point and gNB. We consider for the gNB Rx-Tx reference point the three cases at gNB, at the UL-synchronisation point and at the satellite.

- Option 1: gNB Rx-Tx Time Difference is measured at satellite

  - RTT(UE-Sat) = gNB-RTTD + TA - Common-TA

- Option 2: gNB Rx-Tx Time Difference is measured at UL-synchronisation point

  - RTT(UE-RP) = gNB-RTTD + TA

  - RTT(RP-Sat) = Common-TA

  - RTT(UE-Sat) = RTT(UE-RP) - RTT(RP-Sat)
                       = gNB-RTTD + TA - Common-TA

- Option 3: gNB Rx-Tx Time Difference is measured at gNB

  - RTT(UE-gNB) = gNB-RTTD + TA + K_mac

  - RTT(gNB-Sat) = Common-TA + K_mac

  - RTT(UE-Sat) = RTT(UE-gNB) - RTT(gNB-Sat)
                       = gNB-RTTD + TA - Common-TA

For all options, RTT(UE-Sat)=gNB-RTTD + TA – Common-TA can be obtained, which can be computed based on gNB-RTTD, UE-RTTD and TA-report as we have shown in our previous contribution R1-2300714 for RAN1#112.

Observation 2: In Option 3, kmac is subtracted together with the Common-TA from the RTT between UE and gNB when calculate RTT between UE and satellite is calculated. Hence, LMF has no need to be informed about kmac.

In our view, the verification procedure should follow the overall design principle of the UL/DL timing relationship which is centered around the UL synchronisation point.

Proposal 4: In NTN, consider the UL-synchronization point for the position of the reference point for the definition of gNB Rx – Tx time difference measurement.

# Accuracy of RTT measurement based on Rx-Tx time difference

For TN, the UE Rx-Tx time difference measurement accuracy requirements are defined in Clause 10.1.25.2 of TS 38.133. The following proposals were made in RAN1#112.

Initial proposal 5-1 RAN1 to discuss the Rx-Tx time difference measurements accuracy in NTN. At least, the following factors impacting the accuracy should be discussed: - Impact of satellite movement - Impact of UE speed - Impact of the uplink timing error - Impact of the autonomous timing adjustment at the UE - Impact of radio propagation conditions

Initial proposal 5-2 RAN1 to send an LS to RAN4 asking for the Rx-Tx time difference measurements accuracy in NTN, particularly when the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment at the UE.

We analyze the achievable RTT measurement accuracy based on UE/gNB Rx-Tx time difference below.

UE Rx-Tx time difference measurement is based on the DL-PRS reference signal. DL-PRS is usually located towards the end of a subframe. This is modeled in such a way that DL-PRS is sent at time t0,PRS in a subframe beginning at time instant t0, whereby t0=floort0,PRS1ms*1ms. UE receives DL-PRS at time instant t1,PRS, and its containing subframe at time t1. UE determines the UE Rx-Tx Time Difference RTTDUE via the Timing Advance value tTA, which holds at subframe reception timing t1, as follows

RTTDUE=tTA-roundtTA1ms∙1ms .

This equation determines the time difference between the UE’s received subframe boundary and its closest transmit subframe boundary.

Similarly, gNB Rx-Tx time difference measurement is based on the UL-SRS reference signal. The SRS signal is sent at time t2,SRS in a subframe starting at t2. And it is received at time t3,SRS in a subframe starting at t3. The gNB Rx-Tx time difference RTTDgNB is determined as

RTTDgNB=t3-t0'-roundt3-t0'1ms∙1ms

whereby t0' is the timing of the closest transmit subframe at the gNB.

Computing t0' is complicated by the fact that we start from the UE transmission timing t2,SRS but we don’t have any knowledge about the relative subframe timings. The simulation steps to obtain t0' are in sequence the UE Rx reception timing t1,SRS', the gNB transmission timing t0,SRS', the gNB transmit subframe boundary t0', the UE Rx subframe boundary t1', the UE Tx subframe boundary t2, and finally the gNB Rx subframe boundary t3. Note that this complication only exists for this type of simulation. In practice, these steps are not required; gNB determines gNB Rx-Tx Time Difference instantly.

The unapproximated (or ideal) RTT, here, is RTT=δ1,PRS+δ2,SRS. Based on this equation, LMF can determine the satellite locations and eventually solve for the UE location. On the other hand, the legacy reporting method is based on UE Rx-Tx TD and gNB Rx-Tx TD, and TA report is additionally used for NTN (Option 3-3 in section 3).

RTTr=RTTDUE+RTTDgNB+TAreport-ceilRTTDUESL∙SL

We want to analyze the error, RTT-RTTr, it introduces in the following. We simulate a LEO satellite at 600 km in a circular orbit around the equator. gNB and UE are co-located at the same position. At time t0=0s, the satellite is directly above gNB and UE. The x-axis shows the time t0, which corresponds to the gNB transmit timing of the subframe containing PRS. The PRS-signal is offset from the gNB Tx subframe boundary by 0.6ms, hence, t0,PRS=t0+0.6ms. The validity duration of the common TA polynomial is denoted as VD and set to 10 seconds. In each 10 second segment, the CTA polynomial is obtained based on the minimax approximation of the distance between satellite and gNB/RP. In other words, UE is assumed to always use feeder-link approximation when computing delays. Additionally, we simulate various transmit timings of the SRS signal relative to the PRS signal. For example, t2,SRS-t0,PRS=-0.1s, means that the SRS signal is sent 100ms in advance of the corresponding PRS signal; and t2,SRS-t0,PRS=0.1s, means that the SRS signal is sent 100ms after of the corresponding PRS signal.

By the setup above, the analyzed error, RTT-RTTr, is composed of the approximation error introduced by the common TA polynomial (minimax design), and the error introduced by the satellite movement during determination of the Rx-Tx time differences, which causes small deviations for the computed TA-values and the computed subframe boundaries and the TA-report.The results are shown in the figure below. We observe the following behavior.

Observation 3:

- The error between the true RTT and the  RTT based on (legacy) reporting increases with the time t0, which is equivalent to saying that the error increases with larger distances between satellite and the pair of UE and gNB/RP.

- The error becomes largest at the largest distance between satellite and UE/gNB (when t0 is largest)

- The error is overall smallest if SRS is sent before PRS (orange curve, t2_SRS-t0_PRS=-0.01s)

- The error between the true RTT and the RTT based on the reporting can be kept below 10-6 seconds which corresponds to an uncertainty of 300 meters.

# Determining the triggering time of multi-RTT positioning method

In RAN1#112 it was concluded that the geometry relating the UE and the TRPs (satellites) affects positioning accuracy for network verified UE location based on Multi-RTT. As captured in the RAN1#111 conclusions, for network verification of UE location in NR NTN with single satellite in view with multi-RTT positioning the required over-the-air latency reported in evaluations ranged from less than 10s up to 180s.

When the network verifies the UE location, particularly, in case of a single satellite, the time instants/measurement windows at which the multi-RTT positioning can be triggered by the network for UE location verification (and thereby the over-the-air latency of the procedure) should be assessed and determined prior to any network verified UE location execution. This can be done in a blind manner by the network, but this is not preferable as this may lead to inaccurate positioning/verification or even a useless execution of the verification procedure if the geometry is not appropriate.

In RAN1#112, it was proposed that the geometry relating the UE and the TRPs/satellites is used to predetermine the triggering time of the multi-RTT positioning method and the time instants at which the Rx-Tx time difference measurements will be performed.

- FFS: Whether by LMF or gNB

- FFS: Details on the metric (e.g. the Geometric Dilution of Precision) to be used

- FFS: Details on necessary assistance information for such predetermination.

- FFS: Whether the multi-RTT method is not applied to a UE near satellite orbit region

- FFS: Whether LS should be sent to other WGs (e.g. RAN2/RAN3) on the potential impact on LPP/NRPPs procedures

In our view, any particular method, e.g., GDOP, for determining the triggering time will not be specified. Network will instruct UE with the measurement instants and operators will determine the measurement instant according to theirinternal rules.

Proposal 5: The triggering time of the multi-RTT positioning method and the time instants at which the Rx-Tx time difference measurements will be performed are handled internally by the network.

On the subject whether LMF or gNB are performing the necessary operations, we have argued in our previous contribution R1-2300714 for RAN1#112 that a variety of functional splits exists in which the RTT – based on its components – could be determined across UE, gNB and LMF. This is not a discussion which can be led alone by RAN1.

Proposal 6: Send LS to RAN2/RAN3 on the potential impact on LPP/NRPPs procedures.

# Multiple RTT positioning using Timing Advance

Multiple RTT positioning using Timing Advance reporting instead of the Rx-Tx time difference measurements is an alternative solution that was discussed during the study phase on Network verified UE location. RAN1#111 concluded to not preclude that the UE Rx – Tx time difference is defined as TUE-RX – TUE-TX, where TUE-RX – TUE-TX is directly derived from the timing advance TTA applied by the UE at a given subframe.

Initial proposal 7 Multi-RTT positioning using Timing Advance reporting is also supported in NTN. - FFS: the granularity and the reporting range of TA. Whether this positioning method is used for network verified UE location is subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved.

Based on the subframe timings shown below, the RTT follows as the sum of the one-way delays δ1 and δ2. If the transmit and reception timings t0 to t3 are inserted and suitably rearranged, it follows that RTT is the sum of Timing Advance value and gNB Rx-Tx Time Difference.

Observation 4: Multiple RTT positioning using Timing Advance determines the RTT as the sum of Timing Advance value and gNB Rx-Tx Time Difference. Thus, it duplicates Option 4 of Clause 3.1 on “UE Rx-Tx Time Difference” and is not needed.

# Conclusion

In this document, we discussed issues on Network verified UE location for NR NTN. We observe and propose the following:

Observation 1: When configuring PRS resource, time resources will be indicated to UE. So, UE can know which PRS is configured to be measured. And LMF can likewise distinguish the PRS based on receiving time of measurement result, i.e, when UE carried out the measurement, which can already be signalled back with NR-TimeStamp. Other parameters to describe PRS resource may beneeded only if the Network-verified UE location is extended with the multi-satellite use case.

Observation 2: In Option 3, kmac is subtracted together with the Common-TA from the RTT between UE and gNB when calculate RTT between UE and satellite is calculated. Hence, LMF has no need to be informed about kmac.

Observation 3:

- The error between the true RTT and the RTT based on (legacy) reporting increases with the time t0, which is equivalent to saying that the error increases with larger distances between satellite and the pair of UE and gNB/RP.

- The error becomes largest at the largest distance between satellite and UE/gNB (when t0 is largest)

- The error is overall smallest if SRS is sent before PRS (orange curve, t2_SRS-t0_PRS=-0.01s)

- The error between the true RTT and the RTT based on the reporting can be kept below 10-6 seconds which corresponds to an uncertainty of 300 meters.

Observation 4: Multiple RTT positioning using Timing Advance determines the RTT as the sum of Timing Advance value and gNB Rx-Tx Time Difference. Thus, it duplicates Option 4 of Clause 3.1 on “UE Rx-Tx Time Difference” and is not needed.

Proposal 1: RAN1 prioritizes adopting the legacy R17 definitions of Rx-Tx time difference (Option 3 each) for Network-verified UE location and falls back to other solutions only if problems are identified.

Proposal 2: Option 3-3 should be adopted because TA report (as specified in Rel.17) is most difficult to be faked.

Proposal 3: (Option 3) Keep the current gNB Rx-Tx definition and report an offset which can covers the time duration corresponds to kmac if needed.

Proposal 4: In NTN, consider the UL-synchronization point for the position of the reference point for the definition of gNB Rx – Tx time difference measurement.

Proposal 5: The triggering time of the multi-RTT positioning method and the time instants at which the Rx-Tx time difference measurements will be performed are handled internally by the network.

Proposal 6: Send LS to RAN2/RAN3 on the potential impact on LPP/NRPPs procedures.

# Reference

- RP-223534, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales, RAN1#98e

- TR 38.882, “Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR (Release 18)”

- TR 38.821 V16.0.0

- TS 38.215, “Physical layer measurements (Release 17)”

- R1-2300714, „Discussion on network verified UE location for NTN“, Panasonic, RAN1#112

- TS 38.305, „NG-RAN Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN“

- TS 37.355, “LTE Positioning Protocol (LPP)”

- TS 38.455, “NR Positioning Protocol A (NRPPa) (Release 17)”