3GPP TSG RAN WG1 #112bis-e	R1-2302896

Electronic meeting, 17th – 26th April, 2023

Agenda item:		9.17.14

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for eDSS

Document for:		Discussion and Decision

# Introduction

RAN1#110 made the core agreement for configuring PDCCH on LTE CRS as follows:

Agreement If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs “is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side. Note: depends on UE capability Following options can be used for PDCCH-DMRS channel estimation - legacy CE assumption - RAN1 consider support this, if no RAN4 performance requirements are defined - CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET) Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session. Conclusion RAN1 understands above agreements applies to 15kHz SCS only.

RAN1#110bis further discussed the UE capabilities and RRC configuration of the NR PDCCH on LTE CRS under agenda item 9.17 – Rel-18 Others under an email thread [110bis-e-R18-Others-01] with no conclusion [R1-2210358].

This contribution continues the discussion on UE capabilities for eDSS.

# Discussion

RAN1 has discussed over several meetings during the WI phase, whether the specifications should allow the possibility to configure the UE with a CORESET that does not have a “clean” symbol, i.e. there is no symbol that doesn’t overlap with LTE CRS. However, we do not see this as a UE capability differentiator per-se; it is clear that the all UEs have the ability to demodulate and decode a PDCCH, and this ability is still there even if the PDCCH is overlapping (fully or partially) with LTE CRS. Already with Rel-15 UEs can be configured with a CORESET that doesn’t have clean symbols as long as the UE is not aware of the presence of the LTE CRS in the system, in which case the UE would happily process the PDCCH candidate without ever bothering to worry about what the CRS REs are doing to the PDCCH channel estimate or demodulation performance. Making the UE aware of the LTE CRS presence should not eliminate this UEs ability of handling the fully CRS-overlapping CORESET.

Observation 1: Legacy PDCCH CE is able to process PDCCH with all symbols overlapping with LTE CRS

Proposal 1: Basic UE capability for PDCCH overlapping with LTE CRS supports CORESET configuration with all symbols overlapping with LTE CRS REs

The UE’s support for processing a PDCCH candidate that has REs overlapping with LTE CRS REs configured to the UE is obviously a new UE capability as the legacy UEs are expected to drop the PDCCH candidates that it knows overlap with LTE CRS. This regardless from the fact that the legacy UEs would possess the ability to process the PDCCH candidate, but due to unfortunate overspecification are supposed to drop such a candidate.

Furthermore, as shown in [R1-2206432] (Nokia, Nokia Shanghai Bell), the UE that optimizes its PDCCH demodulator/decoder may reach different performance than one that exploits the “legacy” PDCCH demodulator/decoder that doesn’t take the PDCCH/CRS collisions into account. Specifically, for a given receiver type, the gNB could avoid using or even configuring the PDCCH candidates that it knows have a very high SNR requirement or can be expected to fail no matter what the SNR when colliding with REs. In addition it would be useful for the UE could drop the PDCCH AL2 that are known to fail with very high probability and not count these PDCCH candidates in the PDCCH blind decodin budget.

Observation 2: It is beneficial for the gNB to know to what the UE is capable of in terms of PDCCH processing when overlapping with LTE CRS

Different UE capabilities helping the network to decide what to do with the colliding REs are:

- Is the PDCCH channel estimator using the DMRS of the symbol with the LTE CRS REs

- Is the PDCCH channel estimator using the DMRS REs colliding with the LTE CRS REs

- Is the PDCCH decoder always puncturing the PDCCH REs colliding with the LTE CRS RE

- Is the PDCCH channel estimator and decoder able to leverage relative power difference information between the PDCCH and the LTE CRS REs

Knowing the above capabilities allows the gNB to decide what to do with the PDCCH/PDCCH DMRS REs that collide with the LTE CRS REs. These can be expressed as 4 separate capabilities as outlined in table 1:

Proposal 2: Take the table 1 as the basis for UE capability discussion for the PDCCH overlapping with LTE CRS

Table 1: Set of new Rel-18 UE capabilities for PDCCH overlapping LTE CRS

| FG | FG name | Components | Value range | Note |
| --- | --- | --- | --- | --- |
| X-1 | Support for PDCCH monitoring on LTE CRS | The UE is capable of monitoring a PDCCH candidate that overlaps with LTE CRS RE(s) using all the PDCCH DMRS REs nominally present with the candidate | {Supported} | Basic capability for this feature. Can be supported with the PDCCH channel estimator as implemented in earlier releases. |
| X-2a | Optimized PDCCH CE for PDCCH overlapping with LTE CRS #1 | Support for ignoring the PDCCH DMRS REs colliding with LTE CRS | {Supported} | X-1 is a pre-requisite capability |
| X-2b | Optimized PDCCH CE for PDCCH overlapping with LTE CRS #2 | Support for ignoring the PDCCH DMRS REs of a symbol colliding with LTE CRS and base the PDCCH channel estimate on the symbol(s) with no overlap with LTE CRS, if at least one present | {Supported} | X-1 is a pre-requisite capability |
| X-3 | Optimized PDCCH decoding for PDCCH overlapping with LTE CRS | Support for ignoring the PDCCH REs colliding with LTE CRS | {Supported} | X-1 is a pre-requisite capability |
| X-4 | PDCCH to LTE CRS configuration | Support for PDCCH and PDCCH DMRS to LTE CRS power ratio configuration | {Supported} | X-1 is a pre-requisite capability |

# Conclusion

In this contribution we presented our initial views on UE features for eDSS, and we have made the following observations and proposals:

Observation 1: Legacy PDCCH CE is able to process PDCCH with all symbols overlapping with LTE CRS

Proposal 1: Basic UE capability for PDCCH overlapping with LTE CRS supports CORESET configuration with all symbols overlapping with LTE CRS Res

Observation 2: It is beneficial for the gNB to know to what the UE is capable of in terms of PDCCH processing when overlapping with LTE CRS

Proposal 2: Take the table 1 as the basis for UE capability discussion for the PDCCH overlapping with LTE CRS