3GPP TSG RAN WG1 #112bis-e	R1- 2302897

Electronic meeting, 17th – 26th April, 2023

Agenda item:		9.17.10

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for Multi-carrier Enhancements

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss the our initial views on UE features for Rel-18 Multi-carrier enhancements [1], in particular regarding multi-cell PDSCH / PUSCH scheduling.

# Discussion on multi-cell PDSCH/PUSCH scheduling

Based on the agreements and functionalities for multi-cell PDSCH/PUSCH scheduling, we see a need for discussions related to the following potential specific UE capabilities:

- Separate capabilities for multi-cell PDSCH and multi-cell PUSCH?

  - The operation for multi-cell scheduling of PUSCH & PDSCH is handled through the structure of ‘a set of cells’. As the set of cells is the same for PUSCH & PDSCH based on RAN1 agreements, separating the capabilities of multi-cell PUSCH and multi-cell PDSCH seems to be creating issues with the related UE capability of some of the aspects below. Moreover, we don’t think a separate capability for scheduling using DCI format 0_3 and 1_3 will be needed.

- Supported maximum number of ‘set of cells’ within a PUCCH cell group:

  - We agreed to support up to a maximum of 4 set of cells from specification perspective. There could be some signaling on the maximum number of set of cells supported by the UE for multi-cell PDSCH/PUSCH scheduling if seen needed.

- Supported maximum number of cells per set of cells

  - From specification perspective, up to 4 cells within a set of cells are supported. If seen needed, there could be UE capabilitiy signaling of the component.

- Scheduling of more than one set of cells from a single scheduling cell

  - There is a RAN1 agreement that up to N set of cells can be scheduled from a single scheduling cell, with N being a UE capability. Therefore, such signaling would need to be defined.

- Indication of the scheduled cell combination

  - Two different ways for indicating the scheduled cell combination (which also affects on the DCI content) have been agreed in RAN1.

Proposal 1: Take table 1 as the basis for UE capability discussion for the Multi-cell PDSCH / PUSCH scheduling using DCI format 0_3 / 1_3

Table 1: Starting point for Rel-18 UE capabilities for Multi-cell PDSCH / PUSCH scheduling

| FG | FG name | Components | Value range | Note |
| --- | --- | --- | --- | --- |
| X-1 | Multi-cell PDSCH/PUSCH scheduling | - Multi-cell PDSCH/ PUSCH scheduling for up to M set of cells with a PUCCH cell group from different scheduling cells - Support for up to L cells within a set of cells - Scheduling of PDSCH on one or more cells of a set of cells using DCI format 1_3 - Scheduling of PDSCH on one or more cells of a set of cells using DCI format 0_3 | For component 1: M={1,2,3,4} For component 2: L={2,3,4} | Basic capability for this feature with potential UE signaling on the maximum number of supported set of cells and max. number of cells within a set of cells |
| X-2 | Multi-cell scheduling PDSCH / PUSCH scheduling of different sets from the same scheduling cell | Multi-cell PDSCH/ PUSCH scheduling for up to N set of cells for a PUCCH cell group from the same scheduling cell | For component 1: N={2,3,4} | X-1 is a pre-requisite capability No separate signaling of M and L needed, can be taken from X1 with the total number of set of cells that can be scheduled given by M of X-1. |
| X-3a | Indication of scheduled cell combination based on indicator field in DCI 0_3 / 1_3 | Support for using an indicator in DCI 0_3 / 1_3 to indicate the scheduled cell combination for PDSCH or PUSCH scheduling | {Supported} | X-1 is a pre-requisite capability Note: A UE supporting X-1 (or X-2) needs to support X-3a or X-3b (or both) |
| X-3b | Indication of scheduled cell combination based on FDRA field in DCI 0_3 / 1_3 | Support for using the FDRA field in DCI fomat 0_3 / 1_3 to indicate the scheduled cell combination for PDSCH or PUSCH scheduling | {Supported} | X-1 is a pre-requisite capability Note: A UE supporting X-1 (or X-2) needs to support X-3a or X-3b (or both) |

# Conclusion

In this contribution we presented our initial views on UE features for Rel-18 Multi-carrier enhancements, and we have made the following proposals:

Proposal 1: Take table 1 as the basis for UE capability discussion for the Multi-cell PDSCH / PUSCH scheduling using DCI format 0_3 / 1_3

# References

[1] 	RP-220834, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., Mar. 2022