3GPP TSG RAN WG1 #112bis-e	R1- 2302898

Electronic meeting, 17th – 26th April, 2023

Agenda item:		9.17.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for Network Controlled Repeaters

Document for:		Discussion and Decision

# Introduction

RAN1 will discuss Rel-18 UE features for the first time in RAN#112bis-e, where rapporteur’s input on UE features for each WI will be presented for discussion as well. Hence, in this contribution we provide our initial views on UE features for Network Controlled Repeaters [1].

# Discussion

NCR nodes are supposed to be transparent to UEs, and hence no capability signaling from UEs is expected to support NCRs. However, similarly to IAB nodes, it is relevant to capture features supported for communication between NCR-MT and gNB.

Proposal: No UE capabilities are defined for support of NCR, but feature groups for communication between NCR node and gNB are defined in Rel-18.

Based on the agreements made so far in the WI, the following feature groups can be considered as a starting point for discussion:

Table 1: Starting point for NCR FG discussion

| FG | Description | Observations |
| --- | --- | --- |
| NCR-1 | - Fixed beam for C-link/backhaul link - TDMed UL transmission of C-link and backhaul link | Basic capability for NCR Pre-requisites: - RACH procedure and TA adjustment for the NCR-MT - CSI-RS reception, CSI measurement and reporting by the NCR-MT - SRS transmission by the NCR-MT |
| NCR-2 | Support for simultaneous UL transmission of C-link and backhaul link |  |
| NCR-3 | Support for adaptive beam for C-link/backhaul-link | Candidate values: {Rel15, Rel17, both} |
| NCR-4 | Support for periodic beam indication |  |
| NCR-5 | Support for semi-persistent beam indication |  |
| NCR-6 | Support for aperiodic beam indication Slot offset k | FFS Candidate values |
| NCR-7 | Support for explicit indication of backhaul beam | Optional feature |

Proposal: Consider Table 1 as basis for further discussion on NCR FGs.

# Conclusion

In this contribution we presented our initial views on UE features for NCR, and we have made the following proposals:

Proposal:

# References

- RP-223505, “Revised WID on NR network-controlled repeaters,” ZTE, Sanechips, Dec. 2022