3GPP TSG RAN WG1 #112b	R1-2302900

e-Meeting, April 17th – April 26th, 2023

Source:	Fujitsu

Title:	Discussion on unified TCI framework extension for multi-TRP

Agenda item:	9.1.1.1

Document for:	Discussion and Decision

Introduction

The unified framework introduced in Rel-17 intends to facilitate streamlined multi-beam transmission/reception. The Rel-18 WID for MIMO evolution for downlink and uplink is approved in RAN#94, and the detailed objectives of the extension marked with red colour are listed as follows [1]:

2. Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. 6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. 7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

As is shown above, the use cases of unified TCI framework are extended from single-TRP (STRP) in Rel-17 to multi-TRP (MTRP) in Rel-18. Some agreements have been reached in the few past meetings, and we will share some further considerations on unified TCI framework for MTRP.

Discussion

In our consideration, to keep a clean design on the premise of meeting new requirements, the mechanism of Rel-17 unified TCI framework could be reused as much as possible in Rel-18. In the meanwhile, some designs in Rel-16 TCI for MTRP could also be adopted in Rel-18 to avoid repetitive discussion, which we believe should be one of the basic principles for design of unified TCI framework extension in Rel-18.

On combinations of joint/DL/UL TCI states that can be mapped to a TCI codepoint of the existing TCI field for S-DCI based MTRP, a full set of TCI states for the two pairs of downlink and uplink corresponding to two TRPsmight not be always indicated by DCI. For a serving cell configured with joint DL/UL TCI mode, if a subset of {1st joint TCI state, 2nd joint TCI state} is indicated to UE, how to use the only indicated joint TCI state needs to be considered. In case that UE is switched into STRP operation, the indicated joint TCI state in the subset could be applied as the only unified TCI state. And in case that UE is still in MTRP operation, however, the indicated joint TCI state could be used to update the corresponding TCI state currently applied. Similarly, for a serving cell configured with separate DL/UL TCI mode, if a subset of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} is indicated to UE, there are also at least two possible behaviours for UE. Taking downlink for example, if only one DL/joint TCI state is in the subset, the indicated DL/joint TCI state could be used as the only applied unified TCI state for downlink transmission, implying STRP operation in downlink direction. As another possible way, the indicated DL/joint TCI state could be used to update the corresponding TCI state currently applied.

For either downlink or uplink, when one TCI state is indicated, each aforementioned UE behavior may have its own usage. By applying the single indicated TCI state, it follows the Rel-16 MTRP principle where STRP mode or MTRP mode depends on the number of the indicated TCI states, and the usage seems to be more aligned with the “unified” purpose. By updating the currently applied TCI state, it may provide a faster switch between MTRP and STRP, e.g., a TCI selection field can be indicated to use one TCI state for PDSCH after two TCI states have already been applied. Also, which UE behavior is preferred may depend on whether the TCI selection field is configured or not. If the TCI selection field does not exist, the dynamic switching between MTRP PDSCH and STRP PDSCH can be determined just based on the number of the indicated TCI states as in Rel-16 MTRP.

- If a subset of TCI state(s) is indicated to UE, and only one DL/joint TCI state and/or only one UL TCI state is in the subset, there are two possible alternatives for UE to use the indicated TCI state. It can be considered to support both alternatives and study when to use a particular alternative:

- Alt 1: To apply the indicated TCI state as the only unified TCI state for downlink or/and uplink

- Alt 2: To update the corresponding TCI states currently applied

Figure 1. An example of two SRS resource sets and the associated TX parameters

Under the assumption of Alt 1 in Proposal 1 where a single indicated TCI state is applied as the unified TCI state, it is worth clarifying how to apply the single TCI state to different channels, especially the channels originally configured for MTRP usage. For MTRP configured grant PUSCH, a UE is configured with two SRS resource sets and two sets of SRIs and TPMIs as shown in Figure 1. As for MTRP based PUSCH transmission, the PUSCH repetitions are associated with the two SRS resource sets based on cyclic mapping or sequential mapping. Under the assumption of Alt 1, it can happen that the UE is within a time period when a single UL TCI state is applied. Following unified TCI, the UE will perform transmission by using the single UL TCI state and a single set of power control parameters which are determined based on the indicated one UL TCI state. However, it is not clear which/how SRI, TPMI or SRS resource set is used for transmission. During the time period applying a single UL TCI state, the implication is that STRP PUSCH is preferred. Therefore, one of the two SRS resource sets and the associated SRI/TPMI should be used for all the PUSCH repetitions. For simplicity, it can be defined by default that the first SRS resource set and the associated SRI/TPMI are used.

- In case that a single indicated TCI state is used as the only unified TCI state (i.e., based on Alt 1 in Proposal 1), when MTRP configured grant PUSCH is within a period applying one UL TCI state, the first SRS resource set and the associated SRI/TPMI are used for all the PUSCH repetitions.

Figure 2. An example of PDCCH reception within a time period of one DL TCI state for PDCCH repetition

Under the assumption of Alt 1 in Proposal 1 where a single indicated TCI state is applied as the unified TCI state, similarly, it is worth clarifying how to apply the single TCI state to MTRP PDCCH. PDCCH repetition can be seen as a type of MTRP PDCCH. If a UE is configured with PDCCH repetition, the UE is expected to monitor two linked PDCCH candidates based on two DL TCI states. Under the assumption of Alt 1, it is possible that the UE monitors two linked PDCCH candidates within a period applying one DL TCI state. In this case, it is not clear how the UE monitors PDCCH based on the one DL TCI state. To address this issue, the following alternatives can be considered.

Alt.1: Both of the two linked PDCCH candidates are monitored by using the one DL TCI state. In this case, two PDCCH replicas will be transmitted based on one DL TCI state. Correspondingly, two linked PDCCH candidates will be monitored based on the one DL TCI state. As an example, in Figure 2 (Alt.1), it can be seen as PDCCH repetition from one TRP when there is only one TCI state.

Alt.2: Only one PDCCH candidate is monitored by using the one DL TCI state. In this case, only one PDCCH replica is transmitted based on one DL TCI state, i.e., PDCCH repetition is not performed. Correspondingly, only one PDCCH candidate is monitored based on the one DL TCI state. Although PDCCH repetition is configured, when there is only one TCI state, it falls back to PDCCH without repetition. This is exemplified in Figure 2 (Alt.2).

- In case that a single indicated TCI state is as the only unified TCI state (i.e., based on Alt 1 in Proposal 1), when two linked PDCCH candidates for PDCCH repetition are within a period applying one DL TCI state, the following alternatives should be considered for PDCCH monitoring:

- Alt.1: Both PDCCH candidates are monitored by using the one DL TCI state

- Alt.2: Only one PDCCH candidate is monitored by using the one DL TCI state, i.e., falling back to PDCCH without repetition

It has been discussed which TCI state a channel should be associated with, and some alternatives have been agreed for further down-selection. Similarly, it should be specified which TCI state is associated with the PUSCH scheduled by a fallback DCI. Take DCI format 0_0 as an example. As a fallback DCI, DCI format 0_0 has no “SRS resource set indicator” field. For MTRP unified TCI, a UE should be able to follow two UL TCI states within a period of time or follow a single UL TCI state within another period of time. When PUSCH scheduled by a fallback DCI is within a time period applying two UL TCI states, it should be specified which UL TCI state is assumed to be associated with the scheduled PUSCH. To address this, a fixed rule can be followed e.g., PUSCH scheduled by a fallback DCI is always associated with the first UL TCI state.

- When PUSCH scheduled by a fallback DCI is within a period applying two UL TCI states, the first UL TCI state is used for the PUSCH.

In the RAN1#112 meeting, we have an agreement on PDSCH reception as follows [2].

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value

The agreement provides schemes for PDSCH’s beam selection under certain conditions based on value of the 2-bit TCI selection field. However, in case that UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, UE’s behaviour before a time threshold and the value of time value are still FFS. To our understanding, the time threshold mentioned here is mainly aimed to ensure that UE has already decoded DCI and obtained parameters related to TCI state(s) at least. The higher layer parameter timeDurationForQCL introduced since Rel-15 is also a time threshold based on UE capability which defines minimum time interval required by UE to perform PDCCH reception and applying (legacy) TCI state(s) indicated in PDCCH. The value of timeDurationForQCL could guarantee that UE has indeed decoded DCI and obtained TCI parameters. So we think reusing parameter timeDurationForQCL as the time threshold for the TCI selection field to take effect is able to meet the needs in aforementioned agreement.

- In case that the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, the parameter timeDurationForQCL should be reused as the time threshold for the TCI selection field.

As for the timeline to apply the indicated joint/DL/UL TCI state(s) to channels/signals, recalling the parameter for beam application time in Rel-17, i.e., beamAppTime, when UE is indicated with a DL/joint TCI state different from the previously indicated one, the indicated DL/joint and/or UL TCI state should be applied at least beamAppTime symbols after the last symbol of positive HARQ-ACK corresponding to the beam indication DCI or corresponding to  PDSCH scheduled by the  beam indication DCI. The timeline to apply indicated TCI state(s) based on beamAppTime allows enough time interval from both UE’s perspective and gNB’s perspective to finish decoding DCI and switch to new beam according to the indicated TCI state(s). The time demand in Rel-17 is also necessary in Rel-18, i.e., to guarantee enough time for DCI decoding and beam switching for both UE and gNB, except for a slight difference with regard to timeline that indicated TCI states in Rel-18 are for multi-TRP instead of for single-TRP in Rel-17, so it is reasonable to retain the Rel-17 timeline to apply indicated TCI states.

- In Rel-18 unified TCI framework extension for MTRP operation, the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) should be retained, i.e., the indicated joint/DL/UL TCI state(s) is updated based on the Rel-17 beam application time.

A problem arises here, how to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the scheduling offset is less than a threshold (say, timeDurationForQCL for example) in FR2 in the case that the UE doesn’t support the capability of two default beams for S-DCI based MTRP, which is also the FFS in the agreement in [2]. If only one DL/joint TCI state is applied before the new beam indication DCI, it is natural to continue applying the TCI state before the time threshold. If two DL/joint TCI states are applied before the new beam indication DCI, however, UE’s behavior for PDSCH reception is unclear. A straightforward way is to apply the first of the two indicated TCI states by default which are already applied. Therefore, our proposal is given below.

- In the case that UE doesn’t support the capability of two default beams for S-DCI based MTRP, on how to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the scheduling offset is less than a threshold in FR2, a straightforward rule could be used, i.e.,

- if only one DL/joint TCI state is applied before the beam indication DCI, continue applying the TCI state before the time threshold; or

- if two DL/joint TCI states are applied before the beam indication DCI, apply the first indicated TCI states currently applied by default.

Regarding PUCCH transmission in M-DCI based MTRP, the following agreement has been reached in last meeting [2].

Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set () or the second BFD-RS set () when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set () and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

The higher layer parameter coresetPoolIndex is used to distinguish transmissions corresponding different TRPs for M-DCI based MTRP. For example, UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value, or UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value. When it comes to PUCCH, coresetPoolIndex could still be used to configure the association between a PUCCH resource/resource group and a TRP. Although the same effect can be achieved by directly configuring in RRC the association between a PUCCH resource/resource group and indicated TCI state(s), considering some possible special cases such as a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, using coresetPoolIndex seems a more consistent design compared to using other configurations.

- On PUCCH transmission in M-DCI based MTRP, Opt1 is preferred between Opt1 and Opt2, i.e.,

- Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission.

For a CC list, such as higher layer parameter simultaneousU-TCI-UpdateListN, it is assumed that when UE is configured with a TCI state pool with activated TCI states in any CC in the CC list, the beam indication DCI the UE receives could provide beam indication for any other CC(s) in the CC list, which will reduce the overhead for TCI state configuration for both STRP and MTRP. Furthermore, if a CC list is comprised of a mix of STRP CC(s) and MTRP CC(s), the UE is somewhat free to be configured to work in STRP or MTRP mode within one CC group (list). Besides this, a mixed CC list could allow for more flexible CC combinations within one CC list by removing the limitation imposed on CCs in the list, which could also reduce the overhead for CC grouping.

- For the flexibility and overhead reduction, a CC list (simultaneous TCI update list) consisting of MTRP CC(s) and STRP CC(s) should be supported.

Conclusion

- If a subset of TCI state(s) is indicated to UE, and only one DL/joint TCI state and/or only one UL TCI state is in the subset, there are two possible alternatives for UE to use the indicated TCI state. It can be considered to support both alternatives and study when to use a particular alternative:

- Alt 1: To apply the indicated TCI state as the only unified TCI state for downlink or/and uplink

- Alt 2: To update the corresponding TCI states currently applied

- In case that a single indicated TCI state is used as the only unified TCI state (i.e., based on Alt 1 in Proposal 1), when MTRP configured grant PUSCH is within a period applying one UL TCI state, the first SRS resource set and the associated SRI/TPMI are used for all the PUSCH repetitions.

- In case that a single indicated TCI state is as the only unified TCI state (i.e., based on Alt 1 in Proposal 1), when two linked PDCCH candidates for PDCCH repetition are within a period applying one DL TCI state, the following alternatives should be considered for PDCCH monitoring:

- Alt.1: Both PDCCH candidates are monitored by using the one DL TCI state

- Alt.2: Only one PDCCH candidate is monitored by using the one DL TCI state, i.e., falling back to PDCCH without repetition

- When PUSCH scheduled by a fallback DCI is within a period applying two UL TCI states, the first UL TCI state is used for the PUSCH.

- In case that the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, the parameter timeDurationForQCL should be reused as the time threshold for the TCI selection field.

- In Rel-18 unified TCI framework extension for MTRP operation, the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) should be retained, i.e., the indicated joint/DL/UL TCI state(s) is updated based on the Rel-17 beam application time.

- In the case that UE doesn’t support the capability of two default beams for S-DCI based MTRP, on how to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the scheduling offset is less than a threshold in FR2, a straightforward rule could be used, i.e.,

- if only one DL/joint TCI state is applied before the beam indication DCI, continue applying the TCI state before the time threshold; or

- if two DL/joint TCI states are applied before the beam indication DCI, apply the first indicated TCI states currently applied by default.

- On PUCCH transmission in M-DCI based MTRP, Opt1 is preferred between Opt1 and Opt2, i.e.,

- Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission.

- For the flexibility and overhead reduction, a CC list (simultaneous TCI update list) consisting of MTRP CC(s) and STRP CC(s) should be supported.

Reference

- RP-223276, New WID: MIMO Evolution for Downlink and Uplink, RAN#98, Dec. 12-16, 2022.

- Chair’s notes, RAN1#112, February 27 – March 3, 2023.