3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302902

e-Meeting, April 17th – April 26th, 2023

Source:	Fujitsu

Title:	Discussion on UL precoding indication for multi-panel transmission

Agenda item:	9.1.4.1

Document for:	Discussion and Decision

Introduction

For Rel-18 MIMO, STxMP (Simultaneous Transmission Multi-Panel) will be standardized. In the WID [1], the relevant objectives are as follows. The part marked with red colors will be handled under this agenda item.

6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In the RAN#112 meeting, much progress has been made for STxMP including the precoding indication, the scheme switching, the CW number and so on. In this contribution, we provide more discussions on some remaining issues for single-DCI based STxMP.

Discussion

In the RAN1#112 meeting, it has been summarized in the FL summary [2] and briefly discussed on how to distinguish the STxMP SFN PUCCH scheme and the Rel-17 mTRP PUCCH scheme. For both PUCCH schemes, two TCI states will be configured for a PUCCH resource. Based on the number of TCI states, a UE cannot know which PUCCH scheme to use when transmitting on the PUCCH resource. In our view, whether the STxMP SFN PUCCH scheme or the Rel-17 mTRP PUCCH scheme is used can be configured per PUCCH resource via a RRC signaling. During the discussions in the RAN1#112 meeting, it was also mentioned that the solution depends on whether the STxMP SFN PUCCH scheme can be configured with a Rel-15 PUCCH repetition number. For Rel-17 mTRP PUCCH, the TDM based repetition can follow a Rel-15 PUCCH repetition number. If the STxMP SFN PUCCH cannot be configured with the Rel-15 PUCCH repetition number, then the Rel-15 PUCCH repetition number may be used to distinguish the STxMP SFN PUCCH and the Rel-17 mTRP PUCCH. In our view, it is not necessary to prevent configuring the STxMP SFN PUCCH with a Rel-15 PUCCH repetition number, and whether to use this functionality can be up to the gNB control. Besides, the Rel-15 PUCCH repetition number (nrofSlots) is an optional parameter. In the current standard, it may be ambiguous whether the Rel-15 PUCCH repetition number can be absent, or more specifically, whether both the Rel-15 PUCCH repetition number (nrofSlots) and the Rel-17 PUCCH repetition number (pucch-RepetitionNrofSlots-r17) can be absent. According to TS 38.331, if pucch-RepetitionNrofSlots-r17 is not configured for Rel-17 mTRP PUCCH, the UE will apply the value of nrofSlots. Furthermore, according to the description of nrofSlots, if nrofSlots is not configured, the UE will apply the value of n1. Therefore, one interpretation is that UE will apply the value of n1 for repetition if neither RepetitionNrofSlots-r17 nor nrofSlots is configured. Actually, if pucch-RepetitionNrofSlots-r17 is not configured, this is the only way to let the repetition number being equal to the value of n1. If it is possible that the Rel-17 mTRP PUCCH is not configured with either the Rel-17 or the Rel-15 PUCCH repetition number, it is also necessary that the specific PUCCH scheme is configured per PUCCH resource.

The dynamic switching between the STxMP SFN PUCCH scheme and the Rel-17 mTRP PUCCH scheme is supported.

- Which scheme to use is configured per PUCCH resource.

In the RAN1#112 meeting, the following agreement has been achieved to study the UCI multiplexing for multi-DCI based STxMP PUSCH+PUSCH [3].

Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH:

In our view, for single-DCI based STxMP PUSCH+PUSCH, how to perform the UCI multiplexing should be also studied.

Figure 1. UCI multiplexing for single-DCI based STxMP PUSCH+PUSCH

For single-DCI based STxMP PUSCH+PUSCH, it can happen that only one of the two STxMP PUSCHs overlaps with one sTRP PUCCH on a same panel or two STxMP PUSCHs overlap with two STxMP PUCCHs on two panels respectively. If any one of the aforementioned overlapping happens, it should be specified whether UCI should be multiplexed to only one PUSCH or both PUSCHs. If two STxMP PUSCHs overlap with two STxMP PUCCHs respectively, it may seem natural that UCI should be multiplexed to two PUSCHs. However, if only one of the two STxMP PUSCHs overlaps with one sTRP PUCCH on a particular panel, it should be specified whether UCI is multiplexed to only one PUSCH as shown in Figure 1a) or UCI has to be multiplexed to both PUSCHs as shown in Figure 1b). For the UCI multiplexing with STxMP SFN PUSCH+PUSCH, following Figure 1a) seems to be against the spirit of STxMP SFN PUSCH+PUSCH. Therefore, following Figure 1b) may be more reasonable. However, for the UCI multiplexing with STxMP SDM PUSCH+PUSCH, following Figure 1a) is still STxMP SDM PUSCH+PUSCH, and so is following Figure 1b). To avoid any ambiguity between the gNB and the UE, it should be clarified whether UCI is multiplexed to only one PUSCH or both PUSCHs. Also it should be studied whether the existing equation based on the offset β and the scaling factor α needs to be modified when determining the number of REs used for UCI. For example, for STxMP SDM PUSCH+PUSCH, if the UCI multiplexing is performed as in Figure 1a), it is worth studying whether the equation based on β and α needs to be modified. The reason is that UCI is only multiplexed to a part of the codeword (CW) on the first panel, and this is different from the existing UCI multiplexing.

For single-DCI based STxMP PUSCH+PUSCH, it should be studied whether UCI is multiplexed to only one PUSCH or two PUSCHs for the following scenarios.

- Scenario 1: Only one of two STxMP PUSCHs overlaps with one sTRP PUCCH on a same panel.

- Scenario 2: Two STxMP PUSCHs overlap with two STxMP PUCCHs on two panels respectively.

Conclusion

In this contribution, we have expressed our views on STxMP. The proposals are summarized as follows.

- The dynamic switching between the STxMP SFN PUCCH scheme and the Rel-17 mTRP PUCCH scheme is supported.

- Which scheme to use is configured per PUCCH resource.

For single-DCI based STxMP PUSCH+PUSCH, it should be studied whether UCI is multiplexed to only one PUSCH or two PUSCHs for the following scenarios.

- Scenario 1: Only one of two STxMP PUSCHs overlaps with one sTRP PUCCH on a same panel.

- Scenario 2: Two STxMP PUSCHs overlap with two STxMP PUCCHs on two panels respectively.

Reference

- RP-223276, New WID: MIMO Evolution for Downlink and Uplink, RAN#98, Dec. 12-16, 2022.

- R1-2301821, Summary #3 on Rel-18 STxMP, February 27 – March 3, 2023.

- Chair’s notes, RAN1#112, February 27 – March 3, 2023.