3GPP TSG RAN WG1 #112bis-e	R1-2302905

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.2.2

Source: 	Fujitsu

Title: 	    Views on specification impact for CSI feedback enhancement

Document for:	Discussion/Decision

Introduction

AI/ML-based channel state information (CSI) feedback enhancement is agreed to be one of the use cases in this study item [1]. In the previous RAN WG1 #112 meeting, the following aspects of the potential specification impacts were discussed [2] for the CSI compression using two-sided AI/ML models (Fig. 1):

- Data collection.

- CSI configurations and CSI reports.

- AI/ML performance monitoring.

Fig. 1: The two-sided AI/ML model [1].

Highlights of the agreements achieved are summarized as follows.

Agreement In CSI compression using two-sided model use case, further study potential specification impact of the following output-CSI-UE and input-CSI-NW at least for Option 1: - Option 1: Precoding matrix - 1a: The precoding matrix in spatial-frequency domain - 1b: The precoding matrix represented using angular-delay domain projection. - Option 2: Explicit channel matrix (i.e., full Tx * Rx MIMO channel) - 2a: raw channel is in spatial-frequency domain - 2b: raw channel is in angular-delay domain - Note: Whether Option 2 is also studied depends on the performance evaluations in 9.2.2.1. - Note: RI and CQI will be discussed separately Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded. - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead. Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least: - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection Agreement In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report: - NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration. - How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network. Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: The priority rule regarding CSI collision handling and CSI omission.

In this document, we further share our views on potential specification impacts on AI/ML model pairing, and AI/ML model performance monitoring for the sub use case of CSI compression using two-sided AI/ML models.

AI/ML Model Pairing

The AI/ML-based CSI generation part is often equipped at the UE side, which transforms the channel information into a bit sequence as a part of the CSI. The AI/ML-based CSI reconstruction part is equipped at the NW side, which recovers the channel information. At the NW side, a gNB is expected to support various types of AI/ML-based models manufactured by a number of vendors. On the other hand, however, only some certain types of AI/ML-based models may be supported by a UE. The knowledge of the types of AI/ML-based model supported by a UE may not be assumed to be available by the NW. For this reason, it would be difficult for the NW to configure proper AI/ML-based CSI generation part to the UE. Consequently, the AI/ML model pairing should be done before configurations and AI/ML-based CSI reporting.

Our view is that the NW and UE should align the models they support before communications in multi-vendor collaboration. Specifically, an efficient method of aligning the models from both sides is that UE reports to the NW the AI/ML models it supports. This may be implemented by reporting some identifications (IDs) of the AI/ML models from UE to the NW. The NW, based on the AI/ML models it owns, assigns the available AI/ML to the UE, potentially by configuring some IDs to the UE. In this way, the UE and NW align their supported AI/ML models, and they are ready to communicate.

In the RAN2 #121 meeting, it is agreed that the model ID is assumed to be unique “globally”. The global model ID can be regarded as an identification for each one of the AI/ML models in use.

RAN2 #121 agreement [3] RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified.

Besides global model ID, some “local ID” may be helpful for aligning the AI/ML models available at both sides because not all the AI/ML models are equipped. Pairing ID, which indicates the encoder/decoder pairing information, may serve for this purpose. Specifically, a pairing ID may be allocated to a pair of AI/ML models. For example, the AI/ML-based CSI generation part and the AI/ML-based CSI reconstruction part may both named as Model #1, or be named as Model #1-encoder and Model #1-decoder, respectively. The advantages of the pairing ID are two-folded.

- The pairing information can be observed straightforwardly.

- The information bits used to communicating the AI/ML model availability is significantly saved compared to that of using global model ID.

In the following, we give an example to illustrate the AI/ML model alignment using global model ID or pairing IDs. As the setup, the AI/ML models supported by the UE and NW are given in the first and second columns of Table 1. The AI/ML models deployed at the NW-side are Model #A, #B, #C, #D, and #E. A UE supports Model #A, #C, and #X. As the first step, the UE reports to the NW these three AI/ML models it supports. Upon receiving the report from UE, the NW looks up the list of its available AI/ML models and finds out that the Model #X is beyond its reach. The NW then decides to assign Model #A and #C to this UE.

Table 1: AI/ML model alignment between UE and NW.

| NW-side supported AI/ML models | UE-side report | NW-side can assign (Paired model ID) |
| --- | --- | --- |
| Model #A | Model #A | Model #A |
| Model #B | Model #C | Model #C |
| Model #C | Model #X |  |
| Model #D |  |  |
| Model #E |  |  |

Proposal-1: For the sub use case of CSI compression using two-sided AI/ML models, study the mechanism that UE and NW align their supported AI/ML models in the multi-vendor collaborations.

In the example above, the IDs, i.e., #A, #B, …, can be regarded as global model IDs. A UE reports to the NW, possibly through UE capability report, the global model IDs of the AI/ML models available for some scenario and configurations. Since the global model IDs may cause a large overhead, The NW may assign pairing IDs to the UE to inform the outcome of the AI/ML model alignment. Compared to the global model ID, pairing ID is in the sense of a “local” model ID. The AI/ML models assigned with pairing IDs belong to a subset of the set of all of the AI/ML models. The size of the former may be significantly smaller than the latter.

The IDs, i.e., #A, #B, …, can also be regarded as pairing IDs. In this case, the AI/ML model alignment may be regarded as a two-stage process. The first stage is finished in the model training process, either joint training or separate training. The pairing ID is assigned upon successfully training a pair of AI/ML models. At the second stage, the UE reports the pairing IDs for the AI/ML models available for the scenario and configuration of interest to the NW. The NW may inform the UE the information of the AI/ML models in common.

We have the following proposal.

Proposal-2: For the sub use case of CSI compression using two-sided AI/ML models, study the method for indicating the pairing information of the AI/ML-based CSI generation and reconstruction parts. The pairing ID can be studied as a starting point.

AI/ML Model Performance Monitoring

In the RAN1 #110bis-e meeting, the following agreement is made, regarding the co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode [4].

Agreement (RAN1 #110bis-e) In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

In the RAN1 #112 meeting, the agreement of studying the potential specification impacts on intermediate KPI-based AI/ML model performance monitoring is achieved. The monitoring can be performed at UE side or NW side [2].

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

In this paper, we study the intermediate-KPI-based AI/ML model performance monitoring at NW side. To simplify the presentation, we assume that the target CSI is the true right singular vectors of the channel matrix from realistic channel estimation. The intermediate KPI is assumed to be the square generalized cosine similarity (SGCS). For simplicity, we assume that there is only a unique payload size for an AI/ML model, i.e., AI/ML models with scalability are not considered in this section.

Procedures for Intermediate-KPI-Based Monitoring

At the NW side, the SGCS of the target CSI and the AI/ML-based CSI may be computed. Thresholds for SGCS may be set to monitor the performance of the AI/ML model(s) in use. There may be multiple thresholds depending on the configurations, e.g., the payload size of an AI/ML model. Specifically, the threshold for an AI/ML model which outputs a CSI of a larger payload size is expected to be higher than that outputting a CSI of a smaller payload size. For an easy reference, we call the threshold in this paragraph the “Class A threshold”.

The threshold may also be the performance of the non-AI/ML-based CSI feedback, e.g., the SGCS of the target CSI and the precoding vector corresponding to some PMI. Specifically, since the target CSI is reported to the NW from the UE, the PMI can be computed at the NW side. If the target CSI is nearly the true right singular vector of the channel matrix, e.g., each component of the right singular vector is represented by a float64 number. the legacy PMI computed at the NW side can be regarded as the same as the one computed at the UE side. Let the bitwidth of PMI be similar to that of the precoding matrix related information generated by AI/ML approach. The SGCS of the target CSI and the precoding vector corresponding to the PMI may be used as the threshold for AI/ML model performance monitoring. We refer to this threshold as “Class B threshold”.

The “Class B threshold” is computed at the NW side as presented in the previous paragraph. However, it may be computed at the UE side, and be reported to the NW in some cases. This may happen when the target CSI reported is not the true right singular vectors. Specifically, the overhead of reporting true right singular vectors may be prohibitive. Hence, other approaches for quantizing the target CSI may be adopted in AI/ML monitoring. One of the examples is the Rel-16 e-type II-like codebook with new parameter values, which offers a reasonable accuracy with significant overhead reduction [5]. However, the precoding vectors associated with the Rel-16 e-type II-like target CSI may not be accurate enough, compared with the true right singular vectors, to guarantee that the SGCS of it and its PMI is a good “Class B threshold”. In this case, one possible solution is that UE generates the PMI of the right singular vectors with a similar bitwidth to the precoding matrix related information generated by AI/ML approach , and computes the SGCS of the PMI and the Rel-16 e-type II-like target CSI as the “Class B threshold”. The UE reports the “Class B threshold”, precoding matrix related information generated by AI/ML approach, and Rel-16 e-type II-like target CSI to the NW for the purpose of AI/ML monitoring.

In summary, “Class A threshold” is a threshold configured by NW or available in specification, which may be chosen case by case depending on the demand. “Class B threshold” is the threshold representing the performance of the legacy non-AI/ML-based CSI reporting, e.g., codebook of a certain type.

Follow-up Mechanism

Under the framework of the coexistence of AI/ML-based CSI feedback and legacy non-AI/ML-based CSI feedback modes, there is a choice made by the NW and/or UE whether the mode of CSI feedback is fallen back to the non-AI/ML mode or is kept in the AI/ML mode, depending on the monitoring result. It is straightforward that falling back to the legacy non-AI/ML-based approach is needed if monitoring results indicate a poor performance of the AI/ML model in use, and vice versa. The relevant signaling and procedures of the follow-up mechanism after AI/ML monitoring should be studied.

The decision of whether to fall back to the legacy non-AI/ML-based CSI reporting can be made by comparing the SGCS of the target CSI and the precoding matrix related information generated by AI/ML approach according to some threshold, e.g., the “Class A/B threshold” mentioned in the previous subsection. The signaling required for fall back may be physical layer signaling, such as DCI, or higher layer signaling, such as RRC. We have the following proposal.

Proposal-3: For the CSI compression using two-sided AI/ML models use case, study the procedures and signaling needed for intermediate KPI-based AI/ML performance monitoring and the follow-up mechanism after the monitoring, including the falling back to codebook-based CSI report from AI/ML-based CSI report.

The follow-up mechanism after falling back to non-AI/ML-based CSI reporting should also be studied. One option is to stay on the non-AI/ML-based CSI reporting. Another choice is to re-activate the AI/ML-based CSI reporting when appropriate. In the latter, the mechanism similar to the AI/ML model performance monitoring may be needed to determine whether it is a good idea to re-activate the AI/ML-based CSI reporting mode. As a starting point, the procedures and signaling for the AI/ML model performance monitoring may be re-used in this application scenario.

The activation of the AI/ML-based CSI reporting may also happen at the initial stage. An example is the instance that a UE is turned on. The performance of the AI/ML model to be used may be tested before use. The procedures and the signaling required in the case of AI/ML performance monitoring discussed above may be re-used in this scenario as a starting point. We have the following proposal.

Proposal-4: For the AI/ML model performance monitoring of the CSI compression using two-sided AI/ML models use case, study the potential specification impacts on monitoring the performance of an AI/ML model in inactivate mode, taking at least the following cases into consideration.

- Initial activation of an AI/ML model.

- Re-activation of an AI/ML model.

Conclusions

Proposal-1: For the sub use case of CSI compression using two-sided AI/ML models, study the mechanism that UE and NW align their supported AI/ML models in the multi-vendor collaborations.

Proposal-2: For the sub use case of CSI compression using two-sided AI/ML models, study the method for indicating the pairing information of the AI/ML-based CSI generation and reconstruction parts. The pairing ID can be studied as a starting point.

Proposal-3: For the CSI compression using two-sided AI/ML models use case, study the procedures and signaling needed for intermediate KPI-based AI/ML performance monitoring and the follow-up mechanism after the monitoring, including the falling back to codebook-based CSI report from AI/ML-based CSI report.

Proposal-4: For the AI/ML model performance monitoring of the CSI compression using two-sided AI/ML models use case, study the potential specification impacts on monitoring the performance of an AI/ML model in inactivate mode, taking at least the following cases into consideration.

- Initial activation of an AI/ML model.

- Re-activation of an AI/ML model.

- References

[1] 3GPP SR RP-221347: “Status report for study on AI/ML for NR air interface”, Jun. 2022.

[2] 3GPP “RAN1 chair’s notes (eom4)”, RAN WG1 #112, Athens, Greece, Feb. 2023.

[3] 3GPP “RAN2 chair’s notes (eom)”, RAN WG2 #121, Athens, Greece, Feb. 2023.

[4] 3GPP “RAN1 chair’s notes (v17)”, RAN WG1 #110bis-e, Oct. 2022.

[5] 3GPP R1-2209011: “Evaluations on AI/ML for CSI feedback enhancement,” Fujitsu, Oct. 2022.