3GPP TSG RAN WG1 #112b		R1-2302907

E-meeting, Apr. 17th – Apr. 26th, 2023

Agenda Item:	9.2.3.2

Source:	Fujitsu

Title:	Discussion for specification impacts on AI/ML for beam management

Document for:	Discussion/Decision

Introduction

In RAN1#112 meeting, the following agreements [1] have been achieved about sub use cases and specification impacts on AI/ML for beam management.

Conclusion For the sub use case BM-Case1 and BM-Case2, “Alt.2: DL Rx beam prediction” is deprioritized. Agreement Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Other alternatives are not precluded - Note: At least the performance and spec impact should be considered Conclusion Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) - FFS: Definition/content of confidence/probability information - Note: At least the performance and spec impact should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused). Conclusion Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity: - Configuration/Signaling from gNB to UE for measurement and/or reporting - UE reporting to NW (e.g., for the calculation of performance metric) - Indication from NW for UE to do LCM operations - Other aspect(s) is not precluded - Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring - Other aspect(s) is not precluded

This contribution presents our views on AI/ML for DL beam prediction and the potential specification impacts.

Potential specification impacts

Discussion on the data collection

In RAN1-112 meeting, the following proposal about the AI/ML model training at NW side is discussed in [2].

Proposal 3.2.1: Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study the following options as a starting point for the contents of collected data - Opt.1: UE reports sends M1 L1-RSRPs (corresponding to M1 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set A, Set A+B, Set B), where M1 can be larger than 4 - FFS: the range of M1 - Opt.2: UE reports sends M2 L1-RSRPs (corresponding to M2 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set B), reports sends M3 L1-RSRPs (corresponding to M3 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to another beam set (e.g., Set A), where M2 (M2>0) and M3 can be larger than 4 - FFS: the range of M2, M3 - Opt.3: UE r reports sends M4 L1-RSRPs (corresponding to M4 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set B), reports sends M5 beams (beam pair) indicators based on the measurement corresponding to another beam set (e.g., Set A), where M4 can be larger than 4 - FFS: the range of M4, M5 - Other option(s) is not precluded - Note1: From UE perspective, the measurement and reporting related to one beam set may be separate from/transparent to the operations related to another beam set - Note2: Data collection for model training may be implemented by gNB in a transparent way - Note3: Potential down-selection/prioritization will be discussed later - Note4: Overhead, UE complexity and power consumption should be considered for the above options

According to the above proposal, from our understanding, the opt.1 is for the case that Set B is subset of Set A and opt.2 is for the case that the Set B is different from Set A. However, the opt.3 may be for both cases with less overhead. Generally, we agree on this proposal.

Form this proposal, UE should report the measurement results of beam configured by gNB according to the requirement of AI/ML model. For example, for opt.2, the gNB configures the RSs of Set B and Set A, and UE measures and reports the results of Set B and Set A. It is observed that the UE behaviour on the measurement reporting is different from the legacy method. In legacy method, which measurement results of beams are reported is an implementation issue of UE. However, for NW-side model, which beams will be reported depends on the configuration of gNB.

Proposal 1: Regarding the data collection for AI/ML model trained at NW-side, study the potential specification impacts on the UE behavior of beam reporting.

Regarding the data collection for AI/ML model trained at NW side, the procedure of data collection happens infrequently. In this case, the data to be collected are non-real time and the UE reporting may be from different time occasions. The information about timestamp is important for the training of AI/ML model. When the model is trained, the data for the input of AI/ML model from the collected dataset should have suitable timestamp. It is necessary to study mechanism to generate the attached timestamp information along with the collected data.

Proposal 2: Regarding the data collection for AI/ML model trained at NW-side, considering the non-real time data to be collected, study mechanism on attached timestamp information along with collected data.

- FFS: the method to generate the timestamp

Regarding the data collection for AI/ML model trained at NW side, if UE reports the measurement of Set A to gNB, the dynamic range of measured L1-RSRP for all beams(pairs) is larger than legacy beam reporting since only at most 4 largest L1-RSRPs may be reported in legacy beam reporting. In this case, the quantization for data collection maybe different from the legacy method. the improper quantization on beam reporting may cause the performance degradation for AI/ML model. On the other hand, the characteristics of UE reported data may be different with working stage of AI/ML model (training, inference, monitoring). Considering the overhead of beam reporting, it’s necessary to study quantization scheme on different working stages of AI/ML model.

Proposal 3: Considering the performance of AI/ML model and the overhead of beam reporting, study the necessity and scheme on quantization of beam reporting in the data collection, inference and monitoring on AI/ML model.

Regarding the data collection for AI/ML model trained at NW side, all the options require the number of measurement results in one report to be larger than 4. When the measurement results of Set A are required to be reported, e.g., opt.1, the number of measurement results increase a lot compared with the legacy beam reporting. For example, it’s assumed that the measured L1-RSRPs of 64 beam pairs (16 Tx beams and 4 Rx beams) are used to predict the L1-RSRPs of total 256 beam pairs (32 Tx beams and 8 Rx beams). The measurement results about 256 beam pairs of Set A will be reported to NW for data collection. In this case, the overhead increases 64 times compared with at most four measured results reported to NW.

Proposal 4: Regarding the data collection for AI/ML model training at NW side, the mechanism of reporting overhead reduction is suggested to be studied.

From our understanding, the data collection happens infrequently. The required measurement results can be balanced with multiple reporting occasions. The enhanced signaling/procedure on the reporting configuration is necessary to be studied.

Proposal 5: Study the potential specification impacts on the enhanced signaling/procedure of reporting configuration for data collection.

Discussion on the model inference

Regarding the inference of NW-side model, it has the similar specification impacts on the UE behavior of measurement reporting. The measurement results of Set B are required to be reported from UE and which beams will be reported for model inference depends on the configuration of gNB. However, in legacy beam reporting, which beams are reported depends on the implementation of UE. In this case, the UE behavior on the measurement reporting is different for the NW-side model compared with the legacy method.

Proposal 6: Regarding the inference of NW-side model, study the potential specification impacts on the UE behavior of beam reporting.

For NW-side model, if DL Tx-Rx beam pair prediction is supported, the Rx beam information should be included in the measurement report especially for the variable set B since the Rx beam information is transparent to gNB. In terms of how to define the Rx beam information, one candidate is the physical beam information such as beam angle. The other is the logical beam information such as beam ID, but how to map the logical beam into physical beam needs to be further studied.

Proposal 7: For the DL beam pair prediction with a NW-side model, study the potential specification impacts on the Rx beam information included in report instance.

Proposal 8: Regarding the Rx beam information included in report instance for the DL beam pair prediction with a NW-side model, it is suggested to study

- Physical beam information (e.g., beam angle)

- Logical beam information (e.g., beam ID)

  - FFS: How to map the logical beam into physical beam

For the UE-side model, if the AI/ML model information is transparent to gNB, the gNB does not know which RSs corresponding to the Tx beams is necessary to be transmitted for model inference. A signaling about the required Tx beams of Set B from UE to gNB is necessary for model inference.

Proposal 9: For DL beam (pair) prediction with a UE-side model, study the potential specification impacts of model inference on

- The request to NW about the required Tx beams of Set B

Discussion on the model monitoring

In RAN1-110b meeting, the following proposal on the model monitoring is agreed.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

For the model monitoring with a UE-side AI/ML model, from our understanding, the NW should be aware of the model to make related configuration. For example, the NW only configures part of Tx beams for the inference of AI/ML model when it is activated in UE side. When the AI/ML model is deactivated, the NW may be fallback to the legacy configuration for beam sweeping procedure. In this case, it is suggested that NW makes decisions of model selection/activation/deactivation /switching/fallback operation.

Proposal 10: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, the Alt.2 and Alt.3 are suggested to be prioritized for further study of model monitoring.

For the hybrid model monitoring, when performance metrics are calculated in UE side, it is necessary to consider the robust calculation results with the fluctuation of wireless channel conditions. In this case, the filtering on the instant results can be considered. And gNB can configure the filtering parameters for the metrics calculation.

Proposal 11: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impacts of hybrid model monitoring on

- The configuration of filtering on the performance metric calculation

- Conclusion

In this contribution, we present our views on sub use cases and potential specification impacts of AI/ML beam management. For the discussion, we have the following proposals.

Proposal 1: Regarding the data collection for AI/ML model trained at NW-side, study the potential specification impacts on the UE behavior of beam reporting.

Proposal 2:  Regarding the data collection for AI/ML model trained at NW-side, considering the non-real time data to be collected, study mechanism on attached timestamp information along with collected data.

- FFS: the method to generate the timestamp

Proposal 3: Considering the performance of AI/ML model and the overhead of beam reporting, study the necessity and scheme on quantization of beam reporting in the data collection, inference and monitoring on AI/ML model.

Proposal 4: Regarding the data collection for AI/ML model training at NW side, the mechanism of reporting overhead reduction is suggested to be studied.

Proposal 5: Study the potential specification impacts on the enhanced signaling/procedure of reporting configuration for data collection.

Proposal 6: Regarding the inference of NW-side model, study the potential specification impacts on the UE behavior of beam reporting.

Proposal 7: For the DL beam pair prediction with a NW-side model, study the potential specification impacts on the Rx beam information included in report instance.

Proposal 8: Regarding the Rx beam information included in report instance for the DL beam pair prediction with a NW-side model, it is suggested to study

- Physical beam information (e.g., beam angle)

- Logical beam information (e.g., beam ID)

  - FFS: How to map the logical beam into physical beam

Proposal 9: For DL beam (pair) prediction with a UE-side model, study the potential specification impacts of model inference on

- The request to NW about the required Tx beams of Set B

Proposal 10: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, the Alt.2 and Alt.3 are suggested to be prioritized for further study of model monitoring.

Proposal 11: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impacts of hybrid model monitoring on

- The configuration of filtering on the performance metric calculation

- Reference

[1] Draft_Minutes_report_RAN1#112_v040, Athens, Greece, 27th February – 3rd March 2023

[2] R1-2301897 Summary#4 for other aspects on AI/ML for beam management