3GPP TSG RAN WG1 #112 bis	R1-2302908

E-meeting,  April 17th – 26th,  2023

Agenda Item:	9.2.4.1

Source:		Fujitsu

Title:	Discussions on evaluation results of AIML positioning accuracy enhancement

Document for:	Discussion/Decision

Introduction

There are many simulation results submitted in the previous several RAN1 meetings for AI/ML positioning enhancements, covering multiple aspects such as fundamental performance evaluation under different scenarios and configurations for both direct and assisted AI/ML positioning methods, advanced performance evaluation when imperfections introduced, model generalization performance evaluation under multiple conditions, and model fine-tuning performance. So far almost every aspect of AI/ML positioning has been done, only some minor cases left for companies to evaluate optionally. In this contribution, we present our additional comments and summary on the evaluation results obtained based on the recent agreements made in last RAN1 # 112 meeting.

Discussions

In the previous RAN1 #112 meeting [1], the following agreements have been made.

Agreement For both direct AI/ML positioning and AI/ML assisted positioning, companies include the evaluation area in their reporting template, assuming the same evaluation area is used for training dataset and test dataset. Note: - Baseline evaluation area for InF-DH = 120x60 m. - if different evaluation areas are used for training dataset and test dataset, they are marked out separately under “Train” and “Test” instead. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m] Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Computation complexity; AI/ML. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. - Agreement - The agreement made in RAN1#110 AI 9.2.4.1 is updated by adding additional note: - Note: if complex value is used in modelling process, the number of the model parameters is doubled, which is also applicable for other AIs of AI/ML. Agreement For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity. - The type of information to use as model input. The candidates include at least: time-domain CIR, PDP. - The dimension of model input in terms of NTRP, Nt, and Nt’. - Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference. Agreement For direct AI/ML positioning, study the performance of model monitoring methods, including: - Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output. - Label-free methods, where model monitoring does not require ground truth label (or its approximation). Agreement For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) for model monitoring. Conclusion - No dedicated evaluation is needed for the positioning accuracy performance of model switching. - It does not preclude future discussion on model switching related performance. Agreement For direct AI/ML positioning, study the impact of labelling error to positioning accuracy. - The ground truth label error in each dimension of x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models are not precluded. - [Whether/how to study the impact of labelling error to label-based model monitoring methods] - [Whether/how to study the impact of labelling error for AI/ML assisted positioning.] Observation Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. - The generalization aspects include: - Different drops - Different clutter parameters - Different InF scenarios - Network synchronization error - Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training. - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios.

We have noticed that the following issues raised in the previous RAN1 #111 meeting has not been fixed or completely agreed, so in this contribution, we continue to discuss on them:

- The impact of user density/ size of training dataset.

- The model monitoring metrics details for assisted methods.

In addition, the following aspect raised in RAN1 #112 meeting will also be discussed：

- Label-free model monitoring for direct positioning method.

- 2.1 Impact of user density/size of training dataset

Like other AI/ML applications, more training dataset samples typically gives more chances for the model to learn more features from the samples, in the AI/ML positioning cases, the most crucial feature is the wireless channel characteristics between single/multiple gNBs and the UEs, both the large-scale and small-scale factors may vary per UE-gNB pair, as well as the additional imperfections such as synchronization error or timing error, moreover, the wireless environment is subject to change, and in some scenarios (e.g., in the moving high-speed train), the changings may be rapid and frequent that for the UE, two wireless environments before and after changing have no coherence at all.

The simulation user density faces the same issues, by increasing the number of users for certain areas can definitely add more unknown features for the model to study and generalize, however this benefit is highly dependent on the wireless layout of the scenarios, especially in heavy NLOS scenarios, two adjacent UEs with tiny distance between them may have completely different wireless channels so the model will not be able to handle the generalization issue for one of the two UE even if the model has already learnt the features of the other UE.

Furthermore, in highly uncertain scenarios such as heavy NLOS, the extra samples may become outliers for the model under some conditions, the outliers may affect or even decrease the performance of the overall model inference since the model must adjust the weights accordingly to balance the performance of the outliers and the normal samples.

Observation 1 The impacts to model performance from both the number of dataset samples and the user density are dependent on the wireless environment.

Observation 2 For heavy NLOS scenarios, the relationship between the user density/dataset samples and the model inference performance varies per case.

Therefore, we propose not to simply evaluate the linear connection between the number of data samples or user density and the model inference performance, some more considerations should be taken for the simulation, and furthermore, in the potential data collection procedure for online-training, for example, the distribution of PRUs in each cell can be used for reference.

Proposal 1 In order to accurately evaluate the performance impact introduced by different user densities, study the impacts from not only the number of users/dataset samples, but more information associated with the users/samples.

- 2.2 Model monitoring metrics for assisted positioning method

There could be multiple intermediate results inferred by the AI/ML model, such as TOA, TDOA, AoA, RSRP or even LOS/NLOS probability, however the final purpose for AI/ML positioning is to increase the location accuracy of the UEs so all valid model monitoring metrics must have direct relationship with the location accuracy, e.g., there is public algorithm equations which can calculate the UE location by using TDOA. The metrics for the intermediate results is not suggested to be used for monitoring the model.

It may not be easy to figure out the direct connection to the UE location accuracy for some intermediate output such as LOS/NLOS probability or AoA, but it is not limited to have only one monitoring metrics for one model at one time, that is, the monitoring metrics can be multiple with different applicability within one cell, or the metrics can be changed upon conditions for better model performance. In some cases, the relationship will be easier to be found within a small range of area or wireless channels with certain conditions.

Proposal 2 For the intermediate output without explicit connections to the UE locations, it is suggested to have dynamic model monitoring metrics management framework to study the applicability of different available metrics under certain conditions or configurations.

- 2.3 Label-free model monitoring

One of the agreements from RAN1 #112 meeting is to study label-free model monitoring method for both direct and assist AI/ML positioning. Typically, to monitor the AI/ML model performance, ground truth labels must be collected as reference, however, it is very difficult to obtain error-free labels for positioning cases. Therefore, some label-free monitoring methods can be used to perform the model monitoring. However, before discussing the performance of the label-free monitoring methods, we suggest that some basic consensuses have to be aligned first such as:

- The KPI to evaluate the performance of the model monitoring.

- The baseline method for starting the model monitoring performance evaluation.

- The final purpose for evaluating the model monitoring performance.

- The KPI to evaluate the performance of the model monitoring.

Unlike the basic AI/ML training/inference performance evaluation, the very straightforward KPI is to figure out the positioning accuracy of the AI/ML model, i.e., to fine the difference between the ground truth label and the model inferred values. However, for the model monitoring performance evaluation, it is difficult to determine a single KPI to indicate the success or failure of the model monitoring. Here we list some of the potential KPIs for model monitoring performance evaluation:

- Continue to use positioning accuracy as the only KPI:

One possible label-free method for model monitoring is to use some intermediate values such as RSTD, LOS/NLOS indicator or RSRP to deduce the potential positioning ground truth labels, so basically this “label-free” is equivalent to label-based model monitoring, positioning accuracy can still be used as model monitoring KPI.

- Successful rate of model monitoring follow-up actions:

The purpose of performing model monitoring is to check if the model is still useful in the current scenario, so basically if the model can still be working then no switching actions will be made, and if the model fails to meet the requirements of current application, it has to be switched to another AI/ML model or conventional methods to continue the positioning process. Therefore, the model monitoring has become one duo-classification or multi-classification task, the KPI for evaluating classification problem can be TP/TN/FP/FN or cross entropy possibilities.

- Other metrics for label-free model monitoring:

We have noticed that some companies (e.g., VIVO) had proposed some method to monitor model performance by using input data, and some new metrics per algorithm have been proposed. However, these metrics are algorithm-specific, and has not been validated across companies, dataset, or scenarios, so if new metrics which are heavily dependent on algorithms will be adopted, an alignment among companies are necessary.

Proposal 3 Before evaluating the performance of the model monitoring, the evaluation methodology and the KPI have to be discussed and aligned among companies, especially for introducing new metrics for label-free model monitoring.

- The baseline method for starting the model monitoring performance evaluation.

The baseline method is suggested to help companies to evaluate the model monitoring from a same starting line, just like the necessity to align the KPI, if there is no baseline method provided, different companies may give completely different ways of the evaluation and the results are non-comparable among companies, then these evaluations may lose the generality and become meaningless.

Proposal 4 Study the baseline method for model monitoring performance evaluation in order to achieve comparable results among companies.

- The final purpose for evaluating the model monitoring performance.

Another minor issue is why we need to evaluate the model monitoring, in CSI feedback and beam management, there is no similar issue proposed. We suppose it is because the accurate labels are difficult to be obtained so accurate model monitoring can hardly be achieved for AI/ML positioning, if so, it is necessary to develop alternative ways such as label-free method to support better model monitoring, and there should be some discussions in agenda item 9.2.4.2 for the potential specification impact to support the new model monitoring mechanism.

Observation 3 It seems that the label-free model monitoring issue should be discussed more in potential specification impact rather than evaluation methodology.

From our point of view, we suggest having the second option (successful rate) mentioned above and the following label-free model monitoring methods are proposed to have further study.

- Applicability condition for AI/ML models

If one AI/ML is applicable under all scenarios, then there is no need to do any model monitoring actions, so the model applicability is the root cause for the bad performance of one AI/ML model. For positioning use case, the applicability condition can be related to the input data e.g., peak distribution of CIR/PDP/RSRPP, statistics of RS configuration, or geometric range of data collection.

- Statistics of the historical monitoring results

A potential mechanism to collect historical monitoring results to assist the real-time model monitoring is possible, since positioning final output is highly visible, not only the personal users but also industrial applications can provide feedbacks for the model performance, the gNB or LMF can collect huge numbers of feedbacks from multiple UEs, PRUs or other equipment, and one local historic monitoring database will be available for further model monitoring reference. The signaling and procedure for the maintenance of the database can be further studied.

Proposal 5 For the label-free model monitoring for positioning method, study at least the following methods:

- AI/ML model applicability condition information.

- Historical model monitoring results.

Conclusion

Based on the above discussions, we give the following observations and proposals:

Observations

Observation 1 The impacts to model performance from both the number of dataset samples and the user density are dependent on the wireless environment.

Observation 2 For heavy NLOS scenarios, the relationship between the user density/dataset samples and the model inference performance varies per case.

Observation 3 It seems that the label-free model monitoring issue should be discussed more in potential specification impact rather than evaluation methodology.

Proposals

Proposal 1 In order to accurately evaluate the performance impact introduced by different user densities, study the impacts from not only the number of users/dataset samples, but more information associated with the users/samples.

Proposal 2 For the intermediate output without explicit connections to the UE locations, it is suggested to have dynamic model monitoring metrics management framework to study the applicability of different available metrics under certain conditions or configurations.

Proposal 3 Before evaluating the performance of the model monitoring, the evaluation methodology and the KPI have to be discussed and aligned among companies, especially for introducing new metrics for label-free model monitoring.

Proposal 4 Study the baseline method for model monitoring performance evaluation in order to achieve comparable results among companies.

Proposal 5 For the label-free model monitoring for positioning method, study at least the following methods:

- AI/ML model applicability condition information.

- Historical model monitoring results.

Reference

[1] R1-2212845, “Session notes for 9.2 (Study on AI/ ML for NR air interface)”, Ad-hoc Chair (CMCC), RAN1 #111, Toulouse, France, November 14th – 18th, 2022

- Appendix

The feature lead has announced to collect simulation results by aligned Excel files and suggested not to put same results in the contribution Word file, but the Excel files do not include CDF figures, so we keep this part in the appendix. The appendix shows the previous simulation results on AI/ML positioning.

A.1 Direct AI method basic performance

Figure A-1 shows the performance of the direct AI and conventional DL-TDOA methods for positioning.

Figure A-1	Positioning accuracy CDF of direct AI and DL-TDOA

The percentiles for both the AI and Non-AI methods simulation results are listed in Table A-1 below:

Table A-1 Simulation results percentiles of the direct AI method and non-AI DL-TDOA

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Direct AI | 2.51 | 3.25 | 4.08 | 5.42 |
| Non-AI DL_TDOA | 8.97 | 12.55 | 17.32 | 23.13 |

A.2 Assisted AI method basic performance

Figure A-2 shows the performance of the indirect AI and conventional DL-TDOA methods for positioning.

Figure A-2	Positioning accuracy CDF of indirect AI and DL-TDOA

The percentiles for both the AI and Non-AI methods simulation results are listed in Table A-2 below:

Table A-2 Simulation results percentiles of the indirect AI method and non-AI DL-TDOA

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Indirect AI | 4.12 | 5.61 | 7.24 | 10.05 |
| Non-AI DL_TDOA | 8.97 | 12.55 | 17.32 | 23.13 |

A.3 Model generalization and fine-tuning

The simulation results are shown below in figure A-3:

Figure A-3 Simulation results of direct AI/ML methods for perfect and imperfect synchronizations under original trained and fine-tuned models

The dubbing of the figure legend is similar to our previous contributions, the explanation table can be seen below in table A-3.

Table A-3 The explanation of the legend of figure A-3

| pp | AI/ML model trained with dataset of perfect sync and inferred with dataset of perfect sync. |
| --- | --- |
| ss | AI/ML model trained with dataset of sync error and inferred with dataset of sync errors. |
| sp | AI/ML model trained with dataset of sync error and inferred with dataset of perfect sync. |
| ps | AI/ML model trained with dataset of perfect sync and inferred with dataset of sync errors. |
| ps_ft | AI/ML model trained with dataset of perfect sync and fine-tuned with dataset of sync errors. |
| sp_ft | AI/ML model trained with dataset of sync error and fine-tuned with dataset of perfect sync. |

A.4 Simulation parameters

Table A-4-1	Fine-tuning on sync error dataset based on perfect timing original model.

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| CIR | UE location | UE location | 662 Perfect timing | 662 Sync Error | 662 Sync Error | 30000 | 3000 | 6000 | Num of parameters < 1.5M | FLOPS 1.14M | Before ft: 19m After ft: 11.6m |

Table A-4-2	Fine-tuning on perfect timing dataset based on sync error original model.

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| CIR | UE location | UE location | 662 Sync Error | 662 Perfect timing | 662 Perfect timing | 30000 | 3000 | 6000 | Num of parameters < 1.5M | FLOPS 1.14M | Before ft: 13.6m After ft: 7.9m |