3GPP TSG RAN WG1 #112	R1- 2302912

E-meeting, April 17th – April 26th, 2023

Agenda Item:	9.7.1

Source: 	Fujitsu

Title:	Discussion on NW energy saving techniques in spatial and power domains

Document for:	Discussion/Decision

Introduction

Rel.18 Network (NW) energy saving WID [1] were discussed in RAN1#112 meeting. The following agreements have been achieved w.r.t. network energy saving techniques in spatial and power domains.

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns” Agreement For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration Agreement For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account. Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included Agreement For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline. Agreement Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

In this contribution, we share some further views on CSI related procedures to enable efficient adaptation of spatial elements and PDSCH transmission power.

Discussion

In the following sections, we present our views on the necessary CSI related procedure enhancements from the configuration and signaling aspects for adaptation of spatial elements and adaptation of PDSCH transmission power, respectively.

Adaptation of spatial elements

Spatial element adaptation types

In the last meeting, it was agreed to consider the following two types of spatial element adaptation.

- Type 1: All the spatial elements associated to a logical antenna port are disabled/enabled

An example of Type 1 adaptation is shown in Figure 1(a). As shown in this figure, the mapping relationship between TxRUs and logical antenna ports does not change during adaptation. Figure 2(a) shows an example of different patterns for type 1 adaptation from the perspective of the antenna ports. As shown in this figure, Pattern 2 and Pattern 3, where a portion of antenna ports are disabled can be seen as subsets of Pattern 1, where all antenna ports are active. When it comes to channel measurement of difference Type 1 adaptation patterns, it is possible to utilize a nested CSI-RS resource structure. An example of nested CSI-RS resource structure is shown in Figure 2(b). As shown in this figure, the 16-port CSI-RS resource, in the time, frequency and code domains, is a subset of the 32-port CSI-RS resource.

- Type 2: part/subset of spatial elements associated to a logical antenna port is disabled/enabled

An example of Type 2 adaptation is shown in Figure 1(b). As shown in this figure, the mapping relationship between TxRUs and logical antenna ports changes during adaptation. In this case, the number of antenna ports stays the same, but the channels experienced by the UEs’ DL signals before and after the adaptation may completely get different. Therefore, separate CSI-RS resources are required for channel measurement of different Type 2 adaptation patterns.

| (a). Type 1 spatial elements adaptation | (b). Type 2 spatial elements adaptation |
| --- | --- |

Figure 1. Spatial elements adaptation types

| (a) | (b) |
| --- | --- |

Figure 2. (a) Example of spatial adaptation patterns for Type 1 adaptation

(b) Example of nested CSI-RS resource structure

Observation 1.

- For Type 1 spatial elements adaptation, the mapping relationship between spatial elements and logical antenna ports does not change during adaptation.

  - The nested CSI-RS resource structure can be utilized for channel measurement of different Type 1 adaptation patterns

- For Type 2 spatial elements adaptation, the mapping relationship between spatial elements and logical antenna ports changes during adaptation.

  - Separate CSI-RS resources are required for channel measurement of different Type 2 adaptation patterns

Type 1 and Type 2 spatial element adaptations originate from different original RF/IF structures. Rel-18 NES is an enhancement of network equipment which does not/cannot require a substantial change to the original RF/IF structures of the network equipment. Hence, both Type 1 and Type 2 spatial element adaptation are considered. In this way, different operators/vendors can choose the one suitable for their own equipment/product to implement Rel-18 network power saving feature. Moreover, this also means that there is hardly a need to support both Type 1 and Type 2 adaptation in a single network equipment which may require a large cost and provide almost no additional benefit compared to either one of them.

Hence, we believe that the discussion on the support of Type 1 and Type 2 spatial element adaptation can somehow be organized separately. In this way, we can use the minimum standardization effort/additional signaling overhead to support either of them. The mechanism/signaling design can be simplified by considering the characteristics of Type 1 spatial element adaptation and Type 2 spatial element adaptation, respectively.

Observation 2.

- The reason we support both Type 1 and Type 2 spatial element adaption is because they originate from different original RF/IF structures. Different operators/vendors need to choose one of them to enhance their own equipment/product for the purpose of energy saving.

- Type 1 and type 2 adaptation have different properties which can be handled respectively.

- A unified solution for Type 1 and Type 2 adaptation is not always expected because it can somehow increase the complexity/cost for the update of the network equipment.

  - Type 1 adaptation or Type 2 adaptation is not a configurable feature but presetting property. Almost no network equipment supports both type 1 and type 2 adaptation.

Proposal 1. The CSI related enhancement(s) for the support of type 1 spatial element adaptation and type 2 spatial element adaptation are considered and discussed separately.

Necessary enhancements for configuration

In the last meeting, two alternatives of CSI-RS resource configuration were agreed for further study. Depending on configuration granularity, the second alternative of CSI-RS configuration can be further divided into three sub-alternatives. Figure 3 shows an overview of CSI-RS configuration alternatives.

A1-1) Each CSI-RS resource / resource set / resource setting can be associated with one spatial adaptation pattern

A1-2) Each CSI-RS resource / resource set / resource setting can be associated with multiple spatial adaptation pattern

A1-2-1) Each CSI-RS resource (NZP-CSI-RS-Resource) is associated with multiple spatial adaptation patterns

A1-2-2) Each CSI-RS resource set (NZP-CSI-RS-ResourceSet) is associated with multiple spatial adaptation patterns

A1-2-3) Each CSI-RS resource setting (CSI-ResourceConfig) is associated with multiple adaptation patterns

Figure 3. Overview of CSI-RS configuration alternatives

A1-2-1 can fully explore the feature of nested resource structure to achieve high configuration efficiency. As discussed previously, nested CSI-RS resource structure can be considered when Type 1 spatial element adaptation is adopted. From the perspective of configuration, the parameters to configure the time, frequency and code resource for the CSI-RSs corresponding different spatial adaptation patterns can be shared. This reduces the number of duplicate parameters. However, A1-2-1 cannot support type 2 spatial adaptation considering that different CSI-RS resources are required.  A1-1, A1-2-2 and A1-2-3 can support both type 1 and type 2 spatial adaptations, with some loss of efficiency in Type 1 adaptation case as duplicate parameters in separate NZP-CSI-RS-Resource configurations case cannot be combined. A1-1 can be supported by the current specification without specification impact. For A1-2-2, the current CSI-RS configuration structure can be reused. Specifically, a CSI-RS resource set is configured to consist of multiple CSI-RS resources. Each resource is associated with one spatial pattern, e.g., number of antenna ports. Thus, the amount of standardization work can be minimized.

Regarding CSI report configuration, the following two alternatives were agreed for further study.

A2-1) Each CSI-ReportConfig corresponds to one spatial adaptation pattern

A2-2) Each CSI-ReportConfig corresponds to multiple spatial adaptation patterns

A2-1 can provide a flexible solution so that the gNB can configure the CSI reports differently according to different spatial adaptation patterns. From the perspective of specification impact, the current CSI-ReportConfig IE can be reused.

A2-2 allows to share some common parameters, such as report type, PUCCH resource or report timing for CSI report transmission on PUSCH, to achieve higher efficiency. But a new CSI-ReportConfig IE may be required. Regarding the number of CSI reports, the current maximum number of CSI reports supported by a UE will not be exceeded for A2-2.

Based on the above discussion, Table 1 summaries the evaluation of above alternatives in terms of support of spatial adaptation types, specification impact and configuration efficiency.

Table 1. Evaluation of configuration alternatives

| Alternatives | Support for Type 1 | Support for Type 2 | Spec impact | Configuration efficiency |
| --- | --- | --- | --- | --- |
| A1-1 | ○ | ○ | No impact | Low |
| A1-2-1 | ○ | × | Medium | High |
| A1-2-2 | ○ | ○ | Low | Medium |
| A1-2-3 | ○ | ○ | Medium | Medium |
| A2-1 | - | - | Medium | Low |
| A2-2 | - | - | Medium | High |

As summarized in Table 1, we have the following observations:

- Among the CSI-RS configuration alternatives for spatial element adaptation,

- A1-2-1 can achieve the high efficiency of CSI-RS configuration if there are some parameters in NZP-CSI-RS-Resource configuration that have common values.

- A1-2-2 has low specification impacts as the current CSI-RS configuration structure can be reused.

- Among the CSI report configuration alternatives for spatial element adaptation,

- A2-1 can reuse the current CSI-ReportConfig IE.

- A2-2 can achieve the high efficiency of CSI report configuration if there are some parameters in NZP- CSI-ReportConfigthat have common values.

Observation 3.

- Among the CSI-RS configuration alternatives for spatial element adaptation,

- A1-2-1 can achieve the high efficiency of CSI-RS configuration if there are parameters in NZP-CSI-RS-Resource configuration that have common values.

- A1-2-2 has low specification impacts as the current CSI-RS configuration structure can be reused.

- Among the CSI report configuration alternatives for spatial element adaptation,

- A2-1 can reuse the current CSI-ReportConfig IE.

- A2-2 can achieve the high efficiency of CSI report configuration if there are some parameters in CSI-ReportConfig that have common values.

Proposal 2: Considering the support of spatial adaptation types, specification impact and configuration efficiency, consider the following alternatives for further study:

- CSI-RS configuration

  - A1-2-1) Each CSI-RS resource (NZP-CSI-RS-Resource) is associated with multiple spatial adaptation patterns

    - The common parameters are FFS

  - A1-2-2) Each CSI-RS resource set (NZP-CSI-RS-ResourceSet) is associated with multiple spatial adaptation patterns

- CSI report configuration

  - A2-1) Each CSI-ReportConfig corresponds to one spatial adaptation pattern

  - A2-2) Each CSI-ReportConfig corresponds to multiple spatial adaptation patterns

    - The common parameters are FFS

Necessary enhancements for CSI feedback

There are two potential CSI feedback approaches to obtain up-to-date CSI when dynamic adaptation of spatial is performed.

- Single CSI feedback

In this approach, UEs make new CSI measurements and report after adapting the spatial elements. Figure 4 shows an example of the single CSI feedback. The gNB can determine to perform spatial elements adaptation based on the traffic load and other radio link quality related information, such as RSRP reported by UEs. The gNB informs the UE about updated CSI-RS to obtain CSI report corresponding to current channel state.

Figure 4. An example of single CSI feedback

- Multiple CSIs feedback

In this approach, UEs feedback multiple CSI measurement results corresponding to different spatial patterns. As shown in Figure 5, the CSI-RSs and CSI reports corresponding to different spatial patterns are transmitted in a semi-static manner. The base station determines whether to adapt spatial elements based on these CSI reports and the traffic load. For type 1 spatial elements adaptation, the CSI-RSs corresponding to different spatial patterns can be transmitted in a nested CSI-RS resource.

Figure 5. An example of multiple CSIs feedback

The single CSI feedback provides a low overhead approach for CSI tracking. On the other hand, as the gNB needs to receive the updated CSI reports to determine the most appropriate DL transmission parameters (e.g., MCS, precoder), this approach has the disadvantage that the gNB may need to postpone DL transmission after receiving new CSI report or schedule DL transmission with the parameters quite conservative.

The multiple CSIs feedback allows gNB to transmit PDSCH with proper parameters immediately after adaptation, where the MCS and precoder for DL transmission is determined based on the CSI report received before spatial element adaptation. On the other hand, the drawback is the overhead of CSI-RS transmission and CSI reporting is quite high which can be times of that of single CSI feedback approach. Furthermore, if Type 2 spatial elements adaptation is adopted, to transmit CSI-RSs corresponding to different spatial patterns, the gNB must frequently switch the spatial elements on and off.

Observation 4. To achieve CSI tracking of the dynamic varying channel due to dynamic spatial element adaptation, there are two possible approaches.

- Single CSI feedback approach provides a CSI tracking solution without increasing the overhead of CSI reporting. However, latency may be large as gNB has to postpone DL transmission after receiving new CSI report or schedule DL transmission with the parameters quite conservative.

- Multiple CSIs feedback enables the base station to perform DL transmission of proper parameters immediately after spatial elements adaptation, at the expense of high UE complexity for CSI measurement, high overhead of UCI and more frequent spatial elements on/off operations.

For single CSI feedback in the case of Type 1 spatial adaptation, updating CSI-RS resource in terms of antenna ports is required. In the case of Type 2 spatial elements adaptation, updating CSI report is required so that UEs can report CSI only based on the CSI-RS transmitted after adaptation.

Thus, the key point for the necessary enhancement(s) on signaling aspect is how to efficiently inform the UEs about the updated CSI-RS port/report. Under such consideration, a group common L1 signaling can be introduced. The reason that L1 signaling is needed is for timely updating CSI-RS configuration and CSI report configuration. The reason for group common dynamic signaling is to save signaling overhead. Whenever the gNB applies the spatial elements adaptation, all active UEs served by the gNB have to adjust/change their CSI measurement and report behavior accordingly. If the UE-specific signaling, such as the existing signaling for activating semi-persistent CSI report or triggering aperiodic CSI report, is adopted, the signaling overhead to trigger the UEs’ CSI report behavior change will become significant. On the other hand, if a group-common L1 signaling is adopted the above problems can obviously be alleviated.

Proposal 3. For single CSI feedback, group-common L1 signaling should be considered to efficiently indicate the UEs of the CSI-RS resource/report update.

For multiple CSIs feedback, enhancement(s) to reduce UCI overhead and the CSI processing complexity is required. To achieve this, the most desirable way is that gNB selects and indicates UE to report the CSIs among those have been configured in CSI report configuration.  In this case, gNB needs to dynamically update the CSIs to be reported to enable efficient switching between spatial adaptation patterns.

Proposal 4.  For multiple CSIs feedback, UCI overhead and CSI processing complexity reduction is required. To achieve this, gNB selects the CSIs to be reported and indicates them to UE.

- Dynamic signaling is used for indicating UE of the CSIs need to be measured and reported

Necessary enhancements for CSI-RS based RLM and RRM

Another issue needs to be considered for spatial elements adaptation is that it may also have impact on 1-port CSI-RS transmission.  When TxRU(s) are reduced, corresponding PA(s) and antenna elements are turned off which may lead to CSI-RS transmission power backoff. Eventually, it may lead to false alarm of beam failure and radio link failure, as well as unnecessary event-triggered measurement report, which increases the signaling overhead for UL reporting. Thus, enhancement on RLM and RRM measurement based on CSI-RS should be considered. For example, group-common signaling can be considered to inform UE of transmission power update caused by the spatial element adaptation.

Proposal 5. Enhancements on RLM and RRM measurement on CSI-RS are necessary considering the potential transmission power fluctuation of 1-port CSI-RS due to adaptation of spatial element.

Adaptation of PDSCH transmission power

In current specification, the gNB can indicate the power offset between PDSCH and NZP CSI-RS by RRC parameter powerControlOffset, which is a parameter of NZP CSI-RS resource configuration. UEs calculate CSIs by taking the configured offsets into account. Thus, when the transmission power of PDSCH is adjusted dynamically, power offset between CSI-RS and PDSCH should be changed accordingly. Similar to spatial elements adaption case, both single CSI feedback considering the current power offset and multiple CSIs feedback corresponding to different power offset patterns can be considered. They are applicable in different scenarios based on derivability of CSIs. For multiple CSIs feedback, gNB could select and indicate to UE the CSIs needs to be measured and reported to reduce overhead.

The necessary enhancement from the configuration perspective is to enable a single CSI-RS resource/resource set to be associated with multiple power offsets. The motivation is similar as explained in section 2.1.1, high configuration efficiency can be obtained if each CSI-RS resource is associated with multiple power offsets, while low specification impact is expected if each CSI-RS resource set is associated with multiple power offsets.

The necessary enhancement from the signalling perspective is to enable dynamic update of power offset configuration between CSI-RS and PDSCH. With a similar motivation as explained in section 2.1.3.2, group common L1 signalling can be considered to dynamically update the power offset configuration.

Proposal 6. Support each CSI-RS resource/resource set to be associated with multiple power offsets between CSI-RS and PDSCH.

Proposal 7. Both single CSI feedback and multiple CSIs feedback can be considered for CSI tracking when dynamic adaptation of PDSCH transmission power is adopted.

- For single CSI feedback, group common L1 signaling to update of power offset between CSI-RS and PDSCH should be considered

- For multiple CSIs feedback, gNB could indicate to UE the hypothesis power offsets for CSI measurement and report to reduce UCI overhead.

Conclusions

In this contribution, we discussed the potential enhancement on CSI related procedures. The following observations and proposals were provided.

Observation 1.

- For Type 1 spatial elements adaptation, the mapping relationship between spatial elements and logical antenna ports does not change during adaptation.

  - The nested CSI-RS resource structure can be utilized for channel measurement of different Type 1 adaptation patterns

- For Type 2 spatial elements adaptation, the mapping relationship between spatial elements and logical antenna ports changes during adaptation.

  - Separate CSI-RS resources are required for channel measurement of different Type 2 adaptation patterns

Observation 2.

- The reason we support both Type 1 and Type 2 spatial element adaption is because they originate from different original RF/IF structures. Different operators/vendors need to choose one of them to enhance their own equipment/product for the purpose of energy saving.

- Type 1 and type 2 adaptation have different properties which can be handled respectively.

- A unified solution for Type 1 and Type 2 adaptation is not always expected because it can somehow increase the complexity/cost for the update of the network equipment.

  - Type 1 adaptation or Type 2 adaptation is not a configurable feature but presetting property. Almost no network equipment supports both type 1 and type 2 adaptation.

Observation 3.

- Among the CSI-RS configuration alternatives for spatial element adaptation,

- A1-2-1 can achieve the high efficiency of CSI-RS configuration if there are parameters in NZP-CSI-RS-Resource configuration that have common values.

- A1-2-2 has low specification impacts as the current CSI-RS configuration structure can be reused.

- Among the CSI report configuration alternatives for spatial element adaptation,

- A2-1 can reuse the current CSI-ReportConfig IE.

- A2-2 can achieve the high efficiency of CSI report configuration if there are some parameters in CSI-ReportConfig that have common values.

Observation 4. To achieve CSI tracking of the dynamic varying channel due to dynamic spatial element adaptation, there are two possible approaches.

- Single CSI feedback approach provides a CSI tracking solution without increasing the overhead of CSI reporting. However, latency may be large as gNB has to postpone DL transmission after receiving new CSI report or schedule DL transmission with the parameters quite conservative.

- Multiple CSIs feedback enables the base station to perform DL transmission of proper parameters immediately after spatial elements adaptation, at the expense of high UE complexity for CSI measurement, high overhead of UCI and more frequent spatial elements on/off operations.

Proposal 1. The CSI related enhancement(s) for the support of type 1 spatial element adaptation and type 2 spatial element adaptation are considered and discussed separately.

Proposal 2: Considering the support of spatial adaptation types, specification impact and configuration efficiency, consider the following alternatives for further study:

- CSI-RS configuration

  - A1-2-1) Each CSI-RS resource (NZP-CSI-RS-Resource) is associated with multiple spatial adaptation patterns

    - The common parameters are FFS

  - A1-2-2) Each CSI-RS resource set (NZP-CSI-RS-ResourceSet) is associated with multiple spatial adaptation patterns

- CSI report configuration

  - A2-1) Each CSI-ReportConfig corresponds to one spatial adaptation pattern

  - A2-2) Each CSI-ReportConfig corresponds to multiple spatial adaptation patterns

    - The common parameters are FFS

Proposal 3. For single CSI feedback, group-common L1 signaling should be considered to efficiently indicate the UEs of the CSI-RS resource/report update.

Proposal 4.  For multiple CSIs feedback, UCI overhead and CSI processing complexity reduction is required. To achieve this, gNB selects the CSIs to be reported and indicates them to UE.

- Dynamic signaling is used for indicating UE of the CSIs need to be measured and reported

Proposal 5. Enhancements on RLM and RRM measurement on CSI-RS are necessary considering the potential transmission power fluctuation of 1-port CSI-RS due to adaptation of spatial element.

Proposal 6. Support each CSI-RS resource/resource set to be associated with multiple power offsets between CSI-RS and PDSCH.

Proposal 7. Both single CSI feedback and multiple CSIs feedback can be considered for CSI tracking when dynamic adaptation of PDSCH transmission power is adopted.

- For single CSI feedback, group common L1 signaling to update of power offset between CSI-RS and PDSCH should be considered

- For multiple CSIs feedback, gNB could indicate to UE the hypothesis power offsets for CSI measurement and report to reduce UCI overhead.

- References

[1] RP-223540, “New WID: Network energy savings for NR”, RAN#98e, Dec. 12-16, 2022