3GPP TSG RAN WG1 #112bis-e	R1-2302917

e-Meeting, April 17th – April 26th, 2023

Source: 	Fujitsu

Title:	Discussion on UE features for NR NCR

Agenda item:	9.17.4

Document for:	Discussion and decision

- Introduction

In this contribution, we will discuss UE features for NR NCR based on agreements in previous meetings for NCR.

- Discussion

  - Legacy features

In the legacy, a table of Layer-1 mandatory features (i.e. Table 4.2.15.1-1 in [1]) is introduced for IAB-MT. And the features outside the table are optional, unless stated otherwise.

In general, the UE features for NCR can be defined in a similar manner to save discussion effort. Otherwise, we may need to go through all features one-by-one, which is inefficient.

Proposal 1: Define a new table of Layer-1 mandatory features for NCR. The features outside the table are optional, unless stated otherwise.

Furthermore, the table defined for IAB (Table 4.2.15.1-1 in [1]) can be the baseline for NCR. It should be changed at least based on some agreements/conclusions for NCR. For example:

- Feature group 1-3 (SSB based RLM) should be removed since RLM is optional for NCR as per the agreement below.

Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behaviour of NCR-Fwd when BFR/RLF happen in C link.

- Statements related to CA/DC in component 2) of feature group 3-1 and component 4) of feature group 6-1 should be removed, because it is the common understanding that NCR does not support CA/DC.

- Component 2) of feature group 2-23 should be changed to be “2) 2Tx codebook for FR1 and FR2-1”, because NCR does not support FR2-2 according to R1-2302113 as below.

Agreement The RRC parameters and MAC CE parameters in R1-2302113 are agreed. LS to RAN2 is agreed in R1-2302227.

- Component 7) of feature group 5-1 should be removed due to the following conclusion.

Conclusion For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the dynamic DL/UL operation of NCR-MT and NCR-Fwd is not supported in Rel-18.

Moreover, there are some other remaining issues to be clarified so that the final clean table can be defined for NCR.

Issue 1: FDD operation

During the discussion in previous meetings, the basic assumption is TDD. In our view, it is not clear whether NCR can operate in FDD bands or whether FDD is supported for NCR in Rel-18. If the FDD is not supported, component 5) of feature group 3-1 should be updated.

Issue 2: Discussion/check points in the legacy table for IAB

In Table 4.2.15.1-1 in [1] for IAB, there are still some discussion/check points. Whether the discussion/check points should be maintained in the new table for NCR need to be clarified/discussed.

Issue 3: Additional information for feature group 1-1

In Table 4.2.15.1-1 in [1] for IAB, there is additional information for feature group 1-1. However, the meaning of the additional information seems not clear, and NCR has no relevant discussion yet. The additional information should be clarified to check whether it should be maintained for NCR.

Based on the discussions above, an example for NCR (with comments) is given in Table 1.

Table 1: Layer-1 mandatory features for NCR-MT

| Features | Index | Feature group | Components | Additional information | Comments |
| --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL | 1) CP-OFDM for DL 2) CP -OFDM for UL |  |  |
|  | 0-3 | DL modulation scheme | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |  |  |
|  | 0-4 | UL modulation scheme | 1) QPSK modulation 2) 16QAM modulation |  |  |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1) RACH preamble format 2) SS block based RRM measurement 3) Broadcast SIB reception including RMSI/OSI and paging | Only 1 preamble for component 1), component 2), component 3) except paging | The additional information should be clarified and removed if needed |
|  | 1-3 | SS block based RLM | SS-SINR measurement |  | Remove since RLM is optional for NCR as agreed |
| 2. MIMO | 2-1 | Basic PDSCH reception | 1) Data RE mapping 2) Single layer transmission 3) Support one TCI state |  |  |
|  | 2-5 | Basic downlink DMRS for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. |  |  |
|  | 2-6 | Basic downlink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |
|  | 2-12 | Basic PUSCH transmission | Data RE mapping Single layer (single Tx) transmission Single port, single resource SRS transmission (SRS set use is configured as for codebook) |  |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols |  |  |
|  | 2-16a | Basic uplink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |
|  | 2-22 | Aperiodic beam report | Support aperiodic report on PUSCH |  |  |
|  | 2-32 | Basic CSI feedback | 1) Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2) 2Tx codebook for FR1 and FR2-1 3) 4Tx codebook for FR1 4) 8Tx codebook for FR1 when configured as wideband CSI report 7) a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |  | 1. FR2FR2-1 2.The further discussion points should be clarified and removed if needed. |
|  | 2-50 | Basic TRS | 1) Support of TRS (mandatory) 2) All the periodicity are supported. |  |  |
|  | 2-52 | Basic SRS | 1) Support 1 port SRS transmission 2) Support periodic/aperiodic SRS transmission |  |  |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD |  | 1. Remove statements related to CA/DC in component 2) since NCR does not support CA/DC. 2.Clarify whether FDD is supported for NCR and update component 5) if needed |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as "enabled" 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as "enabled" 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) SR/HARQ multiplexing once per slot using a PUCCH when SR/HARQ-ACK are supposed to be sent by overlapping PUCCH resources with the same starting symbols in a slot 8) HARQ-ACK piggyback on PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on 9) Semi-static beta-offset configuration for HARQ-ACK 10) Single group of overlapping PUCCH/PUCCH and overlapping PUCCH/PUSCH s per slot per PUCCH cell group for control multiplexing |  |  |
|  | 4-10 | Dynamic HARQ-ACK codebook | Dynamic HARQ-ACK codebook |  |  |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - One unicast PDSCH per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment for TDD 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment 9) In TDD support at most one switch point per slot for actual DL/UL transmission(s) 10) DL scheduling slot offset K0=0 12) UL scheduling slot offset K2<=12 For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, interleaving for VRB-to-PRB mapping for PDSCH |  | Remove component 7) since it was concluded the dynamic DL/UL operation of NCR-MT is not supported in Rel-18. |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | 1. Remove statements related to CA/DC in component 4) since NCR does not support CA/DC. |
| 7. Channel coding | 7-1 | Channel coding | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |  |  |
| 8. UL TPC | 8-3 | Basic power control operation | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 9) PHR |  |  |

Proposal 2: For definition of the new table of Layer-1 mandatory features for NCR, take the table defined for IAB (Table 4.2.15.1-1 in [1]) as the baseline.

- At least changes as below should be made based on previous agreements/conclusions for NCR.

  - Remove feature group 1-3 (SSB based RLM).

  - Remove statements related to CA/DC in component 2) of feature group 3-1 and component 4) of feature group 6-1.

  - Change component 2) of feature group 2-23 to be “2) 2Tx codebook for FR1 and FR2-1”.

  - Remove component 7) of feature group 5-1.

- Clarify the following aspects.

  - Whether to support FDD.

    - If FDD is not supported for NCR, component 5) of feature group 3-1 should be updated for NCR.

  - Whether to maintain the discussion/check points in Table 4.2.15.1-1 in [1] for NCR.

  - The meaning of the additional information for feature group 1-1 and whether to maintain the information for NCR.

  - New features

    - C-link and BH-link

For C-link and BH-link, some agreements which are related to UE feature discussion and corresponding comments are provided below.

| Agreements | Comments |
| --- | --- |
| Agreement The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link - Note-1: Fixed beam for C-link/backhaul link is default capability - Note-2: TDMed UL transmission of C-link and backhaul link is default capability. - FFS: How to define the capability for adaptive beam for C-link/backhaul-link | Two features should be defined according to the agreement: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link |
| Agreement If adaptive beams are adopted for C-link and backhaul link, new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link. - Predefined rule is used to define the beam in case there is no indication via the new signalling - FFS: Details of the predefined rule - FFS: Application of predefined rule for other cases - Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link. - The new signalling, if needed, is an optional NCR capability Agreement For NCR-MT which can support adaptive beams in C link, - Rel-15 beam indication framework can be reused - Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports. Agreement The semi-static beam indication for backhaul link is supported as: - If the beam indication framework in Rel-15 is used for NCR-MT - The DL beam is indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link - The UL beam is indicated by SRI on C-link via MAC CE. - If the beam indication framework in Rel-17 is used for NCR-MT - The DL and UL beam are indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link | 1. Support of adaptive beams for C-link and BH-link is prerequisite condition of support of the new signalling (dedicated BH-link beam indication) 2.Two features for the new signalling (i.e. dedicated BH-link beam indication) should be defined respectively for Rel-15/16 and Rel-17 framework (optional for NCR capability). - Dedicated BH-link beam indication when Rel-15/16 beam indication framework is used for C-link - Dedicated BH-link beam indication when Rel-17 beam indication framework is used for C-link |
| Agreement For NCR-MT which can support adaptive beams in C link, - Rel-15 beam indication framework can be reused - Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports. Agreement The following pre-defined rules are applied to determine the beam for backhaul link: - In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link. - In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-15/16 beam indication framework is used for C-link, - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link. Agreement In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-17 beam indication framework is used for C-link, - If no unified TCI is applied for C-link, the beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. (i.e. same as the default beam defined for Rel-15/16 beam indication framework) If there is unified TCI applied for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. | 1. Support of adaptive beams for C-link and BH-link is prerequisite condition of support of BH-link beam determination based on the pre-defined rule. 2.Two features for BH-link beam determination based on the pre-defined rule should be defined respectively for Rel-15/16 and Rel-17 framework (optional for NCR capability). - Dedicated BH-link beam indication when Rel-15/16 beam indication framework is used for C-link - Dedicated BH-link beam indication when Rel-17 beam indication framework is used for C-link |

As a summary, we have the following proposal.

Proposal 3: For NCR C-link and BH-link, define the following optional features.

- Simultaneous UL transmission of C-link and BH-link

  - If it is not supported, TDMed UL transmission of C-link and BH-link is assumed.

- Adaptive beam for C-link and BH-link

  - If it is not supported, fixed beam for C-link and BH-link is assumed.

- Dedicated BH-link beam indication when Rel-15/16 beam indication framework is used for C-link

  - It is optional only when adaptive beam for C-link and BH-link beam is supported.

- Dedicated BH-link beam indication when Rel-17 beam indication framework is used for C-link

  - It is optional only when adaptive beam for C-link and BH-link beam is supported.

- BH-link beam determination when Rel-15/16 beam indication framework is used for C-link

  - It is mandatory when adaptive beam for C-link and BH-link beam is supported.

- BH-link beam determination when Rel-17 beam indication framework is used for C-link

  - It is mandatory when adaptive beam for C-link and BH-link beam is supported.

    - AC-link

For AC-link, some agreements which are related to UE feature discussion and corresponding comments are provided below.

| Agreements | Comments |
| --- | --- |
| Agreement Update the agreements on ‘ON-OFF’ achieved in RAN1 #111 as follows: For FR2, the “ON” state of NCR-Fwd is indicated: - Implicit indication via the AC link beam indication (i.e., if there is an AC link beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) For FR1, the “ON” state of NCR-Fwd is indicated: - Indication via the AC link beam indication (i.e., if there is an AC link beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) - When there is only one beam, the sole purpose of the AC link beam indication is for indicating “ON” state of NCR-Fwd Note: Backhaul link is following the AC link in terms of ON/OFF. | AC-link beam indication is necessary for NCR’s proper operation, at least with respect to ON-OFF. |
| Agreement For each periodic beam indication for access link, one RRC signalling is used including the information defined by the following: - A list of X (1≤X≤Xmax) forwarding resource, each is defined as {Beam index, time resource} - FFS: The value of Xmax, other details Each time resource is defined by {Starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field. - The periodicity is configured as part of the RRC signaling for periodic beam indication - The same periodicity is assumed for all time resource(s) in one periodic beam indication. - The reference SCS is configured as part of the RRC signaling for periodic beam indication - The same reference SCS is assumed for all time resource(s) in one periodic beam indication. | 1. Three features can be defined for periodic, semi-persistent and aperiodic beam indication for access link, respectively. 2. Since AC-link beam indication is necessary for NCR’s proper operation, at least one of periodic, semi-persistent and aperiodic beam indication for access link should be supported or mandatory for NCR. 3.For semi-persistent beam indication for access link, support of beam index update via MAC CE can be defined as an optional feature. |
| Agreement For semi-persistent beam indication: - Alt-0: - RRC configures Y list of forwarding resource, the yth list is consist of Zy 1≤Zy≤Zmax forwarding resources, and each forwarding resource is defined by {beam index, time resource}. - The periodicity and reference SCS is configured as part of the RRC signaling for each list of forwarding resource - MAC-CE activate/de-activate one of Y list, and all the Zy forwarding resources in this list are selected. - MAC-CE can optionally provide update for Zy beam index - Note: The value of Zmax (1≤Z≤Zmax) is 128, where Zmax refers to the maximum beams indicated in one indication. |  |
| Agreement For each aperiodic beam indication for access link, one DCI is used with the information defined by Option-1: - Lmax fields are used to indicate the beam information and each field refers to one beam index ; - Note: The bitwidth of this field is determined by the number of beams used for access link. - Tmax fields to indicate the time resource; - Note: A list of time resource is pre-defined by RRC signalling. The bitwidth of this field for time resource indication is determined by the length of list. - FFS: The value of Tmax - Down-select between Tmax=1 or Tmax=Lmax. - FFS: How to define the association between time indication and beam indication Each time resource is defined by {Starting slot defined as the slot offset, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field. Agreement The DCI Format 5_0 carrying the side control information is monitored by the NCR-MT at least in the UE specific search space. Note: The existing configuration of UE specific search space will be reused. |  |

Proposal 4: For NCR AC-link, define the following optional features.

- Periodic AC-link beam indication

- Semi-persistent AC-link beam indication

- Aperiodic AC-link beam indication

- Update of AC-link beam via MAC CE

  - It is optional only when semi-persistent AC-link beam indication is supported.

- Note: At least one of periodic, semi-persistent and aperiodic AC-link beam indication is supported or mandatory for NCR.

    - ON-OFF

For ON-OFF, some agreements which are related to UE feature discussion and corresponding comments are provided below.

| Agreements | Comments |
| --- | --- |
| Agreement Update the agreements on ‘ON-OFF’ achieved in RAN1 #111 as follows: For FR2, the “ON” state of NCR-Fwd is indicated: - Implicit indication via the AC link beam indication (i.e., if there is an AC link beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) For FR1, the “ON” state of NCR-Fwd is indicated: - Indication via the AC link beam indication (i.e., if there is an AC link beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) - When there is only one beam, the sole purpose of the AC link beam indication is for indicating “ON” state of NCR-Fwd Note: Backhaul link is following the AC link in terms of ON/OFF. | ON-OFF according to AC-link beam indication should be mandatory feature for NCR |
| Agreement For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols - FFS: The behavior over these symbol if dynamic DL/UL operation is supported by NCR-MT and/or NCR-Fwd. Conclusion - For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the dynamic DL/UL operation of NCR-MT and NCR-Fwd is not supported in Rel-18. | ON-OFF according to semi-static UL/DL configuration should be mandatory feature for NCR |
| Agreement Once beam failure is detected in C link by NCR-MT, NCR-Fwd is OFF until the beam failure recovery is completed. Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link. | ON-OFF according to BFD/BFR should be mandatory feature for NCR supporting BFD/BFR. |
| On NCR-MT RLF: - After RLF is declared by NCR-MT, NCR-MT performs cell selection and trigger RRC re-establishment; - If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF; - During RRC re-establishment procedure, NCR-Fwd is OFF. Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link. | ON-OFF according to RLF should be mandatory feature for NCR supporting RLM. |
| - WA: RRC_INACTIVE is optionally supported without any specific enhancements On NCR-Fwd ON/OFF: - When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB. - After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB. - Release to RRC-IDLE is FFS. - The NCR-FWD is switched OFF if the NCR-MT in RRC_INACTIVE state reselects a different cell than the last serving cell on which side control configuration was received. | ON-OFF in case when NCR-MT enters RRC_INACTIVE should be mandatory feature for NCR supporting RRC_INACTIVE. |

Proposal 5: For ON/OFF, at least the following optional features considered.

- ON-OFF according to AC-link beam indication and semi-static TDD configuration

  - It is mandatory for NCR.

- forwarding when NCR-MT enters RRC_INACTIVE

  - It is mandatory when NCR supports RRC_INACTIVE.

- ON-OFF in case of BFD/BFR

  - It is mandatory when NCR supports BFD/BFR.

- ON-OFF in case of RLF

  - It is mandatory when NCR supports RLM.

Based on the discussions above in 2.2, an example for definition of new features is given in Table 2.

Table 2: L1-1 feature list for NR_NCR

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| X. NR_NCR | X-Y1 | Simultaneous UL transmission of C-link and BH-link | Support simultaneous UL transmission of C-link and BH-link |  | If not supported, TDMed UL transmission of C-link and BH-link is assumed. | Optional with capability signalling |
|  | X-Y2 | Adaptive beam for C-link and BH-link | Support adaptive beam for C-link and BH-link |  | If not supported, fixed beam for C-link and BH-link is assumed. | Optional with capability signalling |
|  | X-Y3a | Dedicated BH-link beam indication | Support MAC CEs for dedicated BH-link beam indication when Rel-15/16 beam indication framework is used for C-link | X-Y2 |  | Optional with capability signalling |
|  | X-Y3b | Dedicated BH-link beam indication | Support MAC CE for dedicated BH-link beam indication when Rel-17 beam indication framework is used for C-link | X-Y2 | Optional when NCR supports Rel-17 beam indication framework | Optional with capability signalling |
|  | X-Y4a | BH-link beam determination | Support BH-link beam determination based on pre-defined rules when Rel-15/16 beam indication framework is used for C-link | X-Y2 | Mandatory when NCR supports X-Y2 | Optional with capability signalling |
|  | X-Y4b | BH-link beam determination | Support BH-link beam determination based on pre-defined rules when Rel-17 beam indication framework is used for C-link | X-Y2 | Mandatory when NCR supports X-Y2 and Rel-17 beam indication framework | Optional with capability signalling |
|  | X-Y5 | Periodic AC-link beam indication | Support semi-static configuration/indication of access link beam |  | Support at least one of X-Y5, X-Y6 and X-Y7 | Optional with capability signalling |
|  | X-Y6 | Semi-persistent AC-link beam indication | Support semi-persistent configuration/indication of access link beam |  |  | Optional with capability signalling |
|  | X-Y7 | Aperiodic AC-link beam indication | Support monitoring DCI Format 5_0 scrambled by NCR-RNTI for indication of access link beam |  |  | Optional with capability signalling |
|  | X-Y8 | Update of AC-link beam via MAC CE | Support update of AC-link beam via MAC CE for semi-persistent AC-link beam indication | X-Y6 |  | Optional with capability signalling |
|  | X-Y9a | ON-OFF operation | Support ON-OFF according to AC-link beam indication and semi-static TDD configuration |  |  | Mandatory with capability signalling |
|  | X-Y9b | ON-OFF operation | Support forwarding when NCR-MT enters RRC_INACTIVE |  | Mandatory when NCR supports RRC_INACTIVE | Optional with capability signalling |
|  | X-Y9c | ON-OFF operation | Support ON-OFF in case of BFD/BFR |  | Mandatory when NCR supports BFD/BFR | Optional with capability signalling |
|  | X-Y9d | ON-OFF operation | Support ON-OFF in case of RLF |  | Mandatory when NCR supports RLM | Optional with capability signalling |

- Conclusion

This contribution discusses UE features for NR NCR, and we have proposals as below. For other details, please refer to Table 1 and Table 2 in section 2.

Proposal 1: Define a new table of Layer-1 mandatory features for NCR. The features outside the table are optional, unless stated otherwise.

Proposal 2: For definition of the new table of Layer-1 mandatory features for NCR, take the table defined for IAB (Table 4.2.15.1-1 in [1]) as the baseline.

- At least changes as below should be made based on previous agreements/conclusions for NCR.

  - Remove feature group 1-3 (SSB based RLM).

  - Remove statements related to CA/DC in component 2) of feature group 3-1 and component 4) of feature group 6-1.

  - Change component 2) of feature group 2-23 to be “2) 2Tx codebook for FR1 and FR2-1”.

  - Remove component 7) of feature group 5-1.

- Clarify the following aspects.

  - Whether to support FDD.

    - If FDD is not supported for NCR, component 5) of feature group 3-1 should be updated for NCR.

  - Whether to maintain the discussion/check points in Table 4.2.15.1-1 in [1] for NCR.

  - The meaning of the additional information for feature group 1-1 and whether to maintain the information for NCR.

Proposal 3: For NCR C-link and BH-link, define the following optional features.

- Simultaneous UL transmission of C-link and BH-link

  - If it is not supported, TDMed UL transmission of C-link and BH-link is assumed.

- Adaptive beam for C-link and BH-link

  - If it is not supported, fixed beam for C-link and BH-link is assumed.

- Dedicated BH-link beam indication when Rel-15/16 beam indication framework is used for C-link

  - It is optional only when adaptive beam for C-link and BH-link beam is supported.

- Dedicated BH-link beam indication when Rel-17 beam indication framework is used for C-link

  - It is optional only when adaptive beam for C-link and BH-link beam is supported.

- BH-link beam determination when Rel-15/16 beam indication framework is used for C-link

  - It is mandatory when adaptive beam for C-link and BH-link beam is supported.

- BH-link beam determination when Rel-17 beam indication framework is used for C-link

  - It is mandatory when adaptive beam for C-link and BH-link beam is supported.

Proposal 4: For NCR AC-link, define the following optional features.

- Periodic AC-link beam indication

- Semi-persistent AC-link beam indication

- Aperiodic AC-link beam indication

- Update of AC-link beam via MAC CE

  - It is optional only when semi-persistent AC-link beam indication is supported.

- Note: At least one of periodic, semi-persistent and aperiodic AC-link beam indication is supported or mandatory for NCR.

Proposal 5: For ON/OFF, at least the following optional features considered.

- ON-OFF according to AC-link beam indication and semi-static TDD configuration

  - It is mandatory for NCR.

- forwarding when NCR-MT enters RRC_INACTIVE

  - It is mandatory when NCR supports RRC_INACTIVE.

- ON-OFF in case of BFD/BFR

  - It is mandatory when NCR supports BFD/BFR.

- ON-OFF in case of RLF

  - It is mandatory when NCR supports RLM.

- Appendix

- 3GPP TS 38.306 V17.4.0 (2023-03), NR; User Equipment (UE) radio access capabilities (Release 17)