3GPP TSG-RAN WG1 #112bis-e	R1-2302920

e-Meeting, April 17th – April 26th, 2022

Agenda Item:	9.17.16

Source:	Ericsson

Title:	UE feature for agreed TEI-18 on SR periodicity

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the UE feature for the agreed TEI-18 on SR periodicity.

# 2	Discussion

RAN1 has identified an issue for the periodicities of Scheduling Request (SR) periodicity. The issue is that the periodic SRs does not support 5 (and 10 slot) periodicities in FR2 while at the same time, all the three used FR2 TDD patterns have 5 slot periodicity.

The problem with this is that it can introduce latency in some cases due to SR not aligning, it can also introduce difficulties in avoiding symbol overlap with other channels.

A contribution to the RAN1 TEI18 item was brought up (R1-2300155) to RAN1#112 and was agreed. This is the corresponding UE feature for this agreement to allow 5 and 10 slot periodicities for 120 and 30 kHz SCS.

We note that there is no prerequisite feature group for the SR periodicities, hence the periodic SR feature is in legacy de facto supported “Per UE”. Hence, it is natural if this extension of that feature is also supported “Per UE”.

Table 1: Proposed FGs for BWP with restriction.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI-18 | 55-1 | additionalSR-Periodicities-r18 | Indicates whether the UE supports the following SR periodicities in the periodicityAndOffset parameter as specified in TS 38.331 [9]: -5sl for 30 kHz SCS -5sl and 10sl for 120 kHz SCS | N/A | Yes | N/A | If the network implements the TS 38.331 CR on new SR periodicities and the UE does not according to the capability indication, the network will not assign the new SR periodicities. Legacy behaviour applies. | Per UE | No | No | N/A |  | Optional |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture the UE capability for SR periodicity provided in Table 1.