3GPP TSG RAN WG1 #112bis-e          	          	      		            R1- 2302921

e-Meeting, April 17th – April 26th, 2023

Agenda Item:   	5

Source:   	LG Electronics

Title: 	Discussion on LS on comparison of SL-RSRP and SD-RSRP measurements

Document for:   	Discussion and decision

- Introduction

In this contribution, we will provide our view on the question related to the feasibility/necessity of the measurement reporting event using the comparison of SL-RSRP and SD-RSRP measurements for the indirect-to-indirect path switch procedure in RAN2 LS [1].

- Discussion

First of all, the new Event Z1 below was agreed in RAN2 for the indirect-to-indirect path switch operation. As already discussed in RAN2, we think that the direct comparison of SL-RSRP and SD-RSRP measurements at the Event Z2 could trigger the unnecessary measurement reporting due to the SL pathloss-based power control. Also, even the offset value semi-statically configured by the network is difficult to effectively solve this issue.

- Agreements made in RAN2#119-e meeting:

  - For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2.

    - FFS if we also have an Event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE.

To the question below on LS [1], for the reasons described above, it would be desirable for RAN1 to provide an answer saying that there is no strong technical motivation to newly define the measurement reporting event using the comparison of SL-RSRP and SD-RSRP measurements other than the Event Z1.

- Question: Can the comparison of SL-RSRP and SD-RSRP measurements be used for the purposes of triggering a measurement report?

Observation: The measurement event using the direct comparison of SL-RSRP and SD-RSRP measurements could trigger the unnecessary reporting due to the SL pathloss-based power control. Furthermore, even the offset value semi-statically configured by the network is difficult to effectively solve this issue.

Proposal: RAN1 provides an answer to the question on RAN2 LS [1] as follows:

- From the perspective of RAN1, for the indirect-to-indirect path switch procedure, there is no strong technical motivation to newly define the measurement reporting event using the comparison of SL-RSRP and SD-RSRP measurements other than the Event Z1.

- Conclusion

In this contribution, we discussed how RAN1 provides feedbacks on RAN2’s question in LS [1]. The following observation and proposal are given.

Observation: The measurement event using the direct comparison of SL-RSRP and SD-RSRP measurements could trigger the unnecessary reporting due to the SL pathloss-based power control. Furthermore, even the offset value semi-statically configured by the network is difficult to effectively solve this issue.

Proposal: RAN1 provides an answer to the question on RAN2 LS [1] as follows:

- From the perspective of RAN1, for the indirect-to-indirect path switch procedure, there is no strong technical motivation to newly define the measurement reporting event using the comparison of SL-RSRP and SD-RSRP measurements other than the Event Z1.

- Reference

[1] R1-2302280, “LS on Comparison of SL-RSRP and SD-RSRP measurements,” RAN2, Nokia.