3GPP TSG RAN WG1 #112bis-e	  	          	     		  R1-2302923

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.1.2

Source: 	LG Electronics

Title: 	Discussion on physical channel design framework for sidelink on unlicensed spectrum

Document for:	Discussion and decision

- Introduction

In RAN1#112 meeting [1], followings are agreed for channel access mechanism for SL-U:

Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - FFS: value of K2, K4 - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case - FFS: whether/how to reduce PAPR of PSFCH transmission Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements Agreement For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc. - FFS gap of 0 - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS. Working assumption If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS: value range - FFS: whether a different reference number of symbols is needed for transmission in a shared COT Agreement Down-select one or support both of the followings: - Option 1: Additional candidate S-SSB occasions are excluded from resource pool - Option 2: Additional candidate S-SSB occasions belong to resource pool - Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc. Agreement RAN1 further study the followings: - Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc. Agreement RAN1 further study the followings: - Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

In this contribution, we discuss issues on the physical channel design for NR sidelink transmission on unlicensed band.

- Discussion

  - SL BWP and SL resource pool configuration

In case of SL communication on licensed spectrum, for TDD, cell-specific UL slots can be belonging to a resource pool. For flexible slots, if the SL symbol group in a slot is set to cell-specific UL, then the flexible slots can be belonging to a resource pool as well. According to WID of NR SL evolution, the target NR operating band is n46, n96, and n102. According to Table 5.2-1 in TS 38.101, the duplex mode of these bands is TDD as shown in Table 1.

Table 1: NR operating bands in FR1.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD13 |
| n9614 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD13 |
| n10214 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD13 |
| NOTE 13: This band is restricted to operation with shared spectrum channel access as defined in 37.213. NOTE 14: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. |  |  |  |

In this case, even for unlicensed spectrum operation, the same principle could be adopted for candidate slots that can be belonging to a resource pool. If it is allowed that other flexible slots or DL slots to belong to a resource pool, it would be further discuss how to define prioritization rule between DL reception (e.g., DL discovery burst, PDCCH monitoring, CSI measurement) and SL transmission/reception. If the UE is configured with tdd-UL-DL-ConfigurationCommon for the unlicensed carrier, it seems straightforward to use it as in Rel-16/17 NR SL.

Proposal 1: For SL communication on shared spectrum, a resource pool excludes following slots:

- Slots whose symbols from sl-StartSymbol to sl-StartSymbol + sl-LengthSymbols – 1 are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration if provided or sl-TDD-Config of the received PSBCH if provided

- S-SSB slots (including additional S-SSB slots)

- Reserved slot as specified in section 8 of TS 38.214

    - Interlaced RB-based transmission

Since the sub-channelization without a separate indication of allocated RB set(s) will lose scheduling flexibility, it would be better to consider indicating RB set(s) separately from indicating interlace(s) or sub-channel(s). In this case, a single sub-channel can consists of PRBs belonging to a single or multiple interlaces across RB set(s). The final PRBs allocated for PSSCH transmission will be determined by intersection between the indicated sub-channel(s) and the indicated RB set(s) like NR-U resource allocation type 2. Since it would be necessary to consider how to indicate reserved resources in NR SL, FRIV can be used to indicate RB set(s) instead of RIV. In this approach, we can maximizes the scheduling flexibility and can guarantee the same set of interlaces across different RB sets when more than one RB sets are used for PSSCH transmission.

Proposal 2: For interlaced RB based PSSCH transmission,

- Support explicitly indicating the allocated sub-channel index(s) and RB set index(s)

  - SCI indicates FRIV for subchannel allocation and FRIV for RB set allocation.

  - Subchannel indexing is done in increasing order of the interlace index.

  - PSSCH transmission resource(s) are determined by the intersection of indicated sub-channel(s) and indicated RB set(s).

On PSCCH mapping, it is agreed that the PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH. In this case, it would be necessary to clarify the exact meaning of the lowest sub-channel. In Rel-16/17 NR sidelink, as the sub-channel index decreases, the actual frequency location of the sub-channel is lowered. However, since interlace will be defined in common RB grid, it would be possible that the interlace with higher index can locate lower frequency range compared to interlace with lower index depending on the staring PRB of SL BWP, the starting PRB of RB set, and CRB#0 as shown in Figure 1.

Figure 1: Example of interlace/subchannel indexing within a RB set.

Meanwhile, considering that SCI can indicates reserved resources for future PSCCH/PSSCH transmission, it is necessary that the PSSCH allocation derived by the PSCCH location and the number of sub-channels also needs to be expressed by FRIV (and RB set indicator, if necessary). Moreover, due to the compact design of FRIV, depending on the number of allocated sub-channels, the range of the starting sub-channel index would be limited.

In this case, if the lowest sub-channel implies the sub-channel with the lowest location, it is possible that different PSSCH allocations have the same PSCCH location. For instance, for PSCCH allocation with sub-channels {#1, #2, #3} and PSSCH allocation with sub-channels {#2, #3, #4}, the PSCCH will be commonly mapped on sub-channel #3 as shown in Figure 2. In other words, there could be ambiguity on the PSSCH allocation since the same PSCCH location can be associated with different PSSCH allocations. In case of PSSCH allocation with sub-channel {#3, #4, #0} is not feasible since this combination will be expressed by the existing FRIV.

Figure 2: Example of the meaning of the lowest sub-channel in frequency domain for PSCCH mapping.

On the other hand, if the lowest sub-channel implies the sub-channel with the lowest index, the above ambiguity problem could be resolved. For instance, the PSCCH for PSCCH allocation with sub-channels {#1, #2, #3} will locates sub-channel #1 while the PSCCH for PSSCH allocation with sub-channels {#2, #3, #4} will locates sub-channel #2. Therefore, for a given PSCCH location and the number of sub-channels, TX UE and RX UE can have the same understanding on the PSSCH allocation. Moreover, these combinations are still expressed by the existing FRIV.

Observation 1: For PSCCH mapping, the meaning of the lowest sub-channel of the lowest RB set needs to be carefully clarified.

Observation 2: For PSCCH mapping, if the lowest sub-channel is the sub-channel with the lowest location, since different PSSCH allocations can have the same PSCCH location, there could be ambiguity on the PSSCH allocation even though the RX UE knows the location of PSCCH and the number of sub-channels.

Observation 3: For PSCCH mapping, if the lowest sub-channel is the sub-channel with the lowest index, different PSSCH allocation will have different PSCCH location, so the ambiguity problem will be resolved.

Proposal 3: For PSCCH mapping for interlaced PSSCH transmission, RAN1 clarify that the lowest sub-channel implies the sub-channel with the lowest index.

On TBS determination, it would be necessary to assume that every subchannels consist of the common number of PRBs or interlaces to ensure the same TBS between initial transmission and retransmission. Considering that the number of PRBs belonging to interlace within a RB set can be 9, 10, or 11, for simplicity, 10 PRB is assumed for the interlace size.

Proposal 4: For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- TBS is determined based on a reference number of PRBs of a sub-channel

  - The reference number of PRBs is 10.

  - Time domain resource assignment for PSCCH/PSSCH on shared spectrum

    - Additional starting symbol(s) within a slot

For PSCCH/PSSCH with two candidate starting positions, it is necessary to check whether or how to ensure enabling the same TB size among PSSCH transmission candidates with different starting positions. For TBS determination, if the TBS is determined based on the 1st starting position, the effective code rate of the PSSCH transmission with 2nd starting position could be too high. In this case, the UE still need to transmit retransmission for the same TB. If the TBS is determined based on the 2nd starting position, the peak data rate will be limited.

Meanwhile, the existing TBS determination is based on the higher layer parameter sl-LengthSymbols. If UE want to reduce the coding rate for the case when the 2nd starting position is used, it can be considered that the UE uses lower MCS index and/or indicates PSFCH overhead of 3 OFDM symbols.

Proposal 5: For multiple starting symbols within a slot for a PSCCH/PSSCH transmission, the reference number of symbols for TBS determination is determined by sl-LengthSymbols

- No specification change is supported for TBS determination for the purpose of additional starting position in a slot.

Since PSCCH transmission will be confined within the PSSCH transmission, if more than one starting positions are allowed, the PSCCH transmission candidates would be also increased. In this case, the PSSCH RX UE needs to perform blind decoding for multiple starting positions. Once multiple starting symbols in a slot for a PSCCH/PSSCH transmission is supported, UE should be capable of performing blind decoding for multiple starting symbols as well. It was discussed the possibility that the UE skips blind decoding for PSCCH with the 2nd starting symbol when the UE success to detect PSCCH on the 1st starting symbol. However, it will have negative impact on the SL sensing operation. Due to the hidden-node issue or frequency reuse, it is possible that two different PSCCH/PSSCH transmissions with different starting symbols are overlapping in the same slot. Since each PSCCH will contain SCI indicating reserved resources, it would be beneficial for a UE to decode both PSCCH with different starting symbols for SL Mode 2 operation.

On the other hand, it was discussed the possibility of reducing the PSCCH candidates for each starting position. However, since the PSCCH will be located on the lowest sub-channel of the corresponding PSSCH, reducing the number of PSCCH candidates will reduce the amount of the candidate single-slot resources for PSSCH transmission as well. It will increase the resource collisions among SL transmissions. In those points of views, once the additional starting position is supported for PSCCH/PSSCH transmission in a slot, it is understood that the UE with a capability for the increased PSCCH BD attempts is supported as well.

Proposal 6: UE monitors PSCCHs on two starting symbols within a slot where 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission is supported regardless of detection of PSCCH on the 1st starting symbol.

As per working assumption, the PSSCH RX UE will perform additional AGC procedure for the 2nd starting position since PSCCH/PSSCH transmission with different starting positions can be FDMed in a system perspective. Since these additional AGC symbol would be not be used for decoding or channel estimation, the throughput performance would be also degraded. In Rel-16/17 NR SL, REs used for the PSSCH in the second OFDM symbol is duplicated in AGC symbol. However, AGC procedure itself is RX UE behavior, and it does not always implies such duplication shall be used as TX UE behavior.

Observation 4: Starting symbol candidates intended for AGC purpose does not imply that REs on other symbol will be duplicated in the AGC symbol.

To alleviate performance degradation due to additional AGC procedure at the additional starting position, it is necessary to decide which symbol needs to be avoided or to be selected as the additional starting symbol. First of all, at least PSCCH symbol duration of PSCCH/PSSCH with the 1st starting symbol needs to be avoided to be 2nd starting symbol to guarantee PSCCH detection performance. If the 2nd starting symbol is overlapping with PSCCH symbol duration of PSCCH/PSSCH with the 1st starting position as shown in Figure 3, the RX UE can effectively use only one symbol for PSCCH decoding.

Observation 5: The 2nd starting symbol of PSCCH/PSSCH is not overlapping with PSCCH symbol duration for the 1st starting symbol of PSCCH/PSSCH to guarantee PSCCH detection performance.

Figure 3: Example of the additional starting position of PSCCH/PSSCH overlapping with PSCCH symbol duration of PSCCH/PSSCH with the 1st starting position.

In a similar manner, the 2nd starting symbol needs to avoid the time location(s) for the 2nd SCI of PSCCH/PSSCH with the 1st starting position. To do this, the 2nd starting position would be located later than the upper limit of the 2nd SCI mapping. Alternatively, beta offset indicator needs to be adjusted to ensure that the 2nd SCI mapping of PSCCH/PSSCH with the 1st starting position is not overlapping with the 2nd starting position.

Observation 6: It is necessary to carefully investigate whether or how the 2nd starting symbol of PSCCH/PSSCH is overlapping with the time location(s) for the 2nd SCI mapping for the 1st starting symbol of PSCCH/PSSCH.

Considering channel estimation performance for 2nd SCI and PSSCH, it would be better to avoid possible time location(s) of PSSCH DMRS for the 1st starting symbol. On the other hand, if the 2nd starting symbol is always overlapping with data symbol of PSSCH with the 1st starting symbol, the RX UE will fail to decode certain code block(s) mapped on the data symbol as shown in Figure 4.

Figure 4: Example of the additional starting position of PSCCH/PSSCH overlapping with or without PSSCH DMRS symbol of PSCCH/PSSCH with the 1st starting position.

Since NR SL does not support CBG-based retransmission, the TX UE may need to retransmit the whole TB until the all the code blocks are not skipped due to additional AGC procedure. For instance, if the TX UE transmits PSCCH/PSSCH with the 1st starting symbol, the TX UE still need to retransmit the PSCCH/PSSCH with different starting symbol and/or PSSCH DMRS pattern and/or beta offset to recover the punctured code block. To avoid this situation, it can be considered that the 2nd starting symbol can be overlapping with a certain PSSCH DMRS symbol of PSCCH/PSSCH with the 1st starting symbol as shown in Figure 6. If the number of PSSCH DMRS symbols is large enough, performance degradation on channel estimation due to puncturing PSSCH DMRS symbol would be marginal. For instance, if the 1st starting symbol index of PSCCH/PSSCH is 0, and if the number of SL symbols in a slot is 14, and if the number of PSSCH DMRS symbols can be 2 or 4 in the resource pool, the 2nd starting symbol of PSCCH/PSSCH could be set to OFDM symbol index 4 which is the second DMRS symbol position for the 1st starting symbol. If the TX UE determines that the additional AGC procedure can be performed in the data symbol, the TX UE will select the number of PSSCH DMRS symbols as 2 for the 1st starting symbol. On the other hand, if the TX UE determines that the additional AGC procedure needs to be performed to avoid the data symbol, the TX UE will select the number of PSSCH DMRS symbols as 4 for the 1st starting symbol.

Observation 7: Considering a certain code block(s) can be consistently dropped, it can be considered that the 2nd starting symbol of PSCCH/PSSCH is set to PSSCH DMRS symbol index of a certain DMRS pattern for PSCCH/PSSCH with the 1st starting symbol.

Proposal 7: For multiple starting symbols within a slot for a PSCCH/PSSCH transmission, the 2nd starting symbol is selected so that overlapping with at least PSCCH duration for the 1st starting symbol is avoided.

    - MCSt and SL transmission burst structure

To minimize overhead for channel sensing operation, it can be considered to introduce SL burst transmission for the same TB and/or different TBs. According to TS 37.213 [2], for burst transmission, gNB or UE can skip LBT operation for consecutive DL or UL transmissions without gaps after the gNB or UE accesses the channel according to channel access procedure, respectively.

S-SSB slot(s) could be located in the middle of MCSt. In this case, if the UE does not transmit the S-SSB, the SL transmission burst will be separated due to the time gap greater than 16us. For the separated SL transmission bursts, the UE may need to perform Type 1 channel access procedure separately. Moreover, if the MCSt occupies more than one RB sets, and if the S-SSB transmission occupies only one RB set, the SL transmission burst on other RB sets can be broken.

Meanwhile, the S-SSB period is currently 160msec, and the UE can be provided with at most 3 time allocation for S-SSB occasions where each time allocation can indicate the number of S-SSB occasions within a period and the time interval between S-SSB occasions. To check the necessity of the S-SSB transmission to keep the MCSt or SL TX burst, it would be necessary to check how frequently the S-SSB occasion will break the MCSt or SL TX burst. Considering that the MCSt size does not need to be larger than the maximum COT duration, it can be considered to check the time gap between S-SSB occasions when the S-SSB occasions are evenly distributed over the period. For instance, for 15 kHz SCS, if the number of S-SSB occasions is increased into 2 for 3 of the time allocations of S-SSB occasions, the time gap between S-SSB will roughly 26.67msec, and this time gap is much larger than the maximum COT duration of 6msec. For 30 kHz SCS, if the number of S-SSB occasions is increased into 4 for 3 of the time allocations of S-SSB occasions, the time gap between S-SSB will roughly 13.33msec, and this time gap is much larger than the maximum COT duration of 6msec. For 60 kHz SCS, if the number of S-SSB occasions is increased into 8 for 3 of the time allocations of S-SSB occasions, the time gap between S-SSB will roughly 6.67msec, and this time gap is much larger than the maximum COT duration of 6msec. For the analysis, it is assumed that the number of additional S-SSB occasions is no greater than the maximum number of the existing candidate value.

Observation 8: Considering the S-SSB period and the expected maximum number of S-SSB occasions within a period, the time gap between (contiguous) S-SSB occasions (groups) is much larger than the maximum COT duration even though it is assumed that the S-SSB occasions are evenly distributed over the period.

Moreover, if the S-SSB occasions within a period is contiguous in time domain, the time gap will be further increased. In addition, depending on the CAPC value, the maximum COT duration will be decreased further. In those points of views, it seems not critical issue to design S-SSB repetition for the purpose of maintaining a COT.

Proposal 8: RAN1 deprioritize the design of S-SSB repetition in one or more RB sets to maintain a COT when the COT contains one or multiple RB sets and includes S-SSB slot(s)

On the other hand, PSFCH resource can be TDMed with PSCCH/PSSCH resource within a slot, and the maximum PSFCH resource period is 4 logical slots. Moreover, if the PSFCH transmission occupies only one RB set, the SL transmission burst on multiple RB set will be (partially) broken as well. Unlink S-SSB occasions, the PSFCH occasions are present more frequently. If maintaining a COT consisting of multiple RB set is necessary, it would be necessary to design a single PSFCH transmission over multiple RB sets. To do this, for the same PSSCH resource, multiple PSFCH resource groups across multiple RB sets needs to be mapped. Alternatively, it can be considered that the UE can transmit dummy signals on certain RB set(s) in PSFCH occasion for the purpose of maintaining COT.

For the COT sharing or SL transmission burst, the condition for COT sharing needs to be met. In case of CAPC restriction, since the CAPC value of S-SSB or PSFCH is always 1, the condition will be always met regardless of the CAPC value used to initiate the COT. On the other hand, the transmit power also need to be restricted based on the EDT used to initiate the COT. If the transmit power is larger than the reference power associated with the EDT, it will cause the fairness problem. In this case, the PSFCH transmit power in the middle of the MCSt also need to be limited not to exceed the reference power associated with the EDT for initiating the COT. Otherwise, the UE need to use Type 1 channel access procedure for the PSFCH transmission in the middle of the MCSt.

Meanwhile, if UE performs SL RX in the middle of the MCSt, it will also break the SL transmission burst. If the SL transmission associated with the SL RX in the middle of the MCSt utilizes the COT associated with the MCSt, the UE can perform Type 2A or Type 2B or Type 2C channel access procedure for the remaining transmissions within the MCSt based on the condition including time gap between SL RX and SL TX considering DL-to-UL-to-DL COT sharing in NR-U. However, if the SL transmission associated with the SL RX in the middle of the MCSt does not utilize the COT associated with the MCSt due to the transmit power greater than the reference power associated with the EDT or double COT sharing issue, the UE may need to perform Type 1 channel access procedure for the remaining transmissions within the MCSt. In this point of view, for the purpose of maintaining the COT, PSFCH or S-SSB TX would need to be prioritized over PSFCH or S-SSB RX. Considering the existing PSFCH TX/RX prioritization and synchronization procedure, this change will have a huge impact on the system.

In short, even though there is PSFCH TX or RX in the PSFCH occasion in the middle of MCSt, depending on the condition, the COT would not be maintained. In our understanding, resolve this issue needs to be prioritized over the discussion on whether or how to design SL transmission(s) on one or more RB sets to maintain the COT.

Observation 9: If the transmit power of PSFCH transmission in the middle of the MCSt is greater than the reference power associated with the EDT used to access the channel for MCSs, the COT corresponding to the MCSt will not be maintained even though there is a SL transmission in PSFCH occasion in the middle of the MCSt

Observation 10: If the PSFCH reception in the middle of the MCSt does not utilize the COT initiated by a UE transmitting the MCSt, the COT corresponding to the MCSt will not be maintained even though there is a SL reception in PSFCH occasion in the middle of the MCSt

Proposal 9: RAN1 discusses following issues first to discuss whether/how to design PSFCH repetition on one or more RB sets to maintain a COT when the COT contains one or multiple RB sets and includes PSFCH occasions(s)

- Whether/how to design a single PSFCH transmission over multiple RB sets

- Whether/how the PSFCH transmit power is limited based on the EDT used to access the channel for MCSt

- Whether/how PSFCH TX is prioritized over PSFCH RX in the middle of MCSt regardless of their SL priorities

If RAN1 will design S-SSB repetition for the purpose of maintaining the COT, the above issues for S-SSB TX or RX also needs to be addressed first.

  - SL HARQ procedure

In Rel-16 PSSCH-to-PSFCH mapping, the implicit mapping rule is adopted rather than dynamic indication of PSFCH resources. It is motivated from the fact that if the dynamic indication is adopted, the PSSCH TX UE needs to perform sensing operation for PSFCH further. Since the SL HARQ-ACK enabling/disabling and SL-HARQ-ACK feedback option indicators are present in 2nd SCI, the TX UE would need to success to decode 2nd SCI of other UEs as well for sensing operation. If the TX UE does not perform such sensing operation for PSFCH, even though PSSCH resource collision does not occur, PSFCH resources can be collided.

It was discussed the possibility that the PSCCH/PSSCH TX UE indicates PSFCH resource to be inside the shared COT duration. However, since the existing PSSCH-to-PSFCH mapping already targets the case where the PSCCH/PSSCH RX UE transmits PSFCH as soon as possible upon PSCCH/PSSCH reception, it is still unclear the necessity of dynamic indication. If the dynamic indication targets to indicate PSFCH resource inside the COT duration in the future, it is also ambiguous since the future COT duration would not be always guaranteed. Moreover, delaying the SL HARQ-ACK feedback will have impact on the CWS adjustment which uses the SL HARQ-ACK status, and it will delay UE’s channel access procedure or make the CWS adjustment procedure inefficient due to the lack of SL HARQ-ACK information.

When only the COT initiator UE can disable PSFCH occasion within its own COT duration, it is possible that another UE can enables PSFCH occasion on different RB set(s) in the same time. It will cause AGC problem since the different slot structures are FDMed in the same SL BWP or SL carrier.

Observation 11: If the locations of PSFCH resources are dynamically indicated, UE needs to perform sensing operation for PSFCH further. Otherwise, even though PSSCH resources are not overlapping each other, PSFCH resource collision can occur. Moreover, the dynamic indication of PSFCH occasion will cause additional AGC problem.

Observation 12: Delaying SL HARQ-ACK feedback will affect the CWS adjustment procedure, and it will results on delaying UE’s channel access procedure and/or inefficient CWS adjustment due to the lack of SL HARQ-ACK information.

Observation 13: For dynamic indication of PSFCH resource to be inside the future COT duration, since the future COT duration is not always guaranteed, it is unclear whether it can mitigate PSFCH transmission dropping due to LBT failure.

If we consider multiple PSFCH transmission chances to handle PSFCH transmission dropping due to LBT failure, there could be two domains to be considered. To be specific, since the sensing results could be different across different RB sets, it can be considered that the UE attempts to access multiple RB sets to transmit a single PSFCH transmission. In this case, once the UE successes to access at least one RB set, then the UE can use the PSFCH resource within the RB set for PSFCH transmission. To avoid sensing operation for PSFCH, the implicit mapping rule can be used. For instance, PRB groups for PSFCH transmission is (pre)configured in each RB set, and Rel-16 PSSCH-to-PSFCH resource determination rule is applied to each RB set. If the PSFCH repetition for maintaining a COT is supported, this approach would be needed for the case when the COT consists of multiple RB sets.

Another domain is to increase the PSFCH time-domain occasions associated with the same PSSCH slot. To do this, it can be considered that UE is (pre)configured with more than one sl-MinTimeGapPSFCH and the difference between minimum timing values will be larger than or equal to the value of PSFCH resource period. In this case, for a given PSSCH slot, there could be more than one PSFCH time-domain occasions to be associated with the PSSCH slot. To avoid PSFCH resource collisions between different PSSCH-to-PSFCH timeline, PRB groups for PSFCH transmission is (pre)configured in each timeline, and Rel-16 PSSCH-to-PSFCH resource determination rule is applied to each timeline.

In the above approaches, it requires additional frequency resources for PSFCH transmissions. Therefore, this approach would be applicable when a single PRB is used for PSFCH transmission. On the other hand, if a number of PRBs are used for PSFCH transmission, it would not be possible to reserve frequency domain resources for PSFCH transmission further for this purposes. Alternatively, the dynamic indication of PSFCH resource set can be used together with multi-PRB PSFCH transmission. In this case, to mitigate PSFCH resource collision, the source ID and M_ID can be used to select the final PSFCH resource within the indicated PSFCH resource set.

Proposal 10: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, one or more of followings are supported:

- Option 1: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next RB set.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each RB set.

  - UE can use PSFCH resource in the RB set which UE successes to access.

- Option 2: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next PSFCH occasion.

  - For each min-PSSCH-to-PSFCH timing, UE is (pre)configured with different PRB groups for PSFCH resources.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each PSFCH occasion.

    - FFS: PSFCH occasions are present in the same slot or different slots.

  - UE can use PSFCH resource in the earliest PSFCH occasion which UE successes to access the channel.

Meanwhile, when we consider more than one PSFCH occasions per PSCCH/PSSCH transmission, it is necessary to investigate when and how to perform PSFCH TX/TX or TX/RX prioritization rule with respect to the PSFCH occasion to be used for actual PSFCH transmission. For instance, if PSFCH TX/TX prioritization is performed first, it would be possible that the prioritized PSFCH is dropped due to LBT failure while the UE can access the channel associated with the deprioritized PSFCH. In this case, UE may not transmit any PSFCH transmissions due to LBT failure and PSFCH TX/TX or TX/RX prioritization.

Moreover, it would be necessary to clarify whether the UE performs deprioritized PSFCH RX if the UE drops prioritized PSFCH TX due to LBT failure. Alternatively, if PSFCH TX/TX prioritization is performed after channel sensing operation for each PSFCH or each RB set, the processing time for preparing PSFCH transmissions may not be sufficient. To be specific, after channel sensing, the UE may have few usec, and the UE may need to regenerate OFDM waveform based on the selected PSFCH transmissions within the duration.

Another issue is that when a UE fails to transmit PSFCH on the 1st PSFCH occasion due to LBT failure, it is also possible that the PSFCH on the 2nd PSFCH occasion is dropped again due to PSFCH TX/TX or TX/RX prioritization. In this case, it would be better to use additional PSFCH occasion(s) even for the PSFCH dropping due to prioritization rules. Or, it can be considered to prioritize the dropped PSFCH when the UE tries to transmit the PSFCH in the next PSFCH occasion.

According to agreement on UE-to-UE COT sharing, when performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. In this case, if the PSFCH transmission to the COT initiating UE is dropped, the UE may not use the COT for PSFCH transmission(s). In this case, it can be considered that the PSFCH transmission to the COT initiating UE is prioritized over other PSFCH transmission(s).

Proposal 11: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, RAN1 discusses when and how to perform PSFCH TX/TX or TX/RX prioritization.

- e.g., whether PSFCH TX/TX or TX/RX prioritization is performed before or after channel sensing operation.

- e.g., whether a UE can transmit PSFCH on additional PSFCH occasion even if PSFCH on a PSFCH occasion is dropped due to PSFCH TX/TX or TX/RX prioritization.

- e.g., whether PSFCH transmission to the COT initiator UE is prioritized over other PSFCH transmission(s) or not.

For OCB requirement and PSD requirement, a number of options are listed up to be studied while it is agreed to support at least RB-based interlace PSFCH transmission as follows:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

According to PSD requirement, as the number of PRBs for PSFCH transmission within 1 MHz increases, the maximum transmit power for PSFCH will be degraded. When NR-U interlace is reused for SL-U, in Alt 1-1a, the transmit power will be always halved if the common PRB and dedicated PRB are not overlapping as shown in Figure 5. If the PSD requirement is 10dBm/MHz, then the maximum transmit power for actual SL HARQ-ACK feedback will be 7 dBm. Moreover, considering that the UE can transmit multiple PSFCHs to different UEs and different dedicated PRBs can be within 1MHz, the transmit power for the actual SL HARQ-ACK feedback can be further reduced. Even if the UE transmits a single PSFCH transmission, the total power will be split into common PRBs and dedicated PRB while only dedicated PRB is used to convey actual information. Moreover, since a number of UE can transmit signals on the common PRBs in SFN manner, it will make IBE issue worsened. On the maximum number of simultaneous PSFCH transmission, since the final form of the PSFCH transmission is somewhat different from the interlace form, it is unclear how the maximum number of PSFCH transmissions will be changed. This change needs to be checked by RAN4.

Meanwhile, Alt 3-2a will avoid the case where the transmit power is split between common PRB and dedicated PRB. However, since Alt 3-2a also use small number of dedicated PRB for SL HARQ-ACK feedback, this transmit power limitation problem due to different dedicated PRBs within 1MHz will occur as well. To bypass this problem, it can be considered to use sufficient number of dedicated PRB is used for PSFCH transmission. On the maximum number of simultaneous PSFCH transmission, it can be seen as that the UE transmit multiple single-PRB PSFCH transmission. For instance, if the UE transmits N PSFCH transmissions, it would be equivalent to K2+K4*N single-PRB PSFCH transmissions.

For Alt 1-1a and Alt 3-2a, it seem that if UE drops all the dedicated PRBs due to the limit on the number of PSFCH transmission, the UE does not need to transmit common PRB or interlaces as well.

Figure 5: Example of PSFCH transmission for OCB/PSD requirement.

Observation 14: Due to PSD requirement and simultaneous PSFCH transmissions within 1MHz, Alt 1-1a, and Alt 3-2a can suffer from the transmit power limitation problem. It will degrade the detection performance of SL HARQ-ACK feedback.

Observation 15: For PSFCH transmission to meet OCB requirement, the usage of common PRBs or common interlace (Alt 1-1a, Alt 3-2a) has the limited PSFCH coverage issue, and IBE issue.

On the other hand, Alt 2-2a, Alt 2-3a, and Alt 2-4a use a number of dedicated PRBs for SL HARQ-ACK feedback, so the transmit power limitation problem will be mitigated. On the other hand, since the number of dedicated PRBs for a single PSFCH transmission would be highly increased, it would be necessary to check whether the number of PSFCH resources are sufficient. The total number of RB-based interlace PSFCH resources will be determined based on the number of RB sets within a resource pool, the number of interlaces, the number of cyclic shift pairs. The required number of PSFCH resources will be determined by the number of subchannels within a resource pool, PSFCH resource period. If the number of cyclic shift pairs is larger than PSFCH period, the multiplexing capacity will be sufficient. If the number of cyclic shift pairs is reduced due to tightened delay spared requirement, it can be considered that more than one interlaces are belonging to a subchannel. If it is allowed to support RB-based interlace structure other than NR-U interlace, it can be considered that UE transmits at least two dedicated PRBs whose gap is large enough to cover 80% of the channel to meet the OCB requirement.

Observation 16: RB-based interlace PSFCH transmission (Alt 2-2a, Alt 2-3a, Alt 2-4a) may or may not have multiplexing capacity issue depending on subchannel size, PSFCH resource period, the number of cyclic shift pairs, and the number of PRBs belonging to a single RB-based interlace.

When the PSFCH resource is associated with RB set index and interlaced index, it would be necessary to modify the implicit PSSCH-to-PSFCH resource determination rule as well. According to Rel-16 NR SL, subchannel index and slot index of PSSCH are used to determine RB sub-group of PSFCH, and source ID of PSSCH is used to determine RB index within the RB sub-group and cyclic shift pair index. For interlaced PSFCH transmission, it would be necessary to define how RB set index and interlace index of PSFCH are derived from PSSCH resource and source ID. Due to limited number of frequency resources of the interlaced RB-based PSFCH transmission, subgrouping would need to be extended to code-domain resources. For instance, the UE can determines the subset of interlaces and RB sets and cyclic shifts pairs based on subchannel, RB set, and slot associated with PSSCH transmission. Then, the UE finally selects PSFCH resource within the subset of PSFCH resource candidates based on source ID and/or M_ID. For the groupcast HARQ-ACK feedback Option 2, the PSFCH resource groups associated with the allocated subchannels and RB sets could be used like Rel-16/17 NR SL PSFCH resource determination rule.

Proposal 12: For interlaced RB-based PSFCH transmission with 15kHz SCS or 30kHz SCS,

- Interlaced PUCCH format 0 is considered as a baseline (Alt 2-4a).

- For PSSCH-to-PSFCH determination rule,

  - Among (pre)configured set of interlaces, RB sets, and cyclic shift pairs for PSFCH transmission, PSFCH resource indexing is done in increasing order of first interlace index, and then RB set index, and then cyclic shift pair index.

  - Set of PSFCH resources is partitioned by the RB set of PSSCH, and then is partitioned by the subchannel of PSSCH, and then is partitioned by PSSCH slot.

  - Within the subset of PSFCH resources, the UE selects final PSFCH resource index based on source ID and/or M_ID.

In this approach, unlike Rel-16 NR SL, it will allows CDM among PSFCH resources associated with different PSSCH transmissions of which resources are TDMed and/or FDMed. If the subchannel size of 5 interlaces for 30 kHz SCS, the existing Rel-16 NR SL PSFCH determination rule could be reused for the PSFCH resource period greater than 1 logical slot.

  - Synchronization procedure and channels

In our view, the typical scenario of S-SSB transmission is to use Type 2A channel access procedure rather than Type 1 channel access procedure with a help of SCSt exemption. In this case, the additional candidate S-SSB occasions could be simply achieved by modifying the description of the higher layer parameter sl-NumSSB-WithinPeriod to allow more S-SSB occasions for each SCS or by adding more candidate values for the higher layer parameter sl-NumSSB-WithinPeriod. In this approach, the specification impact or workload could be minimized. Otherwise, to design new S-SSB occasions, it would be necessary to further discuss how to modify sl-SSB-TimeAllocation1, sl-SSB-TimeAllocation2, sl-SSB-TimeAllocation3, and UE behavior for the usage of additional S-SSB occasions.

Proposal 13: For additional candidate S-SSB occasions, support Option 1

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

In RAN1#111 meeting, it was discussed how a UE uses additional S-SSB occasions for S-SSB transmission. Unlike NR-U discovery burst, S-SSB is transmitted by a number of UEs in SFN manner. It is motivated by the fact that the synchronization is important even for UEs who are not actually communicated each other to identify and avoid reserved resources of other UEs. Moreover, the periodicity of DL discovery burst could be shorter than 160msec, but the periodicity of S-SSB is fixed to 160msec. In those points of views, it is necessary to maximize the case when a number of UEs transmit S-SSB on the same occasion in SFN manner. If the usage of additional S-SSB occasions are different depending on the channel sensing results or other factor, it will cause the case where small number of UEs will transmit S-SSB on the same S-SSB occasion, and it will reduce the coverage of S-SSB as well. Extremely, it would be possible that no one transmit S-SSB on a certain S-SSB occasion while RX UEs still expect to receive S-SSB. It could increase false alarm probability as well.

Proposal 14: Regarding additional candidate S-SSB occasions:

- In the same S-SSB period,

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

Depending on the regulation, S-SSB transmission also needs to meet the OCB requirement and PSD requirement. For this purpose, a number of options are listed up in RAN1#111 meeting.

On the time domain resources for S-SSB, unlike NR-U, S-SSB transmission is performed by a UE of which TX power would be limited. In Rel-16 NR SL, 2-symbol duration of S-PSS and S-SSS could achieve energy combining gain. In that point of view, at least for S-PSS and S-SSS needs to have 2 symbol duration as in Rel-16 NR SL. In a similar manner, it seems not preferable to transmit S-PSS/S-SSS and PSBCH in FDM manner since the transmit power will be split into S-PSS/S-SSS and PSBCH.

Observation 17: Considering coverage of SL communication, the S-SSB structure in time domain (i.e., 2-symbol S-PSS and 2-symbol S-SSS) needs to be kept to achieve energy combining gain.

Proposal 15: For S-SSB in SL-U, FDM between S-PSS/S-SSS and PSBCH is not supported.

According to EN 301 893 [3], within a COT duration, the UE may not need to fulfil the OCB requirement temporarily as follows:

- During a Channel Occupancy Time (COT), equipment may operate temporarily with an occupied bandwidth of less than 80 % of its nominal channel bandwidth. The occupied bandwidth shall not be less than 2 MHz.

Whether the S-SSB transmission is inside or outside the COT will be different across different UEs. In this case, if some UEs use S-SSB structure with temporary OCB exemption while other UEs use S-SSB structure for OCB requirement, the SFN transmission will not be supported, and it will cause resource collision between S-SSB with different structure.

Observation 18: If the temporary exemption of OCB requirement is applied, it is necessary to consider whether/how the S-SSB transmission with different structure can be transmitted in SFN manner. Otherwise, S-SSB transmission inside a COT and S-SSB transmission outside a COT will be collided.

Proposal 16: Temporary exemption of OCB requirement for S-SSB is not supported in Rel-18 SL.

In case of interlaced RB-based S-SSB transmission, it would be possible to reuse the existing sequences if the number of PRBs belonging to one interlace within a RB set is 11. Otherwise, it would be necessary to use two interlaces for S-SSB transmission. If two interlaces within a RB set is used for S-SSB transmission, due to PSD requirement, the TX power would be limited compared to the case when one interlace is used.

Observation 19: For interlaced RB-based transmission for S-SSB, depending on the number of PRBs associated with a single interlace, it would be necessary to use two interlaces for S-SSB transmission to ensure bandwidth of 11 PRBs.

In NR-U, the interlaced structure is defined for 15 kHz SCS and 30 kHz SCS, but not for 60 kHz SCS. Considering the discussion on the necessity of the design based on the OCB requirement for 60 kHz SCS, it would not be necessary to design S-SSB transmission with OCB requirement for 60 kHz SCS as well.

Observation 20: For 60 kHz SCS, S-SSB transmission design for the OCB requirement does not need to be discussed in RAN1.

For the OCB and PSD requirement, repetition of S-SSB in frequency domain also discussed in RAN1#111 meeting. However, in this case, PARP could be highly increased. To alleviate this high PAPR problem, phase adjustment is applied to S-SSB repetitions. Moreover, since S-PSS and S-SSS has guard band within 11 PRB, the repetition in frequency domain can be seen as non-contiguous SL transmissions. In this case, the transmission BW and PSD will be quite different across different symbols, it also needs to consider the transient period issue. To mitigate the transient period issue, it can be considered to remove guard REs between S-PSS/S-SSS repetitions or to use guard REs for S-SSB transmission to make contiguous RB transmission. For instance, UE can use a certain sequence that can reduce PAPR or phase discontinuity between S-SSB repetitions.

Observation 21: Repetition of S-SSB in frequency domain would need to consider PAPR issue and the transient period issue due to guard REs between S-SSB repetitions (especially for S-PSS/S-SSS).

Considering that the S-SSB transmission will not be FDMed with other SL channel(s), it seems that the RB-based interlace structure for S-SSB transmission does not need to be the same as that of PSCCH/PSSCH, or PSFCH transmission. For unified design across different SCS, it can be considered that R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs which are equally distributed over a RB set. We compares misdetection probability of S-PSS and S-SSS between R16 mapping rule and the proposed mapping rule as shown in Figure 11. According to the evaluation results shown in Figure 6, for the all the SCS, the proposed scheme achieves the comparable detection performance compared to R16 S-SSB. Moreover, we observed that the PAPR/CM of the proposed scheme does not affect the S-SSB detection performance and S-SSB coverage. There was a comment that the total noise power will increase since the interlaced structure occupies wideband (including unused PRBs), but it is not correct. The total noise power is depending on the BW for actual transmission, and the actual BW for interlaced structure will be 11 PRB.

(a) 15 kHz SCS

(b) 30 kHz SCS

Figure 6: Detection performance of S-PSS and S-SSS.

Proposal 17: For S-SSB in SL-U, support Option 1 to meet OCB and PSD requirement for S-SSB transmission:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH. For details, down-select one of followings:

  - Option 1-1a:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs associated with one or two interlaces within a RB set.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-1b:

    - R16 S-PSS/S-SSS/PSBCH are mapped on PRBs associated with one interlace within a RB set.

      - If the number of PRBs associated with the interlace within the RB set is smaller than 11 PRBs, parts of sequences can be truncated.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-1c:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs equally distributed over a RB set.

      - PRB gap is implicitly or explicitly determined.

    - A (pre)configuration provides the RB set(s) and/or PRB gap for S-SSB mapping.

- Conclusions

In this contribution, we discussed physical channel design framework for sidelink on unlicensed spectrum. Based on the above discussion, our observations and proposals are given as follows:

Observation 1: For PSCCH mapping, the meaning of the lowest sub-channel of the lowest RB set needs to be carefully clarified.

Observation 2: For PSCCH mapping, if the lowest sub-channel is the sub-channel with the lowest location, since different PSSCH allocations can have the same PSCCH location, there could be ambiguity on the PSSCH allocation even though the RX UE knows the location of PSCCH and the number of sub-channels.

Observation 3: For PSCCH mapping, if the lowest sub-channel is the sub-channel with the lowest index, different PSSCH allocation will have different PSCCH location, so the ambiguity problem will be resolved.

Observation 4: Starting symbol candidates intended for AGC purpose does not imply that REs on other symbol will be duplicated in the AGC symbol.

Observation 5: The 2nd starting symbol of PSCCH/PSSCH is not overlapping with PSCCH symbol duration for the 1st starting symbol of PSCCH/PSSCH to guarantee PSCCH detection performance.

Observation 6: It is necessary to carefully investigate whether or how the 2nd starting symbol of PSCCH/PSSCH is overlapping with the time location(s) for the 2nd SCI mapping for the 1st starting symbol of PSCCH/PSSCH.

Observation 7: Considering a certain code block(s) can be consistently dropped, it can be considered that the 2nd starting symbol of PSCCH/PSSCH is set to PSSCH DMRS symbol index of a certain DMRS pattern for PSCCH/PSSCH with the 1st starting symbol.

Observation 8: Considering the S-SSB period and the expected maximum number of S-SSB occasions within a period, the time gap between (contiguous) S-SSB occasions (groups) is much larger than the maximum COT duration even though it is assumed that the S-SSB occasions are evenly distributed over the period.

Observation 9: If the transmit power of PSFCH transmission in the middle of the MCSt is greater than the reference power associated with the EDT used to access the channel for MCSs, the COT corresponding to the MCSt will not be maintained even though there is a SL transmission in PSFCH occasion in the middle of the MCSt

Observation 10: If the PSFCH reception in the middle of the MCSt does not utilize the COT initiated by a UE transmitting the MCSt, the COT corresponding to the MCSt will not be maintained even though there is a SL reception in PSFCH occasion in the middle of the MCSt

Observation 11: If the locations of PSFCH resources are dynamically indicated, UE needs to perform sensing operation for PSFCH further. Otherwise, even though PSSCH resources are not overlapping each other, PSFCH resource collision can occur. Moreover, the dynamic indication of PSFCH occasion will cause additional AGC problem.

Observation 12: Delaying SL HARQ-ACK feedback will affect the CWS adjustment procedure, and it will results on delaying UE’s channel access procedure and/or inefficient CWS adjustment due to the lack of SL HARQ-ACK information.

Observation 13: For dynamic indication of PSFCH resource to be inside the future COT duration, since the future COT duration is not always guaranteed, it is unclear whether it can mitigate PSFCH transmission dropping due to LBT failure.

Observation 14: Due to PSD requirement and simultaneous PSFCH transmissions within 1MHz, Alt 1-1a, and Alt 3-2a can suffer from the transmit power limitation problem. It will degrade the detection performance of SL HARQ-ACK feedback.

Observation 15: For PSFCH transmission to meet OCB requirement, the usage of common PRBs or common interlace (Alt 1-1a, Alt 3-2a) has the limited PSFCH coverage issue, and IBE issue.

Observation 16: RB-based interlace PSFCH transmission (Alt 2-2a, Alt 2-3a, Alt 2-4a) may or may not have multiplexing capacity issue depending on subchannel size, PSFCH resource period, the number of cyclic shift pairs, and the number of PRBs belonging to a single RB-based interlace.

Observation 17: Considering coverage of SL communication, the S-SSB structure in time domain (i.e., 2-symbol S-PSS and 2-symbol S-SSS) needs to be kept to achieve energy combining gain.

Observation 18: If the temporary exemption of OCB requirement is applied, it is necessary to consider whether/how the S-SSB transmission with different structure can be transmitted in SFN manner. Otherwise, S-SSB transmission inside a COT and S-SSB transmission outside a COT will be collided.

Observation 19: For interlaced RB-based transmission for S-SSB, depending on the number of PRBs associated with a single interlace, it would be necessary to use two interlaces for S-SSB transmission to ensure bandwidth of 11 PRBs.

Observation 20: For 60 kHz SCS, S-SSB transmission design for the OCB requirement does not need to be discussed in RAN1.

Observation 21: Repetition of S-SSB in frequency domain would need to consider PAPR issue and the transient period issue due to guard REs between S-SSB repetitions (especially for S-PSS/S-SSS).

Proposal 1: For SL communication on shared spectrum, a resource pool excludes following slots:

- Slots whose symbols from sl-StartSymbol to sl-StartSymbol + sl-LengthSymbols – 1 are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration if provided or sl-TDD-Config of the received PSBCH if provided

- S-SSB slots (including additional S-SSB slots)

- Reserved slot as specified in section 8 of TS 38.214

Proposal 2: For interlaced RB based PSSCH transmission,

- Support explicitly indicating the allocated sub-channel index(s) and RB set index(s)

  - SCI indicates FRIV for subchannel allocation and FRIV for RB set allocation.

  - Subchannel indexing is done in increasing order of the interlace index.

  - PSSCH transmission resource(s) are determined by the intersection of indicated sub-channel(s) and indicated RB set(s).

Proposal 3: For PSCCH mapping for interlaced PSSCH transmission, RAN1 clarify that the lowest sub-channel implies the sub-channel with the lowest index.

Proposal 4: For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- TBS is determined based on a reference number of PRBs of a sub-channel

  - The reference number of PRBs is 10.

Proposal 5: For multiple starting symbols within a slot for a PSCCH/PSSCH transmission, the reference number of symbols for TBS determination is determined by sl-LengthSymbols

- No specification change is supported for TBS determination for the purpose of additional starting position in a slot.

Proposal 6: UE monitors PSCCHs on two starting symbols within a slot where 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission is supported regardless of detection of PSCCH on the 1st starting symbol.

Proposal 7: For multiple starting symbols within a slot for a PSCCH/PSSCH transmission, the 2nd starting symbol is selected so that overlapping with at least PSCCH duration for the 1st starting symbol is avoided.

Proposal 8: RAN1 deprioritize the design of S-SSB repetition in one or more RB sets to maintain a COT when the COT contains one or multiple RB sets and includes S-SSB slot(s)

Proposal 9: RAN1 discusses following issues first to discuss whether/how to design PSFCH repetition on one or more RB sets to maintain a COT when the COT contains one or multiple RB sets and includes PSFCH occasions(s)

- Whether/how to design a single PSFCH transmission over multiple RB sets

- Whether/how the PSFCH transmit power is limited based on the EDT used to access the channel for MCSt

- Whether/how PSFCH TX is prioritized over PSFCH RX in the middle of MCSt regardless of their SL priorities

Proposal 10: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, one or more of followings are supported:

- Option 1: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next RB set.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each RB set.

  - UE can use PSFCH resource in the RB set which UE successes to access.

- Option 2: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next PSFCH occasion.

  - For each min-PSSCH-to-PSFCH timing, UE is (pre)configured with different PRB groups for PSFCH resources.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each PSFCH occasion.

    - FFS: PSFCH occasions are present in the same slot or different slots.

  - UE can use PSFCH resource in the earliest PSFCH occasion which UE successes to access the channel.

Proposal 11: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, RAN1 discusses when and how to perform PSFCH TX/TX or TX/RX prioritization.

- e.g., whether PSFCH TX/TX or TX/RX prioritization is performed before or after channel sensing operation.

- e.g., whether a UE can transmit PSFCH on additional PSFCH occasion even if PSFCH on a PSFCH occasion is dropped due to PSFCH TX/TX or TX/RX prioritization.

- e.g., whether PSFCH transmission to the COT initiator UE is prioritized over other PSFCH transmission(s) or not.

Proposal 12: For interlaced RB-based PSFCH transmission with 15kHz SCS or 30kHz SCS,

- Interlaced PUCCH format 0 is considered as a baseline (Alt 2-4a).

- For PSSCH-to-PSFCH determination rule,

  - Among (pre)configured set of interlaces, RB sets, and cyclic shift pairs for PSFCH transmission, PSFCH resource indexing is done in increasing order of first interlace index, and then RB set index, and then cyclic shift pair index.

  - Set of PSFCH resources is partitioned by the RB set of PSSCH, and then is partitioned by the subchannel of PSSCH, and then is partitioned by PSSCH slot.

  - Within the subset of PSFCH resources, the UE selects final PSFCH resource index based on source ID and/or M_ID.

Proposal 13: For additional candidate S-SSB occasions, support Option 1

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

Proposal 14: Regarding additional candidate S-SSB occasions:

- In the same S-SSB period,

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

Proposal 15: For S-SSB in SL-U, FDM between S-PSS/S-SSS and PSBCH is not supported.

Proposal 16: Temporary exemption of OCB requirement for S-SSB is not supported in Rel-18 SL.

Proposal 17: For S-SSB in SL-U, support Option 1 to meet OCB and PSD requirement for S-SSB transmission:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH. For details, down-select one of followings:

  - Option 1-1a:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs associated with one or two interlaces within a RB set.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-1b:

    - R16 S-PSS/S-SSS/PSBCH are mapped on PRBs associated with one interlace within a RB set.

      - If the number of PRBs associated with the interlace within the RB set is smaller than 11 PRBs, parts of sequences can be truncated.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-1c:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs equally distributed over a RB set.

      - PRB gap is implicitly or explicitly determined.

    - A (pre)configuration provides the RB set(s) and/or PRB gap for S-SSB mapping.

Reference

[1] RAN1 Chair’s Notes, RAN1#112.

[2] 3GPP TS 37.213, “Physical layer procedures for shared spectrum channel access”.

[3] ETSI EN 301 893, “5 GHz WAS/RLAN; Harmonised Standard for access to radio spectrum”.