3GPP TSG RAN WG1 Meeting #112bis-e  	          	     		R1-2302926

April 17 – 26, 2023

_____________________________________________________________________Agenda item: 9.5.1.1

Source: LG Electronics

Title: 	Discussion on SL positioning reference signal

Document for: Discussion and decision

- Introduction

The following agreements on SL positioning reference signal were made in RAN1#112 meeting.

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095 Agreement A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier. Agreement Support SCS values for SL PRS include: - 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2 - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities. Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering. Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

In this contribution, we discuss the solutions for SL PRS design, the structure of SL positioning slot, and SL PRS transmission power control.

- SL PRS

For the sequence of a new reference signal for SL positioning, there was an agreement in RAN1#112 meeting that DL PRS sequence generation based on Gold sequence is reused for SL PRS sequence generation. There was one FFS point regarding the SL PRS sequence ID for SL PRS sequence initialization, as follows.

Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined.

In the legacy DL positioning, DL PRS sequence ID is configured by the network. In SL positioning, it may be impossible that SL PRS sequence ID is configured by the network especially when UE is out of the network coverage.

Considering the purpose of providing the unique and orthogonal property of SL PRS sequence through SL PRS sequence ID, an alternative way of constructing SL PRS sequence ID can be used. For uniqueness, an ID associated to UE transmitting SL PRS can be involved in SL PRS sequence ID. For orthogonalization, CRC field of SCI for example can be used as such in SL DMRS sequence generation.

For ID associated to UE for SL PRS sequence generation, a layer-2 source ID can be used in SL positioning. It provides the identification of the UE transmitting SL PRS per each unicast link. For CRC field of SCI, there are two candidates of the 1st-stage and the 2nd-stage SCI. If the SL PRS resource information is carried by the 2nd-stage SCI, it would be reasonable to use the CRC field of the 2nd-stage SCI. In conventional 2nd-stage SCI, the layer-2 source and destination ID are carried. Therefore, if the CRC field of the 2nd-stage SCI is used for SL PRS sequence generation, the layer-2 source ID of UE is implicitly included in generation of SL PRS sequence ID.

Proposal 1: SL PRS sequence ID used for SL PRS sequence initialization is determined based on the CRC field of the 2nd SCI associated with SL PRS transmission.

In RAN1#112 meeting, there were agreements about the supported SL PRS comb size and the combination of the number of symbols M and the comb size N of the comb pattern, as follows.

Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering.

As for the dedicated resource pool, there was a discussion whether the comb size {1} needs to be supported or not. In our opinion, the comb size {1} is beneficial for a short latency SL positioning service while maintaining the maximum transmission path resolution for positioning accuracy. In addition, it is already supported in a shared resource pool. Therefore, we think the comb size {1} should also be supported in a dedicated resource pool.

Considering there is no backward compatibility issue in a dedicated resource pool, we don’t support to transmit PSSCH DMRS except the PSSCH symbols that carries the SCI associated to SL PRS. Therefore all the symbols except PSCCH and PSSCH can be used for SL PRS transmission symbols.

Proposal 2: In a dedicated resource pool, no DMRS is transmitted in a symbol except in PSCCH and PSSCH symbols, in a slot where SL PRS is transmitted.

In a dedicated resource pool, the number of symbols of PSCCH for SL positioning can be less than that of PSCCH for the legacy SL communication. In SL communication, 2 or 3 symbols can be configured for PSCCH. However, since the bandwidth of the dedicated resource pool is relatively large, only a single symbol can be possible for PSCCH in SL positioning. If there are 2 PSCCH symbols, excluding AGC and TX/RX turnaround symbol, 10 symbols are available for SL PRS transmission. As a consequence, we support the comb size {10} in a dedicated resource pool.

As the maximum comb size equals {10}, we also support the comb size {8} in addition to other smaller number of {2, 4, 6} in a dedicated resource pool.

Proposal 3: For a dedicated resource pool, the comb sizes N={1, 8, 10} are additionally supported.

Following the agreement in RAN1#112 meeting, we support all the fully staggered patterns corresponding to the supported comb sizes. We therefore support the fully staggered patterns for comb sizes={1, 6, 8, 10} in a dedicated resource pool.

Proposal 4: For a dedicated resource pool, the fully staggered patterns (M,N)=(1,1), (6,6,), (8,8), (10,10) are additionally supported.

There were discussions whether there needs to be a constraint on the maximum effective comb size when the number of SL PRS symbols M is less than the comb size N. In our opinion, there is no special needs on such restriction considering that the distance between UEs for SL positioning is relatively shorter than that for Uu link positioning. Depending on the use case and the requirement, a smaller number of SL PRS symbols should be supported for a short latency SL positioning service.

Regarding the number of symbols M for the comb size N in a partially staggered pattern, any even number can be used for simplicity of the comb RE offset rule.

Proposal 5: For a dedicated resource pool, there is no constraints on the maximum effective comb size for partially staggered pattern. Any even numbers M less than N are supported, including:

- (M,6) with M={2,4}

- (M,8) with M={2,4,6}

- (M,10) with M={2,4,6,8}

In a shared resource pool, for the legacy SL communication UE to be able to perform sensing without any restriction on DMRS for RSRP measurement, it’s desirable that PSSCH DMRS be transmitted in a slot where SL PRS is transmitted, as configured in a shared resource pool. According to the number of PSSCH DMRS symbols per configuration, the number of symbols available for SL PRS transmission is listed in Table 1. As shown in the table, the number of symbols available for SL PRS transmission except PSCCH, PSSCH and DMRS symbols vary from 3 to 8. Therefore, we support the comb size {6, 8} in a shared resource pool.

Table 1 Number of SL symbols available for SL PRS transmission

| Number of SL symbols | 2 PSCCH symbols |  |  | 3 PSCCH symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Number of PSSCH DMRS symbols |  |  | Number of PSSCH DMRS symbols |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 6 | 3 |  |  | 1 |  |  |
| 7 | 4 |  |  | 2 |  |  |
| 8 | 5 |  |  | 3 |  |  |
| 9 | 4 | 4 |  | 3 | 3 |  |
| 10 | 5 | 5 |  | 4 | 4 |  |
| 11 | 6 | 6 | 5 | 5 | 5 | 4 |
| 12 | 7 | 7 | 6 | 6 | 6 | 5 |
| 13 | 8 | 8 | 7 | 7 | 7 | 6 |

Proposal 6: For a shared resource pool, the comb sizes N={6, 8} are additionally supported.

Similar to the case in a dedicated resource pool, the fully staggered comb patterns corresponding to all the comb sizes are supported in a shared resource pool.

Proposal 7: For a shared resource pool, the fully staggered patterns (M,N)=(1,1), (6,6), (8,8) are additionally supported.

Similar to the case in a dedicated resource pool, there is no special needs on the restriction on the maximum effective comb size in a partially staggered comb pattern in a shared resource pool. Regarding the number of symbols M for the comb size N in a partially staggered pattern, any even number can be used for simplicity of the comb RE offset rule.

Proposal 8: For a shared resource pool, there is no constraints on the maximum effective comb size for partially staggered pattern. Any even numbers M less than N are supported, including:

- (M,6) with M={2,4}

- (M,8) with M={2,4,6}

There was FFS point regarding the comb RE offset sequences of SL PRS. At least for the fully staggered patterns, the comb RE offsets defined for DL PRS and UL SRS can be reused for the comb size={2,4,6} and {8} respectively. Regarding the comb size={10}, a similar rule used for DL PRS and UL SRS can be used for defining the comb RE offsets for the fully staggered patterns. We propose the comb RE offset according to the comb size for the fully staggered pattern, as shown in Table 2.

Table 2 Comb RE offset for the fully staggered patterns

| Comb size | Symbol number within the sidelink PRS resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 2 | 0 | 1 |  |  |  |  |  |  |  |  |
| 4 | 0 | 2 | 1 | 3 |  |  |  |  |  |  |
| 6 | 0 | 3 | 1 | 4 | 2 | 5 |  |  |  |  |
| 8 | 0 | 4 | 2 | 6 | 1 | 5 | 3 | 7 |  |  |
| 10 | 0 | 5 | 2 | 7 | 3 | 8 | 1 | 6 | 4 | 9 |

For the fully staggered comb pattern,

- if the number of symbols of the fully staggered SL PRS is less than the number of symbols available in a slot when TDM of SL PRS resources are not allowed in a resource pool, or

- if the number of symbols of the fully staggered SL PRS is less than the number of symbols available in a TDM duration,

the remaining symbols in a slot or in a TDM duration not used for the fully staggered SL PRS can be filled with the repeated SL PRS symbols by (pre-)configuration.

In the case above, the comb RE offsets of the repeated SL PRS symbols follow the same comb RE offset of the original fully staggered pattern, as shown in Table 2. For example, if there are 10 available symbols for SL PRS transmission in a slot, and the fully staggered comb pattern with comb size=4 is (pre-)configured, the resultant comb RE offsets with repetition in a slot will be {0,2,1,3, 0,2,1,3, 0,2}.

Proposal 9: For the fully staggered comb pattern, it is supported that SL PRS symbols can be repeated to fill a slot or a TDM duration, with the same order of the comb RE offset of the original SL PRS symbols.

For the partially staggered comb pattern,

- if the number of symbols of the partially staggered SL PRS is less than the number of symbols available in a slot when TDM of SL PRS resources are not allowed in a resource pool, or

- if the number of symbols of the partially staggered SL PRS is less than the number of symbols available in a TDM duration,

the remaining symbols in a slot or in a TDM duration not used for the partially staggered SL PRS can be filled with the repeated SL PRS symbols by (pre-)configuration.

In the case above, there are two ways of repetition. In the first way, the partially staggered SL PRS symbols are simply repeated. This way has a benefit of the improved SL PRS reception due to SINR gain through combining. In the second way, the partially staggered SL PRS symbols can be expanded toward the fully staggered SL PRS, according to the comb RE offset order of the fully staggered SL PRS. If there are still remaining symbols after expansion, the same repetition rule in the fully staggered SL PRS case can be applied. This way has a benefit of the improved positioning accuracy because of the enhanced reception path resolution due to the expanded staggered pattern.

For example, if there are 10 available symbols for SL PRS transmission in a slot, and the partially staggered comb pattern with (M,N)=(4,8) is (pre-)configured, the resultant comb RE offsets if the first way is {0,4,2,6, 0,4,2,6, 0,4}. The second way results in the repeated SL PRS symbols with RE offsets {0,4,2,6, 1,5,3,7, 0,4} We think that both ways have their own benefits and can be supported by (pre-)configuration.

Proposal 10: For the partially staggered comb pattern, it is supported that SL PRS symbols can be repeated to fill a slot or a TDM duration, with the following ways.

- Opt 1. Simple repetition of the partially staggered SL PRS

- Opt 2. Expansion toward the fully staggered SL PRS and its repetition

- SL PRS slot structure

During the SI phase, there were discussions on the definition of SL PRS resource. Our understanding on the SL PRS resource is similar to DL PRS resource defined for DL positioning. Based on the DL PRS resource defined in the conventional DL positioning method, we can define SL PRS resource as a set of resources for a single sample or instance of measurement. That is, the SL PRS resource is defined according to the SL PRS comb size, SL PRS RE offset, and the number of symbols for SL PRS. This is the single instance as a minimum resource unit for SL positioning measurement. The SL PRS resources can be repeated or multiplexed within a slot.

Proposal 11: SL PRS resource is defined as a set of SL resources, which is used for the measurement of a single sample/instance of SL PRS.

In SL communication, the basic unit of SL transmission and reception is a slot (except PSFCH within a slot). A slot can be used for either transmission or reception according to the sensing results in SL mode-2 or scheduling in SL mode-1. Therefore, there is always one AGC symbol at the start of a SL slot, so that RX-UE can adapt its AGC gain before receiving PSCCH/PSSCH of the slot. Also there is always one TX/RX turnaround symbol at the end of a slot, so that UE can prepare its RF module functionality for transmission or reception.

Similarly in SL positioning, the basic unit of SL transmission and reception is SL PRS resource. A SL PRS resource can be used for either transmission or reception according to the sensing results in scheme-2 or scheduling in shceme-1. With the similar reason above, there should always be AGC time duration before the start of SL PRS resource. In our opinion, it is sufficient to have one AGC symbol for AGC time duration in SL positioning. One AGC symbol is also compatible for SL communication slot, considering SL PRS transmission in a shared resource pool.

With similar reason, there should always be TX/RX turnaround time duration after the end of SL PRS resource. In our opinion, it is sufficient to have one TX/RX turnaround symbol for RF module switching in SL positioning. One TX/RX turnaround symbol is also compatible for SL communication slot, considering SL PRS transmission in a shared resource pool. The resultant ‘package’ of SL PRS resource is shown in Figure 1.

Figure 1. SL PRS resource and AGC & TX/RX turnaround symbol

Proposal 12: One AGC symbol is located before every SL PRS resource.

Proposal 13: One TX/RX turnaround symbol is located after every SL PRS resource.

One issue discussed in SI phase is how to fill the AGC symbol before every SL PRS resource. In SL communication, the very first symbol of PSCCH is copied as AGC symbol. In SL positioning, a smarter way of constructing AGC symbol may help to improve the SL positioning performance.

First, if an advance UE has an RF module that enables AGC time less than one symbol duration, some part of AGC symbol can be used for SL positioning as SL PRS function. Second, in the situation above, a copy of one of the SL PRS symbol is helpful for phase tracking, especially when there is no repetition of SL PRS within a slot.

To meet both goals above, we can think of two possibilities. In a first way, the copy of the last symbol of SL PRS can be used for AGC symbol. Definitely it repeats the comb pattern of the last SL PRS symbol, which should be helpful for phase tracking purpose. In a second way, the ‘cyclically-rotated’ comb pattern can be used for AGC symbol. For example, if a comb size=4 and a partially-staggered SL PRS with 2 symbols is used, the RE offsets of the symbols in order are {0, 2}. The RE offsets of a fully-staggered pattern would be {0, 2, 1, 3}. In this case, AGC symbol is made up of the last RE offset pattern (RE offset=3) of the fully-staggered pattern, so that the transmitted signal can be part of the ‘cyclic extension’ of the fully-staggered SL PRS – the red part of the RE offsets {0, 2, 1, 3, 0, 2, 1, 3}. The example of AGC symbol is shown in Figure 2.

Figure 2. AGC symbol as a cyclic extension of comb pattern

Proposal 14: AGC symbol is filled with:

- Opt 1. the copy of the last SL PRS symbol

- Opt 2. the last symbol of the fully-staggered comb pattern corresponding the comb size

There were discussions in RAN1#112 meeting on whether there are any restrictions of the SL PRS configuration when TDM of SL PRS resources is allowed in a resource pool. We can discuss this issues in the following paragraphs.

Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

One issue of TDM of SL PRS resources within a slot is related to AGC function. In one example of Figure 3, four SL PRS resources are being transmitted and received among UEs within a slot. The yellow, green an blue SL PRS resources are constructed with 2 SL PRS symbols, and the orange SL PRS resource is with 4 SL PRS symbols. If UE2 receives the orange SL PRS resource, it adapts its AGC gain based on the AGC symbol associated with the orange SL PRS resource. However, from the 3rd orange symbol timing, the yellow SL PRS resource is transmitted, and during the 3rd orange symbol, UE2 suffers saturated input signals due to lack of AGC time duration over the changed input signal level. UE1 has no such AGC issue because there is no input signal level change during the yellow SL PRS resource. To avoid such problem, the number of SL PRS resources that are TDMed within a slot needs to be same.

Figure 3. TDM example of SL PRS resources with different start position within a slot

In another example of Figure 4, four SL PRS resources are being transmitted and received among UEs within a slot. Though those four SL PRS resources have the same number of SL PRS symbols (=4 symbols), UE1 receiving the yellow SL PRS resource still suffers the AGC problem explained above. The reason of AGC issue in this example comes from the misaligned start positions of the TDMed SL PRS resources. To avoid such problem, the start of the SL PRS resources that are TDMed within a slot needs to be aligned.

Figure 4. TDM example of SL PRS resources with misaligned start positions among SL PRS resources

More flexible way of TDM is shown in Figure 5, where two different number of SL PRS symbols (4 and 6 symbols) are TDMed within a slot. Different from the example in Figure 3, UE2 does not suffer the AGC problem explained above. However, UE2 suffers the input signal level changing from high to low from the 4th orange symbol, which sacrifices the input signal dynamic range of the AGC function. This may slightly degrade the ACG performance, though the overall SL PRS detection performance may not be seriously degraded.

Figure 5. TDM example of SL PRS resources with different number of SL PRS symbols

To avoid such AGC issues with TDMed SL PRS resources, the candidate positions of SL PRS resources in a slot should be determined as fixed according to the number of the symbols of SL PRS resource in a slot. The sensing should be done over the candidate SL PRS resource position basis. Depending on the sensing results, a specific candidate SL PRS resource position can be or cannot be used for SL PRS transmission.

Proposal 15: The candidate positions of SL PRS resource in a slot are determined based on the number of symbols of SL PRS resource.

From the discussions above, it’s obvious that only one number of SL PRS symbols is allowed for transmission in a slot. There could be several possibilities to implement this operation. One number of SL PRS symbols as a part of SL PRS configuration can be (pre)configured per slot, which may be too complex and overhead-demanding for configuration. Or one number of SL PRS symbols can be (pre)configured per resource pool, which seems too restrictive from SL positioning service perspective.

A more flexible but relaxed TDM of SL PRS resources is to allow SL PRS resources having the number of symbols smaller than the maximum number of SL PRS symbols within a slot, if the start positions of SL PRS resources are aligned with others. In this case, multiple SL PRS configurations can be (pre)configured in a resource pool, and a part of such configurations can be allowed in a slot as long as the start timing alignment between SL PRS resources is satisfied. How this operation can achieved can be further investigated.

Proposal 16: The start positions of SL PRS resources that are TDMed within a slot are aligned between UEs.

- Power control

In the SI phase, we have agreed to support OLPC for SL PRS transmission in the SI phase. Similar to the transmission power control of the SL communication, we think that SL PRS transmission power can be controlled based on DL pathloss, SL pathloss, and both of them. Which OLPC option is used can be (pre-)configured by the network.

Proposal 17: The following OLPC options for SL PRS transmission can be considered.

- OLPC based on DL pathloss

- OLPC based on SL pathloss

- OLPC based on both DL and SL pathloss

One issue of OLPC of SL PRS transmission is SL pathloss estimation. In SL communication, SL pathloss is calculated from the estimated PSSCH DMRS RSRP, which is the output value of the layer-3 filter. In SL positioning, RSRP can be measured not only based on PSSCH DMRS but also on SL PRS. As the number of resources of SL PRS is much larger than that of PSSCH DMRS, it’s reasonable to consider SL PRS RSRP for SL pathloss estimation.

Proposal 18: SL PRS RSRP can be used for SL pathloss estimation.

When PSCCH/PSSCH carrying SCI associated with SL PRS is transmitted with SL PRS in a same slot, the number of DMRS resources of PSSCH can be quite small due to the small number of PSSCH symbols. So to improve the RSRP measurement quality, it can also be consider to include both PSCCH DMRS and PSSCH DMRS into the RSRP measurement for SL pathloss estimation.

Proposal 19: Both PSCCH and PSSCH DMRS RSRP can be used for SL pathloss estimation.

If UE’s simultaneous transmission of the multiple SL PRS resources with the different RE offset in a slot is allowed, there is an issue how to divide the total transmission power among the different SL PRS resources. In SL communication, when the multiple PSFCHs are transmitted simultaneously, there are some rules how to assign the transmission power and which transmission of PSFCH is to be dropped. Similarly a certain rule is necessary for the simultaneous transmission of the multiple SL PRS resources in SL positioning.

Proposal 20: Further study is needed for SL PRS power control when multiple SL PRS resources are transmitted simultaneously.

When SL PRS and the associated PSCCH/PSSCH are transmitted in a same slot, there is an issue on how to align the transmission powers of SL PRS and PSCCH/PSSCH. If the total symbol power is to be aligned, depending on the comb size, the EPRE of SL PRS can be much higher than that of PSCCH/PSSCH in frequency domain. It may require a transition period between PSCCH/PSSC and SL PRS. If the EPRE is to be aligned, the total power of SL PRS can be much lower than that of PSCCH/PSSCH in time domain. It may cause the reduction of the SL PRS coverage, which is not desirable from the SL positioning service QoS. How to align the transmission power between two kinds of signals needs further study in this regard.

Proposal 21: Further study is needed whether EPRE or the total symbol power is same between PSCCH/PSSCH and SL PRS, if they are transmitted in a same slot.

If SL PRS and the associated PSCCH/PSSCH are transmitted in a same resource pool, a slot can be used either for SL PRS transmission or for the associated PSCCH/PSSCH transmission. If the transmission resource is selected by UE based on sensing, there are possibilities of resource conflict due to improper sensing results. The interference from SL PRS to PSSCH/PSSCH can be significantly high if a large com size is used and resultantly the power of SL PRS RE is highly boosted.

One possible solution to avoid or minimize such interference is to adjust the EPRE of SL PRS resource according to the MCS of the reserved PSCCH/PSSCH based on sensing. If the reserved transmission of PSCCH/PSSCH uses a higher value of MCS, the EPRE of SL PRS resource can be adjusted to a relatively low level to reduce a potential interference level.

Proposal 22: SL PRS power control for coexistence with SL communication in a shared resource pool needs to be investigated.

- Conclusions

In this contribution, the solutions for SL PRS design, the structure of SL positioning slot, and SL PRS transmission power control were discussed. The following proposals were made as a conclusion.

Proposal 1: SL PRS sequence ID used for SL PRS sequence initialization is determined based on the CRC field of the 2nd SCI associated with SL PRS transmission.

Proposal 2: In a dedicated resource pool, no DMRS is transmitted in a symbol except in PSCCH and PSSCH symbols, in a slot where SL PRS is transmitted.

Proposal 3: For a dedicated resource pool, the comb sizes N={1, 8, 10} are additionally supported.

Proposal 4: For a dedicated resource pool, the fully staggered patterns (M,N)=(1,1), (6,6,), (8,8), (10,10) are additionally supported.

Proposal 5: For a dedicated resource pool, there is no constraints on the maximum effective comb size for partially staggered pattern. Any even numbers M less than N are supported, including:

- (M,6) with M={2,4}

- (M,8) with M={2,4,6}

- (M,10) with M={2,4,6,8}

Proposal 6: For a shared resource pool, the comb sizes N={6, 8} are additionally supported.

Proposal 7: For a shared resource pool, the fully staggered patterns (M,N)=(1,1), (6,6), (8,8) are additionally supported.

Proposal 8: For a shared resource pool, there is no constraints on the maximum effective comb size for partially staggered pattern. Any even numbers M less than N are supported, including:

- (M,6) with M={2,4}

- (M,8) with M={2,4,6}

Proposal 9: For the fully staggered comb pattern, it is supported that SL PRS symbols can be repeated to fill a slot or a TDM duration, with the same order of the comb RE offset of the original SL PRS symbols.

Proposal 10: For the partially staggered comb pattern, it is supported that SL PRS symbols can be repeated to fill a slot or a TDM duration, with the following ways.

- Opt 1. Simple repetition of the partially staggered SL PRS

- Opt 2. Expansion toward the fully staggered SL PRS and its repetition

Proposal 11: SL PRS resource is defined as a set of SL resources, which is used for the measurement of a single sample/instance of SL PRS.

Proposal 12: One AGC symbol is located before every SL PRS resource.

Proposal 13: One TX/RX turnaround symbol is located after every SL PRS resource.

Proposal 14: AGC symbol is filled with:

- Opt 1. the copy of the last SL PRS symbol

- Opt 2. the last symbol of the fully-staggered comb pattern corresponding the comb size

Proposal 15: The candidate positions of SL PRS resource in a slot are determined based on the number of symbols of SL PRS resource.

Proposal 16: The start positions of SL PRS resources that are TDMed within a slot are aligned between UEs.

Proposal 17: The following OLPC options for SL PRS transmission can be considered.

- OLPC based on DL pathloss

- OLPC based on SL pathloss

- OLPC based on both DL and SL pathloss

Proposal 18: SL PRS RSRP can be used for SL pathloss estimation.

Proposal 19: Both PSCCH and PSSCH DMRS RSRP can be used for SL pathloss estimation.

Proposal 20: Further study is needed for SL PRS power control when multiple SL PRS resources are transmitted simultaneously.

Proposal 21: Further study is needed whether EPRE or the total symbol power is same between PSCCH/PSSCH and SL PRS, if they are transmitted in a same slot.

Proposal 22: SL PRS power control for coexistence with SL communication in a shared resource pool needs to be investigated.