3GPP TSG RAN WG1 #112bis-e	                    	R1-2302940

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	5

Source:	ZTE, Sanechips

Title:	Discussion on reply LS on LP-WUR architecture

Document for:	Discussion, Decision

1	Introduction

To further evaluate the RF aspects of LP-WUR architecture, RAN4 would like to know the following clarifications from RAN1: - Whether IoT/wearables/smartphone UE types are all considered for LP-WUR design - Power consumption, coverage and SNR targets - Max occupied RB number in channel bandwidth for LP-WUS, for 1.4MHz and 5MHz RF bandwidth case - Possible supported SCS for LP-WUS, if applicable - Whether WUS can be located in a band separate from the UE’s NR band - Whether FR1 is considered as first priority frequency range - Whether in-band power boosting of LP-WUS is considered from RAN1 perspective

RAN WG1 received a reply LS from RAN WG4 regarding the LP-WUR architecture. Specifically, the questions from RAN WG4 are the following:

2	Discussion

Use cases

In last RAN1#112 meeting [1], we have following agreement:

Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any.

Therefore, IoT/wearables/smartphone UE types are all considered for LP-WUR design in SI stage.

Power consumption, coverage and SNR targets

The coverage and SNR targets are related to RAN1’s evaluation. However, the target coverage has been discussed several times without any conclusion. Considering the PUSCH is the coverage bottleneck channel, and the coverage for OOK-based waveform is better than the PUSCH based on the simulation results [2]. Therefore, the target coverage is better than PUSCH. Correspondingly, the required SNR would be less than PUSCH.

As for the power consumption target, RAN1 made the following agreement. Moreover, RAN1 assume that the deep sleep for NR device has the relative power 1.

Agreement in RAN1#112 The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how. Agreement in RAN1#110bis - The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation Power State; Relative Power (unit); Ramp-up and down transition energy (Note1): (unit multiplied by ms); Ramp-up time; Time for sync/re-sync. Ultra-deep sleep; [0.015]; [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection; [400ms], FFS: 100ms; X. Note1: - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.], - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.], - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio - Companies can report the assumption of X in the initial evaluation. - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated. - The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync. - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state. Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS

Therefore, the target relative power should be no more than 4 according to the current agreement. And it is hard to provide an absolute power consumption in RAN1.

Max occupied RB number

In last RAN1#112 meeting, the bandwidth and guardband related agreement for evaluation is made as following.

| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS |
| --- | --- |

In legacy NR system, the RBs of 5MHz RF bandwidth with 30KHz SCS and15KHz SCS are 11 and 25, respectively. For WUR, the anti-interference ability of the WUR is worse than the MR. Therefore, we think the maximum PRB number in 5MHz RF bandwidth for LP-WUS is no more than 11RBs with 30KHz and 25RBs with 15KHz.

For the 1.44MHz RF bandwidth, if the SCS=15KHz, the supported RBs number is 8 and there is no guardband. If the SCS=30KHz, the supported RBs number is 4 and there is no guardband. However, for LP-WUS, considering the anti-interference ability, it is improper to configure the LP-WUS without guarband.

Supported SCS

Since RAN1 studies two kinds of waveforms, i.e., CP-OFDM and DFT-S-OFDM, at least for DFT-S-OFDM, 15KHz and 30KHz could be applicable. As for CP-OFDM, we think larger SCS, for example, 240KHz or 480KHz, may be needed to guarantee the data rate. For CP-OFDM, RAN1 made the following agreement and it is still under discussion whether/how to support the SCS.

Agreement For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study - FDM/TDM multiplexing with other NR transmissions - link performance - impact to legacy UEs - impact on gNB

Therefore, for DFT-S-OFDM, the supported SCS could be 15KHz and 30KHz in FR1. For CP-OFDM, the potential candidate SCS could be 15KHz, 30KHz, 60KHz, 120KHz, 240KHz, 480KHz. However, it seems larger SCS would have impacts on RAN4. If large SCS is introduced, further confirmation from RAN4 is needed.

LP-WUS location

For the MR, the MR has the capability to search cross all the frequencies. For the WUR, we do not think it is possible. Therefore, when the MR camps on another cell with a frequency that the WUR does not support, we need to confirm whether the WUR is supposed to receive the LP-WUS in another band, otherwise, the WUR may not be available in some cases.

Moreover, since coverage sensitive for WUR may be limited and WUR at high frequency has worse performance, we think it is better to deploy the WUR at a low frequency. Therefore, it is required that the WUR could be deployed at another band. As for the feasibility, we think no problem is observed, since the WUR could be separate from the MR.

Whether FR1 is considered as first priority frequency range

All the RAN1 agreement and evaluation assumption are based on FR1 even FR2 is not precluded. It is natural to say FR1 has higher priority.

In-band power boosting of LP-WUS

Power boosting is not assumed in RAN1’s initial evaluation. But for coverage enhancement, power boosting could be considered.

Based on above analysis, we have the following proposal.

Proposal 1: Take the following into LS reply to RAN4

- IoT/wearables/smartphone UE types are all considered for LP-WUR design in SI stage.

- The target coverage should be better than PUSCH, the target SNR for LP-WUS is less than that for PUSCH and target relative power should be no more than [4] wherein the relative power for MR deep sleep is 1.

- The potential candidate SCS of LP-WUS could be 15KHz, 30KHz, 60KHz, 120KHz, 240KHz, 480KHz

- Regarding the maximum occupied PRB number for LP-WUS,

  - For 5MHz RF bandwidth including the guardband, maximum PRBs for LP-WUS (not including guardband) is 11RBs for 30KHz and 25 RBs for 15KHz.

  - For 1.44MHz RF bandwidth, maximum PRBs for LP-WUS is [2 or 3] RBs for 30KHz and [6 or 7] RBs for 15KHz.

- LP-WUS can be located in a band separate from the UE’s NR band

- FR1 is considered as first priority

- In-band power boosting of LP-WUS could be considered for coverage enhancement from RAN1 perspective

3	Conclusion

Based on the discussion in the previous sections, we propose the following:

Proposal 1: Take the following into LS reply to RAN4

- IoT/wearables/smartphone UE types are all considered for LP-WUR design in SI stage.

- The target coverage should be better than PUSCH, the target SNR for LP-WUS is less than that for PUSCH and target relative power should be no more than [4] wherein the relative power for MR deep sleep is 1.

- The potential candidate SCS of LP-WUS could be 15KHz, 30KHz, 60KHz, 120KHz, 240KHz, 480KHz

- Regarding the maximum occupied PRB number for LP-WUS,

  - For 5MHz RF bandwidth including the guardband, maximum PRBs for LP-WUS (not including guardband) is 11RBs for 30KHz and 25 RBs for 15KHz.

  - For 1.44MHz RF bandwidth, maximum PRBs for LP-WUS is [2 or 3] RBs for 30KHz and [6 or 7] RBs for 15KHz.

- LP-WUS can be located in a band separate from the UE’s NR band

- FR1 is considered as first priority

- In-band power boosting of LP-WUS could be considered for coverage enhancement from RAN1 perspective

Reference

3GPP, RAN1 Chairman’s Notes, RAN1#112

R1-2302948	Evaluation on LP-WUS. ZTE, Sanechips