3GPP TSG RAN WG1 #112bis-e	                    	R1-2302948

e-Meeting, April 17th – April 26th, 2023

# Source: 	ZTE, Sanechips

# Title:	Evaluation on LP-WUS

# Agenda item: 	9.11.1

# Document for:	Discussion and decision

Introduction

In RAN #94 meeting¸ new study item on low-power Wake-up Signal and Receiver for NR [1] was approved and the latest revision was approved in [2].

The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In RAN1#112 meeting, some agreements are captured in the chairman notes [3]. In this contribution, we further discuss evaluation details on LP-WUS and provide some initial simulation results.

Use cases and KPI

According to the discussion in RAN1#110bis-e meeting [4], it is agreed that the IoT use cases, wearables and health monitoring devices, and eMBB use cases including XR/smart glasses are to be further studied in the SI stage.

For the KPIs, system overhead, power consumption, latency, coverage, capacity, and UPT are considered for evaluation, while NW power consumption, system overhead and co-existence impacts can be further discussed. Obviously, the introduction of LP-SS and LP-WUS would bring some impacts on NW power consumption, system overhead and spectrum efficiency impact due to the resource occupation. The more resources occupied by LP-SS/LP-WUS, the more serious impacts it brings. Additionally, co-existence issue also needs to be considered and captured in the TR. Therefore, the following proposal is made.

Proposal 1: The following KPIs on LP-WUS should be further evaluated

- System overhead

- Network power consumption

- Co-existence impacts

In last RAN1#112 meeting [3], we have the following agreement and some remaining issues need to be determined further.

Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any.

For the latency, we need to further discuss which kind of latency is assumed here. In connected mode, the latency may refer to the scheduling delay. In idle/inactive mode, the latency may refer to the paging delay. So, the latency here needs clarification. From our perspective, the latency means the time interval between LP-WUS and PRACH in idle/inactive mode. In connected mode, the latency means the time interval between LP-WUS and PDSCH with data.

Proposal 2: Clarify the latency definition for different RRC states.

Evaluation for UE power consumption in idle mode

For RRC idle/inactive mode, the existing UE power consumption evaluation method in Rel-17 PEI, e.g., numerical calculation, can be reused to evaluate the power consumption for LP-WUS. In section 3.1, the power model and system parameters for power consumption evaluation are discussed and the initial results on power consumption are shown in section 3.2.

- 3.1 Power model

In RAN1#112 meeting [3], the following agreements were achieved for ultra-deep sleep of main radio.

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

For LP-WUS, the main radio is powered off until the LP-WUS is received in idle/inactive mode which means main radio can be powered off for a long time. Therefore, the relative power of ultra-deep sleep should be far less than the relative power of deep sleep. Considering the relative power of deep sleep was defined as 1 or 0.8 unit in existing power consumption model, it is acceptable the relative power of ultra-deep sleep is 0.015 unit for LP-WUS power consumption evaluation.

Proposal 3: The relative power of ultra-deep sleep is 0.015 unit for LP-WUS power consumption evaluation.

For IoT cases and Wearable case, transition energy and ramp-up time used in power consumption evaluation is Alt 1. For eMBB cases which are more complicated than IoT cases and Wearable cases, the Alt 2 is more suitable. It is proposed 40000 units*ms and 800ms are used as a starting point of ramp-up and down transition energy and ramp-up time for ultra-deep sleep for eMBB cases in power consumption evaluation.

Proposal 4: The ramp up and down transition energy and ramp-up time of ultra-deep sleep is 15000 units*ms and 400ms for IoT/Wearable cases and 40000 units*ms and 800ms for eMBB cases for power consumption evaluation.

For LP-WUS, the sync/re-sync may be required after main radio is powered on from UDS. For the power consumption evaluation of PEI, it is assumed that there are up to 3 SSBs before the PO from DS. Considering the sync/re-sync may be achieved via LP-WUS or other signals such as TRSs, it is proposed there are at least 3 SSBs used for sync/re-sync when main radio waked up from the state of ultra-deep sleep.

Proposal 5: For LP-WUS power consumption evaluation, at least 3 SSBs are required for sync/re-sync when main radio wakes up from the state of ultra-deep sleep.

In RAN1#111 meeting [3], the following agreements were achieved for LP-WUR.

Agreement The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed.

For LP-WUS, it is known the relative power of WUR ‘on’ state depends on the WUR architectures design. Based on the discussion of WUR architectures, at least three levels of WUR ‘on’ state relative power should be used for LP-WUS power consumption evaluation, e.g., 0.01, 0.5, and 1 for OOK-based receiver. If the OFDM-based receiver is considered, the WUR-on state power should be further defined and whether WUR-off power could be reused also should be discussed. From our perspective, the WUR-on power for OFDM based receiver should be no less than 20, and no more than 40 since the power for processing SSB and micro-sleep is 50 and 30 respectively for RedCap UE. For OFDM-based receiver, the relative power of WUR off is 0.01 is more suitable.

Proposal 6: For OFDM based receiver, WUR ‘on’ relative power should be no less than 20 and WUR ‘off’ relative power should be 0.01.

Proposal 7: The relative power of WUR on is 0.01, 0.5, 1, 20 and 40 for LP-WUS power consumption evaluation.

It is known that the complexity of WUR is lower than MR, therefore the ramp-up time of WUR is less than MR. 10ms ramp-up time of WUR are used as the starting point for LP-WUS power consumption evaluation. As the WUR-on relative power increasing, the ramp-up time also should be increased.

Proposal 8: For LP-WUS power consumption evaluation, the 10ms ramp-up time of WUR could be used as the starting point.

- 3.2 Power consumption results

- 3.2.1 Comparison with DRX/eDRX with/without PEI

The timeline used for LP-WUS power consumption evaluation is shown in Figure 1.

Figure 1. The timeline for LP-WUS

Therefore, the power consumption of LP-WUS is

PLP-WUS=EMR+EWURT

wherein EWUR=PWURon*T, EMR=p*(kSSB*PSSB*TSSB+PPO*TPO+PLS*TLS+kLS*ELS+PUDS*TUDS+EUDS)+(1-p)*PUDS*T, kSSB is the number of SSBs for sync/re-sync, kLS is the number of light sleep and p is paging rate per T. Relative power and time of various states are shown in the following table.

Table 1. Power state modelling for LP-WUS

|  | Relative power |  | Time duration |  | Energy notation |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB | PSSB | 50(Case 1)/ 100(Case 2) | TSSB | 2ms | - | - |
| Paging | PPO | 120/50(Case 1) 300/100(Case 2) | TPO | 4ms | - | - |
| Light sleep | PLS | 18(Case 1)/ 20(Case 2) | TLS | 44ms | ELS | 100 |
| ultra- deep sleep | PUDS | 0.015 | TUDS | T-kSSB*TSSB-TPO-TLS | EUDS | Case 1：15000 Case 2：40000 |
| WUR on | PWUR on | Alt1:0.01 Alt2:0.5 Alt 3:1 Alt 4: 20 Alt 5: 40 | T | 1s | - | - |
| WUR off | PWUR off | 0.001 (for Alt 1/2/3) 0.01 (for Alt 4/5) |  |  |  |  |
| Case 1: IoT cases and Wearable cases Case 2: eMBB cases |  |  |  |  |  |  |

The timeline of DRX and DRX with PEI is shown in Figure 2 wherein the medium SINR is assumed for power consumption evaluation.

- Figure 2 The timeline for DRX and DRX with PEI

Relative power and time of various states used for DRX are shown in the following Table 2.

- Table 2. Power state modelling for DRX

|  | Relative power |  | Time |  | Transition energy |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB | PSSB | 50(Case 1)/ 100(Case 2) | TSSB | 2ms | - | - |
| Paging | PPO | 120/50(Case 1) 300/100(Case 2) | TPO | 4ms | - | - |
| Light sleep | PLS | 18(Case 1)/ 20(Case 2) | TLS | (*)ms | ELS | 100 |
| Deep sleep | PDS | 0.8(Case 1)/ 2(Case 2) | TDS | (*)ms | EDS | 450 |
| PEI | PPEI | 50(Case 1)/ 100(Case 2) | TPEI | 0.5ms | - | - |
| * value depends on timeline Case 1: IoT cases and Wearable cases Case 2: eMBB cases |  |  |  |  |  |  |

The power consumption of eDRX is:

PeDRX=PDRX*LPTW+PS*(TeDRX-LPTW)TeDRX

Wherein Ps is the relative power of the state outside of PTW. Whether the state outside of PTW is deep sleep or ultra-deep sleep should be determined first. It is assumed the length of out-of-PTW is A, the relative power of deep sleep and ultra-deep sleep is P1 and P2 respectively and corresponding state transition energy is B1 and B2. From the perspective of power consumption, it is the most energy saving that the sleep state outside of PTW is ultra-deep sleep when A*P1+B1>A*P2+B2 and the sleep state outside of PTW is deep sleep when A*P1+B1<A*P2+B2. Take the system configurations used for evaluation as example (eDRX cycle is 61.44s, the length of PTW is 5.12s, P1=0.8 unit, P2=0.015, B1=450 units*ms and maximum value of B2 is 40000 units*ms), the sleep state outside of PTW is ultra-deep sleep because more UE power consumption can be saved based on (61.44-5.12) *0.8*1000+450> (61.44-5.12)*0.015*1000+40000.

Proposal 9: Whether deep sleep or ultra-deep sleep state is assumed outside of PTW should be determined for eDRX power consumption evaluation.

Based on above analysis, the initial power consumption evaluation results are shown in Figure 3.

(a) Case 1                                (b)Case 2

Figure 3. Power consumption evaluation results based on R_E=0.01%

From the power consumption results from Figure 3, it can be observed that:

Observation 1: Compared with DRX with/without PEI, the LP-WUS for one UE or multiple UEs can achieve lower power consumption when PWUR on=0.01, PWUR_on=0.5 and PWUR on=1.

Observation 2: For Case 1, compared with eDRX with/without PEI, the LP-WUS brings more UE power consumption when PWUR on=0.5, 1, or 20.

Observation 3: For Case 2, compared with eDRX with/without PEI, the LP-WUS brings more UE power consumption when PWUR on=1 or 20.

- 3.2.2 LP-WUS monitoring

There are two LP-WUS monitoring schemes, one is always on and other is on-off duty cycled. If the on-off duty cycled monitoring scheme is used, the extra power consumption can be reduced. Of course, the latency is increased and the impact depends on detailed on-off duty design, such as duty cycled length, duty ratio and so on. In Figure 4, the power consumption evaluation results of different LP-WUS monitoring schemes are shown when R_E=1%, duty cycle = 1.28s.

(a) Case 1                         (b) Case 2

Figure 4.  Power consumption evaluation results based on LP-WUS monitoring

From the power consumption results from Figure 4, we have following observation:

Observation 4: Compared with LP-WUS with always on monitoring, the extra power consumption caused by LP-WUS with on-off duty cycled monitoring is small when the WUR-on power is 0.01, 0.5 and 1.

For OFDM sequence-based receiver, we evaluated the following two duty-cycled modes:

Mode 1: duty cycle is 1.28s, duty rate is 4%/5%/6%/8%/10%；

Mode 2: on duration time is 1/2/4/6 ms and duty cycle is 50/60/80/90/100/110/120/130/140 ms

The initial power consumption evaluation results are shown in Figure 5 and Figure 6 wherein R_E=1%.

- Mode 1(Case 1)                         (b) Mode 1(Case 2)

(c)Mode 2(Case 1)                         (d) Mode 2(Case 2)

Figure 5. Power consumption evaluation results for OFDM sequence-based receiver with on-off duty cycle monitoring

From the power consumption results from Figure 5, we have following observations:

Observation 5: For Mode 1, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 4) with on-off duty cycled has power saving gain only when duty cycle rate is < =5% for Case 1 and 6% for Case 2 and duty cycle is 1.28s.

Observation 6: For Mode 2, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 4) with on-off duty cycled has power saving gain when on duration=1ms with duty cycle >90ms, on duration=2ms with duty cycle >110ms, on duration =4ms with duty cycle>130ms for Case 1 and on duration=1ms with duty cycle >80ms, on duration=2ms with duty cycle >100ms, on duration =4ms with duty cycle>120ms for Case 2.

- Mode 1(Case 1)                         (b)  Mode 2(Case 1)

Figure 6. Power consumption evaluation results for OFDM sequence-based receiver with on-off duty cycle monitoring

From the power consumption results from Figure 6, we have following observations:

Observation 7: For Mode 1, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 5) with on-off duty cycled has power saving gain only when duty cycle rate is < =2% for case 1 and duty cycle is 1.28s.

Observation 8: For Mode 2, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 5) with on-off duty cycled has power saving gain when on duration=1ms with duty cycle >300ms, on duration=2/4ms with duty cycle >350ms for Case 1.

- 3.2.3 RRM measurement

For LP-WUS, the power consumption also impacted by different RRM measurement schemes. In order to show the impact of RRM measurement on power consumption, the following three Opts are used in power consumption evaluation.

Opt 1: LP-SS/LP-WUS/preamble based on WUR receiving is used for RRM measurement

Opt 2: RRM measurement is performed only after MR is waked up by LP-WUS

Opt 3: RRM measurement is performed by MR and relaxed to N DRX cycle

For Opt 1, the power consumption is the same as no RRM measurement because RRM measurement is performed by WUR and only one state of WUR on is defined. For Opt 2, the timeline of waked up MR is shown in Figure 7. For Opt 3, no matter whether LP-WUS is detected or not, the MR is powered on and performed RRM measurement in N DRX cycle.

Figure 7 The timeline for WUR with measurement

In Figure 8, the power consumption evaluation results of different RRM measurement are shown.

(a) Case 1                          (b) Case 2

Figure 8. Power consumption evaluation results based on RRM measurement

From the power consumption results from Figure 8 it can be observed that:

Observation 9: For Case 1, when N is relaxed to 16, LP-WUS has power saving gain compared with DRX with PEI when R_E=1% and PWUR on=0.5.

Observation 10: For Case2, when N is relaxed to 30, LP-WUS has power saving gain compared with DRX with PEI when R_E=1% and PWUR on=0.5.

- 3.2.4 FAR

Once LP-WUS false detection happens, the MR is powered on. Much power consumption is wasted since the procedure, such as SSB reception and PEI/PO monitoring, would be implemented on MR. When false detection happens, only when the UE has finished PEI/PO monitoring but does not receive its paging message, the UE can identify that a false detection happens. The results of power consumption evaluation with FAR impact are shown in Figure 9 wherein the FAR=X% indicates the probability of extra MR power on caused by FAR is X% in one DRX cycle, which is defined in details in section 5.

(a) Case 1                         (b) Case 2

Figure 9. Power consumption evaluation results based on FAR

From the power consumption results from Figure 9, we have following observation:

Observation 11: For Case 1/Case 2, compared with DRX without PEI, the LP-WUS has power saving gain when the probability of extra MR power on caused by FAR is lower than 10% /5%.

If UE is not required to monitor PO after receiving LP-WUS, UE can identify that a false detection happens only after UE completes the entire RACH procedure and is not scheduled for a long time. Considering the power consumption model of UE transmitting is not defined, this procedure is not considered in evaluating the FAR impact on power consumption.

- 4. Evaluation for latency

In RAN1#111 meeting [3], the following agreements were achieved for latency evaluation:

For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included

For LP-WUS, if the first PO is the legacy PO, there are three parts of the latency, i.e., time for main radio ramp up, time for sync/re-sync and the delay from sync/re-sync to legacy PO. One example is shown in Figure 10 wherein the latency T1=Tx+Tramp up+T sync/re-sync.

Figure 10. Latency for LP-WUS based on legacy PO mechanism

If the first PO is dynamic, i.e., the PO is located after main radio ramp up and sync/re-sync, there are two parts of the latency. One example is shown in Figure 11 wherein the latency T2=Tramp up+T sync/re-sync. It is assumed the gap between the last SSB and PO is 8ms.

Figure 11. Latency for LP-WUS based on dynamic PO mechanism

The latency results based on DRX/eDRX with/without PEI are shown in Table 3, wherein the paging is uniform arrived, DRX cycle is 1.28s, eDRX cycle is 61.44s, PTW is 5.12s, the Tramp up is 400ms /800ms and T sync/re-sync is 50ms.

Table 3. Initial latency evaluation

|  | Latency | T1 | T2 |
| --- | --- | --- | --- |
|  |  | Reduction (%) | Reduction (%) |
| DRX/DRX+PEI | 0.64s | -115 | 29.6% |
| eDRX/eDRX+PEI | 26.46s | 94.7 | 98.2% |
| T1 | 1.38s | - | 67.3% |
| T2 | 0.45s | -206% | - |

Based on table 3, it can be observed:

Observation 12: Compared with DRX/eDRX, the latency reduction of LP-WUS based on legacy PO is -115%/94.7%.

Observation 13: Compared with DRX/eDRX, the latency reduction of LP-WUS based on dynamic PO is 29.6%/98.2%.

Observation 14: Compared with legacy PO, the latency reduction of LP-WUS based on dynamic PO is 67.3%.

Proposal 10: Dynamic PO should be considered for LP-WUS.

If the LP-WUS is discontinuously monitored by UE, donating [T] ms as the period for a complete on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle. The latency caused by on-and-off cycle should be added for T1 and T2. Obviously, the latency for duty cycle monitoring mechanism would be larger than that for always on monitoring. Moreover, different duty cycle monitoring schemes have different impacts on the latency.

Observation 15: The latency for duty cycle monitoring mechanism would be larger than that for always on monitoring and the different duty cycle monitoring schemes have different impacts on the latency.

Once LP-WUS miss detection happens, the MR is not powered on to receive paging message and send PRACH. The eNB can identify that a miss detection happens only after the eNB has not received PRACH. Then eNB pages the UE again in the next first PO. Therefore, the latency caused by LP-WUS miss detection should be considered. One example is shown in Figure 12. Considering the miss detection of LP-WUS, the latency is T=T1/T2+m%*(Toffset+T1/T2), wherein m% is miss detection rate, Toffset includes two parts, one is the gap between PO and RO and the other is the gap between RO and next LP-WUS. Obviously, the latency impact caused by MDR is small if dynamic PO is used.

Figure 12. Latency for LP-WUS when miss detection happens

Observation 16: The latency impact caused by miss detection is small if dynamic PO is used.

- 5. Evaluation for coverage

- 5.1 FAR definition

According to the discussion, FAR definition is needed to be further determined

- The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold Companies to report the selected reference time duration values and the associated number of WUS attempts/trials

For Alt 1, based on each detection, the adjacent two attempts are highly related when there are some samples are overlapped, which is shown as following figure

Figure 13. Sliding detection with step size =1 sample

For Alt1, we cannot assume each detection is independent except that the step size for sliding is larger than the length of the sequence. For MDR, it can be calculated based on each detection.

Observation 17: For FAR definition, independent detection cannot be assumed for Alt1, unless the step size for sliding is larger than the length of the sequence.

For Alt 2, the FAR is defined in a time duration. Among different time durations, we can assume the FAR is the same and independent, since the correlation can be ignored. The longer time duration, the more independent FAR for different time duration is.

Figure 14. Independent FAR for different time durations

Based on this time duration, the total FAR in a given time could be further derived and unified for always-on monitoring and duty cycle monitoring. For example, the FAR is defined in a time duration T ms and the duty cycle monitoring has a periodicity with a DRX cycle and duration-on T ms. In this case, for one DRX cycle T1, the duty cycle monitoring has a FAR P. For always-on monitoring, the FAR in a DRX cycle can be given as  where N=T1/T.

Assuming that the step size for sliding is m, and the sequence length is L, FAR in a time duration T is P. Then we have the following relationship when m=L

P=,

Where N=T/L, x0 is the FAR for one detection assuming the step size is one sequence and detection is independent. If m<L, we can know that the FAR for each detection should be less than x0. Therefore, in the LLS, the FAR assumption for each detection should be less than x0.

Similarly, the MDR also can be defined in a time duration. In this case, the upper limit of MDR when the step size is one sequence, could be y0. A value for each detection less than y0 could be assumed.

As for the reference time duration, we think a DRX cycle can be considered as the starting point, which also can be used for power evaluation.

Proposal 11: FAR P is defined in a time duration T

- The FAR for each detection is LLS is less than x0, where x0 is the FAR for the detection when the step size for sliding is the sequence length L, satisfying P=

- The reference time duration is a DRX cycle.

- MDR has the similar definition.

## 5.2 LLS results

The assumptions for LLS are shown in Table 4.

Table 4. The assumptions for LLS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz for OOK |
| Waveform | OOK |
| Channel structure | For OOK, the following two Options are considered in LLS - Option 2: Sequence only, - Option 3: Payload+CRC, |
| Payload size | 24bits data + 8 bits CRC |
| Sequence | 32-length sequence by the repetition of [0 1 0 1 0 1 0 1] |
| Coder scheme | Manchester coding 1/2 |
| SCS of OFDM generator for NR signal | 30kHz |
| Configuration for LP-WUS signal | For OOK waveform, - Option 2b: M =4 for SCS = 30 kHz (same as NR signal) |
| WUS duration | Based on payload size and coding scheme or the length of preamble sequence |
| MDR/FAR assumption | - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS:0.1% |
| gNB Channel BW | 20MHz and with Normal CP=2.344us。 |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS - 11 subcarriers with 30KHz SCSs are used for guardband on each side of LP-WUS |
| Filter | 3rd order Butterworth LPF - Filter bandwidth =4.32 MHz, - Cutoff frequency = 4.98/2 MHz |
| Adjacent subcarrier interference | PDSCH with randomly modulated QPSK mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =0 dB |
| Sampling Rate | 7.68 MHz |
| ADC bit width | Ideal ADC |
| Channel Model | TDL-C 300ns |
| Number of Rx for LP-WUS | 1 Rx |
| UE speed | 3 km/h |
| Inter-cell interference | No |
| Phase noise modeling | No |
| Oscillator max frequency error | No |
| Oscillator max time error | No |

The initial results are given in Figure 15.

Figure 15. Initial coverage comparison between OOK signal and PUSCH

Proposal 12: The target coverage of LP WUS should be better than PUSCH.

- Evaluation for system overhead

According to the discussion, the system overhead can be expressed as percentage of the used resources for LP-WUS.

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |

In frequency domain, the bandwidth for LP-WUS includes the guardband bandwidth and signal bandwidth, which can be donated as NLP-WUS RBs. In time domain, the occupied symbols include the symbols for LP-WUS and guard time if any, where the total occupied symbols can be donated as SLP-WUS. Then one transmission of LP-WUS occupies NLP-WUS*SLP-WUS*12 REs in total.

Then the largest system overhead percentage (only in frequency domain) could be calculated as

PMax=NLP-WUS*12/Nband

where the Nband means the total RBs for a band.

If we consider to extend the above formula in the time domain, then M times transmission of LP-WUS in duration T occupies the total RE resources M*NLP-WUS*SLP-WUS*12. And the percentage can be calculated as

P=M*NLP-WUS*SLP-WUS*12/(Nband*T*12)

Satisfying M*SLP-WUS<=T

In a time duration of T, assuming that M times LP-WUS transmission carries X*M bits information, then the data rate is

R1=X*M/T (bit/symbol)

The peak data rate can be

RMax=X/SLP-WUS (bit/symbol)

Satisfying M*SLP-WUS<=T

The value of M can be related to the paging rate, UE number PF, or PO. Of course, if additional LP-WUS is transmitted, e.g., for sync/measurement, the system overhead also needs to be counted. And the following formula also can be considered. Based on above, we calculate the system overhead percentage based on the following formula

If each LP-WUS transmission has the same resource occupation, it can be written as

P=M*NLP-WUS*SLP-WUS*12/(Nband*T*12)

Where

- Nband means the total RBs for a band or carrier in a cell

- NLP-WUS, i means the number of RBs for ith LP-WUS transmission including the guardband bandwidth and signal bandwidth

- SLP-WUS,i means the number of symbols for ith LP-WUS transmission including guard time if any

- Assuming that LP-WUS is transmitted M times in duration T, FFS how to determine M

As for the times of LP-WUS transmission in idle/inactive mode, there are two methods.

Method 1: UEs number in a cell, paging rate for a UE is assumed. Based on UEs number and paging rate, the number of LP-WUS transmission in idle/inactive mode can be obtained.

Method 2: based on PF, PO configuration, obtain the number of LP-WUS in idle/inactive mode

As for the times of LP-WUS transmission in connected mode, we think it is not big issue, since it is UE specific, the overhead for the network is configurable.

Additionally, the system overhead increasement may also be brought by the false wake-up. If the UE is falsely waked up and initiates the RACH procedure without PO monitoring, then the resource occupation during the RACH procedure and RRC connection setup would cause additional system overhead. It can be seen that higher false alarm rate would bring more serious system overhead wasting.

Proposal 13: Discuss the LP-WUS transmission assumption in idle/inactive mode for evaluation and capture how to calculate the system overhead.

- Evaluation for BS power consumption

Another agenda, i.e., network power saving, is under the discussion, which aims to save the network power consumption. However, due to the LP-WUS transmission, which may cause additional network power consumption. It should be carefully considered that the network power saving gain is negated by the LP-WUS transmission. Therefore, evaluation for BS power consumption should be considered. Then the percentage for increased BS power consumption by introducing LP-WUS can be evaluated based on the following formula:

Where r is the percentage for increased BS power consumption by introducing LP-WUS, P2 is the total BS power consumption after introducing LP-WUS, P1 is the total BS power consumption for baseline scheme without introducing LP-WUS.

More specifically, the following scenarios are considered for NWES.

For evaluation purpose, - a load (L) % of a cell is a percentage of resources used for UE specific PDSCH / PUSCH - The following load scenarios are considered Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L≤15. Light load; - Include cell-specific signals and channels, and - 15 < L≤ 30. Medium load; - Include cell-specific signals and channels, and - 30 < L≤ 50. For CA, the companies report whether the load is defined per CC or across all CCs..

Assume that the percentage for the increased BS power consumption by introducing LP-WUS in empty load scenario is r1 and the percentage for the increased BS power consumption by introducing LP-WUS in medium load scenario is r2. It is not difficult to foreseen r2,< r1. Therefore, focusing on the empty load scenario can reflect the maximum BS power impacts brought by LP-WUS, which can be evaluated as a starting point.

Additionally, according to RAN2 spec, the RRM measurement for serving cell is shown as follows:

#### 4.2.2.2 Measurement and evaluation of serving cell The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1. The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2. If the UE has evaluated according to Table 4.2.2.2-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities. If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1]. Table 4.2.2.2-1: Nserv DRX cycle length [s]; Scaling Factor (N1); Nserv [number of DRX cycles]. FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; M1*N1*4. 0.64; 5; 8; M1*N1*4. 1.28; 4; 6; N1*2. 2.56; 3; 5; N1*2. Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length..

That means every M1*N1 DRX cycle, the UE shall measure the signal power or quality. It means, if the measurement period is relaxed for power saving, at least the same measurement times via LP-SS should be guaranteed.

More specifically, if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, M1*N1 is equal to 2 and the LP-SS periodicity should be at least 2DRX cycles including 0.64s and 1.28s. Otherwise, the M1*N1=1 and the LP-SS periodicity should be at least 1 DRX cycle, including 0.32, 0.64, 1.28, 2.56s. Considering the performance loss, more frequent transmission of LP-SS may be assumed, e.g., 0.08s, 0.16s. Then the candidate periodicity for LP-SS could be 80ms, 160ms, 320ms, 640ms, 1280ms, 2560ms.

As for the offset with SSB, at least 0 should be considered for NW power saving. Whether other offset values are considered can be further discussed. For example, for power boosting, 5ms power offset, similar like NCD-SSB may also be a potential value.

Proposal 14: For NW power evaluation, discuss the LP-WUS or LP-SS transmission assumption, and load scenarios in idle/inactive mode.

- Evaluation for RRC connected mode

In this section, LP WUS used for XR traffic and eMBB traffic in RRC connected mode are evaluated. For SLS of XR, the assumptions are in line with TR 38.838.

- 8.1 Evaluation for XR traffic

In this section, XR DL video traffic of (30Mbps, 60fps) is assumed. The packet size follows truncated Gaussian distribution. Both jitter range of [-4,4]ms and [-8,8]ms are evaluated.

Rel-17 UE power saving schemes including PDCCH skipping and SSSG switching are used to compare with LP WUS scheme.

In LP WUS simulation, Main Radio is in micro sleep state if LP-WUS does not wake up the MR. If LP WUS wakes up the MR, the MR will start a drx-onDurationTimer at the next slot after the detection of LP-WUS. After drx-onDurationTimer and drx-inactivityTimer expire, MR returns to micro sleep state. LP WUS is always been monitored when MR is in micro sleep state. The power of LP-WUS is ignored in simulation, because it is negligible compared with the power of micro sleep (45 units). The procedure is shown in Figure 16 as below.

Figure 16 Scheme of LP-WUS simulation

In Rel-17 PS scheme (i.e., PDCCH skipping and SSSG switching) simulation, a SSSG with sparse monitoring occasions (monitor PDCCH every 2 slots) is used in the beginning of DRX onDuration. Once UE detects a data scheduling, UE switches to another SSSG with dense monitoring occasions (i.e., per-slot configuration). When UE finishes data transmission, gNB indicates UE to skip PDCCH monitoring for a duration in the slot that is 2ms after the last packet via dummy DCI. The candidate PDCCH skipping periods are {4ms, 8ms}. CDRX setting (CDRX cycle, drx-onDurationTimer, drx-inactivityTimer) is set to (10, 8, 4). The procedure is shown in Figure 17 as below.

Figure 17 PDCCH skipping and SSSG switching

The simulation results for XR traffic are as shown below.

Table 5 DL only, 30Mbps, 60fps, jitter range = [-4,4]ms, InH

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) | UPT(Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.9% | - | 324 |
| LP WUS | / | 5 | 4 | 11 | 11 | 93.9% | 26% | 242 |
| ## Skipping+switching | ## 10 | ## 8 | ## 4 | ## 11 | ## 11 | ## 90.3% | ## 17% | 235 |

Table 6 DL only, 30Mbps, 60fps, jitter range = [-8,8]ms, InH

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) | UPT(Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.9% | - | 325 |
| LP WUS | / | 5 | 4 | 11 | 11 | 90.18% | 26% | 238 |
| ## Skipping+switching | ## 10 | ## 8 | ## 4 | ## 11 | ## 11 | ## 90.9% | ## 16% | 246 |

According to the simulation results, both LP-WUS and Rel-17 PS scheme have UPT loss. For LP-WUS, the UPT loss is mainly caused by the latency caused by the processing time for MR to wake up, which is 1 slot in this evaluation. In PDCCH skipping and SSSG switching scheme, the UPT loss is mainly caused by the latency caused by sparse PDCCH monitoring and DRX setting.

LP WUS can provide 26% PSG for XR traffic with 30Mbps in InH. PDCCH skipping and SSSG switching can provide 16%-17% PSG for XR traffic with 30Mbps in InH.

Observation 18: LP-WUS can provide 26% PSG for XR traffic with 30Mbps in InH. PDCCH skipping and SSSG switching can provide 16%-17% PSG for XR traffic with 30Mbps in InH.

- 8.2 Evaluation for eMBB traffic

In this section, FTP 3 traffic (packet size: 0.5Mbyte, average interval time: 200ms) is used.

For evaluation of PDCCH skipping-switching-DCI 2-6 scheme, UE skips a duration of 20ms when the UE finishes a packet reception. After PDCCH skipping duration, UE changes to a SSSG with sparse monitoring occasions (monitors PDCCH every 2 slots). If UE receives a data scheduling, UE changes to a SSSG with dense monitoring occasions (monitors PDCCH every 1 slot).

For LPWUS scheme, if the wake-up delay is 3ms, it is assumed that MR can enter into light sleep. If the wake-up delay is 10ms, MR can enter into deep sleep. Moreover, we assume UE always monitors LP-WUS. If LP-WUS indicates to wake up, MR wakes up after a wake-up delay, and starts a DRX On-duration with 4ms. The inactivity timer of DRX is 2ms.

For Genie scheme, UE power consumption assuming that UE is in a sleep state (e.g., micro/light/deep sleep as defined in TR38.840) whenever there is neither DL data reception nor UL transmission.

Table 7 evaluation results for FTP 3 traffic

| Scheme | relative power of LP-WUR “ON” | PSG (Compared to Baseline) | UPT loss (compared to Baseline) |
| --- | --- | --- | --- |
| Baseline-AlwaysOn | - | - | - |
| CDRX (160-8-100) ms | - | 72.7% | 70.9% |
| PDCCH skipping-switching- DCI 2-6 | - | 83.4% | 77.9% |
| LPWUS (wake-up delay 3ms) | 1 | 72.6% | 39% |
|  | 10 | 64.9% |  |
| LPWUS (wake-up delay 10ms) | 1 | 94% | 67.7% |
|  | 10 | 86.4% |  |
| Genie |  | 95.2% | - |

According to the evaluation results, the PSG from LP-WUS is close to genie with the UPT loss of 67.7%. And LPWUS has less UPT loss than PDCCH skipping-switching-DCI 2-6 scheme and CDRX scheme.

Observation 19: LP-WUS can provide 72.6%-94% PSG with 39%-67.7% UPT loss for FTP 3 traffic. PDCCH skipping and switching and DCI 2-6 scheme can provide 83.4% PSG with 77.9% UPT loss for FTP 3 traffic. CDRX can provide 72.7% PSG with 70.9% UPT loss for FTP 3 traffic.

- Conclusions

In this contribution, we have discussed evaluation on LP-WUS. We make the following observations and proposals:

Observation 1: Compared with DRX with/without PEI, the LP-WUS for one UE or multiple UEs can achieve lower power consumption when PWUR on=0.01, PWUR_on=0.5 and PWUR on=1.

Observation 2: For Case 1, compared with eDRX with/without PEI, the LP-WUS brings more UE power consumption when PWUR on=0.5, 1, or 20.

Observation 3: For Case 2, compared with eDRX with/without PEI, the LP-WUS brings more UE power consumption when PWUR on=1 or 20.

Observation 4: Compared with LP-WUS with always on monitoring, the extra power consumption caused by LP-WUS with on-off duty cycled monitoring is small when the WUR-on power is 0.01, 0.5 and 1.

Observation 5: For Mode 1, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 4) with on-off duty cycled has power saving gain only when duty cycle rate is < =5% for Case 1 and 6% for Case 2 and duty cycle is 1.28s.

Observation 6: For Mode 2, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 4) with on-off duty cycled has power saving gain when on duration=1ms with duty cycle >90ms, on duration=2ms with duty cycle >110ms, on duration =4ms with duty cycle>130ms for Case 1 and on duration=1ms with duty cycle >80ms, on duration=2ms with duty cycle >100ms, on duration =4ms with duty cycle>120ms for Case 2.

Observation 7: For Mode 1, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 5) with on-off duty cycled has power saving gain only when duty cycle rate is < =2% for case 1 and duty cycle is 1.28s.

Observation 8: For Mode 2, compared with DRX with PEI, the OFDM sequence-based LP-WUS (Alt 5) with on-off duty cycled has power saving gain when on duration=1ms with duty cycle >300ms, on duration=2/4ms with duty cycle >350ms for Case 1.

Observation 9: For Case 1, when N is relaxed to 16, LP-WUS has power saving gain compared with DRX with PEI when R_E=1% and PWUR on=0.5.

Observation 10: For Case2, when N is relaxed to 30, LP-WUS has power saving gain compared with DRX with PEI when R_E=1% and PWUR on=0.5.

Observation 11: For Case 1/Case 2, compared with DRX without PEI, the LP-WUS has power saving gain when the probability of extra MR power on caused by FAR is lower than 10% /5%.

Observation 12: Compared with DRX/eDRX, the latency reduction of LP-WUS based on legacy PO is -115%/94.7%.

Observation 13: Compared with DRX/eDRX, the latency reduction of LP-WUS based on dynamic PO is 29.6%/98.2%.

Observation 14: Compared with legacy PO, the latency reduction of LP-WUS based on dynamic PO is 67.3%.

Observation 15: The latency for duty cycle monitoring mechanism would be larger than that for always on monitoring and the different duty cycle monitoring schemes have different impacts on the latency.

Observation 16: The latency impact caused by miss detection is small if dynamic PO is used.

Observation 17: For FAR definition, independent detection cannot be assumed for Alt1, unless the step size for sliding is larger than the length of the sequence.

Observation 18: LP-WUS can provide 26% PSG for XR traffic with 30Mbps in InH. PDCCH skipping and SSSG switching can provide 16%-17% PSG for XR traffic with 30Mbps in InH.

Observation 19: LP-WUS can provide 72.6%-94% PSG with 39%-67.7% UPT loss for FTP 3 traffic. PDCCH skipping and switching and DCI 2-6 scheme can provide 83.4% PSG with 77.9% UPT loss for FTP 3 traffic. CDRX can provide 72.7% PSG with 70.9% UPT loss for FTP 3 traffic.

Proposal 1: The following KPIs on LP-WUS should be further evaluated

- System overhead

- Network power consumption

- Co-existence impacts

Proposal 2: Clarify the latency definition and for different RRC states.

Proposal 3: The relative power of ultra-deep sleep is 0.015 unit for LP-WUS power consumption evaluation.

Proposal 4: The ramp up and down transition energy and ramp-up time of ultra-deep sleep is 15000 units*ms and 400ms for IoT/Wearable cases and 40000 units*ms and 800ms for eMBB cases for power consumption evaluation.

Proposal 5: For LP-WUS power consumption evaluation, at least 3 SSBs are required for sync/re-sync when main radio wakes up from the state of ultra-deep sleep.

Proposal 6: For OFDM based receiver, WUR ‘on’ relative power should be no less than 20 and WUR ‘off’ relative power should be 0.01.

Proposal 7: The relative power of WUR on is 0.01, 0.5, 1, 20 and 40 for LP-WUS power consumption evaluation.

Proposal 8: For LP-WUS power consumption evaluation, the 10ms ramp-up time of WUR could be used as the starting point.

Proposal 9: Whether deep sleep or ultra-deep sleep state is assumed outside of PTW should be determined for eDRX power consumption evaluation.

Proposal 10: Dynamic PO should be considered for LP-WUS.

Proposal 11: FAR P is defined in a time duration T

- The FAR for each detection is LLS is less than x0, where x0 is the FAR for the detection when the step size for sliding is the sequence length L, satisfying P=

- The reference time duration is a DRX cycle.

- MDR has the similar definition.

Proposal 12: The target coverage of LP WUS should be better than PUSCH.

Proposal 13: Discuss the LP-WUS transmission assumption in idle/inactive mode for evaluation and capture how to calculate the system overhead.

Proposal 14: For NW power evaluation, discuss the LP-WUS or LP-SS transmission assumption, and load scenarios in idle/inactive mode

- References

RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94-e

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97-e.

3GPP, RAN1 Chairman’s Notes, RAN1#112

3GPP, RAN1 Chairman’s Notes, RAN1#110bis-e