# 3GPP TSG RAN WG1 #112bis-e	                  	 R1-2302949

# e-Meeting, April 17th– 26th, 2023

# Source: 	ZTE, Sanechips

# Title:	LP-WUS receiver architectures

# Agenda item: 	9.11.2

# Document for:	Discussion and decision

Introduction

In RAN #94 meeting¸ new study item on low-power Wake-up Signal and Receiver for NR [1] was approved and the latest revision was approved in [2]. In last RAN1#112 meeting, some agreements regarding the receiver architectures are captured in the chairman notes [3].

Agreement Study the parallel receiver architectures (as examples that can be captured in the TR) for FSK based on the following diagrams: - Parallel homodyne architecture - The observations made for homodyne/zero-IF architecture with baseband envelope detection in RAN1#110b/111 are also applicable here. - Parallel heterodyne architecture - The observations made for heterodyne architecture with IF envelope detection in RAN1#110b/111 are also applicable here. - Note: Other architectures are not precluded. - The OOK receiver architectures agreed for study in RAN1#110bis-e are also examples that can be captured in the TR Agreement Study the receiver architectures (as examples that can be captured in the TR) for FSK with frequency to amplitude conversion based on the following diagrams: - Homodyne architecture with frequency to amplitude conversion - I/Q branches are required for frequency to amplitude conversion in digital BB. - Heterodyne architecture with frequency to amplitude conversion - Companies provide the exact type of frequency to amplitude conversion being studied. - Note: Other architectures are not precluded. Agreement For OFDMA-based signals/channels, study the receiver architectures based on the following diagrams: - I/Q branches are required for digital BB processing. - Digital BB processing may or may not include FFT (companies to provide details on how). - For sequence-based OFDM signals/channels, digital BB processing includes sequence correlation in either time domain (without FFT) or frequency domain (after FFT). - Proponent companies should at least provide details on power consumption reduction compared to the MR regarding the RF and digital BB processing. - Companies are encouraged to provide the break-down for the components. - The potential power reduction compared to the main radio may come from e.g.: - Lower performance LNA/amplifier - Oscillator/PLL with relaxed performance requirements - ADC with lower sampling rate and smaller bit-width - Reduced BB processing complexity compared to the MR - Companies are encouraged to provide the performance analysis corresponding to the considered power consumption considering the impact of e.g. phase noise, I/Q mismatch. - Companies to report whether the LP WUR is assumed to share components with MR. In case of component sharing, the potential impact on the MR ultra-deep sleep state should be considered. - Companies to report the possible number of information bits - In addition, companies should consider the power consumption in the OFF state and the transition energy. Agreement For the study on LP WUR architecture, power consumption relative to the deep sleep state of the MR is provided. - Deep sleep state of non-RedCap UE should be assumed

In last meeting, receiver architectures for FSK detection and OFDM based receiver were preliminarily discussed but no consensus was reached. In this contribution, we further discuss the above two architectures.

FSK receiver architectures

Parallel receiver architectures

In RAN1#112, two kinds of parallel receiver architectures for FSK are given. In this section, take parallel homodyne architecture for example, the receiver architecture is shown as Figure1.

Figure 1. Parallel homodyne architecture for FSK

- Transmitting Waveform Type 1: Single-SC FSK

Take 2FSK as an example, 1bit information can be carried by two different subcarriers. For parallel homodyne 2FSK receiver, an oscillator is needed to convert RF signal to baseband. If lower power consumption LO (such as Ring Oscillator) is used, large frequency offset caused by its frequency drift will have serious impact on the detection performance of 2FSK.

For Ring Oscillator, the maximum frequency error is assumed as 200ppm@2.6GHz, and thus the maximum frequency offset = 200*10^-6*2.6*10^9KHz =520KHz. Moreover, if two subcarriers with SCS=30KHz are allocated for single-SC FSK transmission, in order to mitigate the influence of up to 520KHz frequency offset, a guardband with 520KHz at both side of each 2FSK signal is necessary. The total BW required for guardband is 520*2*2=2080KHz, it is about 2080/60= 35 times of BW required for single-SC FSK. It will cause extremely low frequency efficiency.

Observation 1: For Ring Oscillator with maximum frequency error of 200ppm@2.6GHz, if single-SC FSK with SCS=30KHz is used for LP-WUS transmission, about 35 times of BW required for the single-SC FSK should be allocated for guardband to mitigate the influence of up to 520KHz frequency offset. It will cause extremely low frequency efficiency.

Proposal 1: Single-SC FSK signal is not pursued for parallel receiver architecture.

- Transmitting Waveform Type 2: Multiple-SCs FSK

For Multiple-SCs FSK transmission, multiple SCs are occupied in frequency domain, the problem of extremely low frequency efficiency in Single-SC FSK does not exist. In [4], reception performance of 1-bit and 2-bit Multiple-SCs FSK are evaluated and the corresponding simulation results are shown in Figure 2. It can be seen that the reception performance of 2-bits FSK is obviously worse than that of 1-bit FSK.

Figure 2. Reception performance of 1-bit and 2-bit Multiple-SCs FSK

Observation 2: For Multiple-SCs FSK transmission with parallel receiver architecture, the reception performance of 2-bits FSK is obviously worse than that of 1-bit FSK.

Proposal 2: For Multiple-SCs FSK transmission with parallel receiver architecture, 1-bit FSK is prioritized.

Frequency to amplitude conversion receiver architectures

Based on the agreement on frequency to amplitude conversion receiver architectures in RAN1#112 meeting, bit information is distinguished in accordance with the amplitude value mapped from the frequency location of an FSK signal transmitting. Therefore, this receiver architecture is more applicable for single-SC FSK reception.

Observation 3: Frequency to amplitude conversion receiver architecture could be applicable for single-SC FSK.

If lower power consumption LO (such as Ring Oscillator) is used, large frequency offset caused by its frequency drift will have serious impact on the detection performance of FSK. For example, 2FSK signal is generated in frequency f1 or f0, considering frequency offset caused by frequency drift of LO, the received 2FSK signal is in a range of Range1 = [(f1-foffset), (f1+foffset)] or Range2 = [(f2-foffset), (f2+foffset)], where foffsetis the maximum frequency offset caused by frequency drift of LO. Therefore, the design of frequency to amplitude conversion must ensure an obvious difference of amplitude from frequency mapping of Range1 and Range2. In this way, the design of the conversion is much more complicated.

Observation 4: For frequency to amplitude conversion receiver architecture, the conversion design must ensure obvious amplitude difference between multiple frequency ranges, where each frequency range is composed with potential frequency locations of FSK received signal caused by frequency drift of LO. In this way, the design of the conversion is much more complicated.

Furthermore, for Ring Oscillator, the maximum frequency error is assumed as 200ppm@2.6GHz, thus the maximum frequency offset = 200*10^-6*2.6*10^9KHz=520KHz. In this way, the size of Range1 or Range 2 is 1040KHz, thus this kind of FSK receiver architecture will also cause extremely low frequency efficiency.

Observation 5: For single-SC FSK transmission and frequency to amplitude conversion receiver architecture, it will cause extremely low frequency efficiency.

- Example of frequency to amplitude conversion: Quadrature frequency discriminator

The principle of FM discriminator is shown in Figure 3 which is summarized in RAN1#110bis-e meeting.

Figure 3. Principle of quadrature frequency discriminator

Take 2FSK for an example, the principle of quadrature frequency discriminator includes:

Step 1: 2FSK signal transmitted on a frequency carrier firstly goes through a BPF where central frequency equals to the transmitting frequency band, thus the 2FSK signal is transferred to a FM signal defined as s(t)=Acos(2π(fref+f)t), where fref is a reference frequency, f is a variable frequency and for 2FSK f = f1 or f2which are frequency carriers of 2FSK transmission.

Step 2: FM signal, s(t), is divided into two copies. One copy is directly sent to Mixer, another copy is sent to the Mixer after a time delay, τ. After that, the output signal of the Mixer is y(t), where

y(t)=Acos(2π(fref+f)t)×Acos(2π(fref+f)(t-τ))

=A2/2×cos(2π(fref+f)τ)+A2/2×cos(2×2π(fref+f)t-2π(fref+f)τ)

Step 3: Then y(t) passes an LPF and higher frequency is filtered out, the output signal is y1(f), where

y1(f) = A2/2×cos(2π(fref+f)τ)

From this formula, it can be seen that y1(f) is a value of amplitude and is a function of variable f. Moreover, the function is not linear. In order to get a linear function for good performance of FM to AM conversion, proper assumptions and careful selection of the value of τ should be done firstly.

Assumption 1: Carefully select the value of τ, to make sure cos(2πfrefτ)=0, then sin(2πfrefτ)=±1 . Therefore,

y1(f) =± A2/2×sin(2πfτ)

Assumption 2: If 2πfτ≪1, then sin(2πfτ)≈2πfτ. Therefore,

y1(f) ≈± A2/2×2πfτ = ± A2π∙τ∙f

So based on Assumption 1 and Assumption 2, y1(f) becomes a linear function of frequency carrier f.

Observation 6: Regarding quadrature frequency discriminator, in order to get a good performance of FM to AM conversion, value of phase shift,τ, should be carefully selected to make sure cos(2πfrefτ)=0 and 2πfτ≪1.

For LP-WUS, in order to have significant power saving gain, ring oscillator may be used as the LO in LP-WUS receiver and PLL or FLL may not be supported. Considering the larger frequency drift of Ring oscillator, the reference frequency carrier generated by LO have obvious frequency offset, and thus the Assumption 1 of cos(2πfrefτ)=0 can not be guaranteed. Therefore, the performance from FM to AM conversion will also be degraded.

Observation 7: For FSK receiver using quadrature frequency discriminator, the reception performance will be degraded if lower power consumption oscillator, e.g., ring oscillator, is used as LO since the large frequency drift of the LO leads to a non-linear transfer from FM to AM by quadrature frequency discriminator.

Furthermore, the FSK signal should firstly pass a BPF as described in Step 1, the bandwidth of BPF should be at least include the bandwidth from f1 to f2, in order to ensure the performance of FM discriminator from FM to AM, the other frequency resources among this bandwidth may not be used for other LP-WUSs or legacy NR data transmission, unless additional guardband is defined. Therefore, it results in severe bandwidth utilization efficiency reduction.

Observation 8: For FSK receiver using quadrature frequency discriminator, bandwidth between frequency carriers used for FSK transmission may not be used for other LP-WUSs or legacy NR transmission.

Proposal 3: For FSK receiver using quadrature frequency discriminator, the following issues can be further clarified

- How to get a preferred shifted phase value;

- Bandwidth requirement of FSK;

OFDM based Receiver

- Whether to support FFT

In [4], sequence-based OFDM signal receiver in time domain (Receiver 1, without FFT) or frequency domain (Receiver 2, after FFT) have similar detection performance when Time/Frequency error is not assumed as shown in Figure 4, where the signal is generated by ZC sequence.

Figure 4. Link level performance of sequence-based modulation based on receiver 1 and receiver 2

That is, for the detection of ZC-sequence based signal, if DL synchronization can be ensured for sequence-based OFDM signal receiver, FFT process is not necessary in digital BB processing.

Observation 9: If DL synchronization can be ensured and detection accuracy of sequence-based OFDM signal LR receiver is close to that of MR receiver, the detection performance of ZC-sequence-based OFDM signal in time domain (without FFT) or frequency domain (after FFT) is similar.

When certain time error is assumed for sequence-based OFDM signal receiving, if it is in the protection of CP, the sequence-based OFDM signal detection performance in frequency domain (after FFT) will not be severely degraded. For sequence-based OFDM signal detection performance in time domain (without FFT), a sliding correlation detection in a time window should be used to mitigate the influence of the time error. Moreover, as the increase of time window, the power consumption of sliding correlation detection will increase obviously.

Observation 10: If timing error exists for sequence-based OFDM signal receiving in time domain (without FFT), a sliding correlation detection in a time window should be used to mitigate the influence of the time error.

- As the increase of time window, the power consumption of sliding correlation detection will increase obviously.

- Oscillator/PLL with relaxed performance requirements

In [4], the performance of sequence-based OFDM signal receiver with frequency error is evaluated, and simulation results are shown in Figure 5. It can be seen that sequence-based OFDM signal receiver will be seriously affected by frequency error even only 5ppm max frequency error is assumed.

| - BLER performance based on receiver 1 | - BLER performance based on receiver 2 |
| --- | --- |

Figure 5. Link level performance of sequence-based modulation with/without frequency offset

Therefore, to ensure the detection performance of the OFDM based receiver, a high-accuracy oscillator must be used as LO. Furthermore, PLL or FLL should be supported to correct the frequency error periodically to mitigate the accumulation of frequency error.

Observation 11: For sequence-based OFDM signal receiving, a high-accuracy oscillator and PLL/FLL should be used to mitigate the impact of frequency error on detection performance.

- ADC bits

For sequence-based OFDM signal receiving, take ZC sequence for example, I/Q branches are required in BB processing, in order to ensure the sequence correlation performance, larger ADC bits is necessary compared with MC-OOK based signal. We can take the number of ADC for MR as a starting point for sequence-based OFDM signal of LP-WUS.

Observation 12: Larger ADC bits is needed for sequence-based OFDM signal of LP-WUS.

Proposal 4: For OFDM sequence-based receiver, further clarifications for following aspects are needed.

- oscillator and PLL/FLL

- ADC

- FFT

- LNA

- BB processing including correlation or not, buffer size

Conclusions

In this contribution, we have discussed the receiver architectures for LP-WUS. The following observations and proposals are made.

Observation 1: For Ring Oscillator with maximum frequency error of 200ppm@2.6GHz, if single-SC FSK with SCS=30KHz is used for LP-WUS transmission, about 35 times of BW required for the single-SC FSK should be allocated for guardband to mitigate the influence of up to 520KHz frequency offset. It will cause extremely low frequency efficiency.

Observation 2: For Multiple-SCs FSK transmission with parallel receiver architecture, the reception performance of 2-bits FSK is obviously worse than that of 1-bit FSK.

Observation 3: Frequency to amplitude conversion receiver architecture could be applicable for single-SC FSK.

Observation 4: For frequency to amplitude conversion receiver architecture, the conversion design must ensure obvious amplitude difference between multiple frequency ranges, where each frequency range is composed with potential frequency locations of FSK received signal caused by frequency drift of LO. In this way, the design of the conversion is much more complicated.

Observation 5: For single-SC FSK transmission and frequency to amplitude conversion receiver architecture, it will cause extremely low frequency efficiency.

Observation 6: Regarding quadrature frequency discriminator, in order to get a good performance of FM to AM conversion, value of phase shift,τ, should be carefully selected to make sure cos(2πfrefτ)=0 and 2πfτ≪1.

Observation 7: For FSK receiver using quadrature frequency discriminator, the reception performance will be degraded if lower power consumption oscillator, e.g., ring oscillator, is used as LO since the large frequency drift of the LO leads to a non-linear transfer from FM to AM by quadrature frequency discriminator.

Observation 8: For FSK receiver using quadrature frequency discriminator, bandwidth between frequency carriers used for FSK transmission may not be used for other LP-WUSs or legacy NR transmission.

Observation 9: If DL synchronization can be ensured and detection accuracy of sequence-based OFDM signal LR receiver is close to that of MR receiver, the detection performance of ZC-sequence-based OFDM signal in time domain (without FFT) or frequency domain (after FFT) is similar.

Observation 10: If timing error exists for sequence-based OFDM signal receiving in time domain (without FFT), a sliding correlation detection in a time window should be used to mitigate the influence of the time error.

- As the increase of time window, the power consumption of sliding correlation detection will increase obviously.

Observation 11: For sequence-based OFDM signal receiving, a high-accuracy oscillator and PLL/FLL should be used to mitigate the impact of frequency error on detection performance.

Observation 12: Larger ADC bits is needed for sequence-based OFDM signal of LP-WUS.

Proposal 1: Single-SC FSK signal is not pursued for parallel receiver architecture.

Proposal 2: For Multiple-SCs FSK transmission with parallel receiver architecture, 1-bit FSK is prioritized.

Proposal 3: For FSK receiver using quadrature frequency discriminator, the following issues can be further clarified

- How to get a preferred shifted phase value;

- Bandwidth requirement of FSK;

Proposal 4: For OFDM sequence-based receiver, further clarifications for following aspects are needed

- oscillator and PLL/FLL

- ADC

- FFT

- LNA

- BB processing including correlation or not, buffer size

- References

RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94-e

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97-e.

3GPP, RAN1 Chairman’s Notes, RAN1#112

R1-2302950, “LP-WUS design and related procedure”, ZTE, RAN1#112bis-e