3GPP TSG RAN WG1 #112bis                          R1-2302960

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.1.1.2

Source:	Xiaomi

Title:	Discussion on two TAs for multi-TRP operation

Document for:	Discussion and Decision

- Introduction

The Rel-18 WI on NR MIMO evolution for downlink and uplink is approved in RAN#94-e meeting [1], which includes the following objective.

7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss on two TAs for UL multi-DCI for multi-TRP operation.

- Discussion on two TAs for UL multi-DCI for multi-TRP operation

  - DL reference timing

In RAN1#110b-e meeting [2], the following agreement regarding DL reference timing has been agreed for multi-TRP operation.

Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

In multi-TRP transmission, since the two cooperative TRPs are generally not geographically co-located, the distances between different TRP-UE pairs are typically different. As a result, the DL/UL transmissions between TRPs and UE experience different propagation time delays. In RAN1#110b-e meeting, two DL reference timings are supported where each DL reference timing is associated with one TAG. Rx timing difference between the two DL reference timings no larger than CP length is the basic assumption while timing difference larger than CP length is agreed as an optional UE capability. In this case, the maximum Rx timing difference can be calculated based on the maximum TA difference according to the LS out from RAN4 [3]. When TRP-specific TAs and DL reference timings are adopted, the following case can be considered as illustrated in Fig 1. Without loss of generality, we assume that TRP#1 is closer to UE than the TRP#2. Then the maximum Rx timing difference can be computed as 12TA1-TA2.

Fig 1 Illustration of two TAs and TA reference timings.

When Rx timing difference between the two DL reference timings is larger than CP length, the maximum Rx timing difference can be calculated by 12TA1-TA2 where the maximum TA difference is based on the LS out from RAN4.

  - PRACH configuration

In RAN1#110b-e and RAN1#112 meetings [2][4], the following agreements regarding PRACH configuration have been agreed for inter-cell multi-TRP operation with two TA enhancement.

Agreement Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI Agreement For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order FFS: Explicit indication or implicit indication through PDCCH order

In inter-cell multi-TRP scenario, the UE transmits a preamble towards the serving cell to establish the initial random access. Then the UE performs random access towards the TRP of non-serving cell by autonomous initiation or PDCCH order triggering to acquire the corresponding TA value. In RAN1#112 meeting, the working assumption on supporting one additional PRACH configuration for each configured additional PCI has been confirmed.

When one separate PRACH configuration is configured for additional PCI, a new information about which RACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to use in the PDCCH order should be introduced. Generally, it can be solved by explicit indication or implicit indication via PDCCH order. A new field can be introduced in the PDCCH order to explicitly indicate the serving cell PCI or an activated additional PCI, i.e., using the reserved bits in DCI format 1_0 for indication. Then the UE can distinguish which PRACH configuration to be used in the RACH procedure corresponding to serving cell or non-serving cell.

The explicit indication by using the reserved bits in DCI format 1_0 can be supported to determine which PRACH configuration to be used in the RACH procedure triggered by PDCCH order.

  - Cross TRP PDCCH order

In RAN1#111 meeting [5], the following agreement has been achieved on cross TRP PDCCH order.

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature

For inter-cell multi-DCI based multi-TRP operation, cross TRP PDCCH order has been supported since more scheduling flexibility can be provided by network. Since there is no harm to support cross TRP PDCCH order for intra-cell multi-DCI based multi-TRP operation, a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP can be extended to intra-cell scenario.

For intra-cell multi-DCI based multi-TRP operation, support PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP.

  - TA/TAG identification for intra-cell multi-DCI

In RAN1#110b-e meeting [2], the following agreement has been agreed for intra-cell multi-DCI based multi-TRP operation on TA/TAG identification.

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

In intra-cell multi-TRP, the association between TA command in RAR PDSCH and the TAG should be discussed. A simple way is to explicitly indicate TAG ID as part of TA command in RAR, which uses the reserved bit in RAR. Note that there is one reserved bit in MAC RAR. Similar with Alt 1, Alt 2 indicates TAG ID as part of PDCCH order by introducing a new filed. Although the reserved bits in PDCCH order is much more than that of the RAR, however, this alternative may not be workable for UE-initiated RACH procedure. Alt 3/4/5 share the similar idea which divide SSBs/RACH resources/preambles into two groups. For RACH resources and preambles grouping, it may restrict the scheduling flexibility. For SSBs grouping, since two cooperative TRPs are generally not geographically co-located, which results in different SSB beam directions. As a result, the SSBs can be divided into two groups. If a SSB associated to a RACH procedure belongs to the nth group, then the TA can be obtained via the RACH procedure corresponds to the nth TRP. Regarding Alt 6/7, the TAG ID is associated with CORESETPoolIndex and TCI state, respectively. Since PDCCH order sent by one TRP triggers RACH procedure towards the different TRP has been supported at least for inter-cell multi-DCI based multi-TRP, Alt 6/7 are unworkable in this case. Moreover, these schemes may be only suitable for RACH triggered by PDCCH order. Based on the analysis, we prefer Alt 1 and Alt 3.

Prefer Alt 1 and Alt 3 on TA/TAG identification for intra-cell multi-TRP.

  - Association between TAs and UL channels/signals

In RAN1#110b-e and RAN1#112 meetings [2][4], the following agreements regarding the association between TAs and UL channels/signals have been endorsed.

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111): - Option 1: Associate TAG to TCI-state/spatial relation - Configure TAG ID as part of UL/joint TCI state or spatial relation - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized - Option 2: Associate TAG to CORESETPoolIndex - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured. - FFS: Other signals/channels: AP-SRS, and dynamic HARQ-ACK - Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to - Option 4: TAG association performed as follows: - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured. Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, associating TAG ID with UL/joint TCI state is supported in RAN1#112 meeting. As for the working assumption, we support to confirm it and remove the brackets, i.e., a UE may report that it supports that the activated UL/joint TCI states of UL signals/channels associated to one CORESETPoolIndex correspond to both TAGs.

Support to confirm the working assumption and remove the brackets, i.e., a UE may report that it supports that the activated UL/joint TCI states of UL signals/channels associated to one CORESETPoolIndex correspond to both TAGs.

  - Overlapped region handling

In RAN1#111 and RAN1#112 meetings [4][5], the following conclusion and agreement regarding the association between TAs and UL channels/signals have been endorsed.

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, down-select at least one of the following in RAN1#112bis-e: - Alt 1: Introducing a time gap X between two UL transmissions associated with two different TA values - E.g., X symbols in the slot(s) corresponding to the two UL transmission remain unused - FFS: How X is determined - Alt 2: Reduce the overlapping duration of one of the two UL transmissions - Alt 3: Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap - Other alternatives are not precluded TBD: how to capture the downselected alternative(s) in the specifications in case specification impact is deemed needed.

In Rel-17 UL multi-TRP operation, the UL transmissions toward two TRPs is in a TDM manner. When the TRP-specific TAs are supported, the overlap may be occurred between two consecutive UL transmissions. For example, when TRP#1 is closer to UE than the TRP#2, which leading to TA1<TA2. As illustrated i, in this case, the end of the UL transmission towards TRP#1 overlaps with the start of the UL transmission towards TRP#2 in time domain.

Fig 2 Overlap between two consecutive UL transmissions.

In the existing specification, if two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot. However, in the case of TRP-specific TAs, the overlap may happen more frequently, which degrades the performance loss. If legacy dropping rule is used, dropping the last symbols of the former transmission may have high priority in order to avoid dropping the DM-RS of the latter transmission.

What’s more, reserving a time gap is also an efficient scheme to avoid the overlap, and the time gap can be determined by the maximum uplink timing difference based on the LS to RAN4 [3]. In the LS out from RAN4, the MRTD/MTTD value is defined for a UE capable of supporting RTD > CP while the MRTD/MTTD value is FFS for a UE not capable of supporting RTD > CP. In the case of a UE capable of supporting RTD > CP, the MTTD values for FR1 and FR2 are defined as 34.6 µs and 8.5 µs, respectively. Only when SCS=60 kHz and FR1 are adopted, the time of OFDM symbol is 1/60*1000+1.175=17.84 µs, and the overlapping time satisfies 1 OFDM symbol < Toverlap < 2 OFDM symbols. In other cases, 1 OFDM symbol can over the overlapping time. Although both TRPs cannot always have knowledge of the overlapping region between transmissions corresponding to the two TAs, a static time gap can be reserved. As a result, we also support to introduce a time gap of 2 OFDM symbols between two UL transmissions associated with two different TA values.

Support Alt 1 to introduce a time gap of 2 OFDM symbols between two UL transmissions associated with two different TA values.

Support Alt 2 to reduce the overlapping duration of one of the two UL transmissions, and dropping the last symbols of the former transmission may have high priority in order to avoid dropping the DM-RS of the latter transmission.

  - TAG and TAT

In RAN1#110 meeting [6], the following agreement has been made on number of TAGs.

Agreement For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell.

In current spec, the two TRPs belong to the same TAG and configured with the same TAT. The maximum number of TAGs is four and the TAG is indicated by TAG ID of TAC MAC-CE. The TAG containing the SpCell has the TAG Identity 0. Since configuring two TAGs within a serving cell was agreed in RAN1#110, the maximum number of TAGs maybe be increased, e.g., 8 or 16, and the corresponding bit width of TAG ID can be increased to 3 or 4 bits.

Support to increase the maximum number of TAGs and the corresponding bit width of TAG ID of TAC MAC-CE, e.g., maximum 8/16 TAGs and 3/4 bits TAG ID.

When TAT expires, the UE is out of synchronization and the PUCCH/PUSCH/SRS transmission towards multi-TRP are not allowed in existing UL multi-TRP transmission. Then UE performs random access to obtain the TA and reset TAT. In RAN1#110 meeting, configuring two TAGs within a serving cell for multi-DCI based multi-TRP operation was supported, and each TRP/TAG is configured with a TAT by RRC. Suppose the TRP#1 receives TAC and the corresponding TAT expires firstly, the following two cases can be considered.

Case 1: Random access is triggered to obtain the TA and reset TAT for TRP#1, and TRP#2 maintains its own TA.

Case 2: UE transmits PUCCH/PUSCH/SRS towards TRP#1 based on the TA of TRP#2 and TRP#2 maintains its own TA until the TAT of TRP#2 expires, then the random access is triggered to obtain the TAs and reset TATs for TRP#1 and TRP#2.

The UE can trigger the random access or the random access is triggered by PDCCH order.

Suppose the TAT of TRP#1 is expired, the UE can initiate random access to obtain the TA and reset TAT for TRP#1 or transmits PUCCH/PUSCH/SRS towards TRP#1 based on the TA of TRP#2 until the TAT of TRP#2 expires.

- Conclusion

In this contribution, we discuss about two TAs for UL multi-DCI for multi-TRP operation. According to the above discussions, we provide the following observation and proposals.

- When Rx timing difference between the two DL reference timings is larger than CP length, the maximum Rx timing difference can be calculated by 12TA1-TA2 where the maximum TA difference is based on the LS out from RAN4.

The explicit indication by using the reserved bits in DCI format 1_0 can be supported to determine which PRACH configuration to be used in the RACH procedure triggered by PDCCH order.

For intra-cell multi-DCI based multi-TRP operation, support PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP.

Prefer Alt 1 and Alt 3 on TA/TAG identification for intra-cell multi-TRP.

Support to confirm the working assumption and remove the brackets, i.e., a UE may report that it supports that the activated UL/joint TCI states of UL signals/channels associated to one CORESETPoolIndex correspond to both TAGs.

Support Alt 1 to introduce a time gap of 2 OFDM symbols between two UL transmissions associated with two different TA values.

Support Alt 2 to reduce the overlapping duration of one of the two UL transmissions, and dropping the last symbols of the former transmission may have high priority in order to avoid dropping the DM-RS of the latter transmission.

Support to increase the maximum number of TAGs and the corresponding bit width of TAG ID of TAC MAC-CE, e.g., maximum 8/16 TAGs and 3/4 bits TAG ID.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94-e, Dec. 6-17, 2021.

- RAN1#110b-e Chair’s Notes.

- R4-2217279, Reply LS on MTTD for multi-DCI multi-TRP with two TAs, RAN4#104b-e, Oct. 10-19, 2022.

- RAN1#112 Chair’s Notes.

- RAN1#111 Chair’s Notes.

- RAN1#110 Chair’s Notes.