3GPP TSG RAN WG1 #112-bis                                          	     R1-2302962

e-Meeting, April 17th –26th, 2023

Agenda Item:	9.1.3.1

Source:	Xiaomi

Title:                     Discussion on DMRS enhancement

Document for:	Discussion and Decision

Introduction

In RAN 94e meeting, the work item of further enhancements on MIMO for NR with respect to DL MIMO was proposed as follows [1]:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

  - Striving for a common design between DL and UL DMRS

  - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we will further discuss some possible DMRS enhancements to support larger number of orthogonal DMRS ports and DMRS enhancements for 8 Tx UL operation in Rel18.

Discussion on DMRS enhancement to support lager number of DMRS ports

DMRS port(s) table design

STRP

In RAN1 111 meeting, a possible antenna port table as following was agreed and some controversial rows needed more discussion [5].

111-Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | [1] | [0] | Cat. 1 | [0] | [2] | [0,1,2,3,8] | [Rank 5-8 with one DMRS symbol] |
| 1 | [1] | [1] |  | [1] | [2] | [0,1,2,3,8,10] |  |
| 2 | [1] | [0,1] |  | [2] | [2] | [0,1,2,3,8,9,10] |  |
| 3 | 2 | 0 |  | [3] | [2] | [0,1,2,3,8,9,10,11] |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | [2] | [0-2] |  |  |  |  |  |
| 10 | [2] | [0-3] |  |  |  |  |  |
| 11 | [2] | [0,2] |  |  |  |  |  |
| 12 | [1] | [8] | Cat.2 |  |  |  |  |
| 13 | [1] | [9] |  |  |  |  |  |
| 14 | [1] | [8,9] |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | [1] | [0,1,8] | Cat.3 |  |  |  |  |
| 25 | [1] | [0,1,8,9] |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

And during RAN1 112 meeting, we discussed these rows based on the FL proposal and reached some agreements which are shown below [6]:

112 FL Proposal 2.1.1A

- For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following:

  - For 1 CW,

    - 1) For row 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1),

      - Alt.1-1: Support the rows.

      - Alt.1-3: Remove the rows.

    - 2) For row 9-11, 21-22 (rank 3-4 in Cat.1-2, except for row 23),

      - Alt.2-1: Support the rows.

      - Alt.2-2: Modify values in the rows.

      - Alt.2-3: Remove the rows.

    - 3) For row 23 (e.g. To enable 2+2+3 layers in a CDM group),

      - Alt.3-1: Support the rows.

      - Alt.3-2: Modify values in the rows.

      - Alt.2-3: Remove the rows.

    - 4) Additionally include more rows

      - {0,1,2,8,9} for CJT/NCJT

  - 5) For 2 CWs (row 0-3),

      - Alt.3-1: Support the rows.

      - Alt.3-2: Modify values in the rows (e.g. Table 4-1/4-2 below).

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows:

- For 1 CW,

  - 1) Rows 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1)

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, at least support the following rows:

- For 1 CW,

  - 2) Row 9-11

    - For the above rows, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Working Assumption

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs,

- Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |

For row 23, the use case mentioned in FL proposal is to enable 2+2+3 layers in a CDM group. From our understanding, it is not a valid use case. Because there are only 4 DMRS ports in one CDM group when dmrs-Type=eType1 and maxLength=1. How could it possible to support 2+2+3 layers in a CDM group.

While, it could be used to support 3+2 layers in MU-MIMO, like (0,1,2) + (8,10)/(9,11). To guarantee all the possible use case, we support row 23.

Proposal 1: Support row 23.

About supporting additionally row, {0,1,2,8,9} for CJT/NCJT, it should not be included. First, we are talking about antenna port(s) table design for STRP right now. In addition, for NCJT, the layers combination (2+3) should be 2 CWs, which is already considered in the agreement above. We don’t see any use case of {0,1,2,8,9}. And for CJT, which is introduced to support more UEs in MU-MIMO, at most 4 layers for each UE will be supported in MU-MIMO. In summary, there is no need to support {0,1,2,8,9}.

Proposal 2: Do not support additionally row, {0,1,2,8,9}.

In last meeting, we discussed the DMRS ports for 2 CWs and only a working assumption above is reached. For the DMRS ports combinations for 2 CWs, there might be performance loss since DMRS ports from different CDM groups were indicated to UE for the same CW. However, this antenna table is designed for STRP where the channels corresponding to these antenna ports do not have great difference, which, from our understanding, means the performance loss might be not that much. Therefore, we are not a big fan of these DMRS ports combinations for 2 CWs in the working assumption, but we can live with it if most the companies confirm this working assumption.

Proposal 3: We slightly prefer to modify these DMRS ports combinations for 2CWs, but we can live with it if most the companies confirm this working assumption.

MTRP

About the DMRS ports for MTRP, we came to an agreement as following [6]:

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s):

- For one CW, support at least row 30 in the following table.

  - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

- FFS: other rows are not precluded

Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 30 | 2 | 0,2,3 |

In R16, DMRS ports (0,2,3) is additionally added to support flexible layers combination for 3 layers PDSCH in MTRP NCJT. It could be (2+1) when (0,1,2) is indicated or (1+2) when (0,2,3) is indicated. However, for MTRP CJT, all the three layers of PDSCH will be mapped to these TRPs, which, form our understanding, means there is no need to support this kind of flexible layers combination for 3 layers PDSCH.

Proposal 4: (0,2,3) is not needed for MTRP CJT

Signalling design

DMRS type indication

In RAN1 112 meeting, about the switching between R15 DMRS port and R18 DMRS port, the following conclusion was agreed [6].

Conclusion

Dynamic switching between R15 DMRS port and R18 DMRS port by a scheduling DCI is not supported in Rel-18

In current specification, DMRS type is indicated via RRC parameter dmrs-type which can be set to type1 or type2. In R18, enhanced DMRS will be introduced to support larger number of DMRS ports and there are another two types of DMRS which are denoted as eType1 and eType2. Then, how to indicate UE one of these four DMRS types should be specified.

We can still use RRC parameter dmrs-type to indicate UE one of these four DMRS types. To do that, a new RRC indicator can be introduced to inform UE whether the RRC parameter dmrs-type indicates legacy DMRS type or R18 DMRS type. For example, when the indicator is set to “A” and dmrs-type is “type2”, then, the DMRS type is legacy DMRS Type2. And if the indicator is set to “B” and dmrs-type is “type2”, the DMRS type is R18 DMRS eType2.

Proposal 5: Introduce a new RRC indicator to indicate the legacy DMRS or enhanced DMRS to UE.

DMRS port(s) indication

Because the supported DMRS port is doubled in R18 DMRS, how to indicate UE the DMRS port(s) should also be specified. A straight forward way to do this is to extend the current antenna field in DCI. While, there is another possible way where a “DMRS port(s) offset indicator” field can be introduced to indicate R18 DMRS port(s). For example, if this field is set “1”, DMRS port(s) are incremented with X from the indicated DMRS port(s) by antenna port(s) field.

However, this “DMRS port(s) offset indicator” method has less flexibility of network scheduling. For example, the table for dmrs-Type=1and single symbol, are selected among DMRS port 0~3, then the DMRS port of R18 DMRS indicated to UE can only be selected among port 0~3 or port 4~7 in this method. What’s more, Cat. 3 DMRS port indexes cannot be supported in this method.

Observation 1: Introducing a “DMRS port(s) offset indicator” field to indicate R18 DMRS port(s) may lead to less flexibility of network scheduling and Cat. 3 DMRS port indexes cannot be supported.

To increase the flexibility of DMRS port(s) indication, M>1 bits “DMRS port(s) offset indicator” can be introduced to indicate R18 DMRS port. For example, as shown in Figure1, assuming the DMRS port(s) indicated by antenna ports field in DCI is {0, 1}, if the offset is 2, then the actual DMRS ports are {3, 4}. If the offset is 3, then the actual DMRS ports are {4, 5}.

Fig.1 M>1 bits “DMRS port(s) offset indicator”

Proposal 6: M>1 bits “DMRS port(s) offset indicator” can be introduced to increase the flexibility of DMRS port indication.

While, even introducing M>1 bits “DMRS port(s) offset indicator” can slightly increase the flexibility of DMRS port indication, but, still, some DMRS ports combination cannot be indicated in this method. For example, no matter what the value the DMRS port(s) offset indicator is, DMRS port {0,1,8} cannot be indicated. The simplest and effective method is to increase the size of antenna ports field.

Proposal 7: Increase the size of antenna ports field DCI is a more appropriate way to indicate R18 DMRS port(s) compared with introducing a “DMRS port(s) offset indicator” field.

DMRS port(s) table of PUSCH

Based on the WI, the DMRS for PUSCH also needs to be enhanced to support larger number of orthogonal DMRS ports. Correspondingly, there antenna table for PUSCH should be specified for R18 DMRS. For example, the following enhanced antenna port(s) table for single symbol eType1 DMRS of PUSCH can be supported, in which the exact values can be further discussed.

Table 7.3.1.1.2-8X: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 1 | [8] |
| 7 | 1 | [9] |

Table 7.3.1.1.2-9X: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4 | 1 | [8,9] |
| 5 | 2 | [8,9] |
| 6 | 2 | 10,11 |
| 7 | 2 | [8,10] [9,11] |

Table 7.3.1.1.2-10X: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 2 | [8-10] |
| 2 | 1 | [0,1,8] |
| 3 | 2 | [0,1,8] |
| 4 | 2 | [2,3,10] |

Table 7.3.1.1.2-11X: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 2 | [8-11] |
| 2 | 1 | [0,1,8,9] |
| 3 | 2 | [0,1,8,9] |
| 4 | 2 | [2,3,10,11] |
| 5-7 | Reserved | Reserved |

Proposal 8: For PUSCH with CP-OFDM, enhanced antenna port(s) tables, like the tables shown above, should be introduced and the necessary DMRS port combinations can be further discussed.

Coexistence of R15 DMRS and R18 DMRS in MU-MIMO

In previous meetings, most companies preferred to support coexistence of R15 DMRS ports and R18 DMRS ports in MU-MIMO and we have reached an agreement about the coexistence as shown below [2]:

Agreement

Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports.

- For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed).

- For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH.

  - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs.

- Note: PUSCH above is CP-OFDM waveform.

In RAN1 112 meeting, there is a FL summary about multiplexing between Rel.15 DMRS ports and Rel.18 DMRS ports:

FL Proposal

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 1) For PUSCH, there is no restriction.

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note: MU-MIMO restriction in Rel.17 is applied.

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

    - UE does not expect such MU-MIMO in a CDM group.

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

    - Alt.1: UE does not expect such MU-MIMO in a CDM group.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

    - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

About 3), multiplexing between R15 UE and R18 UE with Rel-18 New ports in a CDM group is still possible. For example, if R15 UE is indicated with DMRS port 0 which applies OCC [+1,+1]T, then for R18 UE, DMRS port(s) using FD-OCC with index 3, which is port 9, can be indicated. As shown in the figure below, the is no interference between R15 UE with port 0 and R18 UE with port 9.

| FD-OCC index | wf(0) | wf(1) | wf(2) | wf(3) |
| --- | --- | --- | --- | --- |
| 0 | +1 | +1 | +1 | +1 |
| 1 | +1 | -1 | +1 | -1 |
| 2 | +1 | +1 | -1 | -1 |
| 3 | +1 | -1 | -1 | +1 |

Fig.2 R15 UE with port 0 and R18 UE with port 9

Proposal 9: About case 3) in the FL proposal, multiplexing between R15 UE indicated with Rel.15 DMRS ports and R18 UE indicated with Rel-18 New ports and in a CDM group is possible.

Enhancements on DMRS ports for 8Tx UL SU-MIMO

The Rel-18 WID for MIMO Evolution for Downlink and Uplink is approved [1], which includes the following objective:

Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

- Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In RAN1#109-e meeting, the following agreement has been made on DMRS configuration [2]:

Agreement

Study the following potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH.

  - Extend DMRS port allocation table for rank 5~8

    - Note: DL DMRS table can be a reference

  - Enhancement for DMRS to PTRS mapping

- Study whether to utilize Rel.18 DMRS ports for more than 4 layers SU-MIMO PUSCH.

- Note: the above study does not imply more than 4 layers SU-MIMO PUSCH is supported.

- Note: other study for potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH is not precluded.

In RAN1#110 meeting, the following agreement has been made on 8Tx transmission and DMRS configuration [3]:

Agreement

For > 4 layers PUSCH, support rank = 5,6,7,8 for both DMRS type 1/2, and for both single-symbol/double-symbol DMRS.

In RAN1#110bis meeting, the following agreement has been made on 8Tx transmission and DMRS configuration [4]:

Agreement

For more than 4 layers SU-MIMO PUSCH, support

- Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports.

  - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports.

    - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed.

  - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

In RAN1#111 meeting, the following agreement has been made on 8Tx transmission and DMRS configuration [5]:

Agreement

- For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS.

  - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following:

    - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

  - For Rel.18 eType1/eType2 DMRS ports,

    - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details).

    - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked.

  - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group.

  - FFS: One or more than one DMRS port combination(s) for each rank and TPMI

  - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17.

  - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

In RAN1#112 meeting, the following agreement has been made on 8Tx transmission and DMRS configuration [6]:

Working Assumption

To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

- Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

  - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

For the uplink transmission with 8Tx, it needs further enhancements on DMRS design if it is agreed to support the maximum number of transmission layers to 8. In current spec, only 4 transmission layers for the PUSCH is supported. It has been decided that whether the supported maximum number of uplink layers is more than 4 or not is up to the UE capability. And for UEs supporting more than 4 layers, DMRS ports indication should be extended.

DMRS enhancements

Since the scenario mainly targeting SU-MIMO for 8Tx enhancement, DMRS port allocation design would be quite aligned with the DL, the DMRS port allocation for RANK5-8 should be introduced to each category of DMRS table. Currently the DMRS port allocation table is categorized according to DMRS type, max number of front-load DMRS symbols, and under each category DMRS tables for Rank 1-4 are specified separately. For DMRS type 1 with single-symbol configuration, the maximum number of DMRS ports is 4 which can’t support the RAN5 indication for 8Tx transmission. And also for DMRS type 2 with single –symbol configuration, the maximum number of DMRS ports is 6 which can support up to RAN6 of DMRS port indication. For DMRS Type 1/2 with double-symbol configuration cases, the supported RANK can be from 5 to 8.

For partial coherent UL codebook, we support layers to DMRS port mapping that layers associated with the same antenna port group can be multiplexed into the same or different DMRS CDM groups, which is aligned with the current principle of UL DMRS ports allocation. No scheduling limitation is required which is not the same principle as the DL.

It is necessary to support more than one DMRS port combinations for each Rank and TPMI that can provide more flexibility to the network especially for higher Rank cases since more DMRS ports can be allocated when new Rel-18 DMRS type is applied.

Proposal 10: Confirm the Working Assumption to introduce separate DMRS port tables for DMRS eType1/eType2.

Proposal 11: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated with the same antenna port group are multiplexed into the same or different DMRS CDM group.

Proposal 12: Support more than one DMRS port combination(s) for each rank and TPMI at least for Rel.18 eType1/eType2 DMRS design.

PT-RS enhancements

Also if DMRS ports are extended to 8 for more than 4 layers PUSCH transmission, PT-RS port(s) associated with DMRS port(s) should be redefined too. The enhancements is to indicate the association between PTRS port(s) and DMRS port(s) for the transmission of one PT-RS port and two PT-RS ports respectively

The following agreement was made in RAN1#110bis meeting on 8Tx and PT-RS enhancements [3]:

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

Agreement

For support of more than 4 layers SU-MIMO PUSCH, study the following potential enhancements for PTRS-DMRS association.

- Whether to support more than 2-port UL PTRS.

- Whether to increase the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2.

In RAN1#112 meeting, the following agreement was made on 8Tx PT-RS enhancements [6]:

Agreement

For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update

- Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2.

  - FFS: Association with The CW with the higher MCS is selected in case of two CWs.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW with the higher MCS |
| 1 | 2nd scheduled DMRS port the CW with the higher MCS |
| 2 | 3rd scheduled DMRS port the CW with the higher MCS |
| 3 | 4th scheduled DMRS port the CW with the higher MCS |

To support more than 4 layers PUSCH with current 2-port PT-RS, the PTRS and DMRS association for 8Tx needs to be enhanced:

- 4bits are needed for the indication of PTRS and DMRS association when 2 PTRS ports are used, 2bits MSB  are for the indication of PTRS port 0, and 2 bits LSB are for the indication of PTRS port 1;

Proposal 13: For Rank>4, support the PTRS and DMRS association of 2-port PTRS for 8Tx when 2-port PTRS is used,

- 4bits are needed for the indication whereas 2bits MSB  are for the indication of PTRS port 0, and 2 bits LSB are for the indication of PTRS port 1;

If antenna ports can be divided into 4 antenna groups, eg. 4 separate panels, 2- port PT-RS may not be enough for the all CPE resources. 4-Port PTRS can be considered from performance point of view. The spec efforts should also be considered.

Proposal 14: Support 4-port PTRS for 8Tx transmission.

Conclusion

In this contribution, we discuss about the DMRS enhancements to support CJT and 8Tx UL transmission. Based on above discusses, we provide the following proposals related to DMRS enhancement for CJT:

Proposals wrt. DMRS enhancement to support lager number of DMRS ports:

Proposal 1: Support row 23.

Proposal 2: Do not support additionally row, {0,1,2,8,9}.

Proposal 3: We slightly prefer to modify these DMRS ports combinations for 2CWs, but we can live with it if most the companies confirm this working assumption.

Proposal 4: (0,2,3) is not needed for MTRP CJT

Proposal 5: Introduce a new RRC indicator to indicate the legacy DMRS or enhanced DMRS to UE.

Observation 1: Introducing a “DMRS port(s) offset indicator” field to indicate R18 DMRS port(s) may lead to less flexibility of network scheduling and Cat. 3 DMRS port indexes cannot be supported.

Proposal 6: M>1 bits “DMRS port(s) offset indicator” can be introduced to increase the flexibility of DMRS port indication.

Proposal 7: Increase the size of antenna ports field DCI is a more appropriate way to indicate R18 DMRS port(s) compared with introducing a “DMRS port(s) offset indicator” field.

Proposal 8: For PUSCH with CP-OFDM, enhanced antenna port(s) tables, like the tables shown above, should be introduced and the necessary DMRS port combinations can be further discussed.

Proposal 9: About case 3) in the FL proposal, multiplexing between R15 UE indicated with Rel.15 DMRS ports and R18 UE indicated with Rel-18 New ports and in a CDM group is possible.

Proposals related to DMRS ports for 8Tx UL SU-MIMO:

Proposal 10: Confirm the Working Assumption to introduce separate DMRS port tables for DMRS eType1/eType2.

Proposal 11: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated with the same antenna port group are multiplexed into the same or different DMRS CDM group.

Proposal 12: Support more than one DMRS port combination(s) for each rank and TPMI at least for Rel.18 eType1/eType2 DMRS design.

Proposal 13: For Rank>4, support the PTRS and DMRS association of 2-port PTRS for 8Tx when 2-port PTRS is used,

- 4bits are needed for the indication whereas 2bits MSB  are for the indication of PTRS port 0, and 2 bits LSB are for the indication of PTRS port 1;

- Proposal 14: Support 4-port PTRS for 8Tx transmission.

References

- RP-213517, “New WID: MIMO Evolution for Downlink and Uplink”, TSG RAN Meeting #94e Electronic Meeting, Dec. 6 - 17, 2021

- RAN1 #109-e meeting Chairman notes, May 2022

- RAN1 #110 meeting Chairman notes, August 2022

- RAN1 #110-bis meeting Chairman notes, October 2022

- RAN1 #111 meeting Chairman notes, November 2022

- RAN1 #112 meeting Chairman notes, February 27th – March 3rd, 2023