3GPP TSG RAN WG1 #112bis-e			R1-2302964

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.4.1

Source:	Xiaomi

Title:	Enhancements on multi-panel uplink transmission

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID for MIMO Evolution for Downlink and Uplink is approved [1], which includes the following objective:

Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

- The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we provide our views on the issues related to STxMP, including S-DCI based transmission and M-DCI based transmission. In addition, issues more related to STxMP are discussed simultaneously with AI9.1.1.1.

- Single-DCI based STxMP

  - Indication of SRI/TPMI for SDM scheme

In RAN1#111 meeting, the following agreements has been made on dynamic switching between SDM scheme and sTRP scheme [5]:

Agreement

For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission:

- Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission.

- FFS: how to interpret each codepoint of “SRS resource set indicator”.

- For the maximal number of layers for sTRP transmission, down-select:

  - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

  - Option2:  Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec.

- Down-select one from the following for maximal number of layers of SDM transmission:

  - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set

  - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

  - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately.

- FFS: To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

In the last RAN1 meeting, the following working assumption has been made on this issue [6]:

Working Assumption

For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following:

- For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1)

- For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1)

- FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same.

  - Note: This corresponds to the case that digital ports are shared between the panels

  - Note: RAN1 supports both implementations that digital ports are shared or separate among panels

The dynamic switching between sTRP and SDM/SFN using the legacy SRS resource set indicator field is necessary for the mechanism of STxMP. For the configuration of the maximal number of layers, it has been agreed that legacy maxRank (or Lmax) is used for the maximal number of layers of sTRP operation.

For SDM scheme, it was agreed as a WA that only symmetric panels are considered for Rel-18 STxMP transmission. Considering the UE implementation, two UE implementation structures can be suggested as below,

- Case 1: the digital ports can be shared among panels;

- Case 2: the digital ports cannot be shared among panels;

Considering different UE structures, we think that both cases are valid UE implementations and can be supported in spec. For Case1, the SRS ports used for SDM and sTRP transmissions per panel are not the same. For Case 2, the SRS ports used for SDM and sTRP transmissions are the same. So how to decide the SRS ports used for SDM transmission from the SRS ports configured for codebook based PUSCH transmission is the issue needs to be solved. In our view, a fixed rule to use a subset of the SRS ports is a simple solution and has less spec impact.

For SDM scheme, different UE structures would lead to different SRS resource/configuration used. So this UE structure information is needed for the gNB scheduling. There are at least two methods for the UE to report the structure,

- Approach 1: Introduce UE capabilities to report whether Case1 or Case2 is supported by UE implementation;

- Approach 2:  Use the maxRank reporting to differentiate different implementations, when the value ofo maxRank for sTRP and maxRank for SDM are the same, Case 2 is implicitly indicated, otherwise Case1 is indicated;

Proposal 1: Confirm the Working Assumption.

Proposal 2: For the two different UE structures, we support both cases as valid UE implementations, and UE can report the according structure information, two approaches can be considered,

  - Approach 1: Introduce UE capabilities to report whether Case1 or Case2 is supported by UE implementation;

  - Approach 2:  Use the maxRank reporting to differentiate different implementations, when the value ofo maxRank for sTRP and maxRank for SDM are the same, Case 2 is implicitly indicated, otherwise Case1 is indicated;

Proposal 3：For configurations that maxRank/Lmax for sTRP and maxRank/Lmax for SDM is not the same, introduce a fixed rule to use only a subset of the SRS ports for a SRS resource for CB based transmission, or a subset of the SRS resources for NCB based transmission.

  - SRS resource set configuration

In RAN1#110bis meeting, the following agreements have been made on SRS resource set configuration for SDM scheme [4]:

Agreement

For SDM scheme of single-DCI based STxMP PUSCH

- Configure two SRS resource sets for CB or NCB .

  - FFS : These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based.

- For codebook -based PUSCH , DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set.

- For non-codebook based PUSCH and codebook -based PUSCH , DCI indicates two SRI fields and each field indicates SRS resource(s)  for each SRS resource set separately.

  - FFS : For codebook -based PUSCH , the two SRS resources indicated by the two SRI fields can have different number of SRS ports

In RAN1#112 meeting, the following agreements has been made on the configuration of SRS resource sets [5]:

Agreement

For the SFN scheme of single-DCI based STxMP PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource

- The DCI indicates two SRI fields and TPMI fields for SFN transmission,

- On the indication of number of layers for CB and NCB PUSCH:

  - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH);

In the last RAN1 meeting, the following agreements has been made on dynamic switching between SDM scheme and sTRP scheme [6]:

Working Assumption

For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following:

- For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1)

- For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1)

- FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same.

  - Note: This corresponds to the case that digital ports are shared between the panels

  - Note: RAN1 supports both implementations that digital ports are shared or separate among panels

With the approach of the WA, it is agreed that only symmetric panels are considered for STxMP transmission. And it is straightforward that the configuration of SRS resource sets for STxMP should be configured with the same number of SRS resources and same number of SRS ports.

So for the two SRS resource sets of usage “codebook” or “non-codebook” without considering the configuration of fullpower mode2,

- Configure same number of SRS resources for each SRS resource set

- Configure same number of SRS ports for each SRS resource

Proposal 4: For the two SRS resource sets of usage “codebook” or “non-codebook”,

- Configure same number of SRS resources in each SRS resource set

- Configure same number of SRS ports for each SRS resource

The above can be applied to either SDM transmission or SFN transmission when full power mode 2 is not configured.

  - PT-RS configuration for SDM scheme

In the last RAN1 meeting, the following agreements has been made on dynamic switching between SDM scheme and sTRP scheme [6]:

Agreement

When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH:

- Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields.

  - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field.

  - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field.

- Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource.

FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission

Agreement

For single-DCI based STxMP SFN scheme,

- Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB

  - Each PTRS port is transmitted in SFN manner

A small issue still pending is how to configure the max number of PTRS ports for SDM and SFN schemes. For SDM scheme, 2 PTRS ports is always required to be configured which is not the case for STRP case. For STRP transmission, either 1 or 2 PTRS ports could be configured. So it would be more flexible to support an additional RRC parameter for the configuration of the max number of ports for SDM transmission.

For SFN scheme, whether the max number of PTRS ports for sTRP can also be applied for SFN needs to be considered. In our view, since an additional parameter can be introduced for SDM scheme, we also prefer to allow the flexibility for SFN scheme.

Proposal 5: Support an additional RRC configuration for the max number of PTRS ports for SDM transmission.

- Multi-DCI based STxMP

In Rel-16, both single-DCI based and multi-DCI based multi-TRP are specified. For single-DCI based multi-TRP, the enhancements are specific to PDSCH schemes. For multi-DCI based multi-TRP scheme, the enhancements are also applicable to PUSCH, e.g. out-of-order PUSCH associated by to a CORESETPoolIndex value. It is supported in Rel-16 that a PUSCH retransmission associated with a different CORESETPoolIndex value from that of the initial transmission. Hence, if PUSCH is not decoded successfully, it can be scheduled to transmit from or toward the other TRP. For multi-DCI based multi-PDSCH transmission, it is supported to have two separate MAC-CE’s for TCI state activation corresponding to the two CORESETPoolIndex values, and the interpretation of the TCI field in the DCI is associated of the CORESETPoolIndex value of the CORESET in which the scheduling DCI is received. These enhancements of multi-DCI based multi-PUSCH transmission are intended to increase the flexibility. However, in Rel-17, the specified multi-TRP based TDM schemes are enhanced under the single-DCI assumption, multi-DCI based PUSCH transmission was only discussed and further enhancements will be specified in Rel-18.

In RAN1#109-e meeting, the following agreement has been made on multi-DCI based PUSCH transmission schemes [2]:

Agreement

For multi-DCI based STxMP PUSCH+PUSCH transmission, study and evaluate the following aspects:

- Two PUSCHs are associated with different TRPs and transmitted from different UE panels. The total number of layers of these two PUSCHs is up to 4.

- Study STxMP of PUSCH+PUSCH transmission where it is some combination of DG-PUSCH, CG-PUSCH and msg3/msgA PUSCH.

- The overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are to be studied and justified for PUSCH+PUSCH.

Note: The above study shall take into account the UE implementation and RF considerations.

Note: Study the conditions required for STxMP PUSCH+PUSCH.

Note: Other aspects are not precluded.

In the previous meetings, the following agreement have been made on multi-DCI based PUSCH transmission schemes [4][5]:

Agreement

Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations:

- DG-PUSCH + DG-PUSCH

- CG-PUSCH + DG-PUSCH

Agreement

- Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission.

For PUSCH, dynamic-grant based PUSCH, configured-grant based PUSCH both need to be enhanced under multi-TRP based transmission scenarios. The enhancements should be applicable for both codebook based UL transmission and non-codebook based UL transmission.

In addition, configured grant PUSCH is also very important for URLLC use cases, and it should be ensured that the enhancements for reliability and robustness of PUSCH are extended to the case of configured grant for both Type 1 and Type 2. The feasibility of such simultaneous transmission is possible and up to gNB scheduler considering the semi-static configuration for CG-PUSCH Type1.

Considering the feasibility of the multi-DCI based PUSCH transmission, different types of overlapping should be studied. The following aspects need to be solved,

- The interference between different DMRS ports corresponding to different PUSCHs

- The interference between DMRS ports of one PUSCH and the data of the other PUSCH

To minimize the interference of the above mentioned aspects, the following rules are suggested when considering different overlapping types of STxMP PUSCH.

- Same DMRS configurations for different PUSCHs, eg. DMRS type, the number of the FL DMRS, etc.

- DMRS port(s) allocation of different PUSCHs belong to different CDM groups

- When partially/fully overlapping occurs for STxMP, the PUSCH can use the “CDM groups without data” in “Antenna Port” field to perform the rate matching towards the DMRS port(s) of the other PUSCH on the overlapped symbols.

Proposal 6: For the fully/partially overlapped PUSCHs based on M-DCI scheduling, the following rules are suggested to minimize the interference between two PUSCHs,

- Same DMRS configurations for different PUSCHs, eg. DMRS type, the number of the FL DMRS, etc.

- DMRS port(s) allocation of different PUSCHs belong to different CDM groups

- When partially/fully overlapping occurs in time domain for STxMP, the PUSCH can use the “CDM groups without data” in “Antenna Port” field to perform the rate matching towards the DMRS port(s) of the other PUSCH on the overlapped symbols.

- Beam indication and Power control issues for STxMP

For S-DCI based and M-DCI based simultaneous multi-panel transmission, power control and beam indication related issues also need to be solved.

  - Beam indication for STxMP

For STxMP based PUSCH/PUCCH transmissions, 2 beams are needed to be indicated (via DCI or MAC-CE) for the transmissions towards different TRPs from different UE panels. TCIs or SRIs can be used for the detailed indication for the 2 beams. For the purpose of scheduling, the first problem need to be solved is how to identify two UL beams to support the simultaneous UL transmission. The association between the indicated TCI(s)/SRI(s) and specific TRP(s)/panel(s) should take the UE capability value set index into consideration. To support STxMP, UE has to let gNB aware whether the selected two beams from UE beam reporting can be simultaneously transmitted or not, and whether they can be transmitted from different panels.

In R17, the beam reporting process is enhanced with the index of UE capability value set to indicate the maximum supported number of SRS ports related to each pair of CRI/SSBRI and L1-RSRP/L1-SINR.But the gNB cannot select two UE panels with same UE capability value set for simultaneous UL transmission, and also the gNB cannot separate the case of the beam reporting for the DL-only panel case.

One approach is enhancements based on group-based beam reporting. UE can report the beam pair with an additional information to differentiate the beam used for Uplink STxMP from which pairs used for DL reception.

Another approach is to enhance the beam reporting mechanism. For the simultaneous uplink transmission, a new configuration of the beam reporting like groupBasedUplinkBeamReporting targeting STxMP can be considered for the report of beams/beam pair(s) for the STxMP transmissions.  UE would guarantee all the supported beams/beam pair(s) can support the STxMP scheduling, the beams/beam pair(s) reported can be differentiated at least for whether to support the STxMP or the TDM transmission mode for the MTRP operation.

Proposal 7: Enhancements can be based on group-based beam reporting for the beam report of STxMP.

Proposal 8: Introduce a new beam reporting configuration “groupBasedUplinkBeamReporting” targeting STxMP for the report of beams/beam pair(s) for the STxMP transmissions also can be considered.

For the current group-based beam reporting, only 2 beams (N=2) or one beam pair can be configured to be reported. From the analysis above, we can see that some beam pair selected cannot even be used for STxMP. Even though one pair of beams is supportive of STxMP, it may not be appropriate for the scheduling of uplink transmission because of the interference situation estimated at the gNB. So more beam pairs should be allowed to be configured for the selection to provide more flexibility for the gNB scheduler and make the schedule of STxMP much more feasible.

Proposal 9: Support reporting in a single reporting instance of more than two (N>=2) different beams/beam pairs that UE can receive and/or transmit simultaneously.

  - Power Control for STxMP

In Rel-17, per-TRP power control has already been supported for the TDM repetition scheme. Since two power control settings and PL-RS for the transmissions from two panels can be indicated by the two TCI states which was discussed in AI9.1.1.1.

In the last RAN1 meeting, the following agreements has been made on dynamic switching between SDM scheme and sTRP scheme [6]:

Agreement

On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state

- FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2)

- FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

In Rel-18, the same principle can be applied to STxMP transmission with per-panel power control for both S-DCI based and M-DCI based PUSCH/PUCCH STxMP. In our view, both per-panel power limit and per-UE power limit should be configured to enable the per-panel power control for STxMP which needs UE reporting of the specific per-panel power limit values. And certain power sharing rules can be considered further.

Proposal 10: Introduce per-panel power limit and per-UE power limit to implement the panel-specific power control for STxMP.

Proposal 11: To meet the per-UE power limit, power sharing has to be guaranteed between two panels for STxMP, the following can be considered

- Same power fallback for both panels when exceeding the power class limitation

- Additional priority rules for power fallback when PUSCHs have different priorities for M-DCI case

Two TPC fields were supported for the closed-loop power control in Rel-17 TDM scheme, same principle can be applied for Rel-18.

Proposal 12: Support the per-panel closed-loop power control, reuse the Rel-17 TPC fields for the indication.

For the open-loop power control, it is very likely that the eMBB traffic has interference only to the transmission from one panel. Thus the power boosting for URLLC transmissions to use a higher P0 indicated when collision happens should be panel-specific.

Proposal 13: Support the per-panel open-loop power control, 2 OLPC indication fields can be indicated, the 2nd OLPC indication field can be configured.

  - Power headroom report for STxMP

In Rel-17, 2 Type-1 PHRs are always reported. When 2 reported PHRs corresponds to actual PHRS, the values are based on two sets of power control parameters. The same principle can be considered as the baseline for the PHR reporting in Rel-18 STxMP.

Proposal 14: Support per-panel PHR triggering and reporting for both S-DCI and M-DCI based STxMP.

Proposal 15: 2 PHs should always be reported, including cases of 2 actual PHRs, 2 virtual PHRs and 1 actual PHR with 1 virtual PHR.

- Conclusion

In this contribution, we provide our views of the enhancements on the precoding indication related issues for multi-panel/multi-TRP uplink transmissions, our proposals are as follows:

Single-DCI based STxMP

Proposal 1: Confirm the Working Assumption.

Proposal 2: For the two different UE structures, we support both cases as valid UE implementations, and UE can report the according structure information, two approaches can be considered,

  - Approach 1: Introduce UE capabilities to report whether Case1 or Case2 is supported by UE implementation;

  - Approach 2:  Use the maxRank reporting to differentiate different implementations, when the value ofo maxRank for sTRP and maxRank for SDM are the same, Case 2 is implicitly indicated, otherwise Case1 is indicated;

Proposal 3：For configurations that maxRank/Lmax for sTRP and maxRank/Lmax for SDM is not the same, introduce a fixed rule to use only a subset of the SRS ports for a SRS resource for CB based transmission, or a subset of the SRS resources for NCB based transmission.

Proposal 4: For the two SRS resource sets of usage “codebook” or “non-codebook”,

- Configure same number of SRS resources in each SRS resource set

- Configure same number of SRS ports for each SRS resource

The above can be applied to either SDM transmission or SFN transmission when full power mode 2 is not configured.

Proposal 5: Support an additional RRC configuration for the max number of PTRS ports for SDM transmission.

Multi-DCI based STxMP

Proposal 6: For the fully/partially overlapped PUSCHs based on M-DCI scheduling, the following rules are suggested to minimize the interference between two PUSCHs,

- Same DMRS configurations for different PUSCHs, eg. DMRS type, the number of the FL DMRS, etc.

- DMRS port(s) allocation of different PUSCHs belong to different CDM groups

- When partially/fully overlapping occurs in time domain for STxMP, the PUSCH can use the “CDM groups without data” in “Antenna Port” field to perform the rate matching towards the DMRS port(s) of the other PUSCH on the overlapped symbols.

Beam indication for STxMP

Proposal 7: Enhancements can be based on group-based beam reporting for the beam report of STxMP.

Proposal 8: Introduce a new beam reporting configuration “groupBasedUplinkBeamReporting” targeting STxMP for the report of beams/beam pair(s) for the STxMP transmissions also can be considered.

Proposal 9: Support reporting in a single reporting instance of more than two (N>=2) different beams/beam pairs that UE can receive and/or transmit simultaneously.

Power control for STxMP

Proposal 10: Introduce per-panel power limit and per-UE power limit to implement the panel-specific power control for STxMP.

Proposal 11: To meet the per-UE power limit, power sharing has to be guaranteed between two panels for STxMP, the following can be considered

- Same power fallback for both panels when exceeding the power class limitation

- Additional priority rules for power fallback when PUSCHs have different priorities for M-DCI case

Proposal 12: Support the per-panel closed-loop power control, reuse the Rel-17 TPC fields for the indication.

Proposal 13: Support the per-panel open-loop power control, 2 OLPC indication fields are needed.

Proposal 14: Support per-panel PHR triggering and reporting for both S-DCI and M-DCI based STxMP.

Proposal 15: 2 PHs should always be reported, including cases of 2 actual PHRs, 2 virtual PHRs and 1 actual PHR with 1 virtual PHR.

- References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94e, December 2021

RAN 1#109-e Meeting chairman notes, May 2022

RAN 1#110 Meeting chairman notes, August 2022

RAN 1#110bis Meeting chairman notes, October 2022

RAN 1#111 Meeting chairman notes, November 2022

RAN 1#112 Meeting chairman notes, February 2023