3GPP TSG RAN WG1 #112bis-e			R1- 2302965

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.1.4.2

Source:	Xiaomi

Title:	Enhancements on 8Tx uplink transmission

Document for:	Discussion and Decision

Introduction

The Rel-18 WID for MIMO Evolution for Downlink and Uplink is approved [1], which includes the following objective:

Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we provide some discussion on the SRI/TPMI enhancements on the 8Tx uplink transmission.

Discussion on 8Tx

In RAN1#110 meeting, the following agreement has been made on 8Tx transmission [2]:

Agreement

8TX PUSCH is supported in Rel-18

Agreement

Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability

- For uplink transmission with rank<=4, single CW is supported

- For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e

The above applies only with regards to the work scope of this agenda item.

Current PUSCH transmission has two transmission modes: codebook based and non-codebook based transmission. Both transmission modes are very important and applied for different scenarios. The enhancements on 8Tx SU-MIMO is applicable to both codebook based UL transmission and non-codebook based UL transmission.

In RAN1#110bis meeting, the following has been approved on 8Tx transmission [3]:

Working Assumption

For uplink transmission with rank>4, support dual CW transmission.

Agreement

If dual CW is supported for uplink transmission with Rank>4 by an 8TX UE, reuse DL Rel-15 codeword to layer mapping for both codebook-based and non-codebook-based transmission.

Proposal 1: Confirm the Working Assumption to support dual CW for RANK>4 8Tx transmission.

In RAN1#111 meeting, the following agreement has been made on 8Tx transmission [4]:

Agreement:

For PUSCH transmission with rank>4 by an 8TX UE, to support dual CW transmission,

- specify MCS, NDI, RV indication for the second CW

- specify PUSCH Scrambling for the second CW

- specify UCI multiplexing on PUSCH for dual CW transmission

- study whether/how Enabling/Disabling the second CW

FFS: Optimization of DCI to indicate the above

Note: Strive to reuse Rel-15 NR DL schemes where possible.

In RAN1#112 meeting, the following agreement has been made on 8Tx transmission [5]:

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support

- Alt.2: A second MCS field (5 bits) is indicated for the second codeword

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated.

- FFS: Details on how to signal

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q∈{0,1},

cinit=nRNTI∙215+q∙214+nID

where nRNTI, and nID are defined similar to the legacy single CW PUSCH transmission.

It has been agreed that the codeword to layer mapping for 8Tx UL PUSCH follows the same mapping principle as the DL PUSCH. The other issues that needs to be solved can be further discussed.

Our preference is to use the principle of the DL as a baseline,

- specify MCS, NDI, RV indication for the second CW

Since the second CW may have totally different configurations of MCS/NDI/RV, separate indication fields is preferred. We don’t see a good solution for the optimization of any joint indication for the reduction of the DCI overhead here.

- specify PUSCH Scrambling for the second CW

To minimize the interference between two codewords, scrambling for the second codeword should be applied.To enable this per-CW scrambling, a new signalling of an additional scrambling identity dataScramblingIdentityPDSCH2 can be specified.

- study whether/how Enabling/Disabling the second CW

A higher signaling can be specified to enable/disable the second CW.

To support the dual CW transmission for rank>4 8Tx, the following is suggested.

Proposal 2: Support separate MCS, NDI and RV indication fields for the second CW, the definition for each indication field can be the same as the first codeword.

Proposal 3: Specify the RRC signaling to enable/disable the second CW.

Non-codebook based

- SRS configuration for non-codebook

In RAN1 previous meetings, the following agreements have been made on SRS for NCB [2][3]:

Agreement

For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources

- Alt3: Support both alternatives.

Agreement

For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

To support the non-codebook based PUSCH transmission of up to RANK 8, the supported maximum number of SRS resources within a SRS resource set with usage of “non-codebook” need to be extended to 8. The spec effort to define new SRI tables would be needed according to the legacy indication method.

Also another approach is that 8 single-port SRS resources can be configured into multiple SRS resource sets. If different SRI indication fields may be used, one of the advantages is that legacy SRI tables can be reused for each SRI indication field which alleviate spec effort.

Proposal 4: For better scheduling flexibility, support configuration of up to 2 or 4 SRS resource sets, each configured with up to 4 or 2 single-port SRS resources accordingly.

- SRI indication scheme for NCB based

In RAN1#110 meeting, the following agreement has been made on NCB based 8Tx transmission [2]:

Agreement

Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE,

- FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded.

- Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms

In RAN1#112 meeting, the following agreement has been made on NCB based 8Tx transmission [5]:

Agreement

For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set,

- All SRS port combinations are supported

- For SRI indication, down-select from,

  - Option 1: Use an NSRS bit length bitmap

  - Option 2: Use a legacy-based solution

- Consideration of Lmax for SRI indication

For NSRS≤4, Rel-15 SRI indication is reused

For non-codebook based PUSCH transmission, the SRS resource set configuration has the following options for the CSI acquisition:

- Alt.1: Up to 8 SRS resources can be configured in a SRS resource set, SRI table needs further extension or redesign, and the bitwidth would be up to 8bits, SRI table may need some re-definition to save DCI bits.

- Alt.2: Up to 8 SRS resources can be configured in a SRS resource set, SRI is indicated in bitmap of the corresponding SRS resources (up to 8 bits), but no effort on the definition of SRI tables. Comparing with option 1, the effort on SRI table definition and certain signaling overhead can be avoided.

- Atl.3: Up to 8 SRS resources can be configured in at least 2 SRS resource sets, two SRI fields are at least needed for the indication according to the current SRI mapping tables. The SRS resource set with a lower ID is associated with the first SRS resource set which is corresponding to the first SRI field, and the SRS resource set of a higher ID is associated the second SRS resource set which is always corresponding to the second SRI field.

- Atl.4: Up to 8 SRS resources can be configured in at least 2 SRS resource sets, one SRI field still can be used to indicate by bitmap corresponding to all SRS resources (up to 8 bits). e.g. the SRS resources in the first SRS resource set of a lower ID is corresponding to the 4 MSB bits and the SRS resources in the second SRS resource set of a higher ID is corresponding to the 4 LSB bits.

Proposal 5: For non-codebook based PUSCH transmission with 8Tx, support Option 1 to use a bitmap corresponding to all SRS resources configured which is preferred for the simplicity without any effort on the design of new SRI tables.

Codebook based

Fully-coherent codebook

In the previous RAN1 meetings, the following agreements have been achieved on Rel-18 UL 8Tx fully-coherent codebook design [4][5].

Agreement For a fully coherent uplink precoding by an 8TX UE, - Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a. - No LS to RAN4 will be needed Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2)` A pair of (N1, N2) can be configured with subject to UE capability. Agreement Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), - Study whether/how to support (O1, O2) = (2,1), (2,2) - whether for all rank, or rank 1-2, or rank 3-8 - applicability of different (O1, O2) values per agreed (N1, N2) - companies are encouraged to submit simulation results

The Rel-15 single panel DL Type I codebook has been agreed as the starting point for Rel-18 UL 8Tx fully-coherent codebook. When considering the single panel UE, the uniform linear array with (N1,N2) = (4,1) and uniform planar array with (N1,N2) = (2,2) are two typical antenna layouts as illustrated in Fig 1.

Fig 1 Antenna layouts for 8Tx UE.

Since (N1,N2) = (4,1) and (N1,N2) = (2,2) have been supported in the RAN1#112 meeting, and a pair of (N1,N2) can be configured with subject to UE capability, a UE capability can be introduced to report the supported antenna configuration to gNB.

Proposal 6: Introduce a UE capability to report the supported antenna configuration to gNB.

Oversampling factors

In Rel-15 UL 4Tx codebook, the fully-coherent codewords are defined as the subset of the Rel-15 DL Type I 4Tx codebook with reduced oversampling factor (O1,O2) = (2,1). The configuration parameters and number of Rel-15 UL 4Tx fully-coherent codewords are summarized in Table 1.

Table 1 Configuration parameters and number of Rel-15 UL 4Tx fully-coherent codewords.

| Number of ranks | Configuration parameters | Number of codewords |
| --- | --- | --- |
| 1 | CodebookMode=1, L=1, N1=2, N2=1, O1=2, O2=1 | 16 |
| 2 | CodebookMode=1, L=1, N1=2, N2=1, O1=2, O2=1, i1,3=0 | 8 |
| 3 | CodebookMode=1-2, L=1, N1=2, N2=1, O1=2, O2=1, i1,1={0,2}, i1,3=0 | 4 |
| 4 | CodebookMode=1-2, L=1, N1=2, N2=1, O1=2, O2=1, i1,1=0, i1,3=0 | 2 |

Given that with the increase of antenna ports, the cardinality of Rel-15 DL Type I 8Tx codebook becomes larger. Considering the UE implementation complexity and the signalling overhead, the Rel-15 DL Type I 8Tx codebook with reduced oversampling factor can be studied for Rel-18 UL 8Tx fully-coherent codewords. Table 2 lists the number of codewords of different oversampling factors.

Table 2 Number of codewords of different oversampling factors.

| Configuration parameters | Number of codewords |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (N1,N2,O1,O2) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total |
| (4,1,4,1) | 64 | 128 | 96 | 96 | 32 | 32 | 16 | 16 | 480 |
| (4,1,2,1) | 32 | 64 | 48 | 48 | 16 | 16 | 8 | 8 | 240 |
| (4,1,1,1) | 16 | 32 | 24 | 24 | 8 | 8 | 4 | 4 | 120 |
| (2,2,4,4) | 256 | 512 | 384 | 384 | 128 | 128 | 128 | 128 | 2048 |
| (2,2,2,2) | 64 | 128 | 96 | 96 | 32 | 32 | 32 | 32 | 512 |
| (2,2,1,1) | 16 | 32 | 24 | 24 | 8 | 8 | 8 | 8 | 128 |

In RAN1#112 meeting [5], precoding matrices generated according to (O1,O2) = (1,1) are supported as the baseline value while whether/how to support (O1,O2) = (2,1) and (2,2) is FFS. To evaluate the impact of oversampling factors, link level simulation has been performed to evaluate the performance of Rel-15 DL Type I 8Tx codebook with different configuration parameters. The detailed EVM is shown in Table 13 and Table 14 of the appendix.

For (N1,N2) = (4,1), we configure the codebook as Rel-15 DL Type I codebook with (O1,O2) = (2,1). In each slot, the global search of codebook is operated to compute the channel capacity corresponds to each codeword, and the codeword with the highest channel capacity is selected as the optimal codeword. The selected codewords are counted in a period of time resource, and the probability of each codeword selected as the optimal one can thus be summarized. In this case, although the oversampling factor is configured as (O1,O2) = (2,1), there is still approximate 50%-70% probability that the selected codewords belong to (O1,O2) = (2,1). For (N1,N2) = (2,2), the probability of selected codewords with (O1,O2) = (1,1) is larger than 95% even (O1,O2) = (2,2) is configured. Numerical results are illustrated in Fig 2.

Fig 2 Probability of low oversampling factor when high oversampling factor is configured.

Observation 1: Although the codebook is configured with high oversampling factors, there is still high probability that the selected optimal codeword belongs to low oversampling factors.

As shown in Fig 3 and Fig 4, the BLER and SE performance are evaluated to further determine the oversampling factors. For (N1,N2) = (4,1), when rank=1, the (O1,O2) = (1,1) oversampling factor shows performance loss of less than 1dB compared to (O1,O2) = (2,1) oversampling factor. The performance degradation of low oversampling factor codebook can be mitigated in the case of rank=4. For (N1,N2) = (2,2), the codebooks with different oversampling factors (O1,O2) = (2,2)/(1,1) exhibit nearly identical performance.

(a) Rank =1                                   (b) Rank = 4

Fig 3 BLER performance.

(a) Rank =1                                   (b) Rank = 4

Fig 4 SE performance.

Observation 2: For (N1,N2) = (4,1), the acceptable performance degradation happens as the oversampling factor reduces. For (N1,N2) = (2,2), different oversampling factors show very similar performance.

What’s more, considering the implementation computational complexity, if (O1,O2) = (1,1) is adopted for (N1,N2) = (4,1), the elements of the codeword is limited into QPSK constellation. In this case, the complicated complex-number multiplication operations of precoding matrix can be replaced by complex-number addition operations.

Observation 3: The codewords with QPSK constellation elements can be used to reduce the implementation computational complexity by replacing the complex-number multiplication operations with the addition operations.

Based on the above analysis, to make a trade-off among performance, signalling overhead, and implementation complexity, we support only (O1,O2) = (1,1) for both (N1,N2) = (4,1) and (N1,N2) = (2,2).

Proposal 7: To make a trade-off among performance, signalling overhead, and implementation complexity, only support (O1,O2) = (1,1) for (N1,N2) = (4,1) and (N1,N2) = (2,2).

Subset selection

To further reduce the signalling overhead, the subset of Rel-15 DL Type I 8Tx codebook with (O1,O2) = (1,1) oversampling factor can be selected following the similar rule of Rel-15 UL 4Tx codebook.

(a) (N1,N2) = (4,1)

When rank=1, all the codewords are reserved. When rank=2/3/4, the index i1,3 can be fixed as i1,3=0 which means the same or neighbouring beams are selected for the layers larger than 1. The index i1,1 can be fixed as i1,1={0,2} and i1,1={0} for rank=5/6 and rank=7/8, respectively.

(a) (N1,N2) = (2,2)

When rank=1, all the codewords are reserved. When rank=2,3,4, the index i1,3 can be fixed as i1,3=0 which means the same or neighbouring beams are selected for the layers larger than 1. The index i1,2 can be fixed as i1,2={0} for rank=5/6, while the indices i1,1 and i1,2 can be respectively fixed as i1,1={0} and i1,2={0} for rank=7/8.

As shown in Fig 3, for both (N1,N2) = (4,1) and (N1,N2) = (2,2), when rank=4, there is almost no performance loss for the codebook with fixed i1,3={0} compared to the codebook with flexible i1,3={0,1,2,3}.

Observation 4: For both (N1,N2) = (4,1) and (N1,N2) = (2,2), when rank=4, there is almost no performance loss for the codebook with fixed i1,3={0} compared to the codebook with flexible i1,3={0,1,2,3}.

The detailed configuration parameters and number of Rel-18 UL 8Tx fully-coherent codewords for (N1,N2) = (4,1) and (N1,N2) = (2,2) are summarized in Table 3 and Table 4, respectively.

Table 3 Configuration parameters and number of 8Tx fully-coherent codewords when (N1,N2) = (4,1).

| Number of ranks | Configuration parameters | Number of codewords |
| --- | --- | --- |
| 1 | N1=4, N2=1, O1=1, O2=1, i1,1={0,1,2,3}, i1,2={0}, i2={0,1,2,3} | 16=4*1*4 |
| 2 | N1=4, N2=1, O1=1, O2=1, i1,1={0,1,2,3}, i1,2={0}, i1,3={0}, i2={0,1} | 8=4*1*1*2 |
| 3 | N1=4, N2=1, O1=1, O2=1, i1,1={0,1,2,3}, i1,2={0}, i1,3={0}, i2={0,1} | 8=4*1*1*2 |
| 4 | N1=4, N2=1, O1=1, O2=1, i1,1={0,1,2,3}, i1,2={0}, i1,3={0}, i2={0,1} | 8=4*1*1*2 |
| 5 | N1=4, N2=1, O1=1, O2=1, i1,1={0,2}, i1,2={0}, i2={0,1} | 4=2*1*2 |
| 6 | N1=4, N2=1, O1=1, O2=1, i1,1={0,2}, i1,2={0}, i2={0,1} | 4=2*1*2 |
| 7 | N1=4, N2=1, O1=1, O2=1, i1,1={0}, i1,2={0}, i2={0,1} | 2=1*1*2 |
| 8 | N1=4, N2=1, O1=1, O2=1, i1,1={0}, i1,2={0}, i2={0,1} | 2=1*1*2 |

Table 4 Configuration parameters and number of 8Tx fully-coherent codewords when (N1,N2) = (2,2).

| Number of ranks | Configuration parameters | Number of codewords |
| --- | --- | --- |
| 1 | N1=2, N2=2, O1=1, O2=1, i1,1={0,1}, i1,2={0,1}, i2={0,1,2,3} | 16=2*2*4 |
| 2 | N1=2, N2=2, O1=1, O2=1, i1,1={0,1}, i1,2={0,1}, i1,3={0}, i2={0,1} | 8=2*2*1*2 |
| 3 | N1=2, N2=2, O1=1, O2=1, i1,1={0,1}, i1,2={0,1}, i1,3={0}, i2={0,1} | 8=2*2*1*2 |
| 4 | N1=2, N2=2, O1=1, O2=1, i1,1={0,1}, i1,2={0,1}, i1,3={0}, i2={0,1} | 8=2*2*1*2 |
| 5 | N1=2, N2=2, O1=1, O2=1, i1,1={0,1}, i1,2={0}, i2={0,1} | 4=2*1*2 |
| 6 | N1=2, N2=2, O1=1, O2=1, i1,1={0,1}, i1,2={0}, i2={0,1} | 4=2*1*2 |
| 7 | N1=2, N2=2, O1=1, O2=1, i1,1={0}, i1,2={0}, i2={0,1} | 2=1*1*2 |
| 8 | N1=2, N2=2, O1=1, O2=1, i1,1={0}, i1,2={0}, i2={0,1} | 2=1*1*2 |

Proposal 8: For Rel-18 UL 8Tx fully-coherent codebook, support the following configuration parameters.

- (N1,N2,O1,O2) = (4,1,1,1)

  - Rank=1: i1,1={0,1,2,3}, i1,2={0}, i2={0,1,2,3}

  - Rank=2,3,4: i1,1={0,1,2,3}, i1,2={0}, i1,3={0}, i2={0,1}

  - Rank=5,6: i1,1={0,2}, i1,2={0}, i2={0,1}

  - Rank=7,8: i1,1={0}, i1,2={0}, i2={0,1}

- (N1,N2,O1,O2) = (2,2,2,2)

  - Rank=1: i1,1={0,1}, i1,2={0,1}, i2={0,1,2,3}

  - Rank=2,3,4: i1,1={0,1}, i1,2={0,1}, i1,3={0}, i2={0,1}

  - Rank=5,6: i1,1={0,1}, i1,2={0}, i2={0,1}

  - Rank=7,8: i1,1={0}, i1,2={0}, i2={0,1}

Construction of Rel-18 UL 8Tx fully-coherent codebook

The Rel-18 UL 8Tx fully-coherent codebook can thus be constructed by adding the codebook for (N1,N2) = (4,1) and (N1,N2) = (2,2). Next, we remove the codewords which belongs to both (N1,N2) = (4,1) and (N1,N2) = (2,2). Table 5 and Table 6 shows the index of the codeword that belongs to both (N1,N2) = (4,1) and (N1,N2) = (2,2) when number of ranks are 1 and 2, respectively. When the number of ranks is larger than 2, there is no codewords that belongs to neither (N1,N2) = (4,1) nor (N1,N2) = (2,2). For example, when rank =1, the first codewords in (N1,N2) = (4,1) is the same as the first codeword in (N1,N2) = (2,2), and the ninth codeword in (N1,N2) = (4,1) is the same as the fifth codeword in (N1,N2) = (2,2).

Table 5 The index of the codeword that belongs to both (N1,N2) = (4,1) and (N1,N2) = (2,2) when rank=1.

|  | Index of the codeword (starts from 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (N1,N2) = (4,1) | 1 | 2 | 3 | 4 | 9 | 10 | 11 | 12 |
| (N1,N2) = (2,2) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table 6 The index of the codeword that belongs to both (N1,N2) = (4,1) and (N1,N2) = (2,2) when rank=2.

|  | Index of the codeword (starts from 1) |  |  |  |
| --- | --- | --- | --- | --- |
| (N1,N2) = (4,1) | 1 | 2 | 5 | 6 |
| (N1,N2) = (2,2) | 1 | 2 | 3 | 4 |

Proposal 9: For Rel-18 UL 8Tx fully-coherent codebook, support the union set of codebook with (N1,N2) = (4,1) and codebook with (N1,N2) = (2,2).

In this case, the number of Rel-18 UL 8Tx fully-coherent codewords with different number of ranks is shown in Table 7. The total number of fully-coherent codewords is 92.

Table 7 Number of Rel-18 UL 8Tx fully-coherent codewords with different number of ranks.

| Number of ranks | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of codewords | 16+16-8=24 | 8+8-4=12 | 8+8=16 | 8+8=16 | 4+4=8 | 4+4=8 | 2+2=4 | 2+2=4 | 92 |

On one hand, based on the agreement achieved in RAN1#111 meeting [4], adding 128-92=36 precoders generated via Alt 2a (non DFT precoders) can be considered as a potential way forward. On the other hand, further increase the number of rank=2 codewords by selecting more candidate of index i1,3 can also be considered. For example, for rank=2, i1,3 can be fixed as {0,1}. Note that the number of rank=2 codewords is smaller than that of the rank=3/4 counterpart.

Proposal 10: For Rel-18 UL 8Tx fully-coherent codebook, further study how to introduce the non DFT codewords or select the indices {i1,1, i1,2, i1,3, i2} candidate.

Partially-coherent codebook

Antenna port groups

In RAN1#110b-e meeting, the following agreement has been made on Rel-18 UL 8Tx partially-coherent codebook [3].

Agreement For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource - For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7} - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7} - For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7} - Alt3: four coherent groups of {0,2}, {4,6}, {1,3} and {5,7} - Note: Other alternatives which are not foreseen are not precluded

For partially-coherent codebook, the first issue that need to be discussed is the number of antenna port groups and the port coherency schemes. Generally, the dual-polarization antenna pair with the similar positions can be considered as coherent transmission. The following two typical antenna configurations can be supported as illustrated in Fig 5.

- Two antenna port groups: e.g., port group {0,1,4,5} and {2,3,6,7}.

- Four antenna port groups: e.g., port group {0,4}, {1,5}, {2,6}, and {3,7}.

Fig 5 Antenna layout for 8Tx partially-coherent UE.

Proposal 11: Support Alt2 for two coherent groups of {0,1,4,5} and {2,3,6,7} for Ng=2. Support Alt1 for four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} for Ng=4.

Partially-coherent codebook for two antenna port groups

In RAN1#112 meeting, the following agreement and FL proposals have been made on Rel-18 UL 8Tx partially-coherent codebook design [5].

Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated.

Agreement For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used

For the partially-coherent codebook design, the rank splitting rule is another important issue. When the number of ranks are 2/3/4, both the balanced splitting and group-selection splitting should be considered to adapt different channel conditions. Similar with the Rel-15 UL 4Tx rank=3 fully-coherent codewords, the (1,2) splitting can be prioritized over the (2,1) splitting for rank=3. When the number of ranks is 5/6/7/8, only balanced splitting should be considered while the unbalanced splitting can be precluded. To reduce the number of rank combination, the (2,3) splitting has a higher priority than the (3,2) splitting for rank=5 while the (3,4) splitting has a higher priority than the (4,3) splitting for rank=7. In this case, there is only when mapping rule between CW and antenna port groups, i.e., the first CW and the second CW are mapped to the first and the second antenna port groups, respectively.

Proposal 12: Support the following rank splitting rule for Ng=2.

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | (2,3), (3,2) |
| 6 |  | (3,3) |
| 7 |  | (3,4), (4,3) |

The partially-coherent codebook for Ng=2 can be designed based on Rel-15 UL 4Tx fully-coherent codebook, and the Rel-15 UL 4Tx partially-coherent codebook should be precluded. Based on the rank splitting rule, the partially-coherent codebook for Ng=2 can be designed as follows.

- For rank = 1

  - A04×18×1 or 04×1A8×1, where A is a rank 1 precoding matrix taken from Rel-15 UL 4Tx fully-coherent codebook

- For rank = 2, 3, 4

  - A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are precoding matrices taken from Rel-15 UL 4Tx fully-coherent codebook or empty matrix

- For rank >4

  - A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are precoding matrices taken from Rel-15 UL 4Tx fully-coherent codebook

To sum up, the partially-coherent codebook for Ng=2 is computed as A104×rank(A2)04×rank(A1)A28×rank where A1 and A2 are precoding matrices taken from Rel-15 UL 4TX fully-coherent codebook or empty matrix, and the rank(A1) and rank(A2) are determined based on the rank splitting rule.

Proposal 13: For PUSCH transmission by a partially-coherent 8TX UE with Ng=2, support the precoding structure A104×rank(A2)04×rank(A1)A28×rank where A1 and A2 are precoding matrices taken from Rel-15 UL 4Tx fully-coherent codebook or empty matrix, the rank=rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and rank(A1) and rank(A2) are determined based on the rank splitting rule.

However, even though not all the rank splitting cases are supported, the candidate of 8Tx partially-coherent codebook is still too large. For example, when rank=5 (only support 2+3 and 3+2), the total number of partially-coherent codewords is 8*4+4*8=64. As a result, the candidate of Rel-15 UL 4Tx fully-coherent codebook should be further down-selected to reduce the signalling overhead.

To reduce the candidate of Rel-15 UL 4Tx fully-coherent codebook, one way is to minimize the cosine similarity between each codewords so that the space can be covered more completely. For example, if we select 4 codewords from Rel-15 UL 4Tx rank=2 fully-coherent codebook (as shown in Table 8), the indices of the selected codewords can be {0,3,4,7} (as highlighted in Table 8).

Table 8 Rel-15 UL 4Tx rank=2 fully-coherent codebook.

| index |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |

Proposal 14: For partially-coherent codebook, the candidate of Rel-15 UL 4Tx fully-coherent codebook should be further down-selected to reduce the signalling overhead, e.g., based on the principle that minimizing the cosine similarity between each codewords.

Partially-coherent codebook for four antenna port groups

There are two alternatives to design Rel-18 UL 8Tx partially-coherent with Ng=4.

(a) Based on Rel-15 UL 2Tx fully-coherent codebook.

In this method, the codeword is designed as A102×rank(A2)02×rank(A1)A202×rank(A3)02×rank(A4)02×rank(A3)02×rank(A4)02×rank(A1)02×rank(A2)02×rank(A1)02×rank(A2)A302×rank(A4)02×rank(A3)A48×rank where A1, A2, A3 and A4 are precoding matrices taken from Rel-15 2TX UL fully-coherent codebook or empty matrix, the rank=rank(A1) + rank(A2) + rank(A3) + rank(A4) is one of 1, 2, 3, 4, 5, 6, 7 or 8. The fully-flexible splitting rule for Ng=4 is shown in Table 9. Since the cases are more complex than that of Ng=2, the rank splitting rule should be further down-selected.

Table 9 Rank splitting rule for Ng=4 when Rel-15 UL 2Tx fully-coherent codebook is used.

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups | Layers split across 3 Antenna Groups | Layers split across 4 Antenna Groups |
| --- | --- | --- | --- | --- |
| 1 | (1,0,0,0), (0,1,0,0), (0,0,1,0), (0,0,0,1) |  |  |  |
| 2 | (2,0,0,0), (0,2,0,0), (0,0,2,0), (0,0,0,2) | (1,1,0,0), (1,0,1,0), (1,0,0,1), (0,1,1,0), (0,1,0,1), (0,0,1,1) |  |  |
| 3 |  | (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) | (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) |  |
| 4 | - | (2,2,0,0), (2,0,2,0), (2,0,0,2), (0,2,2,0), (0,2,0,2), (0,0,2,2), | (2,1,1,0), (2,1,0,1), (2,0,1,1), (0,2,1,1), (1,2,1,0), (1,2,0,1), (1,0,2,1), (0,1,2,1), (1,1,2,0), (1,1,0,2), (1,0,1,2), (0,1,1,2) | (1,1,1,1) |
| 5 | - | - | (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (2,1,0,2), (2,0,1,2), (0,2,1,2), (1,2,2,0), (1,2,0,2), (1,0,2,2), (0,1,2,2) | (2,1,1,1), (1,2,1,1), (1,1,2,1), (1,1,1,2) |
| 6 | - | - | (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) | (2,2,1,1), (2,1,2,1), (2,1,1,2), (1,2,2,1), (1,2,1,2), (1,1,2,2) |
| 7 |  | - | - | (2,2,2,1), (2,2,1,2), (2,1,2,2), (1,2,2,2) |
| 8 |  |  |  | (2,2,2,2) |

Observation 5: The rank splitting rule is pretty complex for Ng=4 partially-coherent codebook constructed by Rel-15 UL 2Tx fully-coherent codebook. And the rank splitting rule should be further down-selected.

Proposal 15: For PUSCH transmission by a partially-coherent 8TX UE with Ng=4, support the precoding structure A102×rank(A2)02×rank(A1)A202×rank(A3)02×rank(A4)02×rank(A3)02×rank(A4)02×rank(A1)02×rank(A2)02×rank(A1)02×rank(A2)A302×rank(A4)02×rank(A3)A48×rank where A1, A2, A3 and A4 are precoding matrices taken from Rel-15 2TX UL fully-coherent codebook or empty matrix, the rank= rank(A1) + rank(A2) + rank(A3) + rank(A4) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and rank(A1), rank(A2), rank(A3), and rank(A4) are determined based on the rank splitting rule for Ng=4 when Rel-15 UL 2Tx fully-coherent codebook is used.

(b) Based on Rel-15 UL 4Tx partially-coherent codebook.

In this method, the codeword is computed as A104×rank(A2)04×rank(A1)A28×rank where A1 and A2 are precoding matrices taken from Rel-15 UL 4TX partially-coherent codebook or empty matrix, and the rank splitting rule can follow the same rule as that of Ng=2.

Proposal 16: For PUSCH transmission by a partially-coherent 8TX UE with Ng=4, support the precoding structure A104×rank(A2)04×rank(A1)A28×rank where A1 and A2 are precoding matrices taken from Rel-15 UL 4Tx partially-coherent codebook or empty matrix, the rank=rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and the rank splitting rule can follow the same rule as that of Ng=2.

It should be noted that when different port coherency schemes are used, each row/column of Rel-15 UL 4Tx/2Tx codewords should be set as the corresponding antenna ports. That is to say, row/column-interleaving operation should be used for Rel-18 UL 8Tx partially-coherent codewords to satisfy different port coherency schemes.

Proposal 17: Row/Column-interleaving operation should be used for Rel-18 UL 8Tx partially-coherent codewords to satisfy different port coherency schemes.

Non-coherent codebook

The Rel-18 UL 8Tx non-coherent codebook can be designed via the 8x1 antenna selection vectors/matrixes. When the number of ranks is L, all  antenna selection vectors/matrixes can be considered. In RAN1#112 meeting [5], all the antenna selection vectors/matrixes are supported for rank=1.

Agreement For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission. 10000000,01000000,00100000,00010000,00001000,00000100,00000010,00000001 with the scaling factor of 122.

However, to reduce the signalling overhead, the following rule can be used for rank>1. When 2≤rank≤X, all the antenna selection vectors/matrixes are used. When X<Rank≤Y, a part of antenna selection vectors/matrixes are used. When Y<Rank≤8, only one antenna selection vector/matrix are used. The value of X and Y can be left for further study, e.g., X=2 and Y=4

Proposal 18: Antenna selection vectors/matrixes can be used for the Rel-18 UL 8Tx non-coherent codebook. Considering the signalling overhead, all antenna selection vectors/matrixes can be used for 1≤rank≤X, a part of antenna selection vectors/matrixes can be used for X<Rank≤Y, and only one antenna selection vector/matrix can be used for Y<Rank≤8. The value of X and Y can be left for further study, e.g., X=2 and Y=4.

TPMI design

In RAN1#111 meeting, the following FL proposal was made on TPMI design [4].

FL Proposal 7.1: For CB-based 8TX PUSCH transmission, - (a) For SRI indication, down-select from - Use a single SRI field - Use two SRI fields - (b) For rank indication, down-select from - Separate indication of TRI and TPMI - Joint indication of TRI and TPMI - (c) For antenna group indication, down-select from - Separate indication of antenna group and TPMI - Joint indication of antenna group and TPMI

The TPMI design is depended on the size of the codebook, as a result, more TPMI design details can be discussed after the codebook design is clearer. The codebook subset configuration rule should also be decided. In the legacy rule, the fully-coherent UE can be configured with 'fullyAndPartialAndNonCoherent' codebook, partially-coherent UE can be configured with 'partialAndNonCoherent' codebook, and non-coherent UE can be configured with 'NonCoherent' codebook. The nested codebook is benefit for satisfying different channel conditions. What’s more, the codebook subset configuration rule should not be restricted by the signalling overhead. As a result, the legacy codebook subset configuration rule can also be supported for Rel-18 8Tx UE.

Proposal 19: For Rel-18 8Tx UE, the legacy codebook subset configuration rule can be reused, i.e., the fully-coherent UE can be configured with 'fullyAndPartialAndNonCoherent' codebook, partially-coherent UEs can be configured with 'partialAndNonCoherent' codebook, and non-coherent UE can be configured with 'NonCoherent' codebook.

The TPMI design is discussed as follows.

(a) Scheme 1: Joint indication of TRI and TPMI (legacy rule)

In Rel-15 UL 4Tx codebook, the precoding matrix is indicated jointly by TPMI and TRI, the number of codewords is 62 and the overhead is 6 bits. In Rel-18 UL 8Tx codebook, if the legacy rule is reused, considering the signalling overhead, the bit width can be set, e.g., at most 2 bits more than that of the Rel-15 UL 4Tx codebook, i.e., 7, or 8 bits. Table 10 lists a feasible TPMI design example with 8 bits.

Table 10 Number of codewords and TPMI design (8 bits).

| Number of ranks | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fully-coherent codewords | 32 | 32 | 16 | 16 | 8 | 8 | 4 | 4 |
| Partially-coherent codewords | 32 | 32 | 16 | 16 | 8 | 8 | 4 | 4 |
| Non-coherent codewords | 8 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of codewords for each rank | 72 | 66 | 33 | 33 | 17 | 17 | 9 | 9 |
| Total number of codewords | 256 |  |  | TPMI |  | 8 bits |  |  |

Proposal 20: For TPMI design, reusing the joint indication of TRI and TPMI (legacy rule). Considering the signalling overhead, the bit width of precoding information and number of layers for Rel-18 UL 8Tx codebook can be set as 7 or 8 bits.

(b) Scheme 2: Joint indication of TRI and TPMI (separate indication of coherence types)

If rounding up the codebook size to the smallest integer by adding precoders generated via Alt 2a is supported, the indication of fully-coherent codebook, partially-coherent, and non-coherent should be separately. For example, using 2 bits to indicate the coherence type as shown in Table 11. In each coherence type, joint indication of TRI and TPMI is supported.

Table 11 A two-bit field to indicate the coherence type

| Index | Coherence type |
| --- | --- |
| 0 | Fully-coherent |
| 1 | Partially-coherent |
| 2 | Non-coherent |
| 3 | Reserved |

Proposal 21: For TPMI design, using 2 bits to indicate the coherence type (fully-coherent, or partially-coherent, or non-coherent), and in each coherence type, legacy joint indication of TRI and TPMI is supported.

What’s more, considering the number of fully-coherent codewords is large than the other two, combining partially-coherent codebook and non-coherent codebook is also a simple way. In this case, 1 bit can be introduced to indicate the fully-coherent and partially-coherent + non-coherent as shown in Table 12. In each coherence type, joint indication of TRI and TPMI is supported.

Table 12 A one-bit field to indicate the coherence type

| Index | Coherence type |
| --- | --- |
| 0 | Fully-coherent |
| 1 | Partially-coherent or non-coherent |

Proposal 22: For TPMI design, using 1 bit to indicate the coherence type (fully-coherent, or partially-coherent + non-coherent), and in each coherence type, legacy joint indication of TRI and TPMI is supported.

(c) Scheme 3: Non-unified indication rule for different coherence types

If non-nested codebook is supported, each coherence type can have different indication methods. For fully-coherent codebook, using i1,1, i1,2, i1,3, and i2 to indicate explicitly is a simple way. However, in this case, introducing precoders generated via Alt 2a (non-DFT precoders) is precluded. For partially-coherent codebook, we can use two/four fields to indicate two/four Rel-15 UL 4Tx/2Tx precoding matrices, the bit width can be determined based on the selected candidate of Rel-15 UL 4Tx/2Tx codebook.

Proposal 23: For TPMI design, considering non-unified indication rule for different coherence types if non-nested codebook is supported. For example, using i1,1, i1,2, i1,3, and i2 to indicate fully-coherent codebook and using two/four fields to indicate two/four Rel-15 UL 4Tx/2Tx precoding matrices.

In addition, if separate/joint indication of antenna group and TPMI as well as the nested codebook are supported, the actual antenna group number of the codewords indicated by the gNB should be defined, e.g., using notation Ng’.

Proposal 24: If separate/joint indication of antenna group and TPMI as well as the nested codebook are supported, the actual antenna group number of the precoding matrix indicated by the gNB should be defined, e.g., using notation Ng’.

Conclusion

In this discussion, we provide our views on the SRI/TPMI enhancements on the 8Tx uplink transmission, our proposals are as follows:

Proposal 1: Confirm the Working Assumption to support dual CW for RANK>4 8Tx transmission.

Proposal 2: Support separate MCS, NDI and RV indication fields for the second CW, the definition for each indication field can be the same as the first codeword.

Proposal 3: Specify the RRC signaling to enable/disable the second CW.

Proposal 4: For better scheduling flexibility, support configuration of up to 2 or 4 SRS resource sets, each configured with up to 4 or 2 single-port SRS resources accordingly.

Proposal 5: For non-codebook based PUSCH transmission with 8Tx, support Option 1 to use a bitmap corresponding to all SRS resources configured which is preferred for the simplicity without any effort on the design of new SRI tables.

Proposal 6: Introduce a UE capability to report the supported antenna configuration to gNB.

Observation 1: Although the codebook is configured with high oversampling factors, there is still high probability that the selected optimal codeword belongs to low oversampling factors.

Observation 2: For (N1,N2) = (4,1), the acceptable performance degradation happens as the oversampling factor reduces. For (N1,N2) = (2,2), different oversampling factors show very similar performance.

Observation 3: The codewords with QPSK constellation elements can be used to reduce the implementation computational complexity by replacing the complex-number multiplication operations with the addition operations.

Proposal 7: To make a trade-off among performance, signalling overhead, and implementation complexity, only support (O1,O2) = (1,1) for (N1,N2) = (4,1) and (N1,N2) = (2,2).

Observation 4: For both (N1,N2) = (4,1) and (N1,N2) = (2,2), when rank=4, there is almost no performance loss for the codebook with fixed i1,3={0} compared to the codebook with flexible i1,3={0,1,2,3}.

Proposal 8: For Rel-18 UL 8Tx fully-coherent codebook, support the following configuration parameters.

- (N1,N2,O1,O2) = (4,1,1,1)

  - Rank=1: i1,1={0,1,2,3}, i1,2={0}, i2={0,1,2,3}

  - Rank=2,3,4: i1,1={0,1,2,3}, i1,2={0}, i1,3={0}, i2={0,1}

  - Rank=5,6: i1,1={0,2}, i1,2={0}, i2={0,1}

  - Rank=7,8: i1,1={0}, i1,2={0}, i2={0,1}

- (N1,N2,O1,O2) = (2,2,2,2)

  - Rank=1: i1,1={0,1}, i1,2={0,1}, i2={0,1,2,3}

  - Rank=2,3,4: i1,1={0,1}, i1,2={0,1}, i1,3={0}, i2={0,1}

  - Rank=5,6: i1,1={0,1}, i1,2={0}, i2={0,1}

  - Rank=7,8: i1,1={0}, i1,2={0}, i2={0,1}

Proposal 9: For Rel-18 UL 8Tx fully-coherent codebook, support the union set of codebook with (N1,N2) = (4,1) and codebook with (N1,N2) = (2,2).

Proposal 10: For Rel-18 UL 8Tx fully-coherent codebook, further study how to introduce the non DFT codewords or select the indices {i1,1, i1,2, i1,3, i2} candidate.

Proposal 11: Support Alt2 for two coherent groups of {0,1,4,5} and {2,3,6,7} for Ng=2. Support Alt1 for four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} for Ng=4.

Proposal 12: Support the following rank splitting rule for Ng=2.

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | (2,3), (3,2) |
| 6 |  | (3,3) |
| 7 |  | (3,4), (4,3) |

Proposal 13: For PUSCH transmission by a partially-coherent 8TX UE with Ng=2, support the precoding structure A104×rank(A2)04×rank(A1)A28×rank where A1 and A2 are precoding matrices taken from Rel-15 UL 4Tx fully-coherent codebook or empty matrix, the rank=rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and rank(A1) and rank(A2) are determined based on the rank splitting rule.

Proposal 14: For partially-coherent codebook, the candidate of Rel-15 UL 4Tx fully-coherent codebook should be further down-selected to reduce the signalling overhead, e.g., based on the principle that minimizing the cosine similarity between each codewords.

Observation 5: The rank splitting rule is pretty complex for Ng=4 partially-coherent codebook constructed by Rel-15 UL 2Tx fully-coherent codebook. And the rank splitting rule should be further down-selected.

Proposal 15: For PUSCH transmission by a partially-coherent 8TX UE with Ng=4, support the precoding structure A102×rank(A2)02×rank(A1)A202×rank(A3)02×rank(A4)02×rank(A3)02×rank(A4)02×rank(A1)02×rank(A2)02×rank(A1)02×rank(A2)A302×rank(A4)02×rank(A3)A48×rank where A1, A2, A3 and A4 are precoding matrices taken from Rel-15 2TX UL fully-coherent codebook or empty matrix, the rank= rank(A1) + rank(A2) + rank(A3) + rank(A4) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and rank(A1), rank(A2), rank(A3), and rank(A4) are determined based on the rank splitting rule for Ng=4 when Rel-15 UL 2Tx fully-coherent codebook is used.

Proposal 16: For PUSCH transmission by a partially-coherent 8TX UE with Ng=4, support the precoding structure A104×rank(A2)04×rank(A1)A28×rank where A1 and A2 are precoding matrices taken from Rel-15 UL 4Tx partially-coherent codebook or empty matrix, the rank=rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and the rank splitting rule can follow the same rule as that of Ng=2.

Proposal 17: Row/Column-interleaving operation should be used for Rel-18 UL 8Tx partially-coherent codewords to satisfy different port coherency schemes.

Proposal 18: Antenna selection vectors/matrixes can be used for the Rel-18 UL 8Tx non-coherent codebook. Considering the signalling overhead, all antenna selection vectors/matrixes can be used for 1≤rank≤X, a part of antenna selection vectors/matrixes can be used for X<Rank≤Y, and only one antenna selection vector/matrix can be used for Y<Rank≤8. The value of X and Y can be left for further study, e.g., X=2 and Y=4.

Proposal 19: For Rel-18 8Tx UE, the legacy codebook subset configuration rule can be reused, i.e., the fully-coherent UE can be configured with 'fullyAndPartialAndNonCoherent' codebook, partially-coherent UEs can be configured with 'partialAndNonCoherent' codebook, and non-coherent UE can be configured with 'NonCoherent' codebook.

Proposal 20: For TPMI design, reusing the joint indication of TRI and TPMI (legacy rule). Considering the signalling overhead, the bit width of precoding information and number of layers for Rel-18 UL 8Tx codebook can be set as 7 or 8 bits.

Proposal 21: For TPMI design, using 2 bits to indicate the coherence type (fully-coherent, or partially-coherent, or non-coherent), and in each coherence type, legacy joint indication of TRI and TPMI is supported.

Proposal 22: For TPMI design, using 1 bit to indicate the coherence type (fully-coherent, or partially-coherent + non-coherent), and in each coherence type, legacy joint indication of TRI and TPMI is supported.

Proposal 23: For TPMI design, considering non-unified indication rule for different coherence types if non-nested codebook is supported. For example, using i1,1, i1,2, i1,3, and i2 to indicate fully-coherent codebook and using two/four fields to indicate two/four Rel-15 UL 4Tx/2Tx precoding matrices.

Proposal 24: If separate/joint indication of antenna group and TPMI as well as the nested codebook are supported, the actual antenna group number of the precoding matrix indicated by the gNB should be defined, e.g., using notation Ng’.

References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94e, December 2021.

RAN1#110 Meeting chairman notes, August 2022.

RAN1#110b-e Meeting chairman notes, October 2022.

RAN1#111 Meeting chairman notes, November 2022.

RAN1#112 Meeting chairman notes, February 2023.

Appendix

Table 13 EVM for probability statistics.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz |
| Scheduled PRBs | 50 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5)λ |
| UE TX antenna configuration (N1,N2O1,O2) | (4,1,2,1) and (2,2,2,2) with 0.5λ |
| UE speed | 3 km/h, 30km/h |
| Number of Layers | Fixed, 1-8 layers |
| Modulation | QPSK |
| Code Rate | 0.4385 |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |
| Receiver | MMSE |

Table 14 EVM for performance evaluation.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz |
| Scheduled PRBs | 50 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (2,2,2,1,1,2,2) with (dH, dV) = (0.5, 0.5)λ |
| UE TX antenna configuration (N1,N2O1,O2) | (4,1,2,1) and (2,2,2,2) with 0.5λ |
| UE speed | 3 km/h |
| Number of Layers | Fixed, 1 layer and 4 layers |
| Modulation | QPSK |
| Code Rate | 0.4385 |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-C (300ns) |
| Receiver | MMSE |