3GPP TSG RAN WG1 #112-bis                                       	         R1-2302966

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.10.1

Source:	Xiaomi

Title:                     Discussion on L1 enhancements for inter-cell beam management in LTM

Document for:	Discussion and Decision

Introduction

In RAN #93 and #94 meeting, most of the companies prefer to support L1/L2 based inter-cell mobility in Rel-18 to reduce mobility latency. And the work item on further NR mobility enhancements was proposed as follows [1]:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

In this contribution, we will further discuss some possible L1 enhancements for inter-cell beam management and the dynamic switch mechanism among candidate cells.

Discussion on L1 measurement for inter-cell beam management

L1 measurement

Intra-frequency measurement

In RAN1 110-bis meeting, an agreement with respect to intra-frequency L1 measurement was adopted in chairman’s note as follows [2]:

110-bis-Agreement

- For Rel-18 L1/L2 mobility, L1 intra-frequency measurement for candidate cell is supported

  - At least the following aspects are for RAN1 further study:

    - RAN1 assumes Rel-17 ICBM CSI measurement as starting point.

    - Whether and how to apply relaxation for the restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement defined in 9.13.2 of TS38.133, where RAN4 impact is foreseen, e.g.

      - SFN offset alignment compared with serving cell

      - BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP

      - Introduction of symbol level gap or SMTC for larger Rx timing difference (i.e. larger than CP length)

    - Commonality with intra-frequency L3 measurement

    - Commonality with L1 inter-frequency measurement for measurement configuration

- Send an LS to RAN4 (CC RAN2)

  - RAN1 to ask RAN4 if the restriction on e.g., SFN offset alignment, BWP setting and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

  - RAN1 assumes Rel-17 ICBM CSI measurement as starting point.

The restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement defined in 9.13.2 of TS38.133 are shown as following:

-	The SSB from the cell with different PCI completely contained in the active BWP or associated with initial downlink BWP of the UE

-	The SSB of the cell with different PCI from serving cell has the same SCS, sfn-SSB-Offset and center frequency as the SSB of the serving cell

-	The timing difference of arrival at UE between the SSBs of serving cell and cell with different PCI is less than CP length of the corresponding SCS

Based on the LS reply form RAN4 [5], RTD between the SSBs of serving cell and neighbour cell on the same carrier can be larger than CP length of the corresponding SCS with additional UE capability. And about the parameter sfn-SSB-offset, the SFN offset alignment can be relaxed if UE performs L3 measurement before L1 measurement. What’s more, some scenarios that the SSB for intra-frequency L1 measurement may not be covered by serving cell active BWP will be considered. Therefore, these restrictions in Rel-17 intra-frequency L1 non-serving cell measurement defined in 9.13.2 of TS38.133 should relaxed.

Observation 1: The restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement can be relaxed based on the LS reply from RAN4

Inter frequency measurement

In last two meeting, we have reached the following agreements about inter-frequency measurement [2][3]:

110-bis-Agreement

- For Rel-18 L1/L2 mobility, further study the potential RAN1 spec impact of L1 inter-frequency measurement

  - The definition and scenarios of L1 inter-frequency measurement is determined by RAN4, and RAN1 assumes at least the following until receiving their confirmation

    - The scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios:

      - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

      - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

  - At least the following aspect is studied:

    - Commonality with L1 intra-frequency measurement for measurement configuration

- Send an LS to RAN4 (CC RAN2)

  - RAN1 would like to confirm our understanding that the supported scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios:

    - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

    - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

  - It is RAN1 understanding that the introduction of measurement gap and SMTC for L1 inter-frequency measurement, if any, is expected to be a RAN4 issue

  - Note: this content is included in the LS agreed for intra-frequency L1 measurement

111-Agreement

- For Rel-18 LTM, L1 inter-frequency measurement is supported from RAN1 point of view.

From RAN1 perspective, inter-frequency L1 measurement should be supported. For inter-frequency measurement, it will take time for UE to retune RF before perform L1 measurement on candidate cell RS, UE cannot transmit to or receive from serving cell during this time. Therefore, a measurement gap/window may need to be introduced in L1 measurement in L1/L2 mobility as shown in Figure 1, otherwise serving cell may transmit data when UE is retuning RF, which will result in data loss. Or, the measurement gap in RRM measurement could be reused when the measurement RS in both case is the same.

Fig.1 UE RF retuning for inter-frequency case

Proposal 1: L1 measurement gap/window needs to be introduced to support inter-frequency beam measurement.

UE based filtering for L1 measurement.

- The intention to introduce UE based filtering for L1 measurement was to avoid ping-pong issue in LTM based on the discussion in last two meetings. We do not think it necessary to further filter L1 measurement results by UE. First, form our understanding, the cell switch decision is up to NW and the ping-pong issue could be partly avoided by NW with an appropriate cell switch decision criterion. That is the L1 measurement results can be further filtered by NW to avoid ping-pong issue. Secondly, based on the LS from RAN2, sequential L1L2 cell change between candidate cells can be supported in L1/L2 mobility, which means UE is actually allowed to switch among these cells more dynamically than conventional handover. NW can decide whether the L1 measurement results need to be filtered or not.

- In addition, if the results are filtered by UE and then reported to NW. NW does not know the best beam(s) between UE and candidate cell(s). When NW work decide to perform cell switch, another beam management procedure should be executed to get the transmission beam of target cell.

In short, we do not support UE based filtering for L1 measurement results in LTM.

- Proposal 2: Do not support UE based filtering for L1 measurement results in LTM.

Reporting

Reporting format

The agreements about reporting format of L1 measurement are listed as follows [2][3]:

110-bis-Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, further study the following mechanisms:

  - Report as UCI on PUCCH or PUSCH

    - Periodic report on PUCCH, semi-persistent report on PUCCH/PUSCH, and aperiodic report on PUSCH

    - Potential enhancements to Rel-17 ICBM report format to accommodate Rel-18 scenarios, e.g.

      - Inter-frequency measurement, if supported

      - Increasing the maximum number of reported beams, which is 4 for Rel-17 ICBM

      - Flexible size beam report, e.g., two-part UCI (e.g., the 1st part contains the best beam/cell and the number (e.g., N) of reported beams/cells, the 2nd part contains the rest (N-1) beams/cells

      - Reducing the reporting overhead by e.g. choosing beams/cells per frequency or across frequencies to report (FFS how)

  - Report on MAC CE

    - Both gNB scheduled and/or UE initiated (if supported) report are studied

111-Agreement

- For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported

  - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported

    - FFS: periodic and semi-persistent PUCCH

  - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

After UE get the L1 measurement results of candidate cells, it should report these results to serving cell to make decision whether UE should switch to one of these candidate cells. Based on the agreement above, the results of serving cell and candidate cell(s) can be included in a single report instance. About the number of beams that can be reported in one instance, there are two alternatives. First, it can be specified as N no matter how many candidate cells are configured to perform L1 measurement. And how to choose the N beams to be reported from multiple candidate cells should be further discussed. Secondly, it can be specified as K×M, where K is the number of cells that are configured to perform L1 measurement and M is the number of beams for each cell should be reported.

Proposal 3: There are two alternatives about the number of beams should be reported in one report instance:

- Alt.1: N beams, no matter how many candidate cells are configured to perform L1 measurement

  - How to choose the N beams to be reported can be further discussed

- Alt.2: K×M beams, where K is the number of cells need to be reported and M is the number of beams for each cell should be reported

  - K could be the number of the candidate cells or cells that are selected from candidate cells

In addition, how to distinguish the measurement results of each cell should specified when the measurement results of serving cell and candidate cells are reported together in a single report instance. First, the PCI or other indicator of candidate cell can be included in the reporting instance. Secondly, a cell list can be configured by serving cell and UE needs to arrange the measurement results based on this cell list. And, other feasible methods also can be used.

Proposal 4: How to distinguish the measurement results of each cell should be specified when the L1 measurement results of serving cell and candidate cells are reported in a single report instance.

Based on the discussion above, for each beam in the report instance, at least the corresponding measurement RS indicator the related measurement quantity, like L1-RSRP, should be included.

Proposal 5: For each beam in the report instance, at least the corresponding measurement RS indicator and measurement quantity need to be included.

Event triggered reporting

About introducing event triggered reporting for L1 measurement, we have reached an agreement as following [3]:

111-Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study

    - At least the following aspects may be considered

      - How to define UE event and exact definition of events,

      - Report container

      - Resource allocation/assignment for UE event triggered report

      - Necessity of indication to gNB when the condition UE event is met, and how

      - Necessity to define the condition to start/stop the reporting,

      - Contents of the report/reporting format, PCI, RS ID, measurement result etc.

      - The interaction with filtered L1 measurement results (if supported)

      - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured.

      - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s).

      - Benefit when L3 measurement is involved

In current reporting mechanism, the L1 measurement results of serving cell and candidate cells should be reported every time the L1 measurement is conducted, which will cause heavy reporting resource overhead. In order to reduce the overhead, event triggered reporting mechanism, in which only these measurement results of the candidate cell that meet a certain condition need to be reported, can be supported in L1 measurement. For example, if the maximum/average value of measurement results of candidate cell A is larger than that of serving cell, then the measurement results of candidate cell A should be reported to serving cell. Therefore, we are supportive about introducing event triggered reporting mechanism.

Proposal 6: Support event triggered beam measurement reporting mechanism.

Then, the exact definition of the events should be specified. In last meeting, some companies support to reuse the L3 event. However, the L3 event is defined based on L3 measurement results. Reusing the L3 event in L1 measurement is not appropriate because the L1 measurement results cannot be reported timely. We prefer to introduce new event(s), which can be called L1 event. For example, the average value of L1 measurement results of a candidate cell is larger than that of serving cell. Then, the L1 measurement results of this candidate cell should be reported.

Proposal 7: We support to introduce new event, denoted as L1 event to distinguish it from L3 event, for event triggered beam measurement reporting. For example,

- L1 event: The average value of L1 measurement results of a candidate cell > the average value of L1 measurement results of serving cell + Offset.

Configuration of L1 measurement and report of candidate cell

In last meeting, we discussed the configuration of L1 measurement of candidate cells and decided to send a LS to RAN2 [4].

Agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

    - Including above agreement into the LS

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

    - Following RAN1 understanding will be provided in the LS

      - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112:

        - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells

          - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

        - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

          - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]

        - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

          - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

      - Note RAN2 has a full flexibility to design the whole RRC structure design.

      - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters.

      - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

While, the necessary information that should be included in the measurement RS configuration still needs to be decided by RAN1. In R17 ICBM, the transmission power used to estimate pathloss is configured via RRC, which is ss-PBCH-BlockPower-r17. In LTM, because TA management is under discussion to support RACH-less cell switch, then after UE receive cell switch command, from our understanding, UE needs to send acknowledgement to targe cell to inform it the cell switching as shown in Figure2. Then, the transmission power used to estimate pathloss is needed to decide the transmit power of this acknowledgement. And even when the cell switch is completed, this transmission power used to estimate pathloss might be used to determine the transmit power of other UL channel(s) of target cell.

Fig.2 Possible procedures for network triggered L1/L2 mobility

Proposal 8: Transmission power used to estimate pathloss, like the ss-PBCH-BlockPower-r17 for R17 ICBM, needs to configured to UE.

Beam indication of target cell

There are some agreements about the beam indication of target cell in L1/L2 mobility as following [3][4]:

111-Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

111-Agreement

- For beam indication timing for Rel-18 LTM,

    - Support Scenario 2: Beam indication together with cell switch command,

      - For Rel-17 unified TCI framework,

        - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

112-Agreement

- At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s)

- FFS: beam indication for mTRP case

In RAN1 111 meeting, we reached an agreement that the design of LTM should be based on R17 unified TCI framework. While, we believe that R17 uTCI is not prerequisite for R18 LTM. That is, Rel-18 LTM should be supported even that serving cell and candidate cell do not support R17 TCI framework.

Proposal 9: Rel-18 LTM should be supported even both serving cell and candidate cell do not support Rel-17 TCI framework.

Another issue with respect to beam indication is the timing of beam indication of target cell. There are three possible scenarios and scenario 2 where beam of target cell is indicated together with cell switch command according to the agreements. Whether scenario 1, in which beam of target cell is indicated before cell switch command and scenario 2 where beam of target cell is indicated before cell switch command should be discussed is still not decided.

For scenario 1, there are two cases that are denoted as scenario 1-1 and scenario 1-2 as shown in Figure 3. In scenario 1-1, the beam of target cell is indicated at T1 when serving cell makes cell switch decision. While, in scenario 1-2, the beam of target cell is indicated before serving cell makes cell switch decision. Apparently, scenario 1-2 is impossible because serving cell don’t know the target cell yet at T0. Based on the evaluation of the beam indication timing in figure 1, scenario 2 has less time overhead and signaling overhead. We don’t see any benefit of scenario 1 and Scenario 3.

Fig.3 Evaluation of the beam indication timing

Proposal 10: For the beam indication of target cell, we do not support scenario 1 and scenario 2.

Discussion on dynamic switch mechanism among candidate serving cells

How to trigger cell switch

Based on the WID in section 1, L1/L2 mobility rely on L1 measurements to trigger the cell switch. From our understanding, there are two possible methods to trigger dynamic handover. First, it is the Network that triggers the handover by sending handover command. When the L1 measurement results of serving cell and candidate cells meet a certain condition, namely handover condition, Network initiates the dynamic handover by sending the handover command. Another way is to let the UE itself triggers the dynamic handover/switch when the handover condition, which can be configured by network, is satisfied.

Proposal 11: There are two possible methods to trigger cell switch

- Network triggered L1/L2 mobility

- UE triggered L1/L2 mobility

In the initial specification work of L1/L2 mobility, we can first discuss the network triggered dynamic handover which should be the base line of L1/L2 mobility. Then, UE triggered dynamic handover can be further discussed if there is sufficient time.

Proposal 12: We can first discuss the network triggered cell switch.

Cell switch command

In last two RAN1 meetings, we have discussed the cell switch command and reached agreements as following [2][4]:

110-bis-Agreement

-       Interested companies are encouraged to perform technical analysis of the cell switch command from a RAN1 point of view, e.g.

-       Necessary information included in the command, which is relevant for RAN1 discussion

-       Necessary number of bits for the information

-       L1 impact or concern to use DCI or MAC CE for L1/L2 cell switch command

112-Agreement

- RAN1 shares the same understanding as RAN2 on agreement:

  - The LTM mobility trigger information is conveyed in a MAC CE

- The same MAC CE is used for the LTM triggering.

It was agreed that the MAC CE, which will be introduced based on RAN2’s discussion, conveys trigger information will be used as cell switch command in RAN1 112 meeting. The necessary information included in cell switch command should be further discussed.

At first, the target cell indicator should be included in cell switch command to inform UE which candidate cell will be the target cell. The indicator could be a PCI or an index of candidate configuration. And based on the discussion in section 2.4, the beam indication of target cell should also be included in this command. In addition, in agenda 9.12.2, the TA of candidate cell will be measure to support RACH-less cell switch. And it was agreed that the TA of target cell is indicated in cell switch command when RACH without RAR is used to acquire the TA of candidate cell(s).

Proposal 13: At least the following information should be included in cell switch command:

- The PCI of target cell or index of target cell configuration

- Beam of target cell

- TA of target cell, at least when RACH without RAR is used to measure the TA of target cell

Conclusion

In this contribution, we discussed the possible L1 enhancements for inter-cell beam management and the dynamic switch mechanism among candidate cells in L1/L2 mobility. Based on above discusses, we provide the following proposals:

Observation 1: The restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement can be relaxed based on the LS reply from RAN4

Proposal 1: L1 measurement gap/window needs to be introduced to support inter-frequency beam measurement.

- Proposal 2: Do not support UE based filtering for L1 measurement results in LTM.

Proposal 3: There are two alternatives about the number of beams should be reported in one report instance:

- Alt.1: N beams, no matter how many candidate cells are configured to perform L1 measurement

  - How to choose the N beams to be reported can be further discussed

- Alt.2: K×M beams, where K is the number of cells need to be reported and M is the number of beams for each cell should be reported

  - K could be the number of the candidate cells or cells that are selected from candidate cells

Proposal 4: How to distinguish the measurement results of each cell should be specified when the L1 measurement results of serving cell and candidate cells are reported in a single report instance.

Proposal 5: For each beam in the report instance, at least the corresponding measurement RS indicator and measurement quantity need to be included.

Proposal 6: Support event triggered beam measurement reporting mechanism.

Proposal 7: We support to introduce new event, denoted as L1 event to distinguish it from L3 event, for event triggered beam measurement reporting. For example,

- L1 event: The average value of L1 measurement results of a candidate cell > the average value of L1 measurement results of serving cell + Offset.

Proposal 8: Transmission power used to estimate pathloss, like the ss-PBCH-BlockPower-r17 for R17 ICBM, needs to configured to UE.

Proposal 9: Rel-18 LTM should be supported even both serving cell and candidate cell do not support Rel-17 TCI framework.

Proposal 10: For the beam indication of target cell, we do not support scenario 1 and scenario 2.

Proposal 11: There are two possible methods to trigger cell switch

- Network triggered L1/L2 mobility

- UE triggered L1/L2 mobility

Proposal 12: We can first discuss the network triggered cell switch.

Proposal 13: At least the following information should be included in cell switch command:

- The PCI of target cell or index of target cell configuration

- Beam of target cell

- TA of target cell, at least when RACH without RAR is used to measure the TA of target cell

References

- RP-213565, “New WID on Further NR mobility enhancements”, TSG RAN Meeting #94e Electronic Meeting, Dec. 6 - 17, 2021

- RAN1 #110-bis meeting Chairman notes, October, 2022

- RAN1 #111 meeting Chairman notes, November, 2022

- RAN1 #112 meeting Chairman notes, February 27th – March 3rd, 2023

- R4-2303308, “Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility”, TSG-RAN WG4 Meeting # 106, February 27th – March 03rd, 2023