3GPP TSG RAN WG1 #112bis-e			R1-2302971

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.12.2

Source:	Xiaomi

Title:	Discussion on power domain enhancements

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID for Further NR coverage enhancements is approved [1], which includes the following objective:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM.

The detailed objectives of the work item are as follows:

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure

  - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

- Study and if necessary specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

Coverage enhancement is one of the key factors for communication networks. At present in real deployments, uplink transmission and coverage have always been the bottleneck of the system performance. In this contribution, we mainly provide our views on the enhancements to realize the high power limit for the UL transmission in CA and DC.

2	Discussion

In RAN1#110bis meeting, the following agreement has been made on power domain enhancements for CA/DC [2] and an LS has been sent to RAN4.

Agreement

For enhancements to realize increasing UE power high limit for CA and DC, RAN1 can study based on RAN4’s input

- Whether RAN1 enhancements to information exchange between UE and gNB are needed to improve scheduling and network performance when using higher power CA/DC.

  - FFS how to realize such information exchange, e.g., signalling enhancement, and what is the spec impact.

In RAN1#111 meeting, the following agreement has been made on power domain enhancements for CA/DC [6].

Agreement

- At least the following enhancements to information exchange between UE and gNB to facilitate higher power transmissions in CA and DC can be considered for study. Enhanced signaling, if necessary and subject to RAN4’s input, to allow:

  - Determination at gNB of power class change at the UE

  - Increased awareness at gNB of energy/power availability at the UE, e.g., a budget.

    - More informative PHR to be sent from UE to gNB, which may include, e.g., P-MPR related information, power headroom for carrier configured for DL but not UL, power class change indication.

  - More effective scheduling decisions in the context of UL CA, e.g., best band combination, preferred carrier for servicing uplink, adaptive load sharing across sharing,

- Other options are not precluded.

For 5G-NR, it is very important to improve the uplink coverage and throughput by supporting the uplink transmissions across multiple bands via CA/DC. But in real deployments, these techniques are not very practical and transmit power limitation is one of the key issues. The power domain enhancements is a straightforward way to increase the UL coverage and worth investigating.

In Rel-17, RAN4 has addressed the issue to enable the higher maximum output power for UL CA and DC cases for HPUE, the maximum output power relaxation for a band combination can be the sum of the individual power classes for each band. And a new UE capability signalling was introduced to support this feature.

In RAN1#112 meeting, the following agreement has been made on power domain enhancements for CA/DC [7].

Agreement

Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input).

  - FFS: details.

The LS reply [8] from RAN4 showed that no consensus reached on the issues/approaches from RAN4 perspective.

In our view, these issues identified are mostly RAN1 issues. From the above, we share proponents’ views that the gNB need to consider the information related to whether and how the UE can deliver high output power due to the regulatory constraints for scheduling. Better scheduling decisions for CA/DC cases can be expected, which would benefit the system with better UL coverage, better throughput, and also help to save UE power consumptions.

Proposal 1: Support to study the enhancements to information exchange between UE and gNB to improve scheduling and network performance when using higher power CA/DC.

Proposal 2: RAN1 study on the mechanism to enable efficient use of the increased full power for CA/DC should be carried out.

According to the current RAN4 spec, different band combination has different regulatory requirements of Pemax which also limits the total output power UE can deliver for UL CA/DC. In practice, UE may not be able to deliver full output power at all times due to SAR compliance. According to the current spec, how UE determines the safe transmit power level and the instantaneous transmit power for each of the bands due to the SAR limit is transparent to gNB. The power class used can also be changed by the UE, e.g. fall back to PC3. The gNB cannot differentiate such power fluctuations either from the SAR requirements or from the channel/link quality change. Also gNB cannot get the accurate information from the current PHR reporting. This misaligned understanding between UE and the gNB may influence the practical use of the high power for CA/DC [3][4][5].

In our view, this power class change indication may not need to be introduced since the current PHR reporting already provide the report of Pcmax which reflect the power class change, but the one-shot reporting after the PC fallback happened may not be that useful. We support the reporting of P-MPR in FR1 which can reflect the power fallback due to SAR requirements. Despite of considering the indication of power class change, some other indication also needs to be considered to let the gNB aware of more information of the current power class sustainable situation. We find the following approaches more meaningful from discussions in the previous meetings,

- P-MPR reporting in FR1 due to SAR requirements ;

- Number of symbols or proportion of symbols in the current SAR window that UE assumes to sustain the high power class without having to fallback to make a power class change;

- UE recommended maxUplinkDutyCycle value that would prevent triggering a power class fallback;

Proposal 3: Consider the following approaches to help UE get a better chance to maintain the high Power class,

- P-MPR reporting in FR1 due to SAR requirements

- Number of symbols or proportion of symbols in the current SAR window that UE assumes to sustain the high power class without having to fallback to make a power class change;

- UE recommended maxUplinkDutyCycle value that would prevent triggering a power class fallback;

For high power UEs, the gNB unawareness of the UE power change due to SAR compliance would degrade the efficiency of the link adaptation scheduling. In our view, the enhancement on this issue would be beneficial for both non-CA and CA/DC cases.

Proposal 4: The enhancement to solve the SAR compliance issue for a better awareness of UE energy/power availability can be applied to both non-CA and CA/DC cases.

Another issue is the unawareness of the UE cumulative power level due to SAR compliance which leads to inappropriate adaptation scheduling to the characteristics of future UL traffic data. Whether high output power can be obtained also has an impact on whether CA/DC operation should be scheduled.

Proposal 5. Support reporting of informative PHR at least to improve the accuracy of the acknowledgement of UE power/energy change due to SAR requirements.

Proposal 6: Further considering at least P/AP triggering and reporting of the enhanced PHR;

3	Conclusion

In this contribution, we provide our views of the enhancements on the power domain enhancements for CA/DC, our proposals are as follows:

Proposal 1: Support to study the enhancements to information exchange between UE and gNB to improve scheduling and network performance when using higher power CA/DC.

Proposal 2: RAN1 study on the mechanism to enable efficient use of the increased full power for CA/DC should be carried out.

Proposal 3: Consider the following approaches to help UE get a better chance to maintain the high Power class,

- P-MPR reporting in FR1 due to SAR requirements

- Number of symbols or proportion of symbols in the current SAR window that UE assumes to sustain the high power class without having to fallback to make a power class change;

- UE recommended maxUplinkDutyCycle value that would prevent triggering a power class fallback;

Proposal 4: The enhancement to solve the SAR compliance issue for a better awareness of UE energy/power availability can be applied to both non-CA and CA/DC cases.

Proposal 5. Support reporting of informative PHR at least to improve the accuracy of the acknowledgement of UE power/energy change due to SAR requirements.

Proposal 6: Further considering at least P/AP triggering and reporting of the enhanced PHR;

4	References

RP-221858, New WID: Revised WID on Further NR coverage enhancements, China Telecom, RAN#96, June 2022

RAN 1#110bis-e Meeting chairman notes, October 2022

R1-2210014, Power-domain enhancements, Qualcomm Incorporated, RAN1#110bis, October 2022

R1-220926, Discussion on power domain enhancements for CA/DC, Fujitsu, RAN1#110bis, October 2022

R1-220926, Discussion on power domain enhancements, NTT DOCOMO, RAN1#110bis, October 2022

RAN 1#111 Meeting chairman notes, November 2022

RAN 1#112 Meeting chairman notes, February 2023

R4-2303701, Reply LS on enhancements to realize increasing UE power high limit for CA and DC, NTT DOCOMO, RAN1#112, February 2023

R4-2303560, WF on coverage enhancement part 1, Huawei, HiSilicon, RAN1#112, February 2023