3GPP TSG RAN WG1 #112bis-e			R1-2302972

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.12.3

Source:	Xiaomi

Title:	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID for Further NR coverage enhancements is approved [1], which includes the following objective:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM.

The detailed objectives of the work item are as follows:

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure

  - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

- Study and if necessary specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

Coverage enhancement is one of the key factors for communication networks. At present in real deployments, uplink transmission and coverage have always been the bottleneck of the system performance.

2	Discussion

In NR, two waveforms of DFT-s-OFDM and CP-OFDM can be applied for the uplink. Unlike LTE, a consistent CP-OFDM waveform for the uplink same as downlink is preferred which can benefit the system design and implementation. Considering the better PAPR/CM characteristics of DFT-s-OFDM waveform, it is more suitable for cell-edge UEs to achieve a better coverage and performance. In Rel-16, some PAPR enhancements was also introduced for the DMRS with Pi/2 BPSK modulation for the DFT-s-OFDM waveform to maintain a low PAPR/CM performance. Currently, the waveform used in the uplink is RRC-configured and the switching between CP-OFDM and DFT-s-OFDM needs RRC reconfiguration. This limitation imposes a large barrier to switch over to DFT-s-OFDM waveform for cell-edge UEs. Thus we agree the flexible assessment of DFT-s-OFDM should be pursued.

A lot of debates were held at the stage of the WID, in our understanding of the scope, the dynamic switching is to enhance the switching between CP-OFDM and RANK-1 DFT-s-OFDM.

In RAN1#110bis meeting, the following agreement has been made on DSW [2]:

Agreement

- Dynamic waveform switching enhancement in R18 is only applicable to PUSCH channel.

In RAN1#112 meeting, the following agreement has been made on supported PUSCH type [4]:

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

First, it was agreed that only dynamic grant PUSCH is supported by DWS in Rel-18 and the indication signaling cannot be applied to the subsequent PUSCH transmissions.

In RAN1#110bis meeting, the following agreement has been made on mechanism of DSW [2]:

Working Assumption

Support at least one of the following options for the dynamic waveform indication in R18:

Alt 1: Indication from an UL scheduling DCI

- Alt 1-A: New field in scheduling DCI

- Alt 1-B: Reuse existing field in scheduling DCI

  - Alt 1-B-1: Explicit indication by repurposing field, e.g.

    - Add one column to TDRA table

    - Add one column to MCS table(s)

    - Other solutions not precluded

  - Alt 1-B-2: Implicit determination from condition(s) on scheduling information, e.g.

    - RA type, MSB of RA

    - Number of RBs (below threshold or multiple of 2,3,5)

    - Location of RB allocation within carrier and the associated MPR

    - MCS below threshold

    - Number of PUSCH repetitions (or whether PUSCH repetition is used) and/or TBoMS

    - Number of DMRS CDM group(s) without data

    - Precoding information and number of layers

    - SRI

    - Condition over multiple types of scheduling information

    - Other types of scheduling information not precluded

- Indicated waveform applies at least to the scheduled PUSCH transmission

  - FFS: Whether it also applies to subsequent transmissions, and of which type

- FFS: DCI formats can contain the indication

- FFS: Indication applies only if condition(s) are satisfied (e.g. PDCCH occasion, /RNTI, /Search space of the scheduling DCI, latest PHR reported by the UE, etc.)

Alt 2: Indication from a non-UL scheduling DCI

- FFS: DCI formats that can provide the indication (e.g. Downlink DCI, UE-group common DCI)

- FFS: Types of subsequent transmissions to which indication is applicable

In RAN1#111 meeting, the following agreement has been made on mechanism of DSW [3]:

Agreement

For DCI based solution,

- For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI

  - Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion

  - Note: It does not imply that the waveform switching indication applies to other transmission or not

- Indication from non-UL scheduling DCI is not supported.

Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed

Working Assumption

Support new 1-bit field for dynamic waveform indication from UL scheduling DCI

- Note: no change of the current size alignment procedure between UL DCI and DL DCI

In RAN1#112 meeting, the following agreement has been made on mechanism of DSW [4]:

Agreement

For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Note: no change of the current size alignment procedure between UL DCI and DL DCI.

Second, it was agreed that 1-bit new DCI field is introduced for DSW.

In RAN1#110bis meeting, the following agreement has been made on assistance information [2]:

Agreement

To study and if necessary, specify, enhancements to assist the scheduler in determining waveform switching, such as:

- Reporting power headroom related information

- Other solutions are not precluded

In RAN1#111 meeting, the following agreement has been made on assistance information [3]:

Agreement

Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching:

- Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform

  - Target waveform can be same or different from waveform of an actual PUSCH transmission

  - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission

  - FFS determination of target waveform, target RB allocation, target modulation order

  - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms

- PHR triggering enhancements, e.g.

  - Network-triggered PHR

  - PH becomes lower (higher) than a threshold

  - PHR triggered by waveform switching

- Reporting of recommended waveform or request to switch waveform

- Other solutions not precluded

We think it is necessary for the gNB to obtain certain information on waveform switching assistance information, and that can be achieved by enhancing the current PHR reporting. Two PHs of different waveforms can be reported as Type 1 power headroom reporting to provide the gNB enough power difference information of the two waveforms. The reference PUSCH used to calculate the PH of the target waveform can be the same as the current PUSCH, i.e. same RB allocation, same MCS.

Also the triggering conditions needs to be enhanced. In our view, PH compared with a threshold for a certain waveform can be used to trigger the PHR reporting. When UE is already at cell edge, it also makes sense to trigger PHR reporting when continuous data error occurs.

Another approach is to let UE directly report of the proposed waveform to the gNB with the current one actual PH.

Proposal 1: Support assistance information reporting for the switching of waveforms.

Proposal 2: Support the enhancement of the PHR triggering and reporting mechanism to help determine a waveform switching.

Proposal 3: Support reporting of two PHs of different waveforms for the Type 1 PHR, and the current PUSCH can be used as the reference PUSCH for the calculation of the PH of the target waveform.

Proposal 4: Support considering the options for the triggering enhancements of the PHR,

- PH compared with a threshold for a certain waveform can be used to trigger the PHR reporting;

- continuous data error compared with a threshold;

Proposal 5：Support UE reporting of the proposed waveform to the gNB with the current one actual PH.

3	Conclusion

In this contribution, we provide our views of the enhancements on the dynamic switching between uplink waveforms, our proposals are as follows:

Proposal 1: Support assistance information reporting for the switching of waveforms.

Proposal 2: Support the enhancement of the PHR triggering and reporting mechanism to help determine a waveform switching.

Proposal 3: Support reporting of two PHs of different waveforms for the Type 1 PHR, and the current PUSCH can be used as the reference PUSCH for the calculation of the PH of the target waveform.

Proposal 4: Support considering the options for the triggering enhancements of the PHR,

- PH compared with a threshold for a certain waveform can be used to trigger the PHR reporting;

- continuous data error compared with a threshold;

Proposal 5：Support UE reporting of the proposed waveform to the gNB with the current one actual PH.

4	References

RP-221858, New WID: Revised WID on Further NR coverage enhancements, China Telecom, RAN#96, June 2022

RAN 1#110bis-e Meeting chairman notes, October 2022

RAN 1#111 Meeting chairman notes, November 2022

RAN 1#112 Meeting chairman notes, February 2023