3GPP TSG RAN WG1 #112bis-e                                       	      R1-2302973

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.16

Source:	Xiaomi, China Unicom, OPPO, ZTE

Title:                     Rel-18 TEI on pathloss RS for Type 1 CG-PUSCH

Document for:	Discussion and Decision

Background

In RAN1#99 [1], the following agreements were archived on the update of pathloss reference for PUSCH via MAC-CE.

Agreement (RRC impact)

On power control for PUSCH, PUCCH, and SRS, the total number of maximum configurable pathloss RSs, in including those supported in Rel-15, by RRC is 64

- Note: Such pathloss reference signals are for configuration purpose only, and UE is still only required to track up to 4 pathloss RSs for any PUSCH, PUCCH, and SRS transmissions.

  - “Up to 4 pathloss RSs” applies the total number of pathloss RSs for PUSCH, PUCCH, and SRS

Agreement (RRC impact)

For the feature of MAC CE based pathloss RS updates for PUSCH/SRS in Rel-16,

- Introduce one new RRC parameter to enable the feature of MAC CE based pathloss RS updates for PUSCH/SRS in Rel-16, i.e.,

  - enablePLRSupdateForPUSCHSRS

Agreement

When enablePLRSupdateForPUSCHSRS is configured, if a grant-based or grant-free PUSCH transmission is scheduled/activated by DCI format 0_1 that does not include a SRI field, the RS resource index qd corresponding to the PUSCH-PathlossReferenceRS-Id mapped with sri-PUSCH-PowerControlId = 0 is used for path-loss measurement of PUSCH transmission. In this case, UE expects to be configured with sri-PUSCH-PowerControl

While in this contribution, we give our views on the update of pathloss reference signal for Type 1 CG-PUSCH in Rel-16.

Pathloss reference signal for Type 1 CG-PUSCH

According to the agreement listed above, we can see that the pathloss reference signal for Type 2 CG-PUSCH and dynamic-grant PUSCH can be updated by the SRI field in DCI. But for Type 1 CG-PUSCH, even the spatial relation based on srs-ResourceIndicator configured in rrc-ConfiguredUplinkGrant can be updated by MAC CE, but the corresponding pathloss reference signal can’t be updated and the pathlossReferenceIndex configured in rrc-ConfiguredUplinkGrant should be used refer to the text in 38.213[2]. In this case the pathloss reference signal will be not matched with the spatial relation indicated by srs-ResourceIndicator.

So we propose the following text proposal.

7.1.1 UE behaviour

……

-	For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig,

- a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

……

7.1.1 UE behaviour

……

<Unchanged parts are omitted>

-	For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig,

- if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, the UE determines a RS resource index  from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the sri-PUSCH-PowerControlId indicated by the srs-ResourceIndicator value included in rrc-ConfiguredUplinkGrant

- if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

- ……

<Unchanged parts are omitted>

Conclusion

In this contribution, we discuss about the update of pathloss reference signal for Type 1 CG-PUSCH in Rel-16. Based on above discusses, we provide the following text proposal.

7.1.1 UE behaviour

……

<Unchanged parts are omitted>

-	For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig,

- if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, the UE determines a RS resource index  from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the sri-PUSCH-PowerControlId indicated by the srs-ResourceIndicator value included in rrc-ConfiguredUplinkGrant

- if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

- ……

<Unchanged parts are omitted>

References

- RAN1-99 meeting Chairman notes.

- TS 38.213 g80, 3GPP Physical layer procedures for control