3GPP TSG RAN WG1 #112bis-e			R1-2302981

e-Meeting, April 17th –24th, 2023

Source:            Xiaomi

Title:			  Discussion on evaluation on NR duplex

Agenda item:    9.3.1

Document for:   Discussion

Introduction

In RAN1#112 meeting, we further discussed remaining issues related to evaluation on NR duplex evolution [1].

In this contribution, we share views on the remaining issues for SLS and LLS. Furthermore, we provide preliminary SLS evaluation results based on the current assumptions.

Discussion

Remaining issues on SLS evaluation

Channel model

SBFD Deployment Case 3-2: 2-layer scenario B

gNB-to-UE link

In RAN1#112 meeting, we further discussed gNB-UE channel modelling for 2-layer Scenario B and the following agreement was achieved:

Agreement For Deployment case 3-2 (2-layer Scenario B), update Indoor-TRP to outdoor UE channel model as follows: Large-scale channel parameters; Indoor TRP to Outdoor UE: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 1: - UMi-Street canyon in TR 38.901 (hBS =3 m) - Option 2: - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - For both options, O2I penetration loss between indoor TRP and outdoor UE follows Table A.2.1-12 in TR38.802 (d2D-in is the distance between the indoor TRP and the building boundary along the direction from Indoor TRP to outdoor UE. The d2D-in may be different for different indoor-TRP-outdoor-UE links associated with the same indoor TRP). Fast fading parameters; Indoor TRP to Outdoor UE: - Option 1: - 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 1: - UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA - Option 2: - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901.

In RAN1#110bis, InH-Office in TR 38.901 for Indoor office layer and InF in TR 38.901 for indoor factory layer were agreed for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B). In RAN1#112, UMi-Street canyon in TR 38.901 was agreed for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B). Considering that indoor TRP to outdoor UE is also O2I link, it is straightforward that O2I channel model parameters related to fast fading is used for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B).

O2I parameters should be used for indoor TRP to outdoor UE in deployment case 3-2 (2-layer Scenario B) if fast fading channel model is applied.

In UMi-Street Canyon, O2I channel model parameters in table 7.5-6 Part-1 in TR 38.901 are used for Macro TRP to indoor UE. Since indoor TRP to outdoor UE is also O2I link, it is straightforward that O2I channel model parameters for Macro TRP to indoor UE in TR 38.901 are reused for indoor TRP to outdoor UE if option 1 is adopted. However, O2I channel model parameters are not modelled for indoor office and indoor factory in TR 38.901, i.e. only LOS and NLOS are defined. For simplicity, O2I fast fading channel model parameters table 7.5-6 Part-1 in TR 38.901 for UMi-Street canyon could be reused for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B) if option 2 is adopted.

For fast fading parameters, O2I channel model parameters for UMi-Street canyon/UMa in table 7.5-6 Part-1 in TR 38.901 could be reused for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B) when option 2 is adopted.

LOS probability

For Large-scale channel model of InH-Office in TR 38.901, d2D-in is used for the LOS probability calculation. In RAN1#112 meeting, it was agreed that d2D-in used for LOS probability in Table 7.4.2-1 in TR 38.901 is the 2D distance between BS and UE (d2D) for LOS probability calculation.

Agreement For indoor office scenario agreed for SBFD and dynamic/flexible TDD evaluation, the LOS probability of Indoor - Open office in Table 7.4.2-1 of TR38.901 is used. - d2D-in used for LOS probability in Table 7.4.2-1 in TR 38.901 is the 2D distance between BS and UE (d2D).

Channel model of InH-Office in TR 38.901 is also used for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B), indoor TRP to indoor TRP and indoor UE to indoor UE in Deployment Case 1(indoor office scenario) and Deployment case 3-2 (2-layer Scenario B). For simplicity, d2D-in used for LOS probability in Table 7.4.2-1 in TR 38.901 is the 2D distance between BS and UE (d2D) for indoor TRP to outdoor UE, 2D distance between BS and BS (d2D) for indoor TRP to indoor TRP and 2D distance between UE and UE (d2D) for indoor UE to indoor UE, respectively.

When channel model of InH-Office in TR 38.901 is used for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B), indoor TRP to indoor TRP and indoor UE to indoor UE in Deployment Case 1 and Deployment case 3-2 (2-layer Scenario B), the 2D distance between BS and UE (d2D) for indoor TRP to outdoor UE , 2D distance between BS and BS (d2D) for indoor TRP to indoor TRP and 2D distance between UE and UE (d2D) for indoor UE to indoor UE, respectively, can be used as d2D-in for LOS probability in Table 7.4.2-1 in TR 38.901.

LLS evaluation

Uplink channel for coverage performance evaluation in LLS

In RAN1#112 meeting, we preliminarily discussed the candidate channel for the purpose of evaluating coverage performance. The relevant agreement was achieved as below:

Agreement For link level evaluation of coverage performance, focus on the following uplink channels. - PUSCH with 1Mbps target data rate for FR1 - PUSCH with 5Mbps target data rate for FR2-1 - FFS: PUCCH - Note: the data rate is based on TR38.830

In the coverage performance evaluation for PUCCH in LLS, PUCCH format 1 and PUCCH format 3 are selected for PUCCH coverage performance evaluation in TR38.830 as below.

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 4bits (3 bits A/N + 1 bit SR)/11/22 bits UCI | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI |

It is the common understanding in Rel-17 coverage enhancement that PUSCH and PUCCH are the bottleneck channel from coverage perspective. Hence, coverage performance of PUSCH and PUCCH were extensively evaluated. Regarding to the potential benefits on coverage, we should focus on the bottleneck channels identified in previous study. Therefore, PUCCH format 1 and PUCCH format 3 should be considered to evaluate how much coverage gain we can harvest from SBFD technology.

Accordingly, we have the following proposal.

For link level evaluation of coverage performance, PUCCH should be evaluated with following assumptions.

- PUCCH format 1, 2bits UCI

- PUCCH format 3, 4bits (3 bits A/N + 1 bit SR)/11/22 bits UCI

Schemes for link level evaluation of PUSCH coverage performance

In RAN1#112, RAN1 achieved a consensus that PUSCH should be used to evaluate the coverage improvement due to SBFD technology. We discussed the simulation cases with achieving the following agreement:

Agreement Regarding the schemes for link level evaluation of PUSCH coverage performance, - For baseline legacy TDD, consider - Single slot PUSCH transmission - For SBFD, consider the following techniques of coverage enhancement: - Case 1: SBFD with single slot PUSCH - Case 2: SBFD with PUSCH repetition type A - Case 3: SBFD with TBoMS PUSCH - Case 4: SBFD with PUSCH repetition type A and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - Case 5: SBFD with TBoMS PUSCH and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate. Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots

For case 4 and 5, joint channel estimation across SBFD and non-SBFD slots should be further determined. For UL transmissions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following aspects were raised in AI9.3.2 in RAN1#112:

-	Whether/how to have separate resources

-	Whether/how to have separate FH parameters

-	Whether/how to have separate UL power control parameters

-	Whether/how to have separate beam/spatial relation

If separate parameters and/or assumptions are used for SBFD and non-SBFD slots, channel for SBFD and non-SBFD slots are quite different and joint channel estimation should be used for SBFD and non-SBFD slots separately.

Similarly, for SBFD antenna configuration option-1 and SBFD antenna configuration option-3, the channel for SBFD and non-SBFD slots are quite different since TXRUs are different in SBFD and non-SBFD slots.

For case 4 and 5, joint channel estimation should not be used across SBFD slot and non-SBFD slot for the following cases:

- When separate resource/ separate FH parameters/ separate UL power control parameters/ separate beam/spatial relations are used for SBFD and non-SBFD slots.

- When SBFD antenna configuration option-1/3 is assumed.

If UL transmissions across SBFD and non-SBFD slots have same resource, same FH parameters, same UL power control parameters, and same beam/spatial relations, the channel for SBFD and non-SBFD slots are nearly the same when SBFD antenna configuration option-2 is used.

For case 4 and 5, joint channel estimation can be used across SBFD and non-SBFD slots if the following conditions are satisfied:

- Same resource, FH parameters, UL power control parameters, and beam/spatial relations are used for SBFD and non-SBFD slots.

- SBFD antenna configuration option-2 is assumed.

If PUCCH is considered for coverage performance evaluation in LLS, the schemes for link level evaluation of case 2 and case 4 for can be reused. Accordingly, the schemes for link level evaluation of PUCCH coverage performance can be described as below:

- For baseline legacy TDD, consider

  - Single slot PUCCH transmission

- For SBFD, consider the following techniques of coverage enhancement:

  - Case 1: SBFD with PUCCH repetition

  - Case 2: SBFD with PUCCH repetition and DMRS bundling

    - FFS: DMRS bundling across SBFD and non-SBFD slots

- UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate.

Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots

Regarding the schemes for link level evaluation of PUCCH coverage performance,

-	For baseline legacy TDD, consider

•	Single slot PUCCH transmission

-	For SBFD, consider the following techniques of coverage enhancement:

•	Case 1: SBFD with PUCCH repetition

•	Case 2: SBFD with PUCCH repetition and DMRS bundling

o	FFS: DMRS bundling across SBFD and non-SBFD slots

-	UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate.

Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots

Preliminary SLS calibration results

In this section, we provide preliminary SLS calibration results for FR1 and FR 2-1. Urban Macro and InH for FR1, Dense Urban and InH for FR2-1 are simulated. Coupling loss for gNB-gNB, gNB-UE and UE-UE are shown in the following figures.

Figure 1 SLS calibration results for Urban Macro in FR1.

Figure 2 SLS calibration results for InH in FR1.

Figure 3 SLS calibration results for Dense Urban in FR2-1.

Figure 4 SLS calibration results for InH in FR2-1.

SLS evaluation results for SBFD

In this section, we provide preliminary SLS evaluation results for SBFD deployment case 1 to investigate the performance gain offered by SBFD. The simulation is conducted with considering the parameters agreed in RAN1#112. At the same time, we provide results for DL transmission when SBFD is introduced in order to see the impacts on DL direction.

We provide the SLS evaluation results for FR1. Urban Macro with single layer scenario is assumed wherein 20 UEs are dropped per sector. In each sector, two UE clusters are dropped and 8 UEs are uniformly distributed within each cluster. Indoor Hotspot scenario is also evaluated. More detail simulation assumptions can be found in Appendix. Regarding the evaluation metrics, UL/DL user perceived throughput, UL/DL latency and UL/DL RU are used in the simulation to comprehensively evaluate SBFD technique. The aforementioned metrics are defined in previous meeting [1].

Basically, the following cases are simulated:

SBFD subband and slot configurations in FR1:

- 100MHz channel bandwidth and 30kHz SCS (273 PRB) for legacy TDD and SBFD

- Legacy TDD-DDDSU: Legacy TDD with TDD UL-DL configuration DDDSU.

- SBFD-XXXXU-UL: On top of DDDSU frame structure, the first four slots are SBFD slot with < ND, NU, NG > = <104, 55, 5>, where ND, NU, NG are RB number for DL subband, UL subband and guardband respectively. UL sensitivity degradation of 1dB is assumed when modelling self-interference.

DL/UL FTP packet size in traffic model

- Asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL

- Asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL

DL/UL traffic load in traffic model

- {DL: UL} = {Low, Low}

- {DL: UL} = {Medium, Medium}

- {DL: UL} = {High, High}

- 2.4.1 System simulation results for Urban Macro scenario

In this section, the evaluation results for Urban Macro scenario are provided which take different assumptions into account.

UL and DL User Perceived Throughput

In this section, we provide results of average-UPT for DL and UL respectively. The definition is shown as below:

  - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

In order to reflect the traffic load in real world, we also provide results corresponding to different RU. The RU is determined by the baseline legacy TDD.

Figure 5 UL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 6 UL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Figure 7 DL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 8 DL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Based on the simulation results, we can have the following observations:

- DL user perceived throughput is degraded significantly once UL subband is configured in DL slots, no matter which kind of UPT is used.

  - The degradation of performance is caused by less available DL resources in each SBFD slot

- UL user perceived throughput is improved with introduction of UL subband in SBFD slots.

  - Significant performance gain can be observed when UL subband is introduced.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

When UL subband is introduced in DL slot, UL user throughput is improved significantly:

- Degradation of DL user throughput is also observed, which depends on the UL subband configuration.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

UL and DL latency

In this section, we provide results DL and UL respectively. The definition of packet-latency CDF is shown below:

- Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

  - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs.

  - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

Figure 9 UL Packet-latency (mean/5%/50%/95%) assuming different Target RU for legacy TDD  (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 10 UL Packet-latency (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Figure 11 DL Packet-latency (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 12 DL Packet-latency (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Based on the simulation results, we can have the following observations:

- DL latency is increased once UL subband is configured in DL slots.

  - The degradation of performance comes from less available DL resources in each SBFD slot

- UL latency is reduced with introduction of UL subband in SBFD slots.

  - Performance gain can be observed when UL subband is introduced.

- In ratio, the magnitudes of improvement for UL performance are close to degradation for DL performance.

When UL subband is introduced in DL slot, UL latency can be reduced:

- Increasement of DL latency is also observed, magnitudes of degradation for DL performance are close to improvement for UL performance.

UL and DL resource utilization

Based on the simulation results, the DL and UL resource utilization at 5%, 50% and percentile are summarized in Figure 13 and Figure 14, respectively.

Figure 13 DL resource utilization for Urban Macro scenario

Figure 14 UL resource utilization for Urban Macro scenario

Based on the simulation results, we can have the following observations:

- DL resource utilization is increased once UL subband is configured in DL slots across light traffic load, medium traffic load and high traffic load.

  - The increasement of DL resource utilization comes from less available DL resources in each SBFD slot

- UL resource utilization is reduced with introduction of UL subband in SBFD slots across light traffic load, medium traffic load and high traffic load.

When UL subband is introduced in DL slot, UL resource utilization can be reduced. Increasement of DL RU is also observed.

- 2.4.2 System simulation results for Indoor Hotspot scenario

In this section, the evaluation results for Indoor Hotspot scenarios are provided which take different assumptions into account.

UL and DL User Perceived Throughput

In this section, we provide results of average-UPT for DL and UL respectively. The definition is shown as below:

  - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

In order to reflect the traffic load in real world, we also provide results corresponding to different RU. The RU is determined by the baseline legacy TDD.

Figure 15 UL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 16 UL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Figure 17 DL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 18 DL Average-UPT (mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Based on the simulation results, we can have the following observations:

- DL user perceived throughput is degraded significantly once UL subband is configured in DL slots, no matter which kind of UPT is used.

  - The degradation of performance is caused by less available DL resources in each SBFD slot

- UL user perceived throughput is improved with introduction of UL subband in SBFD slots.

  - Significant performance gain can be observed when UL subband is introduced.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

Hence similar observation for Indoor Hotspot can be observed as Urban Macro scenario.

UL and DL latency

In this section, we provide results DL and UL respectively. The definition of packet-latency CDF is shown below:

- Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

  - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs.

  - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

Figure 19 UL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 20 UL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Figure 21 DL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL)

Figure 22 DL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD (asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL)

Based on the simulation results, we can have the following observations:

- DL latency is increased once UL subband is configured in DL slots.

  - The degradation of performance comes from less available DL resources in each SBFD slot

- UL latency is reduced with introduction of UL subband in SBFD slots.

  - Significant performance gain can be observed when UL subband is introduced.

- In ratio, the magnitudes of improvement for UL performance is close to degradation for DL performance.

Hence similar observation for Indoor Hotspot can be observed as Urban Macro scenario.

UL and DL resource utilization

Based on the simulation results, the DL and UL resource utilization at 5%, 50% and percentile are summarized in Figure 23 and Figure 24, respectively.

Figure 23 DL resource utilization for Indoor Hotspot

Figure 24 UL resource utilization for Indoor Hotspot

Based on the simulation results, we can have the following observations:

- DL resource utilization is increased once UL subband is configured in DL slots across light traffic load, medium traffic load and high traffic load.

  - The increasement of DL resource utilization comes from less available DL resources in each SBFD slot

- UL resource utilization is reduced with introduction of UL subband in SBFD slots across light traffic load, medium traffic load and high traffic load.

Hence similar observation for Indoor Hotspot can be observed as Urban Macro scenario.

Conclusion

In this contribution, we provide our views on evaluation on NR duplex evolution. We have the following observation:

- O2I parameters should be used for indoor TRP to outdoor UE in deployment case 3-2 (2-layer Scenario B) if fast fading channel model is applied.

- When UL subband is introduced in DL slot, UL user throughput is improved significantly:

- Degradation of DL user throughput is also observed, which depends on the UL subband configuration.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

When UL subband is introduced in DL slot, UL latency can be reduced:

- Increasement of DL latency is also observed, magnitudes of degradation for DL performance are close to improvement for UL performance.

When UL subband is introduced in DL slot, UL resource utilization can be reduced. Increasement of DL RU is also observed.

Based on the aforementioned discussion and observation, we have the following proposals:

- For fast fading parameters, O2I channel model parameters for UMi-Street canyon/UMa in table 7.5-6 Part-1 in TR 38.901 could be reused for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B) when option 2 is adopted.

When channel model of InH-Office in TR 38.901 is used for indoor TRP to outdoor UE in Deployment case 3-2 (2-layer Scenario B), indoor TRP to indoor TRP and indoor UE to indoor UE in Deployment Case 1 and Deployment case 3-2 (2-layer Scenario B), the 2D distance between BS and UE (d2D) for indoor TRP to outdoor UE , 2D distance between BS and BS (d2D) for indoor TRP to indoor TRP and 2D distance between UE and UE (d2D) for indoor UE to indoor UE, respectively, can be used as d2D-in for LOS probability in Table 7.4.2-1 in TR 38.901.

For link level evaluation of coverage performance, PUCCH should be evaluated with following assumptions.

- PUCCH format 1, 2bits UCI

- PUCCH format 3, 4bits (3 bits A/N + 1 bit SR)/11/22 bits UCI

For case 4 and 5, joint channel estimation should not be used across SBFD slot and non-SBFD slot for the following cases:

- When separate resource/ separate FH parameters/ separate UL power control parameters/ separate beam/spatial relations are used for SBFD and non-SBFD slots.

- When SBFD antenna configuration option-1/3 is assumed.

For case 4 and 5, joint channel estimation can be used across SBFD and non-SBFD slots if the following conditions are satisfied:

- Same resource, FH parameters, UL power control parameters, and beam/spatial relations are used for SBFD and non-SBFD slots.

- SBFD antenna configuration option-2 is assumed.

Regarding the schemes for link level evaluation of PUCCH coverage performance,

-	For baseline legacy TDD, consider

•	Single slot PUCCH transmission

-	For SBFD, consider the following techniques of coverage enhancement:

•	Case 1: SBFD with PUCCH repetition

•	Case 2: SBFD with PUCCH repetition and DMRS bundling

o	FFS: DMRS bundling across SBFD and non-SBFD slots

-	UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate.

Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots

Reference

  - RAN1 Chairman notes, RAN1#112 meeting

  - R1-2302119, Final summary on evaluation on NR duplex evolution, Moderator (CMCC)

Appendix

Simulation assumptions for SBFD SLS

Table 1 Simulation parameters for Urban Macro in FR1

| Parameters | Values |
| --- | --- |
| Carrier frequency | 4 GHz |
| System bandwidth | Baseline: 100MHz |
| Numerology | 14 OFDM symbol per slot, SCS = 30kHz |
| BS Tx power | 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD For SBFD antenna configuration Option-2, in both DL-only symbols and SBFD symbols, the maximum BS transmit power for SBFD is always the same as that for legacy TDD Baseline (Option 1): BS transmit power spectrum density per Tx chain is kept the same for SBFD symbols and DL-only symbols. |
| UE Tx power | 23 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |
| Inter-BS distance | 500 m |
| Minimum BS-UE (2D) distance | 35 m |
| Minimum UE-UE (2D) distance | 1 m |
| BS antenna height | 25 m |
| UE distribution | UE Clustering |
| Number of UEs per Macro TRP | 20 |
| UE cluster number per macro cell (X) | 2 |
| UE number per cluster | 8 |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h |
| Indoor UE height (m) | 1.5m |
| Outdoor UE height (m) | 1.5m |
| Radius of cluster (R) | 25m |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | 60m |
| Minimum distance between two UE cluster centers (Dinter-cluster) | 50m |
| gNB-UE Channel model (large -scale) | Macro-to-UE: UMa in TR 38.901 For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |
| UE-UE Channel model (large-scale) | Baseline: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m). For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 Regarding baseline (Option 2) of UE-UE channel model for Dense urban/Urban macro scenarios, use NLOS when two indoor UEs are in different buildings. |
| SBFD and slot configuration | Alt 2 (TDD{DDDSU}, SBFD{XXXXU}) • For FR1 o Baseline: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5> |
| Traffic model | Each UE is assigned both UL traffic and DL traffic. FTP model 3 with DL/UL FTP packet size: - Asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL - Asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL DL/UL traffic load: - {DL:UL}={Low, Low} - {DL:UL}={Medium, Medium} - {DL:UL}={High, High} |
| BS antenna configurations for legacy TDD | M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization |
| BS antenna configurations for SBFD | For SBFD gNB: - SBFD antenna configuration option-2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna radiation pattern | reuse Table 9 in Report ITU-R M.2412 |
| UE antenna configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain |
| BS receiver noise figure | 5 dB |
| UE receiver noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Ideal |
| UE processing capability | UE processing capability 1 |
| Open loop power control parameters | P0= -80 dBm, alpha = 0.8 |
| Handover margin (dB) | 3 dB |
| UE attachment | Based on RSRP from port 0 |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |
| DL/UL modulation | Up to 256QAM |
| Transmission scheme | SU-MIMO with single layer for both DL and UL |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - |
| SI suppression capability | UL receiver sensitivity degradation of 1dB |
| Co-site inter-sector suppression capability | 100dB for spatial isolation BS ACLR is 45dB BS ACS is 46dB |
| Output | Average-UPT CDF related metrics and Packet-Latency CDF related metrics UL/DL UPT (5%,50%, 95%), UL/DL Latency, Resource utilization |

Table 2 BS antenna element pattern for Urban Macro

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

Table 3 Simulation parameters for indoor Hotspot

| Parameters | Values |
| --- | --- |
| Carrier frequency | 4 GHz |
| System bandwidth | Baseline: 100MHz |
| Numerology | 14 OFDM symbol per slot, SCS = 30kHz |
| BS Tx power | 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD For SBFD antenna configuration Option-2, in both DL-only symbols and SBFD symbols, the maximum BS transmit power for SBFD is always the same as that for legacy TDD Baseline (Option 1): BS transmit power spectrum density per Tx chain is kept the same for SBFD symbols and DL-only symbols. |
| UE Tx power | 23 dBm |
| Layout | 12 TRPs per 120m x 50m x 3m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS |
| Inter-BS distance | 20 m |
| Minimum BS-UE (2D) distance | 0 m |
| Minimum UE-UE (2D) distance | 1 m |
| BS antenna height | 3 m |
| UE distribution | Uniform |
| Number of UEs per Macro TRP | 10 |
| UE cluster number per macro cell (X) | - |
| UE number per cluster | - |
| UE outdoor/indoor proportion | 100% indoor in houses: 3km/h |
| Indoor UE height (m) | 1.5m |
| Outdoor UE height (m) | - |
| Radius of cluster (R) | - |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | - |
| Minimum distance between two UE cluster centers (Dinter-cluster) | - |
| gNB-UE Channel model (large -scale) | InH-Office in TR 38.901 Penetration loss is not modelled. |
| gNB-gNB Channel model (large-scale) | InH-Office in TR 38.901 (hUE =3m) Penetration loss is not modelled. |
| UE-UE Channel model (large-scale) | InH-Office in TR 38.901 (hBS =1.5m) |
| SBFD and slot configuration | Alt 2 (TDD{DDDSU}, SBFD{XXXXU}) • For FR1 o Baseline: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5> |
| Traffic model | Each UE is assigned both UL traffic and DL traffic. FTP model 3 with DL/UL FTP packet size: - Asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL - Asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL DL/UL traffic load: - {DL:UL}={Low, Low} - {DL:UL}={Medium, Medium} - {DL:UL}={High, High} |
| BS antenna configurations for legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) dH,dV = (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna configurations for SBFD | For SBFD gNB: - SBFD antenna configuration option-2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (4,4,2,1,1).. - Number of TxRUs: same as legacy TDD dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna radiation pattern | reuse Table 10 in Report ITU-R M.2412 |
| UE antenna configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain |
| BS receiver noise figure | 5 dB |
| UE receiver noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Ideal |
| UE processing capability | UE processing capability 1 |
| Open loop power control parameters | P0= -60 dBm, alpha = 0.6 |
| Handover margin (dB) | 3 dB |
| UE attachment | Based on RSRP from port 0 |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |
| DL/UL modulation | Up to 256QAM |
| Transmission scheme | SU-MIMO with single layer for both DL and UL |
| Mechanic tilt | 180° in GCS (pointing to the ground) |
| Electronic tilt | 90° in LCS |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | - |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - |
| SI suppression capability | UL receiver sensitivity degradation of 1dB |
| Co-site inter-sector suppression capability | 100dB for spatial isolation BS ACLR is 45dB BS ACS is 46dB |
| Output | Average-UPT CDF related metrics and Packet-Latency CDF related metrics UL/DL UPT (5%,50%, 95%), UL/DL Latency, Resource utilization |