3GPP TSG RAN WG1 #112bis-e	                                                 	R1-2302983

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.3.3

Source:	Xiaomi

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Document for:	Discussion

Introduction

In RAN1#112 meeting [1], the following agreements were achieved for dynamic TDD:

Agreement For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement. Agreement Study the benefit of knowledge among gNBs of configurations such as - SBFD time/frequency configuration Agreement For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. Agreement For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics. Agreement For the study of L1/L2 based UE-to-UE co-channel CLI measurement, measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered. Enhancement of measurement resource can be studied. Agreement For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework. - Use existing CSI framework as the baseline. - Others are not precluded. Agreement For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs.

In this contribution, we discuss the possible enhancements on dynamic/flexible TDD.

Discussion

UE-to-UE CLI handling schemes

UE-to-UE CLI measurement and reporting

In previous meeting, it was agreed that L1/L2 UE-to-UE CLI measurement can be considered as the potential enhancements for the purpose of UE-to-UE CLI mitigation. However, the time manner of measurement, i.e. periodic, semi-persistent or aperiodic measurement, has not been determined yet. In our view, at least the periodic measurement should be supported. Actually, the periodic measurement for L3 UE-to-UE CLI measurement has been supported since Rel-16. Regarding to L1/L2 UE-to-UE CLI measurement, periodic measurement can be used to offer timely CLI measurement results if proper measurement periodicity is configured. On the other hand, aperiodic measurement can be triggered based on the gNB’s decision as a supplement for periodic measurement. For instance, gNB can trigger aperiodic measurement for more accurate assessment on interference level if gNB recognizes channel state suddenly degrades and additional measurement is needed.

Proposal 1: For L1/L2 based CLI measurement, at least periodic and aperiodic CLI measurement resource should be supported.

As mentioned above, both periodic CLI measurement resource and aperiodic CLI measurement resource can be considered. In RAN1#112 meeting, it was agreed that existing CSI framework can be used as the baseline of L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism. As aperiodic CSI-RS resources can only be associated with aperiodic CSI-RS reporting, it is natural that aperiodic CLI reporting should be considered as well. Periodic reporting is semi-statically configured and doesn’t need dynamic trigger signaling. It is beneficial for reducing signaling overhead. Furthermore, periodic measurement resources are typically associated with periodic measurement reporting.

Proposal 2: For L1/L2 based CLI reporting, at least periodic and aperiodic CLI reporting should be supported.

In addition, event-triggered reporting was also discussed without consensus [2]. Event-triggered reporting has been supported for L3 UE-to-UE CLI measurement in Rel-16. The report is conditioned on a configured threshold, i.e., the UE reports CLI measurement result only when the CLI value is larger than the threshold. Similarly, the event-triggered reporting for L1/L2 UE-to-UE CLI measurement is triggered automatically only when the CLI value is larger than a threshold. It is beneficial for the reporting overhead reduction as UE conducts reporting only if necessary. Furthermore, event-triggered reporting doesn’t need triggering indication from gNB side and it is more flexible, i.e. UE informs gNB the fluctuation of CLI at any allowed time occasion, more timely reporting can be achieved thanks to removal of alignment delay and signaling delay.

Proposal 3: For L1/L2 based CLI reporting, the event-triggered reporting should be supported.

- CLI measurement metrics

For L1/L2 based UE-UE CLI measurement, it was agreed that SRS-RSRP and CLI-RSSI can be further studied as baseline metrics. In addition, CSI and CQI are also supported by several companies [3][4]. CSI and CQI offer more accurate channel information. Nevertheless, calculation of CSI/CQI needs non-linear operations which introduces higher complexity compared with SRS-RSRP and CLI-RSSI.

Observation 1: CSI and CQI may need high calculation complexity with non-linear operations.

- L1/L2 based CLI measurement and reporting framework

In RAN1#112 meeting, it was agreed that existing CSI framework can be used as baseline for L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism. From our understanding, the RRC signaling relevant to CSI measurement and reporting can be almost reused. For example, the CSI framework can be extended by including the CLI-related metrics as follows in order to supporting L1/L2 based CLI measurement and reporting:

- Updating reportQuantity within CSI-ReportConfig by including CLI-RSRP and CLI-RSSI.

- Updating CSI-ReportConfig by including CLI measurement resources.

- Updating reportConfigType within CSI-ReportConfig by including the event-trigged reporting if supported.

In this case, UE-to-UE CLI can be regarded as a CSI following the above procedure. The UE is able to perform CLI measurement and reporting based on the configuration of CSI-ReportConfig.

Proposal 4: For L1/L2 based UE-to-UE CLI measurement and reporting, the configuration can be realized via updating CSI-ReportConfig:

- Adding CLI-RSRP and CLI-RSSI as component of reportQuantity.

- Adding CLI measurement resources as component of CSI-ReportConfig.

- Adding event-triggered reporting as component of reportConfigType.

- Subband CLI reporting

The wideband measurement is supported in Rel-16 CLI mechanism, i.e. CLI measurement result is averaged on the whole frequency of CLI resource. The L3 based wideband measurement can reflect the overall interference level. However, the CLI level may be varied across different subbands due to the situation of resource occupancy. For instance, the UE may suffer more severe interference within the subband wherein uplink transmission is on-going for another UE. For SBFD, UE suffers more UE-to-UE inter-subband CLI if its downlink reception is allocated near to UL subband. Hence, subband CLI reporting should be considered for the UE-to-UE CLI measurement and reporting enhancement in order to obtain more accurate measurement.

Proposal 5:  Subband CLI reporting can be considered for UE-to-UE CLI mitigation.

Time alignment of UE-to-UE handling

Figure 1 The time offset for the reception of CLI RS.

The victim UE needs to receive CLI-RS transmitted by the aggressor UE for CLI measurement, e.g., SRS. As shown in Figure 1, there is a time offset between DL reception timing and CLI-RS arrival timing at UE side. In Rel-16, the time offset is handled by UE implementation. However, the basic assumption is that victim UE and aggressor UE have similar UL timing [5] . On the other hand, the UL timing of Macro UE and indoor UE have significant difference in HetNet scenario, which is the most promising scenario for DTDD.  Hence, the time alignment scheme for UE-to-UE CLI handling needs further study at least for HetNet scenario.

The time offset between the CLI-RS arriving time and DL frame boundary at victim UE side is equal to: NTA,offsetTc+ T1+T2- T3 [5], wherein NTA,offset is TA-related information of aggressor UE, and T1, T2 and T3 are the propagation delay between gNB2 and aggressor UE, the delay between gNB 1 and victim UE, the delay between UE1 and UE2, respectively.  NTA,offset is designed to provide sufficient time for gNB UL/DL transition, which is identical among gNBs within the same TDD band by default [7]. Therefore, it can be assumed that the NTA,offset of the aggressor UE is equal to that of victim UE (NTA, offset' for distinguish).

As for the propagation delay between gNB2 and aggressor UE, considering that the aggressor UE locates under the Micro cell in HetNet scenario as shown in Figure 1, T2 is negligible due to the small cell radius of Micro cell. Taking the layout of indoor office scenarios shown in Figure 2 as an example, the propagation delay T2 is not larger than 0.067us with inter-site distance (ISD) of 20m. Similarly, the propagation delay between UE1 and UE2 is also negligible as they are close to each other once UE-to-UE CLI is an issue. Accordingly, the time offset mentioned above is approximately equal to: NTA,offsetTc+ T1 = (NTA, offset' +NTA/2)Tc , where NTA, offset' and NTA are TA-related information of victim UE.

Figure 2 Layout of indoor office scenarios[6]

As for other scenarios where victim UE and aggressor UE locate at same type of serving cells, the UL timing of victim UE and aggressor UE can be considered approximately same, i.e., T1≈T2. Recalling that  NTA, offset'= NTA,offset, and T3 is negligible as analyzed above, the time offset in this scenario is approximately equal to NTA,offsetTc+ T1+T2 ≈ (NTA, offset' +NTA)Tc, wherein NTA, offset' and NTA are TA-related information of victim UE.

Observation 2：The time offset between DL reception timing and CLI-RS arrival timing at victim UE side can be determined by UE’s TA information.

Based on the aforementioned observation, the time alignment offset can be realized by UE’s implementation. No more enhancement is needed.

Proposal 6: The misalignment between CLI-RS arrival timing and DL timing at victim UE side can be handled by UE implementation.

- The reception window for multiple aggressors

Multiple aggressor UEs may transmit the CLI-RS simultaneously. Based on the aforementioned analysis, the time offset between DL reception timing and CLI-RS arriving timing at victim UE side can be determined by the TA information of victim UE. Hence, the arriving time of CLI-RSs from multiple aggressor UEs should be approximately same. Accordingly, the misalignment on arrival timing of multiple aggressor UEs may not exceed the CP of DL symbol at victim UE side.

Observation 3：In most scenarios, the arriving time of CLI-RSs are within the same CP duration even the CLI-RSs are from multiple aggressor UEs.

The aforementioned observation is obtained based on the ideal situation that the time offset between DL reception timing and CLI-RS arriving timing is totally determined by the victim UE’s TA information. Actually, the above ideal situation may be not true as it is achieved based on some approximate assumption. For example, the time offset between arriving timing of CLI RS 1 and CLI RS 2 may exceed CP duration, as shown in Figure 3. The simultaneous reception of multiples CLI-RSs will suffer severe inter-symbol interference (ISI). In our view, that problem can be solved by gNB, e.g., only the aggressor UEs of the same serving cell can be configured to transmit CLI-RS on the same OFDM symbol.

Figure 3 The reception of CLI RSs from multiple aggressors.

Proposal 7: The simultaneous reception of multiple SRSs from different aggressor UEs for CLI measurement can be realized by gNB.

Spatial domain enhancement

Currently, the Rx beam for UE-to-UE CLI measurement is determined by UE implementation. gNB is unable to identify the CLI level corresponding to different beams as it cannot obtain any beam information from UE. Hence, the DL beam on which CLI RS reception is conducted should be considered for L1/L2 UE-to-UE CLI measurement. To be specific, a reference signal (e.g., SSB, CSI-RS) can be considered to indicate the reception beam of CLI RS at victim UE side, as shown in Figure 4. In this case, once the CLI measurement results with different Rx beams are reported by victim UE, the gNB is able to identify the CLI levels on different Rx beams. The beam based CLI measurement is beneficial for more accurate CLI mitigation. For instance, the gNB can schedule the Rx beams with lower CLI level for UE’s PDSCH/PDCCH reception, e.g., Rx beam associated with CSI-RS3 in Figure 4.

Figure 4 The reception of CLI RSs configured with different Rx beams

Proposal 8: Support beam based CLI measurement for UE-to-UE CLI mitigation.

gNB-to-gNB CLI handling

The gNB-to-gNB CLI measurement and reporting scheme is an on-going discussion for dynamic/flexible TDD, which is not able to be realized by current standard. In our view, the overall procedure for the gNB-to-gNB measurement and reporting framework can be described as below:

- Step1: Exchange the information of CLI measurement related configuration between CUs if the victim gNB and aggressor gNB belong to different CUs. For instance, the information can be exchanged via Xn;

- Step2: Transmit the CLI-RS by the aggressor gNB based on the measurement configuration provided by CU2; Generally speaking, the RSSI and RSRP are recommended as the measurement metrics for gNB-to-gNB CLI measurement.

- Step3:  Receive the CLI-RS and perform CLI measurement by the victim gNB based on the measurement configuration provided by CU1.

- Step4: Victim gNB reports the measurement results.

- Step5: The victim gNB and aggressor gNB perform the CLI-related operations to mitigate CLI. For instance, the CLI can be mitigated by power, beam related enhancement.

Observation 4：The above procedure of the gNB-to-gNB measurement and reporting can be a starting point.

gNB-to-gNB measurement and reporting

Regarding the resources for gNB-to-gNB CLI measurement, it has been agreed that the periodic NZP CSI-RS/SSB can be studied as a baseline in RAN1#111 meeting. If the periodic CLI-RS is supported for periodic gNB-to-gNB CLI measurement, periodic reporting can also be used in nature.  The periodic reporting can provide timely measurement results with less overhead for information exchange, which is beneficial for the gNB-to-gNB CLI measurement mitigation.

Proposal 9: Support periodic reporting for gNB-to-gNB CLI mitigation.

RSRP and RSSI has been agreed to be baseline metrics for UE-to-UE CLI measurement. Regarding to gNB-to-gNB CLI measurement, it is natural to adopt RSRP and RSSI as baseline metrics. The RSRP and RSSI can reflect the CLI level without high computational complexity.

Proposal 10: RSRP and RSSI can be considered as baseline metrics for gNB-to-gNB CLI measurement.

UE and gNB transmission and reception time

Another issue is how to handle the misalignment between CLI-RS arrival timing and UL timing at victim gNB side. If the aggressor gNB transmits the CLI RS based on its DL symbol timing and the UEs served by victim gNB transmit the UL data based on its UL symbol timing, the time gap between the CLI RS and UL data is equal to NTA, offsetTc+Tdelay, wherein  Tdelay is the propagation delay between aggressor gNB and victim gNB. An example is shown in Figure 5, wherein TTA, offset =NTA, offsetTc.

Figure 5 Timing of CLI-RS and UL data at victim gNB

Currently, the default value of  TTA, offset for FR1 TDD are 13us and 20.3us, and 7us for FR2. Obviously, the arriving time gap shown in Figure 5 exceeds the CP duration of one OFDM symbol. Accordingly, simultaneous reception of CLI RS of UL data may suffer severe ISI.

Observation 5: There is severe ISI between CLI RS and UL data at victim gNB side with non-zero NTA, offset .

One possible solution is that gNB drops uplink data from served UE and only receives the CLI RS on configured CLI symbols. In this sense, the CLI RS can be received by gNB implementation. No additional specification work is needed. However, gNB may receive the CLI RS across two UL symbols. At least two symbols are not available for UL transmission for victim gNB, as shown in Figure 6. Besides, except the timing for UL data reception (Timing 1 in Figure 6), the gNB has to determine another timing for CLI RS (Timing 2 in Figure 6). It brings additional complexity at gNB side.

Figure 6 The reception of CLI RS with existing TA schemes

Observation 6: One CLI RS symbol may result in two UL symbol unavailable at victim gNB side due to the misalignment of timing between CLI-RS arrival and UL timing.

Another solution is to configure the UE served by victim gNB with the zero NTA, offset. With zero NTA, offset, the arriving time gap between the CLI RS and UL data decreases to Tdelay, as shown in Figure 7. Taking 500m cell radium for Macro cell as an example, the propagation delay is equal to 1.67 us, which is within the CP duration assuming 15kHz and 30kHz SCS. As for the scenario with higher SCS, the cell radius will decease correspondingly. To summarize, the zero NTA, offset is sufficient for the timing alignment of UL data and CLI RS reception. The gNB is able to reuse existing timing for both UL data and CLI RS reception. On the other hand, the NTA, offset provides guard period for gNB UL/DL transition [7]. If zero  NTA, offset is configured, then the time for guard period disappears. At least one OFDM symbol should be reserved for UL/DL transition, as shown in Figure 7.

Figure 7 Timing of CLI-RS and UL data at victim gNB

Observation 7: For each UL/DL transition at victim gNB, at least one OFDM symbol is not available for the victim gNB if zero NTA, offset is configured.

Spatial domain enhancement

Agreement For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. Agreement For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs.

The spatial domain solutions are based on the Tx & Rx beam based CLI measurement. For instance, the aggressor gNB transmits the NZP-CSI-RS on a Tx beam (e.g., Tx 1 in Figure 8) by configuration. When the NZP-CSI-RS arrives at the victim gNB, the victim gNB will perform the CLI measurement by receiving the RS with an Rx beam (e.g., Rx 2 in Figure 8). To be specific, the CLI level may be varied across different Tx-Rx beam pairs. For the scenario shown in Figure 8, the CLI level of Tx 1 & Rx 2 beam pair is much higher than that of Tx 3 & Rx 2 beam pair.

The Tx-Rx beam based CLI measurement relies on the indication of Tx and Rx beams. In RAN1#112 meeting, it was agreed to adopt RS ID (e.g., NZP-CSI-RS resource ID, SSB index) to indicate the Tx beams. However, the indication of the Rx beam of victim gNB has not been determined. Currently, the Tx beam of periodic CSI-RS at gNB side is indicated by the associated reference signals (e.g., CSI-RS, SSB) with corresponding quasi-colocation (QCL) relationship. To be specific, the Tx beam is indicated by a Transmission Configuration Indicator (TCI) state configured under NZP-CSI-RS-Resource IE. Considering channel reciprocity in TDD system, the RS associated with one Tx beam of gNB can be regarded as same as that of Rx beam for the same gNB. Hence, the associated RS to indicate Tx beam can also be adopted to represent the Rx beam for the same gNB. Besides, different SRS resources are also associated with different UL beams. In this case, existing mechanism can be reused by indicating the ID of the associated RS (e.g., CSI-RS, SSB, SRS) with corresponding QCL relationship to indicate the Rx beam, as Figure 8 shows.

Proposal 11: Rx beam can be indicated by the associated RS (e.g., CSI-RS, SSB, SRS) for gNB-to-gNB CLI management.

Figure 8 Tx & Rx beam based CLI measurement between gNBs

Following the procedure mentioned above, the victim gNB is able to obtain the CLI results of different beam pairs

(Rx&Tx beam pairs). For CLI mitigation, possible spatial solutions are concluded as follows:

- Option 1: The victim gNB avoids (schedules) UL resources on Rx beams with high (low) CLI.

- Option 2: The aggressor gNB avoids (schedules) DL resources on Tx beams with high (low) CLI. The restricted/recommended Tx beams are indicated by victim gNB or central NW (e.g., OAM).

- Option 3: The victim gNB reports the CLI measurement result of different beam pairs to the central NW. Based on the measurement result, the central NW determines the restricted/recommended Rx & Tx beams and indicates the beams to victim gNB and aggressor gNB, respectively.

As mentioned above, even with one specific Rx beam, the corresponding CLI levels are varied across different Tx beams. In this case, simply restricting Rx by Option 1 or restricting Tx by Option 2 cannot mitigate the CLI effectively, as the Rx/Tx beams are varied from time to time. Accordingly, Option 3 to configure (avoid) Tx/Rx beam pairs with lower (higher) CLI is prioritized. To increase the scheduling flexibility and reduce the influence of restricted beams, the central NW can configure different beam pairs on different slots.

Proposal 12:  The restricted/recommended beam pairs, i.e., restricted/recommended Rx beams for victim gNB and restricted/recommended Tx beams for aggressor gNB, should be configured for gNB-to-gNB CLI mitigation.

Coordinated scheduling

Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not.

The coordination-based solutions are based on the long-term measurement results, the gNB can make proper scheduling decisions, e.g. choose clean time/frequency resources for data transmission to avoid severe interference. The victim gNB can also request the required time or frequency domain resource or TDD configuration to avoid the gNB-gNB CLI.

For the time domain resource coordination, it is possible that the victim gNB can request aggressor gNB to reserve time resource to protect its UL transmission. For example, some slots can be reserved as “UL” only resource to protect its transmission of the important information. Then the aggressor gNB can set the protected slots as the UL slots to avoid the CLI.

For the frequency domain resource coordination, the victim gNB can request to reserve some frequency resource to protect its UL transmission. For example, some UL RBs/subbands/RB sets can be reserved to protect its transmission of the important information. Then the aggressor gNB can hold its DL transmission on the reserved UL resources.

Another issue that needs to be discussed is uplink resources muting scheme. To be specific, the resources for gNB-to-gNB CLI measurement are muted for UL transmission. The scheme may be implemented by gNB scheduling, but such methods will reduce the scheduling flexibility, especially for the periodic UL transmission scheduling. In this case, the indication of UL muting resources to UE may be a better choice. The UE can abandon the UL transmission on the UL muting resources indicated by gNB to reduce the interference to gNB-gNB CLI measurement. In this case, the measurement accuracy can be improved.  Similar mechanism with rate matching patterns (RMP) can be re-used with potential enhancement for the indication of reserved resources.

Proposal 13: The RMP can be considered with potential enhancement to support UL reserved resource indication.

Power control

For the gNB-gNB CLI mitigation, aggressor gNB can ramp down its DL transmission power to reduce the interference level and the victim gNB ramp up UE’s UL transmission power to improve its signal strength.

For the case the aggressor gNB adjust its DL transmission power to reduce the interference level, based on the CLI measurement result, the victim gNB can request the wanted DL power level from the aggressor gNB on intended resources.

For the case the victim UE adjust its UL transmission power to boost its signal strength, based on the CLI measurement result, the victim gNB configures different power parameters on different sets of resources, the UEs can then adjust its UL transmission power based on the configurations. The power control scheme by taking the CLI interference level from the aggressor into account may need to be studied.

Proposal 14: The power adaptation schemes to alleviate the CLI issue can be further studied.

Conclusions

In this contribution, we provide the potential scenario and candidate solutions for dynamic/flexible TDD. The following proposals are made based on our analysis:

For UE-to-UE CLI handling:

Observation 1: CSI and CQI may need high calculation complexity with non-linear operations.

Observation 2：The time offset between DL reception timing and CLI-RS arrival timing at victim UE side can be determined by UE’s TA information.

Observation 3：In most scenarios, the arriving time of CLI-RSs are within the same CP duration even the CLI-RSs are from multiple aggressor UEs.

Proposal 1: For L1/L2 based CLI measurement, at least periodic and aperiodic CLI measurement resource should be supported.

Proposal 2: For L1/L2 based CLI reporting, at least periodic and aperiodic CLI reporting should be supported.

Proposal 3: For L1/L2 based CLI reporting, the event-triggered reporting should be supported.

Proposal 4: For L1/L2 based UE-to-UE CLI measurement and reporting, the configuration can be realized via updating CSI-ReportConfig:

- Adding CLI-RSRP and CLI-RSSI as component of reportQuantity.

- Adding CLI measurement resources as component of CSI-ReportConfig.

- Adding event-triggered reporting as component of reportConfigType.

Proposal 5:  Subband CLI reporting can be considered for UE-to-UE CLI mitigation.

Proposal 6: The misalignment between CLI-RS arrival timing and DL timing at victim UE side can be handled by UE implementation.

Proposal 7: The simultaneous reception of multiple SRSs from different aggressor UEs for CLI measurement can be realized by gNB.

Proposal 8: Support beam based CLI measurement for UE-to-UE CLI mitigation.

For gNB-to-gNB CLI handling:

Observation 4：The above procedure of the gNB-to-gNB measurement and reporting can be a starting point.

Observation 5: There is severe ISI between CLI RS and UL data at victim gNB side with non-zero NTA, offset .

Observation 6: One CLI RS symbol may result in two UL symbol unavailable at victim gNB side due to the misalignment of timing between CLI-RS arrival and UL timing.

Observation 7: For each UL/DL transition at victim gNB, at least one OFDM symbol is not available for the victim gNB if zero NTA, offset is configured.

Proposal 9: Support periodic reporting for gNB-to-gNB CLI mitigation.

Proposal 10: RSRP and RSSI can be considered as baseline metrics for gNB-to-gNB CLI measurement.

Proposal 11: Rx beam can be indicated by the associated RS (e.g., CSI-RS, SSB, SRS) for gNB-to-gNB CLI management.

Proposal 12:  The restricted/recommended beam pairs, i.e., restricted/recommended Rx beams for victim gNB and restricted/recommended Tx beams for aggressor gNB, should be configured for gNB-to-gNB CLI mitigation.

Proposal 13: The RMP can be considered with potential enhancement to support UL reserved resource indication.

Proposal 14: The power adaptation schemes to alleviate the CLI issue can be further studied.

References

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1#112, Athens, Greece, February 27th – March 3rd, 2023.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1#111, Toulouse, France, November 14th – 18th, 2022.

- R1-2300947, Intel Corporation, Potential enhancements on dynamic/flexible TDD, 3GPP TSG RAN WG1, Meeting #112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2301412, Qualcomm Incorporated, On potential enhancement on dynamic/flexible TDD, 3GPP TSG RAN WG1, Meeting #112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-1902292, Samsung, Cross-link interference measurements and reporting at a UE, 3GPP TSG RAN WG1 #96, Athens, Greece, 25th February – 1st March, 2019.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

- R4-1804119, Intel Corporation, On NR NTA offset, 3GPP TSG-RAN WG4 Meeting #86bits, Melbourne, Australia, April 16 – April 20, 2018.