3GPP TSG RAN WG1 #112bis-e			R1-2302990

e-Meeting, April 17th – April 26th, 2023

Source: 	     Xiaomi

Title:	Discussion on resource allocation for SL positioning reference signal

Agenda item: 	9.5.1.3

Document for:  Discussion

Introduction

In RAN#98-e meeting, the WID on expanded and improved NR positioning has been approved [1], including the following objectives:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

  - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, the issues and solutions on resource allocation of sidelink positioning reference signal are discussed, and our opinions are given.

SL PRS resource (pre)configurations

Dedicated SL PRS resource pool (pre)configurations

In RAN1#112 meeting, the following agreement was made:

Agreement

For a dedicated resource pool for positioning:

- The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214).

  - FFS: additional slots that can be used for SL PRS is not precluded

- Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration

RAN1 has agreed that SL dedicated resource pool shall be defined on the same BWP as the BWP used for SL communications. Therefore, SL dedicated resource pool shall also be defined on SL slots similar as that defined for SL communication resource pool. That is, only slots with enough UL symbols are considered. The reserved slots and the slots (pre)configured for S-SSB transmission shall be precluded from slots that may belong to SL PRS dedicated resource pool.

Proposal 1: The UE determines the set of slots that may belong to a SL PRS dedicated resource pool in the same way as for SL communication pool.

Similar as SL communication resource pool, a bitmap can be (pre)configured to indicate which SL slot is assigned to the SL PRS dedicated resource pool. The start PRB position and the number of contiguous PRBs included in the dedicated resource pool are also (pre)configured. The start symbol and length of symbols in a slot of the dedicated SL PRS resource pool are also (pre)configured.

Proposal 2: The set of slots assigned to a SL PRS dedicated resource pool are (pre)configured.

Proposal 3: the start PRB position, the number of contiguous PRBs, the start symbol and the length of symbols in a slot are (pre)configured for a SL PRS dedicated resource pool.

Comb based structure is supported for SL PRS transmission. To enable comb based FDM between different UEs, the comb size of SL PRS shall be (pre)configured in a dedicated resource pool. Furthermore, to reduce the signalling cost to indicate the frequency domain resource for SL PRS, SL PRS subchannel can be defined as the minimum frequency domain resource allocation granularity. The size of SL PRS subchannel shall be (pre)configured for the dedicated resource pool.

In RAN1#112 meeting it has been agreed that TDM multiplexing of SL PRS from different UEs is supported. The number of TDM multiplexing SL PRS resources in each slot of the dedicated resource pool shall be (pre)configured.

Proposal 4: SL PRS comb size, SL PRS subchannel size, and the number of TDM multiplexing SL PRS resources in a slot of dedicated resource pool shall be (pre)configured.

Control and SL PRS multiplexing

In RAN1#112 meeting, the following agreement was made:

Agreement

For a dedicated resource pool for Positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further.

- Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

For both option 2 and option 3 the receiver UE can blindly detect PSCCH to find out whether there is associated SL PRS transmission, and can decode necessary information from PSCCH or PSCCH/PSSCH for SL positioning purpose. The candidate information which can be transmitted with association with SL PRS may include source UE ID, destination UE ID, SL PRS time/frequency location indication for the current transmission, reservation information for the future SL PRS transmission(s), etc. Whether PSSCH carrying 2nd stage SCI associated with SL PRS transmission is needed depends on the discussion of how many/which information is necessary to be associated with SL PRS transmission.

To simplify the design, fixed size and content of SCI associated with SL PRS is preferred, and thus option 2 shall be supported. The candidate information to be included in the control information associated with the SL PRS transmission includes:

1) Time and frequency domain information related to associated SL PRS transmission in the same slot

2) Time and frequency domain information related to reserved resource for future SL PRS transmission(s)

3) Priority information

4) Source and destination ID information

5) SL PRS measurement assistant information

Therefore, we propose:

Proposal 5: For a dedicated resource pool for SL positioning, PSCCH which carriers SCI associated with SL PRS transmission(s) is included.

An example of the slot structure of dedicated SL PRS resource pool can be seen as in Figure 1. A SL PRS transmission may occupy (pre)configured number of symbols in a slot. For each SL-PRS transmission, there will be an associated physical layer control information transmission in the resource pool, which can indicate or reserve the resource of the current or the future SL-PRS transmission(s). The current PSCCH/PSSCH design in SL communication can be the baseline for the design of channel to convey the information. The case of TDM multiplexing of SL PRS from different UEs is shown in the right figure of Figure 1.

Alt 1: 1 SL PRS resource in a slot                          Alt 2: >1 SL PRS resource in a slot

Figure 1 Slot structure of SL PRS dedicated resource pool

In a slot, a mapping relationship shall be defined between the resource used to convey SCI and the resource used to transmit SL PRS associated with the SCI. By this way, the information size in SCI used to indicate the time/frequency resource of SL PRS could be reduced. In addition, the resource selection procedure of control information and the associated SL PRS can be simplified. The details of mapping relationship can be further discussed.

Proposal 6: A mapping relationship is defined between resource of PSCCH and the associated SL PRS resource in a dedicated resource pool.

SL PRS (pre)configuration in shared resource pool

For shared resource pool, SL PRS can be transmitted together with SL PSCCH/PSSCH transmission, as shown in Figure 2. The existing resource reservation and selection mechanisms of SL communication can also be mostly reused.

Figure 2 Slot structure for shared resource pool

To avoid collision with PSCCH and the front loaded PSSCH DMRS symbol, SL PRS shall be mapped to the last several symbols before the gap symbol. The starting symbol or the number of symbols for SL PRS transmitted together with PSCCH/PSSCH can be (pre)configured per resource pool. If SL PRS is transmitted together with PSCCH/PSSCH, different UEs will not transmit SL PRS in the same subchannel of the same slot. Otherwise PSCCH/PSSCH collision will happened. As SL PRS is frequency domain comb based, SL PRS RE and PSSCH RE can be FDMed in the same symbol to improve the resource efficiency.

Proposal 7. The start symbol or the number of symbols for SL PRS transmitted together with PSCCH/PSSCH is (pre)configured for the shared resource pool.

Scheme 1 and scheme 2

In RAN1#110 meeting, the following agreement was made:

Agreement

Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS.

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

Scheme 1

For scheme 1, to support partial coverage scenario i.e., SL PRS transmission from in-coverage UE to out-of-coverage UE(s), the NW shall allocate the SL PRS resource only to the transmitter of SL PRS. Similar as SL communication, the receiver of SL PRS can blindly detect the control information associated with the SL PRS transmission, and then receive SL PRS in the indicated time-frequency domain resource.

CG type 1, CG type 2 and dynamic resource allocation are supported in SL communication. For gNB based scheduling of SL PRS, all of type 1 CG, type 2 CG and dynamic resource allocation can be considered to support different SL positioning requirements in different application scenarios.

Therefore, we propose:

Proposal 8: For gNB based scheduling, CG type 1, CG type 2 and dynamic grand based scheduling are all supported.

Scheme 2

In RAN1#112 meeting, the following agreements were made:

Agreement

For the scheme 2 sensing-based resource allocation,

- Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool.

  - Study if/what changes are needed

- Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc)

- From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

The existing NR sidelink mode 2 resource (re)selection procedures can be mostly reused for SL PRS resource selection, however, there are still some issues to be further investigated.

On candidate resource definition

The definition of SL PRS candidate resource shall be clarified so the initial set of candidate SL PRS resources can be generated in Scheme 2 resource selection. Different from PSSCH in SL communication, SL PRS can be comb based FDMed, therefore, in a dedicated resource pool, a SL PRS resource can be determined by the time domain position and OFDM symbol length, frequency domain position and bandwidth, and the comb offset of the resource. Note that the SL PRS comb factor is assumed to be (pre)configured in the dedicated resource pool. The resource selection procedure of SL PRS shall determine both the time/freq. resource and comb offset of SL PRS transmission.

Proposal 9: A candidate SL PRS resource is determined by the time/frequency position/size and comb offset in a dedicated resource pool.

On S-RSRP measurement

S-RSRP measurement is used in mode 2 sidelink resource selection to determine whether the interference level is strong enough to exclude a candidate resource. The S-RSRP measurement can be obtained by measuring PSCCH DMRS or the associated PSSCH DMRS. To use which option to obtain the S-RSRP measurement is (pre)configured for a resource pool. For SL-PRS in a dedicated resource pool, there may not be associated PSSCH transmission. Therefore, how the SL PRS RSRP measurement is achieved shall be determined.

Two options can be considered:

Option 1: the S-RSRP measurement is obtained from measuring DMRS of PSCCH conveying the SCI indicating the reserved SL PRS resource;

Option 2: the S-RSRP measurement is obtained from measuring SL PRS associated with the SCI indicating the reserved SL PRS resource;

For option 1, a sensing UE can achieve the S-RSRP measurement results from PSCCH only without measuring the associated SL PRS signals, and thus less complexity and energy consumption can be expected; While for option 2, as SL PRS generally has much wider bandwidth, the measurement result is expected to be more accurate. RAN1 shall determine which option or both option(s) shall be supported.

Proposal 10: S-RSRP measurement used to exclude SL-PRS candidate resource in scheme 2 resource selection is obtained

- Option 1: from measuring DMRS of PSCCH conveying the SCI indicating the reserved SL PRS resource;

- Option 2: from measuring SL PRS associated with the SCI indicating the reserved SL PRS resource.

On different multiplexing

Different from PSCCH/PSSCH, SL PRS resources can be comb-based FDMed. However, comb-based FDMed SL PRS resources can produce more interference than TDM or PRB level FDMed SL PRS resource due to IBE, etc. From resource (re)selection perspective, a SL PRS TX UE shall select candidate SL PRS resources without PRB level overlapping with higher priority.

A simple solution is to also perform candidate resource exclusion when the reserved resource and the candidate SL-PRS resource occupies the same PRB but use different comb offset. A higher S-RSRP threshold can be set for candidate resource comb FDMed with reserved resource, compared to the S-RSRP threshold used to exclude the candidate resource which has resource overlapping with reserved resource in both PRB and RE level. In such a way, TDM or PRB level FDMed SL PRS resource can be prioritized.

Proposal 11: For candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset, resource exclusion is performed with an additional S-RSRP threshold.

IUC message

When performing ranging between two UEs and one-way RTT is used, the UE initiates the ranging procedure can proactively provide IUC message to the other UE to help resource allocation of the peer UE to send back the SL PRS signal, e.g., by providing preferred resource set to the peer UE.

Proposal 12: IUC message from a UE to the other UE to assist SL PRS resource allocation shall be supported.

Conclusion

In this contribution, the design of sidelink PRS resource allocation is discussed. Based on the discussion, the following proposals are proposed:

Proposal 1: The UE determines the set of slots that may belong to a SL PRS dedicated resource pool in the same way as for SL communication pool.

Proposal 2: The set of slots assigned to a SL PRS dedicated resource pool are (pre)configured.

Proposal 3: the start PRB position, the number of contiguous PRBs, the start symbol and the length of symbols in a slot are (pre)configured for a SL PRS dedicated resource pool.

Proposal 4: SL PRS comb size, SL PRS subchannel size, and the number of TDM multiplexing SL PRS resources in a slot of dedicated resource pool shall be (pre)configured.

Proposal 5: For a dedicated resource pool for SL positioning, PSCCH which carriers SCI associated with SL PRS transmission(s) is included.

Proposal 6: A mapping relationship is defined between resource of PSCCH and the associated SL PRS resource in a dedicated resource pool.

Proposal 7. The start symbol or the number of symbols for SL PRS transmitted together with PSCCH/PSSCH is (pre)configured for the shared resource pool.

Proposal 8: For gNB based scheduling, CG type 1, CG type 2 and dynamic grand based scheduling are all supported.

Proposal 9: A candidate SL PRS resource is determined by the time/frequency position/size and comb offset in a dedicated resource pool.

Proposal 10: S-RSRP measurement used to exclude SL-PRS candidate resource in scheme 2 resource selection is obtained

- Option 1: from measuring DMRS of PSCCH conveying the SCI indicating the reserved SL PRS resource;

- Option 2: from measuring SL PRS associated with the SCI indicating the reserved SL PRS resource.

Proposal 11: For candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset, resource exclusion is performed with an additional S-RSRP threshold

Proposal 12: IUC message from a UE to the other UE to assist SL PRS resource allocation shall be supported.

# Reference

[1] RP-223549, Expanded and Improved NR Positioning, Intel, RAN#98-e, Dec. 2022.