3GPP TSG RAN WG1 #112bis-e                                       	       R1-2302992

e-Meeting, April 17th –26th, 2023

Agenda Item:	9.5.3

Source:	Xiaomi

Title:                     Discussion on Low Power High Accuracy Positioning

Document for:	Discussion and Decision

Introduction

In RAN- 98 e-meeting, the WI of Rel-18 positioning [1] is approved, and the objectives on LPHAP can be seen as below:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including:

  - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4]

    - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning.

      - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap.

    - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs

  - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3].

    - SRS for positioning configurations in multiple cells [RAN2, RAN1].

      - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work.

    - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3].

    - SRS for positioning activation/request procedure(s) [RAN2, RAN1].

  - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2].

  - Specify solutions for alignment between eDRX and PRS configurations [RAN2].

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we focus on the discussion about SRS enhancement for LPHAP (Low Power High Accuracy Positioning).

Potential enhancement on SRS

Common parameters of SRS in multiple cells

In RAN1-112 meeting [2], the following agreements were archived on SRS configuration in multiple cells.

Agreement

For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following:

- An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area:

  - BWP parameters

    - locationAndBandwidth

    - subcarrierSpacing

    - cyclicPrefix

Agreement

For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area:

- srs-PosConfig

  - SRS-PosResourceSet

    - srs-PosResourceSetId

    - srs-PosResourceIdList

    - resourceType

  - SRS-PosResource

    - srs-PosResourceId

    - transmissionComb

    - resourceMapping

    - freqDomainShift

    - freqHopping

    - groupOrSequenceHopping-r16

    - resourceType

    - FFS: whether sequenceId is configured commonly across cells or per cell

In Rel-17, the UL positioning methods were introduced for RRC Inactive state, but the positioning SRS configuration for RRC inactive will be released when UE performs cell reselection. The network should provide the new SRS configuration to UE, and it will lead more UE power consumption since the additional signaling exchange procedures are needed. Therefore, if positioning SRS configuration provided by network can still be used by the UE when UE performs the cell reselection, the UE power consumption can be reduced.

For SRS configured per cell group, the time resource/frequency resource/ SRS sequence ID/ DMRS port /P0/Alpha can be common for cells in cell group.

- Proposal 1: The SRS sequence ID/ DMRS port /P0/Alpha for SRS per cell group should be common.

TA, spatial relation and pathloss RS update for SRS in multiple cells

In RAN1-112 meeting [2], the following agreements were archived on parameter update of SRS configuration in multiple cells.

Agreement

From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2.

- For TA validation, use of area-specific RSRP change threshold is feasible

  - FFS: which RS is the reference RS for the RSRP change threshold

Agreement

For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area:

  - For the UL timing advance, further study the following options, including the DL reference timing for each option:

  - Option 1: UE maintains the TA obtained from the last serving cell within the validity area

  - Option 2: UE autonomously adjusts the TA

    - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell.

    - FFS: whether there is RAN1 specification impact

  - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH

Agreement

For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options:

  - Option 1: Spatial relation information is absent in the configuration

    - FFS: different approaches for down selection at least include the following:

      - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter

      - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter

    - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.)

  - Option 2: Spatial relation information is provided in the configuration

    - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells

    - FFS signaling to configure the spatial relation information, e.g., via SRS activation message.

  - Note: UE power consumption needs to be taken into account

  - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration.

Agreement

For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options:

  - Option 1: Pathloss RS is absent in the configuration.

    - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.).

  - Option 2: Pathloss RS is provided in the configuration

    - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area

    - FFS signaling to configure the pathloss RS, e.g., via SRS activation message.

  - Note: UE Power consumption needs to be taken into account

  - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area.

  - FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.

But the timing advance, spatial relation and pathloss reference signal need to be updated even in one cell. While on TA validation for SRS, from our point of view, legacy rule can be reused. Since in legacy system, UE consider the TA to be valid when the current RSRP value of the downlink pathloss reference has not increased/decreased by more than a configured threshold. If the RSRP value has increased/decreased by more than a configured threshold, UE will consider the TA to be invalid. Thus the same rule can be used for the validation determination of TA, spatial relation and pathloss reference signal for SRS configured per cell group. In addition, the TA, spatial relation and pathloss reference signal should also be considered to be invalid when cell reselects. And UE can autonomously adjust the TA according to the SSB whose RSRP is higher than a threshold.

- Proposal 2: The TA can be adjusted autonomously by UE based on a SSB whose RSRP is higher than a threshold when it is invalid and the legacy mechanism for valid decision can be reused.

- While for spatial relation and pathloss RS, we prefer option 2, i.e., the spatial relation information and the pathloss RS is provided in the configuration. And the validation of the spatial relation information/pathloss RS can be decided based on the same mechanism as TA validation. If it is invalid, UE can update the spatial relation information and pathloss RS based on a selected SSB. And the selected SSB can be the SSB whose RSRP value is higher than a threshold or whose RSRP is largest among all SSBs.

- Proposal 3: The spatial relation information and pathloss RS for SRS can be provided in the configuration and the same validation mechanism as TA validation can be reused. When invalid, it can be updated based on a selected SSB whose RSRP value is higher than a threshold.

SRS configuration request/activation per cell group

If the cell group changes, the positioning SRS will be not valid. In this case, UE in RRC_ INACTIVE need to request the new SRS configuration from gNB. Thus, random access can be used for SRS configuration request. In addition, the UL transmission based on SDT procedure can also be considered for SRS configuration request. And as same as PRS request, UE can send request to LMF via LPP. So for SRS request, it is also possible for UE to send request to LMF.

- Proposal 4: Consider SRS configuration request by random access or UL transmission based on SDT to gNB, and by LPP to LMF.

While for SRS activation, for UE in INACTICE state, it is possible to activate SRS transmission based on DL transmission of SDT or paging. For the scheme by paging, a new DCI field can be introduced to paging DCI or reserved bitmap in existed DCI field such as short message indicator or short message can be reused for SRS activation. While for the scheme by DL transmission of SDT. For example, based on Msg 2 or Msg 4 during random access procedure. And the SRS activation can be indicated together or separately with SRS reconfiguration.

- Proposal 5: Consider SRS activation by paging DCI or DL transmission based on SDT.

Conclusion

In this contribution, we discussed about LPHAP (Low Power High Accuracy Positioning). Based on above discussion, we provide the following observations and proposals.

- Proposal 1: The SRS sequence ID/ DMRS port /P0/Alpha for SRS per cell group should be common.

- Proposal 2: The TA can be adjusted autonomously by UE based on a SSB whose RSRP is higher than a threshold when it is invalid and the legacy mechanism for valid decision can be reused.

- Proposal 3: The spatial relation information and pathloss RS for SRS can be provided in the configuration and the same validation mechanism as TA validation can be reused. When invalid, it can be updated based on a selected SSB whose RSRP value is higher than a threshold.

- Proposal 4: Consider SRS configuration request by random access or UL transmission based on SDT to gNB, and by LPP to LMF.

- Proposal 5: Consider SRS activation by paging DCI or DL transmission based on SDT.

References

- RP-223549 New WID on Expanded and Improved NR Positioning

- Chairman notes of RAN1-112 meeting