3GPP TSG RAN WG1 #112-bis-e			R1-2302997

Online, April 17th – April 26th, 2023

Source: 	     Xiaomi

Title:		     Discussion on XR-specific capacity enhancements

Agenda item:    9.8.1

Document for:  Discussion

Introduction

In RAN1#112 meeting, we discussed some high priority issues for XR-specific capacity enhancements and some agreements were achieved [1], which can be found in Appendix. In addition, in RAN#99 meeting, WID on XR Enhancements for NR was updated [2], and the remaining issue about PDCCH monitoring resumption may need clarification in RAN1.

In this contribution, we discuss the potential issues on the enhancements for XR-specific capacity and provide our views. Besides, PDCCH monitoring resumption after UL NACK is discussed in subsection 2.3.

Discussion

2.1	Dynamic Indication for Multi-PUSCHs CG

2.1.1	Dynamic Indication for Multi-PUSCHs CG

In RAN1#112 meeting, the following two options are agreed for dynamic indication based on the proposals offered by the companies.

Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI: - Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain. - Applicable numbers can be determined from information obtained from configuration. - FFS details - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain) - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. - Other options are not precluded. Proponent companies to provide details.

For option 1, the UE can indicate the information of the unused TO(s) to the gNB by providing the number of consecutive TO(s) in time domain or a time duration/range containing at least one TO. From the perspective of gNB, the gNB only cares about whether the time-frequency domain resources in TO are used by the UE, while option 1-2 will provide some additional information of time-frequency domain resources in the indicating process, which is unnecessary and leads to increased signaling overhead. From this point of view, option 1-1 is preferred over option 1-2. For option 1-1, there are multiple potential alternatives for determining the consecutive TO(s), as follows.

- Alt. 1: Start TO + Number of TO

- Alt. 2: Start TO + End TO

- Alt. 3: Start TO + Default end TO

Figure 1: Three alternatives for option 1-1

For Alt. 3, the specification can pre-define a default end TO, e.g., the last TO in each CG period is the default end TO. Assuming that the time duration of the dynamic indication is a CG period, as shown in Fig. 1, each of the three alternatives can determines the number of the consecutive TO(s). Technically speaking, each alternative is workable, and how to do down selection requires further discussion.

Observation 1: Option 1-2 provides unnecessary information in the process of indicating the information of the unused TO.

Proposal 1: For option 1-1, the following three potential alternatives require further discussion.

- Alt. 1: Start TO + Number of TO

- Alt. 2: Start TO + End TO

- Alt. 3: Start TO + Default end TO

For option 2, the UE can indicate the information of the unused TO(s) to the gNB by providing a bitmap where a bit corresponds to a TO or more than one TO. Since the number of TO occupied by the XR UL transmission is random, option 2-2 may cause some indication errors. For example, if only one TO is not used by the UE and the indication granularity of option 2-2 is two TOs, the TO can only be abandoned or an additional used TO is indicated as “unused”. Option 2-1 doesn't have that problem, and option 2-1 has the advantage of being flexible enough for dynamic indication. Compared to option 1-1, option 2-1 can indicates all unused TOs in the time duration. Assuming that the time duration of the dynamic indication is a CG period, and the timeline for the rescheduling of gNB cannot be met between adjacent TO. As shown in Fig. 2, there are TO0, TO1, TO2, TO3, TO4, TO5 in a CG period. CG-UCI carrying dynamic indication is transmitted in TO0. TO1 and TO4 are occupied by the XR UL transmission. TO2, TO3 and TO5 are not used by UE. Option 2-1 accurately indicates that the status of TO2, TO3 and TO5 is unused. However, since option 1-1 can only indicate the consecutive CG PUSCH TO(s), only TO2 and TO3 can be indicated. Or TO2, TO3, TO4 and TO5 are all indicated as “unused”, although TO4 is used. If timeline is not considered in option 1-1, at least two CG-UCIs are required to indicate TO2, TO3 and TO5. Although option 2-1 is more flexible than option 1-1, it is clear that option 2-1 has a higher signaling overhead than option 1-1, especially if a larger number of TO are included in the CG period.

Figure 2: Comparison of option 1-1 and option 2-1

Observation 2: Option 2-2 will cause some indication errors when the indication granularity does not match the unused TO.

Observation 3: Option 1-1 has the problem of missing indication for the unused TO or error indication for the used TO.

Observation 4: Option 2-1 has a higher signaling overhead compared with option 1-1, especially if the time duration includes a large number of TO.

In order to balance signaling overhead with indication flexibility, we provide an alternative indication approach as option 2-3 in this section. The UCI provides a table row index that corresponds an unused TO pattern. The unused TO pattern includes at least one TO, and the pattern table can be predefined by the specification or can be semi-statically configured by the gNB to the UE. If the pattern table is large enough, option 2-3 is just as flexible as option 2-1. In addition, the gNB can be configured with different TO pattern tables to balance signaling overhead with indicating flexibility.

Observation 5: Option 2-3 can balance signaling overhead with indicating flexibility by configuring the appropriate pattern table.

Proposal 2: Option 2-3 should be taken into account as follows:

  - Option 2-3: The UCI provides a table row index where the index corresponds to an unused TO pattern within a time duration/range. All TOs in the unused TO pattern within the time duration/range are “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

Proposal 3: Option 1-1, option 2-1 and option 2-3 should be prioritized for Multi-PUSCHs CG.

In the above options, the TO indicated by the UCI needs to be within a time duration/range. The time duration/range corresponding to dynamic indication information also needs to be discussed first to ensure that the gNB and UE can reach the same time duration/range for the application of dynamic indication information. Whether the protocol predefines a fixed time range for dynamic indication or the gNB configures a time range for the UE needs further discussion.

Proposal 4: The time duration/range corresponding to dynamic indication information should be discussed in RAN1.

2.1.2	Transmission occasion of the UCI

In RAN1#112 meeting, the following four options are agreed for transmission occasion of the UCI based on the proposals offered by the companies.

Agreement For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI: - Option 1: A transmitted CG PUSCH, includes the UCI. - FFS details - Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC. - FFS details - Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion. - FFS details - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods - Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s). - FFS details - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods. Other options are not precluded. Proponent companies to provide details.

From our point of view, RRC configuration based on option 2 or protocol pre-definition based on option 3 actually limit the number of TO carrying UCI within a time duration, that is, to avoid the gNB monitoring UCI on some meaningless TO. The ultimate purpose of UE indicating unused TOs to gNB is that make the gNB reuse the time-frequency resources corresponding to these unused TOs. Assuming the time duration is a CG period, some TOs later in the CG period do not need to carry the UCI. Because the dynamic indication for the unused TO needs to consider timeline. Even if a TO later in a CG period carries the UCI, the TO that is indicated unused may not meet the timeline. An extreme example is where the last TO in the CG period including the UCI is meaningless. In addition, if the earlier TO in the CG period carries the UCI, dynamic indication based on the UCI can better indicate more TOs in the CG period, while also providing sufficient time for the gNB to reallocate the time-frequency resources in these TO. Therefore, it makes more sense to send the CG PUSCH carrying the UCI to the first half of a CG period. For example, based on option2 or 3, a number N can be configured by RRC or predefined by specification, and the first N PUSCH TOs within a CG period includes UCI.

Observation 6: Sending CG PUSCH carrying the dynamic indication based on UCI in the TO in the first half of CG period can better meet the timeline, and the UCI can also indicate more TOs in the CG period.

Proposal 5: Details of option 2 and option 3 should be updated as follows:

- Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC.

  - A number N can be configured by RRC, and the first N PUSCH TOs within a CG period/ a multiple CG periods includes the UCI, and N≥1

- Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion.

  - A number N can be predefined by specification, and the first N PUSCH TOs within a CG period/ a multiple CG periods includes the UCI, and N≥1

2.1.3 Timeline for dynamic indication

One objective of the discussion for XR-specific capacity enhancements is to improve system throughput. Therefore, gNB should allocates idle resources of unused TO(s) indicated by a UE to the other UEs through dynamic scheduling or other means. From the perspective of the UEs scheduled by the gNB on the unused TO(s), they need to have sufficient time to receive PDCCH sent by the gNB before sending their PUSCH. Therefore, sufficient time needs to be reserved for the gNB to perform uplink scheduling after receiving the dynamic indication sent by UE, otherwise the indicated idle resources cannot be reused in a timely manner. The reserved time needs to include at least the time for gNB to send a DCI and the time for UE to process the DCI and prepare the PUSCH. So the timeline for dynamic indication needs to be discussed in RAN1.

Proposal 6: RAN1 should prioritize the discussion of the timeline for dynamic indication.

In order to ensure that the gNB can reschedule unused TO(s), three potential options can be considered to define timeline, as follows:

- Option 1: From the TO including the UCI to the time window

- Option 2: From the TO including the UCI to the first TO in the time duration

- Option 3: From the TO including the UCI to the first unused TO in the time duration

Figure 3: Timeline of three options

When the first TO in the time duration is not an unused TO, option 3 can provide more rescheduling time for the gNB, which can ensure that the unused TO can be reused by other UEs. Option 1 and option 2 can provide more preparation time for UE that sends dynamic indication information, and can determine the usage state of TO in the time duration more accurately, avoiding TO that is indicated as used, but in fact the UE does not use the TO. Technically speaking, all options are workable, and the details of timeline based on the three options should be further discussed.

Proposal 7: Three potential options can be considered to define the timeline for dynamic indication, as follows:

- Option 1: From the TO including the UCI to the time window

- FFS details

- Option 2: From the TO including the UCI to the first TO in the time duration

- FFS details

- Option 3: From the TO including the UCI to the first unused TO in the time duration

- FFS details

2.1.4	UCI for dynamic indication

In RAN1#112 meeting, the following agreement are agreed for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision.

Agreement Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision - Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI. - FFS on details - Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI. - FFS on details - Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI. - FFS on details

In the last RAN1 meeting, we agree that the physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH. Therefore, both CG-UCI and the new UCI are carried by CG PUSCH and cannot indicate the information of the first TO in a CG period. Alt. 2 and Alt. 3 do not need define the new UCI and involve less protocol standardization than Alt. 1. Compared with Alt. 3, Alt. 2 will lead to a larger UCI size due to the new field is added to the CG-UCI. Considering the impact on the specification and UCI size, Alt. 3 is preferred.

Proposal 8: Information about unused CG PUSCH transmission occasions can replace some field(s) of the CG-UCI.

2.1.5 Multiple dynamic indications with overlapping time windows

From the perspective of UE, the UE can reserve a TO that is indicated to be used but actually unused. The main reason of this feature is that the UE does not know whether some TOs can be released to provide the gNB for reuse. When time durations of different dynamic indications do not overlap, the information for the usage status of the reserved TO can only be indicated once and cannot be updated, which may cause the time-frequency resources corresponding to the reserved TO to be wasted. Assuming that the UE sends different dynamic indications with overlapping time windows, the UE will have chance to update the information for the usage status of the reserved TO, which can avoid that time-frequency domain resources corresponding these TOs are not eventually allocated by the gNB. As shown in Fig. 4, although TO 2 and TO 3 are indicated as ‘used’ by UCI0, the UE can update the indication information of TO 2 and TO3 as “unused” by UCI1, and the gNB still reallocates the time-frequency resources corresponding these TOs to other UEs. Especially when a CG period is too long in the time domain, the need for UEs to send multiple dynamic indications increases. Therefore, RAN1 needs to discuss whether UE is allowed to send multiple dynamic indications with overlapping time windows.

Figure 4: Different dynamic indications with overlapping time windows

Observation 7: The time-frequency resources corresponding to the reserved TO will be wasted if the usage status of the reserved TO cannot be updated.

Proposal 9: Whether UE is allowed to send multiple dynamic indications with overlapping time windows should be discussed in RAN1.

2.1.6 Reusable resource

In RAN#99 meeting, updated WID for RAN1 has confirmed that UE indicates the unused CG PUSCH transmission occasion by a UCI. From the perspective of gNB, the reusable resource of the indicated TO can be allocated flexibly. For example, the reusable resources can be scheduled to the UE that sents the dynamic indication to the gNB or the other UEs, and it depends on the implementation of the gNB.

Observation 8: The resource of the unused CG PUSCH transmission occasion can be allocated flexibly based on gNB’s implementation.

Proposal 10: The gNB can allocate reusable resources to any UE, including the UE that sends UCI carrying the dynamic indication.

2.2	Multi-PUSCHs CG configuration

2.2.1	HARQ process ID

In RAN1#112 meeting, the following five options are agreed for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG based on the proposals offered by the companies.

Conclusion RAN1 discusses to decide how to determine the HARQ process ID of CG PUSCHs of a multi-PUSCHs CG. Agreement For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives: - Alt. 1: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period. - Alt 1-1; X = 1 - Alt 1-2: X is the number of configured PUSCHs in a period - Alt 1-3: X is provided by RRC configuration. - FFS details - Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - FFS details - Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - FFS on potential enhancements different from previous alternatives - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period. - FFS details - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period. - FFS details - Alt. 4: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS on potential enhancements different from previous alternatives - Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS details - Alt 6: FFS other solutions Note: The case of one TB map to multiple PUSCHs is not considered here.

Configuring different HARQ process IDs for different PUSCHs facilitates scheduling flexibility, but consumes more HP IDs. The same HARQ process ID can be used for multiple PUSCHs, which limits scheduling, but saves HARQ process ID resources. When the number of TO that can be configured in a CG period is small, it makes sense to configure different HARQ process IDs for different PUSCHs so that the UE does not have to wait and can freely choose the CG PUSCH to send. However, if the number of TO that can be configured in a CG period is large, the trade-off between scheduling flexibility and the number of HARQ process IDs needs to be considered. For example, TOs are grouped within a CG period, and PUSCH within the same group share the same HARQ process ID. Technically speaking, all of the above alternatives provided in RAN1#112 meeting are workable, and are applicable to different TO number configuration scenarios. Currently the maximum number of TO that can be configured in a CG period is not determined, so the discussion of HARQ process IDs needs to be postponed.

Observation 9: The number of TO that can be configured in a CG period should be discussed before the discussion of the HP process ID

Proposal 11: RAN1 should postpone discussing HP process IDs until the maximum number of TO that can be configured in a CG period is agreed.

2.2.2	TDRA

In RAN1#112 meeting, the following three alternatives based on three different frameworks are agreed for Multi-PUSCHs CG.

Agreement For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study: - Alt-A: TDRA determination based on repetition framework. - Alt-A1: Follow the time domain resource mapping of Type A repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The same SLIV in N PUSCH in consecutive slots per CG period - FFS for non-consecutive slots - FFS details, including related RRC parameters - Alt-A2: Follow the time domain resource mapping of Type B repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The SLIV used for 1st PUSCH per CG period. - N consecutive nominal PUSCHs with same duration per CG period - Note: N is not necessarily the repetition factor. FFS details, including related RRC parameters - Alt-B: TDRA determination based on NR-U framework - N and M configured by higher layers - Single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration. - FFS details, including related RRC parameters - Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots. - FFS details, including related RRC parameters - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots. - FFS details, including related RRC parameters

Among the alternatives given above, the key difference is whether multiple TOs in a CG period can configure different time domain resources. In R16, different time domain resources with overlapping for different CG periods can be configured by the gNB, so as to select the most appropriate time domain resources to send when the uplink transmission service arrives, thus reducing the time delay. However, in a CG period, if there is time domain resource overlap on different TOs, resources will be wasted. For example, if two TOs with the same frequency resource within the CG period partially overlap in the time domain, then one TO is used by the UE, then the other TO can only be indicated to "used". Even if non-overlapping time domain resources are configured at different TOs, the gain on delay cannot be obtained. In summary, we find no motivation to allocate different time domain resources for different TOs within a CG period, and Alt-A and Alt-B should be prioritized for determination of the TDRA of CG PUSCHs associated to a multi-PUSCHs CG.

Proposal 12: Alt-A and Alt-B should be prioritized for determination of the TDRA of CG PUSCHs associated to a multi-PUSCHs CG.

2.2.3	FDRA

Configuring different FDRAs for different TOs within a CG period can lead to some resource fragmentation, which can result in lower resource utilization across the system. Therefore, FDRA of CG PUSCHs in a multi-PUSCHs CG configuration should be same.

Proposal 13: FDRA of CG PUSCHs in a multi-PUSCHs CG configuration should be same.

2.2.4	MCS

It is not necessary to configure different MCS for CG PUSCHs in a multi-PUSCHs CG configuration. The gNB can configure the MCS for the UE based on the worst case. In this case, the time-frequency resources corresponding to TO will be large. However, considering that unused TOs can be indicated by UEs and reused by the gNB, it will not cause serious waste of resources. In addition, conservative MCS can improve the reliability of XR service transmission.

Proposal 14: MCS of CG PUSCHs in a multi-PUSCHs CG configuration should be same.

2.3	PDCCH monitoring resumption after UL NACK

To ensure short latency for retransmission, we support the proposal for PDCCH monitoring resumption after UL NACK if the NACK transmission falls into the duration of PDCCH skipping, and the details needs to be discussed.

Case 1, UE is configured with C-DRX.

According to current spec, when UE transmits a UL NACK, UE will start drx-HARQ-RTT-TimerDL for corresponding HARQ process, and when drx-HARQ-RTT-TimerDL expires, UE will start drx-RetransmissionTimerDL, and during drx-RetransmissionTimerDL, UE will keep active in order to monitor DL grant for retransmission. And if PDCCH skipping is applied, and UE needs to resume PDCCH monitoring for retransmission scheduling, it is reasonable that UE resumes PDCCH monitoring when drx-RetransmissionTimerDL is running.

Proposal 15: When UE is configured with C-DRX, UE resumes PDCCH monitoring at least when drx-RetransmissionTimerDL is running

Case 2, UE is not configured with C-DRX.

when UE transmits a UL NACK, UE will wait for gNB scheduling for retransmission and the waiting will last for some time to allow gNB preparation for that scheduling. UE can resume PDCCH monitoring immediately after UL NACK transmission to wait retransmission scheduling.

Another question is, how long the PDCCH monitoring will last, or PDCCH skipping is terminated after the resuming. Our opinion is, to reduce UE complexity, it is preferred to terminate PDCCH skipping after the resuming.

Proposal 16: When UE is not configured with C-DRX, UE start resuming PDCCH monitoring in the next slot after UL NACK transmission, and PDCCH skipping is terminated after the resuming.

Conclusion

In this contribution, we discuss possible issues on Rel-18 XR-specific capacity enhancements. Regarding the conditions where the dynamic indication for the CG PUSCH transmission occasion is required, we have the following conclusion:

Observation 1: Option 1-2 provides unnecessary information in the process of indicating the information of the unused TO.

Observation 2: Option 2-2 will cause some indication errors when the indication granularity does not match the unused TO.

Observation 3: Option 1-1 has the problem of missing indication for the unused TO or error indication for the used TO.

Observation 4: Option 2-1 has a higher signaling overhead compared with option 1-1, especially if the time duration includes a large number of TO.

Observation 5: Option 2-3 can balance signaling overhead with indicating flexibility by configuring the appropriate pattern table.

Observation 6: Sending CG PUSCH carrying the dynamic indication based on UCI in the TO in the first half of CG period can better meet the timeline, and the UCI can also indicate more TOs in the CG period.

Observation 7: The time-frequency resources corresponding to the reserved TO will be wasted if the usage status of the reserved TO cannot be updated.

Observation 8: The resource of the unused CG PUSCH transmission occasion can be allocated flexibly based on gNB’s implementation.

Observation 9: The number of TO that can be configured in a CG period should be discussed before the discussion of the HP process ID

Proposal 1: For option 1-1, the following three potential alternatives require further discussion.

- Alt. 1: Start TO + Number of TO

- Alt. 2: Start TO + End TO

- Alt. 3: Start TO + Default end TO

Proposal 2: Option 2-3 should be taken into account as follows:

  - Option 2-3: The UCI provides a table row index where the index corresponds to an unused TO pattern within a time duration/range. All TOs in the unused TO pattern within the time duration/range are “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

Proposal 3: Option 1-1, option 2-1 and option 2-3 should be prioritized for Multi-PUSCHs CG.

Proposal 4: The time duration/range corresponding to dynamic indication information should be discussed in RAN1.

Proposal 5: Details of option 2 and option 3 should be updated as follows:

- Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC.

  - A number N can be configured by RRC, and the first N PUSCH TOs within a CG period/ a multiple CG periods includes the UCI, and N≥1

- Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion.

  - A number N can be predefined by specification, and the first N PUSCH TOs within a CG period/ a multiple CG periods includes the UCI, and N≥1

Proposal 6: RAN1 should prioritize the discussion of the timeline for dynamic indication.

Proposal 7: Three potential options can be considered to define the timeline for dynamic indication, as follows:

- Option 1: From the TO including the UCI to the time window

- FFS details

- Option 2: From the TO including the UCI to the first TO in the time duration

- FFS details

- Option 3: From the TO including the UCI to the first unused TO in the time duration

- FFS details

Proposal 8: Information about unused CG PUSCH transmission occasions can replace some field(s) of the CG-UCI.

Proposal 9: Whether UE is allowed to send multiple dynamic indications with overlapping time windows should be discussed in RAN1.

Proposal 10: The gNB can allocate reusable resources to any UE, including the UE that sends UCI carrying the dynamic indication.

Proposal 11: RAN1 should postpone discussing HP process IDs until the maximum number of TO that can be configured in a CG period is agreed.

Proposal 12: Alt-A and Alt-B should be prioritized for determination of the TDRA of CG PUSCHs associated to a multi-PUSCHs CG.

Proposal 13: FDRA of CG PUSCHs in a multi-PUSCHs CG configuration should be same.

Proposal 14: MCS of CG PUSCHs in a multi-PUSCHs CG configuration should be same.

Proposal 15: When UE is configured with C-DRX, UE resumes PDCCH monitoring at least when drx-RetransmissionTimerDL is running

Proposal 16: When UE is not configured with C-DRX, UE start resuming PDCCH monitoring in the next slot after UL NACK transmission, and PDCCH skipping is terminated after the resuming.

Reference

  - 3GPP TSG RAN WG1 #112 RAN1 Chair’s Notes, February 27th – March 3rd, 2023

  - RP-230786 Updated WID on XR Enhancements for NR, RAN

- Appendix

Agreement - Multi-PUSCHs CG is supported for Type-1 configured grant. - Multi-PUSCHs CG is supported for Type-2 configured grant. Agreement For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study: - Alt-A: TDRA determination based on repetition framework. - Alt-A1: Follow the time domain resource mapping of Type A repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The same SLIV in N PUSCH in consecutive slots per CG period - FFS for non-consecutive slots - FFS details, including related RRC parameters - Alt-A2: Follow the time domain resource mapping of Type B repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The SLIV used for 1st PUSCH per CG period. - N consecutive nominal PUSCHs with same duration per CG period - Note: N is not necessarily the repetition factor. FFS details, including related RRC parameters - Alt-B: TDRA determination based on NR-U framework - N and M configured by higher layers - Single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration. - FFS details, including related RRC parameters - Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots. - FFS details, including related RRC parameters - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots. - FFS details, including related RRC parameters Agreement For the PUSCHs parameters in a multi-PUSCHs CG configuration, the configuration/indication parameters except MCS and FDRA of CG PUSCHs in a multi-PUSCHs CG configuration are the same - FFS: For MCS and FDRA, study further to decide whether/how to be different. - FFS: Applicability to type-1 and type-2 - Note: TDRA and HP ID are not in this scope of the above statement. Conclusion RAN1 discusses to decide how to determine the HARQ process ID of CG PUSCHs of a multi-PUSCHs CG. Agreement For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives: - Alt. 1: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period. - Alt 1-1; X = 1 - Alt 1-2: X is the number of configured PUSCHs in a period - Alt 1-3: X is provided by RRC configuration. - FFS details - Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - FFS details - Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - FFS on potential enhancements different from previous alternatives - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period. - FFS details - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period. - FFS details - Alt. 4: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS on potential enhancements different from previous alternatives - Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS details - Alt 6: FFS other solutions Note: The case of one TB map to multiple PUSCHs is not considered here. Agreement The physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH. Agreement Encoding and multiplexing for “the UCI that provides information about unused CG PUSCH transmission occasions” in a CG PUSCH applies encoding and multiplexing procedures for CG-UCI as baseline. - FFS on details XK：CG Agreement Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision - Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI. - FFS on details - Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI. - FFS on details - Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI. - FFS on details XK：alt- Agreement For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI: - Option 1: A transmitted CG PUSCH, includes the UCI. - FFS details - Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC. - FFS details - Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion. - FFS details - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods - Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s). - FFS details - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods. Other options are not precluded. Proponent companies to provide details. Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI: - Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain. - Applicable numbers can be determined from information obtained from configuration. - FFS details - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain) - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. - Other options are not precluded. Proponent companies to provide details.