3GPP TSG RAN WG1 #112b-e	R1-2303002

Meeting, Fen 11th – 18th, 2022

Source:	CableLabs

Title:	SL-U Channel Access Mechanism Clarifications

Agenda Item:	9.4.1.1

Document for: 	Discussion and Decision

# Introduction

During RAN1 #110 [1] and RAN1 #111 [3], some agreements have been reached, concerning Type 2A/2B/2C access.

Since SL-U is a communication between UE to UE (not defined by NR-U specifications[2]), the applicability of NR-U channel access mechanisms to SL-U operation requires some clarifications.

This paper discusses differences between NR-U and SL-U channel access procedures and also clarifies the use of Type 2A/2B/2C Clear Channel Access (CCA) procedure when applied to SL, aligning them with [2] specifications.

# Background

During RAN1 #110, the following agreement, concerning Type 2A/2B/2C transmissions, had been reached.

Agreement

- Type 2A/2B/2C SL channel access procedures

  - Type 2A channel access procedure is applicable to the following case:

    - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy

    - FFS any other transmission by a UE (e.g., other than COT sharing)

    - FFS whether Type 2A is used also for the case of short control signalling transmission

  - Type 2B channel access procedure is applicable to the following case:

    - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy

    - FFS the case when the gap is between 16 and 25us

    - FFS any other transmission by a UE (e.g., other than COT sharing)

  - Type 2C channel access procedure is applicable to the following case:

    - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us.

    - FFS any other transmission by a UE (e.g., other than COT sharing)

    - FFS whether Type 2C is used also for the case of short control signalling transmission

  - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures

  - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs

----------------------------

NR-U [2] applies restrictions on the usage of Type 2A/2B/2C CCA procedure. These limitations were intended to be discussed in “FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures.”

This paper aligns the SL-U channel access to NR-U and LAA LTE specficcations [2], while considering differences between SL and Uu operations.

During RAN1 #111, the following agreement, concerning Type 2A transmissions, applicable to S-SSB, had been reached.

Agreement

- Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met:

  - Time duration is at most 1ms per transmission

  - The duty cycle of the S-SSB transmissions is at most 1/20

  - FFS: details of EDT

  - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined

- FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

We discuss the alignment of this agreement with [2] requirements focusing on FFS points.

# Discussion

## DL Type 2A/2B/2C channel access clarifications

[2] provides the DL Type 2A conditions on sections #4.1.2 and 4.1.3 (see Appendix 4.1). We observe that some of the DL Type 2A aspects applies to SL-U (e.g. DL Type 2A support for S-SSB), while others don’t (UE sharing a gNB COT).

Observation 1

SL-U re-uses DL Type 2A support for S-SSB as agreed by [4].

SL-U is a UE to UE type of communication. Accordingly, COT sharing by a gNB following a UE transmission, as described by section #4.1.2 [2], doesn’t apply to SL-U.

Proposal 1.  SL-U UE doesn’t employ DL Type 2A, as described by section #4.1.2 [2], except the S-SSB usage of DL Type 2A procedure.

Proposal 2.

DL Type 2B/2C communication, as specified by #4.1.2.2 and #4.1.2.3 [2] do not apply to the SL-U case.

[2] provides the UL Type 2A conditions in sections #4.1.2 and 4.1.3 (see Appendix 4.2).

2.2 UL Type 2A/2B/2C channel access clarifications

[2] provides the UL Type 2A/2B/2C conditions on sections #4.1.2 and 4.1.3 (see Appendix 4.2), which apply to SL-U (Mode 2), since SL-U is based on UE to UE communications

Proposal 3

FFS if there are any exceptions from the Channel Access Procedures for Uplink Transmissions for the SL-U.

2.3 Applicability of Type 2A channel access procedure to S-SSB

We consider that the energy detection threshold procedure described by section #4.1.5 [2] based on discovery burst (e.g., XThresh_max formula) is applicable to the S-SSB case with TA=5dB).

Proposal 4

The EDT procedure, defined by NR-U specs [2], section #4.1.5 applies to S-SSB.

While somehow ambiguous, we believe that the ‘observation period’ agreed during RAN1 #111, as cited above, relates to DL Type 2A and more specifically to the transmission duration of DL Type 2A. If this assumption holds, then DL Type 2A specifications as defined by #4.1.2 [2] applies to S-SSB

Proposal 5

The transmission period and the duty cycle defined for DL Type 2A, apply to S-SSB.

# Conclusions

WE discussed a number of clarifications intended to align Type 2A/2B/2C channel access procedures when applied to SL-U to the NR-U specifications [2].

Observation 1

SL-U re-uses DL Type 2A support for S-SSB as agreed by [4].

Proposal 1.  SL-U UE doesn’t employ DL Type 2A/2B/2C, as described by section #4.1.2 [2], except the S-SSB usage of DL Type 2A procedure.

Proposal 2.

DL Type 2B/2C communication, as specified by #4.1.2.2 and #4.1.2.3 [2] do not apply to the SL-U case.

Proposal 3

FFS if there are any exceptions from the Channel Access Procedures for Uplink Transmissions for the SL-U.

Proposal 4

The EDT procedure, defined by NR-U specs [2], section #4.1.5 applies to S-SSB.

Proposal 5

The transmission period and the duty cycle defined for DL Type 2A, apply to S-SSB.

# References

[1] R1-2208321, “3GPP TSG RAN WG1 Meeting #110. Final report’, August 2022

[2] “3GPP TS 37.213. Physical layer procedures for shared spectrum channel access”

[3] “3GPP TSG RAN WG1 Meeting #111. Final minutes report v100’, November 2022

# Appendix

4.1 DL Type 2A channel access TS37.213 specifications

TS37.213 v17.4.0 section #4.1.2

### 4.1.2	Type 2 DL channel access procedures

This clause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by sensing slots that are sensed to be idle before a downlink transmission(s) is deterministic.

If an eNB performs Type 2 DL channel access procedures, it follows the procedures described in clause 4.1.2.1.

Type 2A channel access procedures as described in clause 4.1.2.1 are only applicable to the following transmission(s) performed by an eNB/gNB:

-	Transmission(s) initiated by an eNB including discovery burst and not including PDSCH where the transmission(s) duration is at most 1ms, or

-	Transmission(s) initiated by a gNB with only discovery burst or with discovery burst multiplexed with non-unicast information, where the transmission(s) duration is at most 1ms, and the discovery burst duty cycle is at most 120, or

-	Transmission(s) by an eNB/ gNB following transmission(s) by a UE after a gap of 25μs in a shared channel occupancy as described in clause 4.1.3.

Type 2B or Type 2C DL channel access procedures as described in clause 4.1.2.2 and 4.1.2.3, respectively, are applicable to the transmission(s) performed by a gNB following transmission(s) by a UE after a gap of 16μs or up to 16μs, respectively, in a shared channel occupancy as described in clause 4.1.3.

#### 4.1.2.1	Type 2A DL channel access procedures

An eNB/gNB may transmit a DL transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_dl=25μs. Tshort_dl consists of a duration Tf=16μs immediately followed by one sensing slot and Tf includes a sensing slot at start of  Tf. The channel is considered to be idle for  Tshort_dl  if both sensing slots  of Tshort_dl are sensed to be idle.

An UE may transmit a SL transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_dl=25μs. Tshort_dl consists of a duration Tf=16μs immediately followed by one sensing slot and Tf includes a sensing slot at start of  Tf. The channel is considered to be idle for  Tshort_dl  if both sensing slots  of Tshort_dl are sensed to be idle.

#### 4.1.2.2	Type 2B DL channel access procedures

A gNB may transmit a DL transmission immediately after sensing the channel to be idle within a duration of Tf=16μs. Tf includes a sensing slot that occurs within the last 9μs of Tf. The channel is considered to be idle within the duration Tf if the channel is sensed to be idle for a total of at least 5μs with at least 4μs of sensing occurring in the sensing slot.

#### 4.1.2.3	Type 2C DL channel access procedures

When a gNB follows the procedures in this clause for transmission of a DL transmission, the gNB does not sense the channel before transmission of the DL transmission. The duration of the corresponding DL transmission is at most 584μs.

4.2 UL Type 2A channel access TS37.213 specifications

TS37.213 v17.4.0 section #4.2.1

### 4.2.1	Channel access procedures for uplink transmission(s)

------------------------------------------------------------------------------

If a DL assignment triggering SRS but not scheduling a PUCCH transmission indicates Type 2 channel access procedures, the UE shall use Type 2 channel access procedures.

If a UE is scheduled by an eNB/gNB to transmit PUSCH and SRS in contiguous transmissions without any gaps in between, and if the UE cannot access the channel for PUSCH transmission, the UE shall attempt to make SRS transmission  according to uplink channel access procedures specified for SRS transmission.

If a UE is scheduled by a gNB to transmit PUSCH and one or more SRSs by a single UL grant in non-contiguous transmissions, or a UE is scheduled by a gNB to transmit PUCCH and/or SRSs by a single DL assignment in non-contiguous transmissions, the UE shall use the channel access procedure indicated by the scheduling DCI for the first UL transmission scheduled by the scheduling DCI. If the channel is sensed by the UE to be continuously idle after the UE has stopped transmitting the first transmission, the UE may transmit further UL transmissions scheduled by the scheduling DCI using Type 2 channel access procedures or Type 2A UL channel access procedures without applying a CP extension if the further UL transmissions are within the gNB Channel Occupancy Time. Otherwise, if the channel sensed by the UE is not continuously idle after the UE has stopped transmitting the first UL transmission or the further UL transmissions are outside the gNB Channel Occupancy Time, the UE may transmit the further UL transmissions using Type 1 channel access procedure, without applying a CP extension.

A UE shall use Type 1 channel access procedures for PUCCH transmissions unless stated otherwise in this clause. If a DL grant determined according to Clause 9.2.3 in [7, TS38.213] or a random access response (RAR) message for successRAR scheduling a PUCCH transmission indicates Type 2 channel access procedures, the UE shall use Type 2 channel access procedures.

When a UE uses Type 1 channel access procedures for PUCCH transmissions or PUSCH only transmissions without UL-SCH, the UE shall use UL channel access priority class p=1 in Table 4.2.1-1.

A UE shall use Type 1 channel access procedure for PRACH transmissions and PUSCH transmissions without user plane data related to random access procedure that initiate a channel occupancy. In this case, UL channel access priority class p=1 in Table 4.2.1-1 is used for PRACH transmissions, and UL channel access priority class used for PUSCH transmissions is determined according to Clause 5.6.2 in [9].

When a UE uses Type 1 channel access procedures for PUSCH transmissions on configured resource, the UE determines the corresponding UL channel access priority p  in Table 4.2.1-1 following the procedures described in Clause 5.6.2 in [9].

When a UE uses Type 1 channel access procedures for PUSCH transmissions with user plane data indicated by a UL grant or related to random access procedure where the corresponding UL channel access priority p is not indicated, the UE determines p in Table 4.2.1-1 following the same procedures as for PUSCH transmission on configured resources using Type 1 channel access procedures.

When a UE uses Type 2A, Type 2B, or Type 2C UL channel access procedures for PUSCH transmissions indicated by a UL grant or related to random access procedures, where the corresponding UL channel access priority p is not indicated, the UE assumes that the channel access priority class p=4 is used by the gNB for the Channel Occupancy Time.

A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds Tulmcot,p where the channel access procedure is performed based on the channel access priority class p  associated with the UE transmissions, as given in Table 4.2.1-1.

--------------------------------------------------------------------------