3GPP TSG-RAN WG1 Meeting #112-bis-e	R1-2303014

E-meeting, April 17th – 26th, 2023

Agenda Item:	9.9.1

Source:	Ericsson

Title:	On coverage enhancements for NR NTN

Document for:	Discussion

# 1	Introduction

A work item (WI) for enhancements of NR NTN was approved at RAN#94-e. The latest WID can be found in [1]. One objective of the WI is NTN-specific coverage enhancements. A 6-month study phase has been pursued, focusing on evaluating the coverage performance and identifying the candidate physical radio channels that have coverage issues specific to NTN. Based on the outcome, RAN#97 agreed to study and specify support for some UL enhancements.

In this contribution, we discuss signaling, performance and other aspects for Msg4 HARQ-ACK PUCCH repetitions and DMRS bundling for PUSCH. We also discuss the support of Msg3 repetitions for CFRA.

# 2	Discussion

## 2.1	WID objectives

The objectives for coverage enhancement in the NTN enhancements WID [1] are copied below.

The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage).

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port).

Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO.

The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps.

The detailed objectives are for NTN:

- To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4]

- To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

## 2.2	Link budget calculations

A link budget analysis for the reference scenario agreed at RAN#97-e is shown in Table 1.

Table 1: Link budget analysis for LEO1200 uplink with Set-1 parameters and 30° elevation angle.

| TX EIRP [dBm] |  | 17.5 |
| --- | --- | --- |
| RX G/T [dB/K] |  | 1.1 |
| Free space path loss [dB] |  | 164.5 |
| Atmospheric loss |  | 0.066 |
| Shadow fading margin [dB] |  | 3 |
| Scintillation loss [dB] |  | 2.2 |
| Polarization loss [dB] |  | 3 |
| Additional losses [dB] |  | 0 |
| Target SNR [dB] | 1 PRB | -8.1 |
|  | 2 PRBs | -11.1 |

## 2.3	PUCCH for Msg4 HARQ-ACK

### 2.3.1	Scope: For Msg4 HARQ-ACK PUCCH, or all PUCCH when no dedicated PUCCH is configured?

If the network configures dedicated PUCCH resources to the UE in the RRCConfig message in Msg4, PUCCH repetition can be configured for the dedicated PUCCH resources. Static PUCCH repetition on dedicated PUCCH is mandatory for the UE since Rel-15. Dynamic PUCCH repetition on dedicated PUCCH was introduced as an optional UE feature in Rel-17 and can also be configured in Msg4 depending on UE capabilities.

If dedicated PUCCH resources are configured in Msg4, PUCCH repetition can be configured on those PUCCH resources and there is no need for further PUCCH repetition on common PUCCH.

However, configuration of a dedicated PUCCH resource in Msg4 is up to network decision, and there could be scenarios in which the network lets the UE continue using the common PUCCH for some time. One example is during initial access from RRC_IDLE state, in which case the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent. Then the network needs to wait until the full UE identity is received in Msg5 before it can retrieve the UE capabilities. In this case, PDSCH transmission other than Msg4 might occur when there is only common PUCCH resource allocated to the UE for HARQ-ACK transmission.

RRC configuration of dedicated PUCCH resources in Msg4 is up to network implementation.

During initial access from RRC_IDLE state, the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent, and may decide to wait with configuring dedicated PUCCH until later.

If repetition is motivated for Msg4 HARQ-ACK, it can as well be needed for the HARQ-ACK corresponding to following PDSCHs. Therefore, until the dedicated PUCCH is provided, PUCCH repetitions on common PUCCH is useful also for other cases than for HARQ-ACK of the PDSCH carrying Msg4.

The solutions to support PUCCH repetition for Msg4 HARQ-ACK are applicable for any PUCCH when dedicated PUCCH is not configured.

To allow early configuration of dedicated PUCCH resources with the Rel-17 feature dynamic PUCCH repetition enabled, we propose that a UE that supports “PUCCH repetition when dedicated PUCCH is not configured” also supports the Rel-17 feature dynamic PUCCH repetition on dedicated PUCCH.

A UE that supports the Rel-18 feature for PUCCH repetition when dedicated PUCCH is not configured shall also support the Rel-17 feature dynamic PUCCH repetition (slotBasedDynamicPUCCH-Rep-r17).

### 2.3.2	Whether UE requests repetition or indicates repetition capability

In RAN1#112, the following working assumption was agreed:

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK

  - FFS: value of X (the maximum configurable value of the RSRP threshold)

  - Down-select one from the following alternatives for the RSRP threshold.

    - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

    - Alt B: New RSRP threshold is introduced.

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

The solution outlined by the working assumption supports both that the UE requests repetition and indicates repetition capability, depending on network configuration. Therefore, we think the working assumption should be confirmed.

Confirm the working assumption from RAN1#112 (“For PUCCH repetition for Msg4 HARQ-ACK, a RSRP threshold can be configured via SIB…”).

In the WA, two options for how to instruct the UE to indicate its capability are mentioned:

- Not configuring the RSRP threshold

- Configuring the RSRP threshold with value X (the maximum configurable value)

Functionally, the two options are equivalent. The difference is only a matter of RRC signaling. It can be noted that option 1 has the advantage that it saves SIB bits in case the network wants the UE to indicate its capability. However, the decision should be left to RAN2.

The two options how to instruct the UE to indicate its capability (not configuring the RSRP threshold or configuring the RSRP threshold with value X) are equivalent from a functional point of view. Not configuring the RSRP threshold saves bits in SIB.

How to instruct the UE to always indicate its capability (not configuring the RSRP threshold or configuring the RSRP threshold with value X) is an RRC signaling decision that should be decided by RAN2.

In the WA, two options for how to define the RSRP threshold for Msg4 HARQ-ACK PUCCH repetition are listed:

Alt A: The same RSRP threshold as for R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

Alt B: A new RSRP threshold is introduced.

Simulations comparing the performance of Msg3 PUSCH and Msg4 HARQ-ACK PUCCH are presented below. Simulation assumptions are summarized in Appendix A, Table 8 and Table 9. Results are shown in Figure 1. It can be seen that the SNR limit for when repetitions are needed differs by 2 dB for Msg4 HARQ-ACK PUCCH (at 1% ACK to MD target) and Msg3 PUSCH (at 10% iBLER target).

Figure 1: Performance of Msg4 HARQ-ACK PUCCH without repetitions and Msg3 PUSCH without repetitions.

Further, Rel-17 Msg3 PUSCH repetition and Rel-18 Msg4 HARQ-ACK PUCCH repetition are optional features both for the network and the UE. Support for one of the features should not require support of the other.

For these reasons, we propose to have separate thresholds for triggering request for Msg3 repetition and Msg4 HARQ-ACK PUCCH repetition from the UE.

The SNR limit for when repetitions are needed is different for Msg4 HARQ-ACK PUCCH and Msg3 PUSCH.

Rel-17 Msg3 PUSCH repetition and Rel-18 Msg4 HARQ-ACK PUCCH repetition are optional features both for the network and the UE. Support for one of the features should not require support of the other.

Define a new RSRP threshold for triggering request for Msg4 HARQ-ACK PUCCH repetitions.

### 2.3.3	How the UE requests repetition/indicates repetition capability

In RAN#112, the following working assumption was agreed:

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE.

- Option A: PRACH preamble and/or occasion

  - FFS: whether PRACH resource partitioning is needed for indication of [repetition request  or capability report]

  - FFS: whether or not indication of repetition factor is assumed

  - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings

- Option B: Higher layer signaling in Msg3 PUSCH

  - FFS: which signaling is used

  - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2.

- Option C: Physical layer signaling in Msg3 PUSCH

  - FFS: which signaling is used, e.g. DMRS ports

Regarding Option A (PRACH preamble and/or occasion), PRACH resource partitioning is used for request for Msg3 repetition. The UE requests repetition of Msg3 by using separate PRACH resources/preambles if the RSRP is below a configured threshold. Using PRACH resource partitioning also to request Msg4 HARQ-ACK PUCCH repetition would result in further fragmentation of the PRACH resources. E.g., assuming that Rel-17 Msg3 repetition and Rel-18 Msg4 HARQ-ACK PUCCH repetition are independent features that are both needed in a coverage limited scenario, four PRACH resource partitions would be needed to support all combinations just for these two features. Even if it is assumed that UE supporting Rel-18 Msg4 HARQ-ACK PUCCH repetition always supports Rel-17 Msg3 repetitions, three PRACH resource partitions would be needed.

PRACH partitioning is also used for RedCap, slicing and SDT, and is being considered for the Rel-18 PRACH repetition feature. All these features could potentially be used in NTN but would then imply further PRACH resource partitioning. Excessive partitioning of the PRACH resources may cause PRACH capacity problems and increase complexity. Therefore, we think that for Msg4 HARQ-ACK PUCCH repetition request, of which knowledge is not needed until after Msg3 is received, PRACH partitioning should not be used.

PRACH resource partitioning is used for Msg3 PUSCH repetition, RedCap, slicing, SDT and other features that are potentially useful in NTN. It is also considered for Rel-18 PRACH repetition.

Excessive partitioning of the PRACH resources may cause capacity problems for PRACH and increases complexity.

Knowledge of the UE’s need for Msg4 HARQ-ACK PUCCH repetition is not needed in the network until after Msg3 is received.

Regarding Option C (physical layer signaling in Msg3 PUSCH), it has been proposed to use dedicated scrambling or a dedicated DMRS port For Msg3 PUSCH. Both these solutions require blind detection, which has negative impact on gNB complexity.

For request for Msg4 HARQ-ACK PUCCH repetition through physical layer signaling in Msg3 PUSCH, the proposed solutions require blind detection, which has negative impact on gNB complexity.

Due to this, we propose the following:

Higher layer signaling in Msg3 PUSCH (Option B) is used to request Msg4 HARQ-ACK PUCCH repetition.

It is possible to convey information in the MAC subheader for Msg3, shown in Figure 2 (from TS 38.321, clause 6.1.2 [6]):

Figure 2: R/LCID MAC subheader

The header consists of an LCID field and two reserved bits. The reserved bits are set to 0 by legacy UE. The LCID field is encoded as shown in the table below (from TS 38.321, clause 6.2.1 [6]):

Table 2: Values of LCID for UL-SCH

| Codepoint/Index |  | LCID values |
| --- | --- | --- |
| 0 |  | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for a RedCap UE |
| 1–32 |  | Identity of the logical channel of DCCH and DTCH |
| 33 |  | Extended logical channel ID field (two-octet eLCID field) |
| 34 |  | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |  |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |  |
| 37–42 |  | Reserved |
| 43 |  | Truncated Enhanced BFR (one octet Ci) |
| 44 |  | Timing Advance Report |
| 45 |  | Truncated Sidelink BSR |
| 46 |  | Sidelink BSR |
| 47 |  | Reserved |
| 48 |  | LBT failure (four octets) |
| 49 |  | LBT failure (one octet) |
| 50 |  | BFR (one octet Ci) |
| 51 |  | Truncated BFR (one octet Ci) |
| 52 |  | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for a RedCap UE |
| 53 |  | Recommended bit rate query |
| 54 |  | Multiple Entry PHR (four octets Ci) |
| 55 |  | Configured Grant Confirmation |
| 56 |  | Multiple Entry PHR (one octet Ci) |
| 57 |  | Single Entry PHR |
| 58 |  | C-RNTI |
| 59 |  | Short Truncated BSR |
| 60 |  | Long Truncated BSR |
| 61 |  | Short BSR |
| 62 |  | Long BSR |
| 63 |  | Padding |

The reserved bits and/or the reserved LCID values (37-42, 47) can potentially be used for Msg4 HARQ-ACK PUCCH repetition related signaling.

In the MAC subheader for the PUSCH carrying Msg3, there are two reserved bits and seven reserved LCID values that can potentially be used for Msg4 HARQ-ACK PUCCH repetition related signaling.

It can be noted (see Table 2) that some LCID codepoints are used for indicating a CCCH for a RedCap UE. It should be possible to independently indicate CCCH for RedCap and indicate support for/request Msg4 HARQ-ACK PUCCH repetitions. Considering also that there are two sizes of CCCH (48 and 64 bits), four of the seven reserved LCID codepoints would be needed to support the request for Msg4 HARQ-ACK PUCCH repetition either with/without RedCap, and with CCCH size of 48/64 bits with full flexibility.

If LCID codepoints are used to request Msg4 HARQ-ACK PUCCH repetitions, four of the seven reserved LCID codepoints would be needed to also support simultaneous indication of CCCH for a RedCap UE and CCCH size of 48 and 64 bits.

Therefore, we propose to instead use one of the reserved bits in the MAC subheader when requesting Msg4 HARQ-ACK PUCCH repetition.

The UE requests Msg4 HARQ-ACK PUCCH repetition using an R bit in the MAC subheader of Msg3.

### 2.3.4	How gNB indicates the repetition factor to UE

At RAN1#111, the following working assumption was agreed:

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- One or more repetition factors may be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

    - FFS: whether UE requests repetition or indicates repetition capability

  - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

    - FFS: whether UE requests repetition or indicates repetition capability

    - FFS: whether repetition factor is indicated by UE

  - FFS: UE behavior when repetition factor is not configured via SIB

  - FFS: whether one or more UE capabilities are needed for the above is for further discussion

If only one repetition factor is configured via SIB (first sub-bullet in the working assumption from RAN1#111), it is our understanding that there is no dynamic indication in DCI that the network can use to request the UE to perform the number of repetitions configured in SIB. In this case, all UE indicating support for/requesting Msg4 HARQ-ACK PUCCH repetitions will transmit the configured number of repetitions, while (obviously) UE without repetition capability will not. To allocate the appropriate amount of UL resources, the network needs to know the UE capability. Therefore, the UE needs to indicate support for/request repetitions as proposed in section 2.3.3 even if only one repetition factor is configured.

Regardless of whether one or more Msg4 HARQ-ACK PUCCH repetition factors are configured in SIB, the UE indicates support for/requests Msg4 HARQ-ACK PUCCH repetition.

If multiple repetition factors are configured via SIB (second sub-bullet in the working assumption from RAN1#111), there is dynamic indication that the network can use to request the UE to perform repetitions according to one of the number of repetitions configured in SIB. For dynamic indication, the following was agreed at RAN1#112:

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

  - Alt 1-2: New field with one or two bits

- Alt 2: Field in DCI scheduling Msg3 PUSCH

  - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor

  - Note: it is assumed that there is impact on DCI design

- Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH

  - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc.

- Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

Our view on each of the high-level alternatives (Alt 1, Alt 2, Alt 3 and Alt 4) is summarized in the following observations:

Indicating the repetition factor for PUCCH in an existing field in the DCI scheduling the Msg4 PDSCH (i.e., Alt 1 in the agreement from RAN1#112) is the most natural solution since this DCI also schedules the PUCCH that is to be repeated. Also, it has the advantage that the repetition factor can be chosen based on measurements on Msg3 PUSCH and selected as closely as possible before the Msg4 HARQ-ACK is to be transmitted by the UE.

Indicating the repetition factor for PUCCH in an existing field in the DCI scheduling Msg3 (i.e., Alt 2 and Alt 4 in the agreement from RAN1#112) has the disadvantage that the repetition factor cannot be chosen based on measurements on Msg3 PUSCH, and that the independent features Msg3 repetition and Msg4 HARQ-ACK PUCCH repetition are coupled.

Indicating the repetition factor for PUCCH using CRC scrambling of DCI scheduling Msg4 (i.e., Alt 3 in the agreement from RAN1#112) has the disadvantage that it reduces the error correcting capability of the CRC and increases receiver complexity.

For these reasons, we make the following proposal, assuming that PUCCH repetition should be supported for any PUCCH when dedicated PUCCH is not configured:

Indicate the repetition factor for PUCCH in the DCI scheduling the PUCCH (i.e., Alt 1 in the agreement from RAN1#112).

Introduction of a new field in DCI (Alt 1-2 in the agreement) would increase the DCI format length. This requires double decoding by the UE, which should be avoided.

Repurpose an existing DCI field for indicating the repetition factor for PUCCH (i.e., Alt 1-1 in the agreement from RAN1#112).

Regarding which existing DCI field to use, some pros and cons are summarized in Table 3.

Table 3: Pros and cons of repurposing different DCI fields.

| Alt | Description | Pros | Cons |
| --- | --- | --- | --- |
| 1-1a | MCS |  | Limiting to PDSCH MCS selection |
| 1-1b | PRI (repetition factor per PUCCH resource) | Same solution as Rel-17 dynamic PUCCH repetition for dedicated PUCCH. Available in all DCI formats for which HARQ-ACK PUCCH is needed when dedicated PUCCH is not configured |  |
| 1-1c | HARQ process number |  | Reducing number of HARQ processes may cause stalling if the RTT is large. |
| 1-1d | DAI | Good solution for Msg4 HARQ-ACK PUCCH since the bits are reserved in DCI format 1_0 with CRC scrambled by TC-RNTI. | Not a viable solution for other DCI formats since the DAI bits are not reserved in these DCI formats. |
| 1-1e | PDSCH-to-HARQ_feedback timing indicator |  | Constrains UL scheduling |

The preferred solution depends on whether the scope of the work is only Msg4 HARQ-ACK PUCCH, or any PUCCH when dedicated PUCCH is not configured. If the former, our view is that DAI should be used. If the latter, the PRI field should be used.

If more than one repetition factor is configured in SIB, the network indicates in the DCI scheduling Msg4 PUSCH the repetition factor to use (among the configured) using
  – DAI if the scope is only repetition for Msg4 HARQ-ACK PUCCH.
  – PRI (i.e., repetition factor configured per PUCCH resource) if the scope is repetition for any PUCCH when dedicated PUCCH is not configured.

### 2.3.5	Inter-slot frequency hopping

Intra-slot frequency hopping is already supported for common PUCCH. Inter-slot frequency hopping is supported on dedicated PUCCH resources. Inter-slot frequency hopping on common PUCCH should be introduced only if it is justified by performance gains needed to reach the coverage performance target.

The performance of intra-slot and inter-slot frequency hopping are compared in Figure 3. Simulation assumptions are summarized in Table 8 in Appendix A.

Figure 3: Performance of Msg4 HARQ-ACK PUCCH repetitions with different hopping pattern

It can be observed that intra-slot and inter-slot frequency hopping can bring 1.3 dB and 0.8 dB gain separately comparing with no frequency hopping, proving that intra-slot frequency hopping can reap more frequency diversity. Even if inter-slot frequency hopping can bring more configuration flexibility, considering the performance and specification impact, we propose do not support inter-slot frequency hopping for Msg 4 HARQ-ACK PUCCH repetitions.

Inter-slot frequency hopping performs worse than intra-slot frequency hopping for Msg4 HARQ-ACK PUCCH repetitions.

Do not introduce support for inter-slot frequency hopping for Msg4 HARQ-ACK PUCCH repetitions.

### 2.3.6	Capacity of the common PUCCH resources

In current specification, the common PUCCH resources are provided by Table 9.2.1-1 in TS 38.213 [4], copied below.

Table 4 (Table 9.2.1-1 from TR 38.820): PUCCH resource sets before dedicated PUCCH resource configuration.

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

The table contains 16 PUCCH resource sets. In a given PUCCH resource set, there are at most 16 PUCCH resources. UE can determine the PUCCH resource based on the PUCCH resource indicator in DCI 1_0 or DCI 1_1 and the index of the CCE in which the PDCCH scheduling the PUCCH was received. If common PUCCH repetitions is configured in the cell, it will cost many PUCCH resources and other UE may not have adequate PUCCH resource or need to wait long time for transmission of PUCCH. To solve this problem, possible ways are adding new PUCCH resource sets that are only used for PUCCH repetition or expanding the resources in one set. Other solutions are preferred to be discussed.

RAN1 needs to further study common PUCCH resource sets capacity problem.

### 2.3.7	Msg4 HARQ-ACK PUCCH repetition in terrestrial networks

The need for Msg4 HARQ-ACK PUCCH repetitions in terrestrial networks was evaluated during the Rel-17 coverage enhancement study. The results in TR 38.830 [9] indicate that the Msg4 HARQ-ACK PUCCH might be the bottleneck in some FDD scenarios (see e.g. Table 5.1.1.5-2 in TR 38.830), but it was not chosen as a candidate for coverage enhancement in Rel-17 since too few companies simulated its performance. However, since it is now specified in Rel-18, it makes sense to make this enhancement available also in terrestrial networks. Therefore, the solution for Msg4 HARQ-ACK PUCCH repetition should target use in both NTN and TN.

Msg4 HARQ-ACK PUCCH repetition can be useful also in terrestrial networks.

The solution for Msg4 HARQ-ACK PUCCH repetition should target use in both NTN and TN.

## 2.4	PUSCH for VoIP

### 2.4.1	Signalling enhancements

The WID objective for DMRS bundling enhancements for PUSCH was slightly updated at RAN#98-e [1] but still leaves it open if enhancements are necessary:

- To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

At RAN1#112, the following proposal was discussed but not agreed:

Proposal (not agreed)

For NTN-specific PUSCH DMRS bundling, further discuss whether nominal/actual TDW determination defined in R17 can be reused for NTN or not, e.g., the following options.

- Option 1: Actual TDW determination based on timing of TA pre-compensation update causing phase discontinuity and power inconsistency. UE determines the timing.

  - Nominal TDW is configured by gNB as in the existing spec.

  - UE reports information on the timing. FFS details

  - Actual TDW is determined by the reported information and events defined in R17.

    - New event is defined based on TA pre-compensation update. FFS details

- Option 2: Nominal/actual TDW determination based on gNB configuration and/or indication

  - Option 2-1: Determined as in the existing spec. No spec change is assumed.

    - Nominal TDW is configured by gNB as in the existing spec.

      - Note: Based on gNB implementation, nominal TDW length can be configured to current timing drift, derived from satellite ephemeris, to allow UE to update time/frequency pre-compensation sufficiently often.

    - Actual TDW is determined by events defined in R17.

    - Note: no new event is defined for the TA pre-compensation update.

  - Option 2-2: Determined by new gNB configuration/indication.

    - FFS details

      - e.g., TA pre-compensation timing is indicated by gNB

      - e.g., further segmented configuration is provided by gNB

    - FFS: Whether new event is defined based on new gNB configuration/indication.

- FFS: whether to define another new event to decide actual TDW (e.g., antenna switching, update of epoch time, etc.)

When DMRS bundling is used, the UE needs to transmit coherently across bundled UL slots. This means that updates of the TA and frequency offset should be avoided. However, updates due to open-loop TA and Doppler shift pre-compensation are sometimes necessary to maintain timing and frequency of the received signal at gNB within RAN4 accuracy requirements.

A similar problem exists in IoT NTN and was solved in Rel-17 by so called segmented UL transmission. When repetitions are transmitted by the UE, pre-compensation updates are prohibited within segments of consecutive slots, while pre-compensation updates are allowed between the segments. The segment length is configured by RRC signaling by the network. How the network determines the segment length is up to implementation, but it will typically be derived from the drift rate that in turn can be estimated from the satellite ephemeris.

The configuration of Rel-17 PUSCH DMRS bundling for NR NTN allows the network to configure a nominal TDW (time-domain window) during which coherent transmission is required, while TA updates and other operations which cause power consistency and phase continuity not to be maintained are allowed between the TDWs. The length of the nominal TDW for PUSCH is configured by UE-specific RRC signaling using the pusch-TimeDomainWindowLength-r17 IE (see TS 38.331 [7]) and can take any value in the range (2..32) slots (also depending on UE capability). Therefore, the Rel-17 feature DMRS bundling already supports a similar configuration framework as Rel-17 IoT NTN segmented uplink transmissions.

The Rel-17 feature DMRS bundling already supports configuration of transmission segments equivalent to Rel-17 IoT NTN segmented uplink transmission through UE-specific RRC configuration of the nominal TDW length. No enhancement of the configuration is necessary.

The gNB can e.g. determine the max Doppler/timing drift rate in a cell based on serving satellite ephemeris, and/or estimated/predicted timing drift based on the UL signal from the UE, and configure the UE with a nominal TDW such that pre-compensation updates can be paused within the TDW without exceeding requirements on time/frequency accuracy.

The gNB can e.g. determine the max Doppler/timing drift rate in a cell based on serving satellite ephemeris, and/or estimate/predicte timing drift based on the UL signal from the UE, and configure the UE with a nominal TDW such that pre-compensation updates can be paused within the TDW without exceeding requirements on time/frequency accuracy.

Reuse the Rel-17 configuration of nominal TDW without modifications, and do not add new events defining the actual TDW.

To increase the TDW length without creating ISI, advanced UE implementations can be considered that compensate their local clock so that the Doppler shift effect is cancelled. If the oscillator frequency is adjusted to cancel the Doppler shift at the start of the TDW, time drift is initially zero and increases only when the Doppler shift changes during the TDW, which should have a very limited effect. E.g., for LEO 1200, the max one-way Doppler drift rate is 0.09 ppm/s (see table 6.1.1.1-8 of TS 38.821 [8]), and the max (service link) Doppler shift after 20 ms is 2*0.09*0.020 ppm = 0.0036 ppm or 7 Hz at 2 GHz carrier frequency, which is negligible. Alternatively, UE implementations that can adjust the pre-compensation digitally while maintaining coherent transmissions could be considered.

With advanced UE implementations that pre-compensate its local clock to cancel the Doppler shift effect at the start of the TDW, or can update time/frequency pre-compensation while maintaining coherent transmission, the need for TDW length limitation due to pre-compensation updates can be avoided.

Consider defining a UE capability to distinguish between different UE implementations, such as:
1) UE that can pre-compensate its local clock to cancel the Doppler shift effect at the start of the TDW
2) UE that can update time and frequency compensation inside the TDW while maintaining coherent transmission
3) UE that can only update time and frequency pre-compensation at TDW borders

### 2.4.2	Time drift and phase drift

At RAN1#112, the following observation was agreed:

Observation

For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

- Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed.

  - FFS: whether/how to consider the initial timing error at the beginning

  - FFS: TA pre-compensation update is assumed

- Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed.

- FFS: impact of phase difference limit

The observation that the timing error limit can be satisfied with at most 13 slots assumes that there is no initial timing error at the start of the TDW. But not all of the timing error budget can be used for managing the timing drift due to paused updating of the open-loop TA, since there are also other sources of timing error (e.g., UE position inaccuracy, satellite ephemeris/propagation inaccuracy, TAC granularity, TA adjustment inaccuracy, DL synchronization inaccuracy and other UE-internal sources of error). In other words, there is likely a timing error already at the start of a transmission of DMRS bundled slots.

The timing accuracy requirement for NTN UE is 29Ts (assuming 15 kHz SCS) but not all of this timing error budget can be used for managing the timing drift due to paused updating of the open-loop TA within the TDW.

RAN1 to discuss assumption for initial timing error at the start of the TDW.

The time drift also gives rise to phase rotation that is proportional to the sub-carrier frequency relative to the center frequency. The phase drift is caused by the time drift of the received signal in combination with a fixed receiver window.

The following observation was proposed by the FL at RAN1#112:

Proposed observation (not agreed)

For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

- Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots for carrier bandwidth 5 MHz at the edge of the carrier bandwidth, if TA pre-compensation update, phase pre-compensation, and RX phase post-compensation are not assumed, and if 70.5 (us/s) timing drift rate is assumed.

  - FFS: whether to assume phase pre-compensation at UE and/or RX phase post-compensation at gNB

The proposal was not agreed but still, phase drift is an important topic that needs further discussion.

According to the phase continuity requirement in 38.101-1 [2]:

- Phase difference between any slot p-1 and slot p must be ≤ 25° (for supported DMRS bundling configurations ≤8 slots)

- Phase difference between slot 0 and any slot p must be ≤ 30° (for supported DMRS bundling configurations of 16 slots)

- The difference is measured per subcarrier and the requirement applies to the maximum phase difference among all subcarriers

A timing drift of 70.5 ppm causes a phase rotation Δφ per 1 ms slot for subcarrier n (where n is the offset from DC subcarrier) as follows:

Δφ = 360 (°) × timing drift × slot length (s) × SCS (Hz) × n = 360×70.5e-6×1e-3×15e3×n = ωn, where ω=0.38°

Consequently, the 25° difference between consecutive slots is exceeded if n>65, as illustrated in Figure 4.

Figure 4: PRBs in a 5 MHz bandwidth where the phase difference between consecutive slots exceeds 25° due to time drift.

Due to time drift, a phase difference between consecutive slots of more than 25° is experienced in a large part of a 5 MHz bandwidth.

However, if a PRB allocation near the carrier edge is narrow, the phase drift of the different subcarriers in the allocation will be very similar, and appear almost like a frequency offset of the whole signal from the receiver point of view.

If the PRB allocation of a UE is narrow, the phase drift of the different subcarriers will be very similar and appear almost like a frequency offset of the whole signal from the receiver point of view.

Therefore, CFO compensation in gNB might be able to compensate for most of the phase drift, ideally leaving only a small residual phase drift per subcarrier. This is illustrated in Figure 5, where the UE is allocated a single PRB at the edge of a 5 MHz band. The maximum phase difference between consecutive slots in the allocation is 57°. But after the ideal CFO, the maximum phase difference between consecutive slots only 2.1°.

Figure 5: Phase drift before and after ideal CFO compensation in a single PRB allocation.

In section 2.4.4.4, simulation results are presented that confirm this. The results show that DMRS bundling performance with a simple (non-ideal) CFO compensation is almost the same at the edge and near the center of a 5 MHz carrier, despite the large phase drift at the edge. Also, it is shown that the CFO compensation can handle the phase drift without performance degradation for narrow PRB allocation while there is a noticeable degradation for wider allocations.

RAN1#111 concluded the following regarding phase variation due to constant frequency error:

Conclusion

For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1.

Since phase drift due to timing drift appears similar to a frequency offset for a narrow PRB allocation, it can be discussed if the conclusion above applies also for phase drift in some cases. Therefore, it is unclear if the phase continuity requirement in 38.101-1 should be modified for NTN or not, e.g., to allow for a linear phase drift that can be compensated by CFO compensation. This is up to RAN4 to decide.

The impact on time drift and phase drift on PUSCH DMRS bundling performance for NTN needs further study.

Send an LS to RAN4 to ask whether the phase continuity requirement in clause 6.4.2.5 38.101-1 should be modified for NTN UE e.g. to allow for a linear phase drift that can be compensated by CFO compensation.

### 2.4.3	Post-compensation in gNB

To handle the time drift within the TDW, post-compensation in gNB has been suggested. It should be noted that in the gNB receiver, the FFT may be applied to the whole UL signal with an RX window that cannot be adjusted individually per UE. The UEs in a spotbeam will not necessarily have aligned TDWs and will therefore autonomously update their TAs at different times. Therefore, each UE must meet the transmit timing accuracy requirements in TS 38.133 [3] (e.g. 29Ts for 15 kHz SCS) to avoid creating ISI. The TDW length limit cannot be increased by post-compensation of the RX timing.

The TDW length cannot be increased by post-compensation of the RX timing in gNB.

The phase drift can be post-compensated to enhance channel estimation within a TDW. Since the exact UE position is not known by the gNB, the post-compensation can be based on the drift at a reference point in the beam. Phase drift post-compensation is a receiver enhancement in gNB that is not expected to have specification impact.

### 2.4.4	Performance evaluation

Simulations of DMRS bundling have been performed. Simulation assumptions are according to agreements from RAN1#109-e. Specific simulation assumptions are listed in Table 10 in appendix A. Time drift is modeled in the simulation as described in section 2.4.2.

#### 2.4.4.1	Ephemeris-based phase drift post-compensation

To evaluate DMRS bundling performance with phase drift post-compensation, a beam at the edge of the satellite footprint (with minimum elevation angle 30°) is considered, as shown in Figure 6. A cell diameter of 1000 km is assumed, which is the worst-case according to TR 38.821 [8]. The UE is located at the outer edge of the beam.

Figure 6: A spotbeam at the edge of the satellite footprint.

The drift rate in the beam ranges from 35 ppm to 71 ppm. The numbers have been derived from Figure 7 that shows simulated (one-way, per link) drift rate for the whole satellite footprint versus offset from nadir. When evaluating phase drift post-compensation, the middle-point in this range, i.e., 53 ppm, has been assumed in the post-compensation.

Figure 7: One-way satellite-to-UE Doppler shift versus offset from nadir for a LEO 1200 satellite.

Simulation results with 20 repetitions and DMRS bundling (without voice frame aggregation) is shown in Figure 8. The 2% BLER performance is summarized in Table 5. Simulation assumptions are according to Table 10 in Appendix A. It can be seen that timing drift degrades BLER performance up to 0.5 dB, but also that phase drift post-compensation in gNB reduces the loss to around 0.1 dB.

Figure 8: PUSCH BLER for VoIP (AMR 4.75) with 20 repetitions, different TDW lengths, with/without phase drift post-compensation.

Table 5: PUSCH BLER performance for VoIP (AMR 4.75) with 20 repetitions and DMRS bundling.

| Timing drift | Phase drift post-compensation | Simulated SNR at 2% BLER for different TDW lengths |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TDW=20 | TDW=10 | TDW=7 | TDW=5 | TDW=4 |
| No | No | -4.2 | -3.7 | -3.4 | -3.1 | -2.8 |
| Yes | No | -3.7 | -3.5 | -3.3 | -3.0 | -2.8 |
| Yes | Yes | -4.1 | -3.6 | -3.3 | -3.0 | -2.8 |

However, as observed in RAN1#112, the TDW length might be limited to 13 slots or less to avoid excessive timing errors. As seen in Table 5, when the TDW is less than 13, the loss due to timing drift is small, and the gain due to phase drift post-compensation is also small.

For a given TDW length, post-compensation of the sub-carrier phase drift in gNB can improve BLER performance for PUSCH. However, for the TDW lengths that are possible without exceeding RAN4 timing accuracy requirements, phase drift post-compensation has only limited gain in the evaluated scenario.

Post-compensation of phase drift can be implemented in gNB without specification impact.

#### 2.4.4.2	TX antenna switching

TX antenna switching has been suggested e.g. in [11]. Below we evaluate performance of 20 PUSCH repetitions with DMRS bundling with TDW lengths of 10 and 5 slots, and one antenna switch after 10 slots. Uncorrelated channels have been assumed for the two TX antennas. Timing drift is modelled and phase drift post-compensation is used. Simulation assumptions are according to Table 10 in Appendix A. The results are shown in Figure 9 and summarized in Table 6.

Figure 9: PUSCH BLER for VoIP (AMR 4.75) with 20 repetitions, different TDW lengths, with/without TX antenna switching, with phase drift post-compensation.

Table 6: PUSCH BLER performance for VoIP (AMR 4.75) with 20 repetitions, DMRS bundling and antenna switching.

| Antenna switching | Simulated SNR at 2% BLER for different TDW lengths |  |
| --- | --- | --- |
|  | TDW=10 | TDW=5 |
| No | -3.6 | -3.0 |
| Yes | -5.9 | -5.4 |

The antenna switching gives a gain of 2.3-2.4 dB in the evaluated scenarios.

For PUSCH DMRS bundling, TX antenna switching between the TDWs can provide a substantial gain.

Antenna switching can be implemented in the UE without specification impact.

#### 2.4.4.3	Voice frame aggregation

Voice frame aggregation has been suggested e.g. in [11]. Here we evaluate performance of 2x voice frame aggregation at application layer. Without voice frame aggregation, one packet is assumed to consist of 184 bits, of which 95 are payload bits. With voice frame aggregation, two voice frames are bundled at application layer, resulting in a TBS of 278 bits every 40 ms. The number of PUSCH repetitions is 32 and DMRS bundling with TDW lengths of 16 and 8 slots are evaluated. An antenna switch is performed after 16 slots. Uncorrelated channels have been assumed for the two TX antennas. Timing drift is modelled and phase drift post-compensation is used. Simulation assumptions are according to Table 10 in Appendix A. The results are shown in Figure 10 and summarized in Table 7.

Figure 10: PUSCH BLER for VoIP (AMR 4.75) with 2x voice frame aggregation, 32 repetitions, different TDW lengths, with/without TX antenna switching, with phase drift post-compensation.

Table 7: PUSCH BLER performance for VoIP (AMR 4.75) with voice frame aggregation, 32 repetitions, DMRS bundling and antenna switching.

| Antenna switching | Simulated SNR at 2% BLER for different TDW lengths |  |
| --- | --- | --- |
|  | TDW=16 | TDW=8 |
| No | -4.4 | -3.8 |
| Yes | -6.4 | -6.0 |

Comparing Table 7 with Table 6, it can be seen that performance is improved with 2x voice frame aggregation. For the case of TDW=16, the RAN4 transmit timing accuracy requirements may be violated due to timing drift but the TDW length of 8 slots should be possible. The gain in SNR is despite that fewer PUSCH slots are transmitted per voice frame (16 compared to 20). Still, there is a gap to the target SNR (-8.1 dB) of >2 dB.

Application level voice frame aggregation can improve the coverage of PUSCH for VoIP due to reduced overhead and increased number of repetitions.

Application level voice frame aggregation has no RAN specification impact.

Several performance enhancements without specification impact have been evaluated for PUSCH DMRS bundling and were found to give relevant gains. Still, a gap to the target CNR for PUSCH VoIP of >2 dB is observed.

#### 2.4.4.4	Impact of time drift for narrow and wide PRB allocations

The impact of timing drift on the NTN-specific PUSCH DMRS bundling is suggested to be evaluated. Here we evaluated the PUSCH performance with voice frame aggregation, two voice frames are bundled at application layer, resulting in a TBS of 278 bits every 40 ms. The number of PUSCH repetitions is 32 and DMRS bundling with TDW lengths of 8 slots is applied. No ephemeris-based phase drift post-compensation is applied. For more detailed parameter settings, refer to Table 11 with voice frame aggregation in Appendix A.

Results in Figure 11 show that DMRS bundling performance with a simple (non-ideal) CFO compensation is almost the same at the edge and near the center of a 5 MHz carrier at 2% SNR, despite the large phase drift at the carrier edge.

Figure 11: PUSCH BLER for VoIP for 2 PRBs allocated at carrier edge and carrier center with timing drift 71ppm.

DMRS bundling performance with CFO compensation is almost the same at the edge and at the center of a 5 MHz carrier, despite the large phase drift at the carrier edge.

With the same parameter settings as Figure 11, we evaluate how the phase drift impacts performance for different allocation bandwidths. Figure 12 shows that the timing drift impact is neglectable for narrow, i.e. 1 or 2 PRBs allocation. But for wider, i.e. 4 and 8 PRBs allocation, the timing drift can cause ~0.2 dB and ~0.5 dB loss at 2% BLER.

Figure 12: PUSCH BLER for VoIP with narrow and wide PRB allocations with/without 71ppm timing drift

Timing drift gives almost no degradation on DMRS bundling performance with CFO compensation for narrow (1-2) PRB allocations, but larger degradation for wider (4-8) PRB allocations.

## 2.5	Msg3 repetition for CFRA

At RAN2#120, the following agreement was made:

From RAN2 perspective we don’t consider msg3 repetition enhancements in R18 NR NTN (apart from msg3 for CFRA, if decided by RAN1)

The need for Msg3 repetitions for CFRA (contention-free random access) has not been discussed in RAN1 but deserves some attention based on the agreement above. During the study phase, RAN1 evaluated the performance of Rel-17 Msg3 PUSCH repetitions and concluded that there is no need for further enhancements. In this study, RAN1 assumed that up to 16 repetitions can be used. The margin with 16 repetitions reported by different companies was in the order of 0.7 to 4.4 dB. However, RAN1 did not consider that Msg3 repetitions are not supported in case of CFRA. Without repetitions, the performance target is not met. Therefore, the conclusion of RAN1 only applies for the case of CBRA (contention-based random access).

There is a coverage gap for Msg3 PUSCH for the case of CFRA since repetitions are not supported in this case.

Support for Msg3 PUSCH repetitions for the CFRA case is not expected to have any RAN1 specification impact. To close the coverage gap, only signalling support for the case of CFRA in RAN2 specifications needs to be added.

Therefore, we propose the following:

Conclude that there is a coverage gap for Msg3 PUSCH in case of CFRA that can be solved by Msg3 PUSCH repetition.

Send an LS to RAN2 informing them of the coverage gap for Msg3 in case of CFRA and recommend them to specify signalling support for Msg3 repetitions in this case.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	If dedicated PUCCH resources are configured in Msg4, PUCCH repetition can be configured on those PUCCH resources and there is no need for further PUCCH repetition on common PUCCH.

Observation 2	RRC configuration of dedicated PUCCH resources in Msg4 is up to network implementation.

Observation 3	During initial access from RRC_IDLE state, the network does not know the UE identity, and therefore not the UE capabilities, when Msg4 is sent, and may decide to wait with configuring dedicated PUCCH until later.

Observation 4	The two options how to instruct the UE to indicate its capability (not configuring the RSRP threshold or configuring the RSRP threshold with value X) are equivalent from a functional point of view. Not configuring the RSRP threshold saves bits in SIB.

Observation 5	The SNR limit for when repetitions are needed is different for Msg4 HARQ-ACK PUCCH and Msg3 PUSCH.

Observation 6	Rel-17 Msg3 PUSCH repetition and Rel-18 Msg4 HARQ-ACK PUCCH repetition are optional features both for the network and the UE. Support for one of the features should not require support of the other.

Observation 7	PRACH resource partitioning is used for Msg3 PUSCH repetition, RedCap, slicing, SDT and other features that are potentially useful in NTN. It is also considered for Rel-18 PRACH repetition.

Observation 8	Excessive partitioning of the PRACH resources may cause capacity problems for PRACH and increases complexity.

Observation 9	Knowledge of the UE’s need for Msg4 HARQ-ACK PUCCH repetition is not needed in the network until after Msg3 is received.

Observation 10	For request for Msg4 HARQ-ACK PUCCH repetition through physical layer signaling in Msg3 PUSCH, the proposed solutions require blind detection, which has negative impact on gNB complexity.

Observation 11	In the MAC subheader for the PUSCH carrying Msg3, there are two reserved bits and seven reserved LCID values that can potentially be used for Msg4 HARQ-ACK PUCCH repetition related signaling.

Observation 12	If LCID codepoints are used to request Msg4 HARQ-ACK PUCCH repetitions, four of the seven reserved LCID codepoints would be needed to also support simultaneous indication of CCCH for a RedCap UE and CCCH size of 48 and 64 bits.

Observation 13	Indicating the repetition factor for PUCCH in an existing field in the DCI scheduling the Msg4 PDSCH (i.e., Alt 1 in the agreement from RAN1#112) is the most natural solution since this DCI also schedules the PUCCH that is to be repeated. Also, it has the advantage that the repetition factor can be chosen based on measurements on Msg3 PUSCH and selected as closely as possible before the Msg4 HARQ-ACK is to be transmitted by the UE.

Observation 14	Indicating the repetition factor for PUCCH in an existing field in the DCI scheduling Msg3 (i.e., Alt 2 and Alt 4 in the agreement from RAN1#112) has the disadvantage that the repetition factor cannot be chosen based on measurements on Msg3 PUSCH, and that the independent features Msg3 repetition and Msg4 HARQ-ACK PUCCH repetition are coupled.

Observation 15	Indicating the repetition factor for PUCCH using CRC scrambling of DCI scheduling Msg4 (i.e., Alt 3 in the agreement from RAN1#112) has the disadvantage that it reduces the error correcting capability of the CRC and increases receiver complexity.

Observation 16	Inter-slot frequency hopping performs worse than intra-slot frequency hopping for Msg4 HARQ-ACK PUCCH repetitions.

Observation 17	Msg4 HARQ-ACK PUCCH repetition can be useful also in terrestrial networks.

Observation 18	The Rel-17 feature DMRS bundling already supports configuration of transmission segments equivalent to Rel-17 IoT NTN segmented uplink transmission through UE-specific RRC configuration of the nominal TDW length. No enhancement of the configuration is necessary.

Observation 19	The gNB can e.g. determine the max Doppler/timing drift rate in a cell based on serving satellite ephemeris, and/or estimate/predicte timing drift based on the UL signal from the UE, and configure the UE with a nominal TDW such that pre-compensation updates can be paused within the TDW without exceeding requirements on time/frequency accuracy.

Observation 20	With advanced UE implementations that pre-compensate its local clock to cancel the Doppler shift effect at the start of the TDW, or can update time/frequency pre-compensation while maintaining coherent transmission, the need for TDW length limitation due to pre-compensation updates can be avoided.

Observation 21	The timing accuracy requirement for NTN UE is 29Ts (assuming 15 kHz SCS) but not all of this timing error budget can be used for managing the timing drift due to paused updating of the open-loop TA within the TDW.

Observation 22	Due to time drift, a phase difference between consecutive slots of more than 25° is experienced in a large part of a 5 MHz bandwidth.

Observation 23	If the PRB allocation of a UE is narrow, the phase drift of the different subcarriers will be very similar and appear almost like a frequency offset of the whole signal from the receiver point of view.

Observation 24	The impact on time drift and phase drift on PUSCH DMRS bundling performance for NTN needs further study.

Observation 25	The TDW length cannot be increased by post-compensation of the RX timing in gNB.

Observation 26	For a given TDW length, post-compensation of the sub-carrier phase drift in gNB can improve BLER performance for PUSCH. However, for the TDW lengths that are possible without exceeding RAN4 timing accuracy requirements, phase drift post-compensation has only limited gain in the evaluated scenario.

Observation 27	Post-compensation of phase drift can be implemented in gNB without specification impact.

Observation 28	For PUSCH DMRS bundling, TX antenna switching between the TDWs can provide a substantial gain.

Observation 29	Antenna switching can be implemented in the UE without specification impact.

Observation 30	Application level voice frame aggregation can improve the coverage of PUSCH for VoIP due to reduced overhead and increased number of repetitions.

Observation 31	Application level voice frame aggregation has no RAN specification impact.

Observation 32	Several performance enhancements without specification impact have been evaluated for PUSCH DMRS bundling and were found to give relevant gains. Still, a gap to the target CNR for PUSCH VoIP of >2 dB is observed.

Observation 33	DMRS bundling performance with CFO compensation is almost the same at the edge and at the center of a 5 MHz carrier, despite the large phase drift at the carrier edge.

Observation 34	Timing drift gives almost no degradation on DMRS bundling performance with CFO compensation for narrow (1-2) PRB allocations, but larger degradation for wider (4-8) PRB allocations.

Observation 35	There is a coverage gap for Msg3 PUSCH for the case of CFRA since repetitions are not supported in this case.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The solutions to support PUCCH repetition for Msg4 HARQ-ACK are applicable for any PUCCH when dedicated PUCCH is not configured.

Proposal 2	A UE that supports the Rel-18 feature for PUCCH repetition when dedicated PUCCH is not configured shall also support the Rel-17 feature dynamic PUCCH repetition (slotBasedDynamicPUCCH-Rep-r17).

Proposal 3	Confirm the working assumption from RAN1#112 (“For PUCCH repetition for Msg4 HARQ-ACK, a RSRP threshold can be configured via SIB…”).

Proposal 4	How to instruct the UE to always indicate its capability (not configuring the RSRP threshold or configuring the RSRP threshold with value X) is an RRC signaling decision that should be decided by RAN2.

Proposal 5	Define a new RSRP threshold for triggering request for Msg4 HARQ-ACK PUCCH repetitions.

Proposal 6	Higher layer signaling in Msg3 PUSCH (Option B) is used to request Msg4 HARQ-ACK PUCCH repetition.

Proposal 7	The UE requests Msg4 HARQ-ACK PUCCH repetition using an R bit in the MAC subheader of Msg3.

Proposal 8	Regardless of whether one or more Msg4 HARQ-ACK PUCCH repetition factors are configured in SIB, the UE indicates support for/requests Msg4 HARQ-ACK PUCCH repetition.

Proposal 9	Indicate the repetition factor for PUCCH in the DCI scheduling the PUCCH (i.e., Alt 1 in the agreement from RAN1#112).

Proposal 10	Repurpose an existing DCI field for indicating the repetition factor for PUCCH (i.e., Alt 1-1 in the agreement from RAN1#112).

Proposal 11	If more than one repetition factor is configured in SIB, the network indicates in the DCI scheduling Msg4 PUSCH the repetition factor to use (among the configured) using   – DAI if the scope is only repetition for Msg4 HARQ-ACK PUCCH.   – PRI (i.e., repetition factor configured per PUCCH resource) if the scope is repetition for any PUCCH when dedicated PUCCH is not configured.

Proposal 12	Do not introduce support for inter-slot frequency hopping for Msg4 HARQ-ACK PUCCH repetitions.

Proposal 13	RAN1 needs to further study common PUCCH resource sets capacity problem.

Proposal 14	The solution for Msg4 HARQ-ACK PUCCH repetition should target use in both NTN and TN.

Proposal 15	Reuse the Rel-17 configuration of nominal TDW without modifications, and do not add new events defining the actual TDW.

Proposal 16	Consider defining a UE capability to distinguish between different UE implementations, such as: 1) UE that can pre-compensate its local clock to cancel the Doppler shift effect at the start of the TDW 2) UE that can update time and frequency compensation inside the TDW while maintaining coherent transmission 3) UE that can only update time and frequency pre-compensation at TDW borders

Proposal 17	RAN1 to discuss assumption for initial timing error at the start of the TDW.

Proposal 18	Send an LS to RAN4 to ask whether the phase continuity requirement in clause 6.4.2.5 38.101-1 should be modified for NTN UE e.g. to allow for a linear phase drift that can be compensated by CFO compensation.

Proposal 19	Conclude that there is a coverage gap for Msg3 PUSCH in case of CFRA that can be solved by Msg3 PUSCH repetition.

Proposal 20	Send an LS to RAN2 informing them of the coverage gap for Msg3 in case of CFRA and recommend them to specify signalling support for Msg3 repetitions in this case.

# References

RP-230809, "NR NTN (Non-Terrestrial Networks) enhancements", Thales, RAN#99, March 2023

3GPP TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

3GPP TS 38.133, "NR; Requirements for support of radio resource management"

3GPP TS 38.213, "NR; Physical layer procedures for control"

3GPP TS 38.214, "NR; Physical layer procedures for data"

3GPP TS 38.321, "NR; Medium Access Control (MAC) protocol specification"

3GPP TS 38.331, "NR; Radio Resource Control (RRC) protocol specification"

3GPP TR 38.821, "Solutions for NR to support Non-Terrestrial Networks (NTN)"

3GPP TR 38.830, "Study on NR coverage enhancements"

R1-2212865, "Summary #5 on 9.11.1 Coverage enhancement for NR NTN", Moderator (DOCOMO), RAN1#111, November 2022

R1-2212136, “Coverage enhancements for NR NTN”, Qualcomm, RAN1#111, November 2022

# Appendix A Simulation assumptions

Table 8: Assumptions for PUCCH simulations.

| Parameter | Value |
| --- | --- |
| Antenna configuration | 1T2R |
| Modulation | BPSK |
| Channel model | NTN-TDL-C Rural |
| Elevation Angle | 30° |
| Frequency hopping pattern / FH offset | {No, intra-slot, inter-slot FH per slot} / 50 PRBs |
| Channel bandwidth | 1 PRB |
| PUCCH format | format 1 |
| Payload | 1 bit |
| PUCCH duration | 14 OS |
| Repetitions | No reps / 2 reps |
| Performance metric | 1% ACK missed and 0.1% NACK to ACK at 1% DTX to ACK rate |

Table 9: Assumptions for PUSCH of Msg 3 simulations.

| Parameter | Value |
| --- | --- |
| Antenna configuration | 1T2R |
| Channel model | NTN-TDL-C Rural / TDL-C |
| Elevation Angle | 30° |
| Allocation PRBs | 2 PRBs |
| TBS | 56 bits |
| MCS index | 0 |
| Waveform | DFT-s-OFDM |
| DMRS setting | 1+2 DMRS |
| Performance metric | 10% iBLER |

Table 10: Assumptions for PUSCH simulations of VoIP.

| Parameter | Value |
| --- | --- |
| Antenna configuration | 1T1R with/without TX antenna switching |
| Modulation order | QPSK |
| Channel Model | NTN-TDL-C Rural |
| Elevation Angle | 30° |
| Frequency hopping | No |
| Channel bandwidth | In section 2.4.4.4: 1, 2, 4, 8 PRBs In other sections: 1 PRB |
| DMRS configuration | Type I, 2 DMRS symbols, no multiplexing with data |
| PUSCH duration | 14 OS |
| Repetitions | Without voice frame aggregation: 20 slots w/ type A repetition With voice frame aggregation: 32 slots w/ type A repetition |
| TDW length | Without voice frame aggregation: 4, 5, 7, 10, 20 With voice frame aggregation: 8, 16 |
| HARQ configuration | Feedback disabled |
| TBS/MCS | Without voice frame aggregation: MCS-9 (TBS=184) from Table 6.1.4.1-1 of 38.214 [5]. With voice frame aggregation: TBS=278 with QPSK |
| Carrier frequency offset (CFO) | 0.1 ppm |
| CFO compensation | Inter-slot correlation-based CFO compensation |
| Timing drift model | Initial timing error at start of TDW: 8Ts Timing drift during TDW: 71 ppm (2.18Ts/slot) |
| Phase drift post-compensation | - No post-compensation - 53 ppm post-compensation (see section 2.4.4.1) |
| Performance metric | 2% BLER |
| Target CNR | -8.1 dB |