3GPP TSG RAN WG1 #112bis		                                       R1-2303025
eMeeting, Apr 17 – Apr 26, 2023

Agenda Item:	9.7.2

Source:	InterDigital Inc.

Title:	Discussion on enhancements on cell DTX/DRX mechanism

Document for:	Discussion and Decision

# Introduction

In RAN1#112, the following agreements were made on cell DTX/DRX [1]:

Agreement - RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX - Further discussions on other aspects are not precluded Agreement At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS. - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded

This contribution discusses the techniques and the associated impacts when supporting the cell DTX/DRX mechanism.

# Discussion

The Cell DTX/DRX mechanism allows the gNB to operate in reduced downlink transmission and and uplink reception activity by informing the UE on whether/when the cell stays inactive. When receiving a cell activity indication informing on the reduced cell activity, some adjustments can be applied at the UE (e.g. on which siganals and channels can be transmitted or received) in alignment with the Cell DTX/DRX pattern.

During the RAN1#112 meeting [1], a number of issues with potential RAN1 impacts were discussed. In the following, we discuss these aspects:

    - UE behavior and procedures when cell DTX/DRX is in operation

    - Configuration and indication of cell DTX/DRX information to UE

## UE behavior and procedures when cell DTX/DRX is in operation

During any of the non-active periods, the gNB can reduce/omit some time occasions for the transmisison/reception of common signals and UE-specific signals and resources to achieve network and UE energy savings. To avoid any impact to legacy UEs, the common signals and channels (e.g. SSBs, SIBs and paging channels) that are used by legacy UEs are assumed to be transmitted as usual during the cell DTX non-active occasions. In RAN1#112 [1], several signals and channels that are not expected to be received or transmitted by the UE during the non-active periods of cell DTX/DRX were discussed.

From the candidate list of signals/channels in DL and UL identified during RAN1#112, the following signals/channels are also discussed in RAN2 in the post-meeting email discussion [2].

- DL

  - PDCCH scrambled with UE-specific RNTI (e.g. containing DCI with dynamic grants/ resource assignments for PUSCH/PDSCH)

  - SPS-PDSCH

  - Dynamic PDSCH assignments

- UL

  - SR

  - CG-PUSCH

  - DG-PUSCH

The corresponding UE behavior for receiving/transmitting the above signals/channels during non-active periods of cell DTX/DRX can be left to RAN2 discussion, as such channels are either generated by the MAC layer or controlled by C-DRX. RAN1 can focus on the following signals/channels and the correspdoning UE behavior during the non-active periods of cell DTX/DRX:

- DL

  - Periodic/Semi-persistent CSI-RS (including TRS)

  - PRS

- UL

  - PUCCH with HARQ feedback for SPS-PDSCH

  - PUCCH with HARQ feedback for DG-PDSCH

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

Reception of DL signals and channels during non-active periods of cell DTX

For DL reference signals such as periodic/semi-persistent CSI-RS (e.g. PTRS and TRS), it is reasonable to assume that for maximizing energy savings, the network does not transmit the RS and hence, the UE does not perform measurements on CSI-RS during non-active periods of cell DTX. Since the CSI-RS for RRM, RLM and BFD can be UE-specific and are not shared with legacy UEs, the UE is not expected to perform measurements during non-active periods. The UE and network can adapt the CSI-RS measurement occasions to the active periods of cell DTX, for example, based on cell specific signalling of cell DTX mode.

In the case when the non-active duration of cell DTX is long, there is a possibility for the UE measurement quality of CSI-RS (e.g., for RLM, RRM) could go below RAN4 requirements. To address this, some relaxation to the measurement requirement can be considered when the cell DTX duration is longer than a threshold. For example, two different measurement requirements can be configured and the set of requirements to apply can depend on parameters such as periodicity and non-active duration of cell DTX.

Proposal 1: UE is not expected to measure periodic/semi-persistent CSI-RS (including PTRS, TRS, BFD, and RLM RS) during non-active periods of cell DTX

The PRS resource configuration can be common for multiple UEs and is provided to the UE by LMF based on exchange of control signaling with one or multiple cells. For network energy savings, a similar assumption as that of periodic/semi-persistent CSI-RS can be made for periodic PRS. In this case, the network is not expected to transmit and the UEs are not expected to measure PRS during the non-active periods of cell DTX. For minimizing any adverse impact to positioning accuracy due to not performing PRS measurements during cell DTX non-active periods, certain relaxation to the PRS measurements and accuracy requirement can be considered (e.g. with two different sets of requirements) for scenarios where the cell DTX duration which may be longer than a threshold.

Proposal 2: UE is not expected to measure PRS during non-active periods of cell DTX

One discussion in RAN2 is whether the UE is expected to monitor PDCCH during Cell DTX non-active times (e.g. during the C-DRX Active Time when C-DRX retransmission timers are running), in order to complete pending HARQ transmissions and retransmissions. While monitoring PDCCH outside of the Cell DTX active period for dynamic assignments may not be necessary, i.e. to maximimize network and UE power savings, it can be necessary for retransmissions. Delaying retransmissions to only occur during the active time can cause significant delays to complete the transmission of a single TB, as the Cell DTX/DRX cycles can be quite long. It’s therefore important to complete retransmissions and also provide feedback for pending TBs as soon as possible in order to maximize energy saving while not affecting UE’s QoS significaly.

Observation 1: To maximize network and UE power savings, UE should not monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active periods, even if the UE is in C-DRX Active time. Such can be decided by RAN2.

Observation 2: To maintain HARQ retransmission timely, UE should monitor PDCCH for dynamic grants/assignments for retransmissions during the UE’s C-DRX Active time per legacy behaviour, even during the Cell DTX non-active period. Such can be decided by RAN2.

Transmisison of UL signals and channels during non-active periods of cell DRX

For UL signals/channels, since CSI-RS is not expected to be transmitted during the non-active periods, the UE is not expected to transmit the periodic/semi-persistent CSI reports when the reporting occasions overlap with the non-active periods of cell DRX. There is no point in transmitting CSI reports if the NW is not blind decoding on PUCCH resources during the cell DRX non-active periods. If the UE is configured to perform CSI-RS measurements during DTX active periods, the UE may transmit the CSI report using the PUCCH resource configured for the current or next cell DRX active period. Although this may have some impact on the measurement accuracy, it can be up to the network on how to strike a balance between energy savings, measurement accuracy and channel state reporting frequency, for example, by aligning the cell DTX/DRX pattern with the CSI-RS measurement and reporting occasions.

Proposal 3: UE is not expected to transmit periodic/semi-persistent CSI-RS reports during non-active periods of cell DRX

Periodic/semi-persistent SRS resources can be dedicated and configured on a per-UE basis. The network can configure the SRS resources and the peioridicty of SRS such that they do not overlap with non-active periods of cell DRX. In this regard, it makes sense to assume that in the UL the UE is not expected to transmit periodic/semi-persistent SRS during non-active periods of cell DRX for similar reasons why CSI-RS is not expected to be transmitted in DL during cell DTX non-active periods. There is no point in transmitting SRS if the NW is not blind decoding associated resources during the DRX non-active period. This is also aligned with the legacy case where the UE is not expected to transmit SRS in the non-active periods when configured with C-DRX.

Proposal 4: UE is not expected to transmit periodic/semi-persistent SRS during non-active periods of cell DRX

In the case of dynamic PDSCH assignments, the UE is indicated in DCI the resource assignment for the PDSCH along with the PUCCH resource and K1 timing offset (PDSCH-to-HARQ-timing-indicator) for transmitting the HARQ feedback. Since the network can decide on the timing for sending the DCI and the K1 value, it is reasonable to assume that if DCI is received containing the PUCCH resource and K1 indication, the UE can transmit the HARQ feedback, even if the PUCCH overlaps with non-active periods of cell DRX. Clearly the network can avoid any HARQ feedback from UE during the non-active period of cell DRX by indicating a K1 value such that the PUCCH occasion does not overlap in the non-active period or overlaps in the next active period of cell DRX.

Proposal 5: UE transmits HARQ feedback for Dynamic PDSCH assignments if the PUCCH resource is provided in DCI (per legacy), even when the PUCCH overlaps with non-active period of cell DRX

For SPS-PDSCH, the gNB can send in the DCI the indication for activating the DL SPS configuration along with the K1 value indicating the timing offset for the PUCCH resource carrying the HARQ feedback for SPS-PDSCH. Unlike for dynamic PDSCH assignments, for SPS the same K1 value is used for all SPS-PDSCH occasions for the duration the DL SPS configuration is active. In legacy, if the PUCCH indicated by the K1 value overlaps in a DL slot in TDD configuration, the UE cancels the PUCCH. In NES scenario, if the PUCCH indicated by K1 overlaps with the non-active period of cell DRX, cancelling the PUCCH and not providing HARQ feedback in time can impact QoS of the DL data, and can also result in resource inefficiency since the network may retransmit the same PDSCH even when received correctly by the UE. Similar to DG-PDSCH such issue can be avoided by the gNB by indicating a K1 value for the SPS-PDSCH occasions that does not overlap with the non-active periods of cell DRX. On the other hand, if the gNB provides in DCI the K1 value for PUCCH that may overlap with non-active periods of cell DRX, the UE can assume the PUCCH to be valid and transmit the HARQ feedback for SPS-PDSCH, per legacy procedure assumptions.

Proposal 6: UE transmits HARQ feedback for SPS-PDSCH if the PUCCH resource is provided in DCI (per legacy), even when the PUCCH overlaps with non-active period of cell DRX

## Configuration and indication of cell DTX/DRX information to UE

During the SI phase [3], a periodic cell DTX/DRX pattern was discussed and assumed to be used as baseline design. A periodic pattern for cell DTX/DRX can consist of periodically recurring set of occasions/cycles, where each successive occasion can consist of an active period and a non-active period.

It was agreed during SI that cell DTX/DRX mode can be activated/deactivated via dynamic L1/L2 signalling and UE-specific RRC signaling. In this case, both UE-specifc and group common L1/L2 signaling can be considered for activation/deactivation of cell DTX/DRX mode. For example, the UE can be preconfigured with one or multiple cell DTX/DRX configurations or patterns via RRC signaling. The corresponding patterns and the parameters for cell DTX/DRX can be left to RAN2 to decide. When transitioning to a cell DTX/DRX mode, the network can signal the UEs with the ID/index of a preconfigured cell DTX/DRX pattern to be activated or deactivated. The evaluation results showing significant energy saving gains with a marginal impact to the UPT when supporting dynamic activation/deactivation of cell DTX/DRX are provided in the following Section 3.

Proposal 7: Support dynamic signaling for indicating the activation/deactivation of a cell DTX/DRX configuration

In this regard, group common L1/L2 signaling can be useful for reducing the overhead by indicating simultaneously to multiple UEs the activation/deactivation of a cell DTX/DRX configuration. Group common signaling also allows multiple UEs to switch to a DRX configuration that is aligned with the activated cell DTX/DRX configuration. On the other hand, UE-specific signaling allows more flexibility for the network to control the timing of the signaling, especially in scenarios where the different UEs may be using different CDRX configurations. However, UE-specific signaling comes at the cost of higher overhead compared to group-common signalling. Based on the benefits, group-common signaling can be used as baseline, for indicating the activation/deactivation of cell DTX/DRX configuration.

Proposal 8: Support group common signaling as baseline for indicating the activation/deactivation of a cell DTX/DRX configuration

Whether L1 or L2 signaling is to used for indicating the activation/deactivation of a cell DTX/DRX configuration should also be discussed. The benefit of L1 signaling in DCI include fast and dynamic indication to multiple UEs on the cell DTX/DRX activation/deactivation. The existing DCI formats for group common signaling (e.g. DCI 2_0, DCI 2_1) can be extended to support the new bit-fields associated with cell DTX/DRX activation/deactivation. However, in terms of reliability it is possible for the DCI to be misdetected by the UEs during PDCCH monitoring, resulting in inability to properly align the transmission or receptions with the indicated cell DTX/DRX configuration. In the case of L2 signaling, a MAC CE multiplexed in PDSCH can be used for indicating the activation/deactivation of cell DTX/DRX. However, MAC CE is applicable only for UE-specific case. In this regard, given the advantages and the potential spec impact effort between the L1 and L2 signaling options, group common signaling in DCI is preferable.

Proposal 9: DCI is used for group common signaling for indicating the activation/deactivation of a cell DTX/DRX configuration

# Evaluation results

In this section we present evaluation results for cell DTX and DRX operation. We consider an Urban macro deployment. Details regarding the gNB energy consumption model can be found in our previous contribution [5]. Details regarding the simulation assumption can be found in the Appendix.

## Evaluation methodology

The following linear approximation of the downlink power model is assumed for a given network transceiver [5]:

PinNES state= Relative PowerNES state . PActive DL;

PActive DL= P0+ΔpPout  (2)

Pout: the load dependent RF output power. 
This can be approximated as Pout=cell loadslot . Pmax  ;  0<Pout≤Pmax , where Pmax is the maximum RF output power at maximum load.

P0: The linear model parameter to represent power consumption at the zero RF output power.

∆p: The slope of the cell load dependent power consumption

Relative PowerNES state: cell power consumption per sleep mode relative to the ON state, which can be calculated based on the relative power (P). We utilize the gNB power consumption per sleep mode scaling based on the agreed Cat-2 (Set 2) parameters [Appendix B]. The relative power levels are provided in Table 1 below. Based on these parameters, we use a power scaling of 2.1/26 for “light sleep”. For our evaluations, we consider cell sleep durations of up to 120ms, and treat this as light sleep.

Table 1: Relative power scaling, using agreed Cat-2 parameters

| NES state | Power level, relative units |
| --- | --- |
| Active transmission (100% PRB) | 26 |
| Active reception (100% PRB) | 5.8 |
| “Micro”-sleep | 5.0 |
| “Light”-sleep | 2.1 |
| “Deep”-sleep | 1 |

The resultant energy consumed by the cell in a given power savings state (on, off, or sleep) can therefore be:

EinPowSavingState=PinPowSavingState×TimePowSavingState 		(2)

We consider a network energy savings scheme where the macro cell will enter a sleep state if cell load falls below a low threshold. Cell load is measured by RU (ρ). A low RU (ρl) is used to indicate the minimum RU below which a cell may go to sleep for a certain sleep duration, before waking up. A UE that is served by a sleeping cell needs to wait for the cell to wake up. For power consumption, we consider the following parameters: Pmax = 79.43 W (based on agreed assumptions for Set 2 FR1), NTRX=1 (normalized), Δp= 4.7 [4]. The energy consumed for the “ON” state is calculated using Eqn. (2), where PinPowSavingsState corresponds to the Pin relating to the “ON” equations from Eqn. (1). Pout in Eqn. (1) is calculated based on cell load as measured by instantaneous uplink cell load (RU) for the cell: Pout=RUinst*Pc,max. Uplink power in the ON (active reception) is estimated as 5.826*PinON, DL, based on relative power level scaling for active reception vs. active transmission from Table 1, with the uplink cell load (RU) is used instead of DL load in a given slot. Downlink and uplink power in the (light) sleep state is estimated as 2.126*PinON, DL.

## Evaluation results

For both and downlink and uplink evaluations, we utilize a low load RU threshold (ρl) of 10% to evaluate the impact of different cell sleep thresholds on system performance. A cell for which the RU falls below the low load threshold is a candidate for entering a sleep state. We consider sleep durations of 40, 80 and 120ms for the cell.

In the baseline scheme, the cell is always ON, meaning the UE can be served as soon as it has downlink/uplink traffic. In cell DTX/DRX, the cell may enter the sleep state (based on the low load threshold criteria). A UE that is served by a sleeping cell will have to wait for the cell to transition out of the sleep state (to an ON state) before its downlink/uplink traffic needs can be served. We consider sleep durations of {40, 80, 120} ms, with active (ON) duration of {2, 3, 5} ms, respectively. Additionally, we consider a transition time for both ramp up, from NES (sleep) to active state state, as well as ramp down from active to a NES state based on reference configuration set 1 (Cat 2).

For these evaluations, we evaluate DTX/DRX independently.

Downlink

Table 2: cell DTX with 40 ms gNB sleep cycle

| User traffic load: λ = 0.24 |  |  |  | User traffic load: λ = 1.0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) Mean UPT (Mbps) | Cell DTX (Sleep) Mean UPT (Mbps) | UPT gain/loss (DTX vs Baseline) (%) | ESG (DTX vs Baseline) (%) | Baseline (Always ON) Mean UPT (Mbps) | Cell DTX (Sleep) Mean UPT (Mbps) | UPT gain/loss (DTX vs Baseline) (%) | ESG (DTX vs Baseline) (%) |
| 47.97 | 46.47 | -3.11% | +60.54% | 14.86 | 14.59 | -1.82% | +10.99% |

Table 3: cell DTX with 80 ms gNB sleep cycle

| User traffic load: λ = 0.24 |  |  |  | User traffic load: λ = 1.0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) Mean UPT (Mbps) | Cell DTX (Sleep) Mean UPT (Mbps) | UPT gain/loss (DTX vs Baseline) (%) | ESG (DTX vs Baseline) (%) | Baseline (Always ON) Mean UPT (Mbps) | Cell DTX (Sleep) Mean UPT (Mbps) | UPT gain/loss (DTX vs Baseline) (%) | ESG (DTX vs Baseline) (%) |
| 47.97 | 45.94 | -4.23% | +65.28% | 14.86 | 14.23 | -4.24% | +12.20% |

Table 4: cell DTX with 120 ms gNB sleep cycle

| User traffic load: λ = 0.24 |  |  |  | User traffic load: λ = 1.0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) Mean UPT (Mbps) | Cell DTX (Sleep) Mean UPT (Mbps) | UPT gain/loss (DTX vs Baseline) (%) | ESG (DTX vs Baseline) (%) | Baseline (Always ON) Mean UPT (Mbps) | Cell DTX (Sleep) Mean UPT (Mbps) | UPT gain/loss (DTX vs Baseline) (%) | ESG (DTX vs Baseline) (%) |
| 47.97 | 45.21 | -5.74% | +69.13% | 14.86 | 13.94 | -6.20% | 13.94 |

Tables 2 - 4 present results for 40ms, 80ms, and 120 ms sleep durations, respectively. We utilize two different user traffic arrival rates and a low load RU threshold of (ρl) = 10% for each sleep cycle duration, allowing us to investigate the impact of varying sleep cycles as well as varying cell load on system performance. The results demonstrate that significant energy savings gain can be achieved while ensuring nearly comparable user level QoS (throughput), where the ESG offered by the DTX scheme is greater at lower user traffic loads vs. higher user traffic loads (~60 - 69 % vs. 11~12% based on sleep cycle duration). This comes with a marginal drop UPT with shorter (40ms) sleep duration (~ 3% for low load and ~2 % at high load). Longer sleep cycles can yield higher energy savings, but this comes with a higher penalty in UPT.

Observation 3: Utilizing a cell DTX can yield significant energy savings gain, with marginal negative impact on user level QoS (throughput).

Uplink

For the uplink, in addition to cell DRX, we consider dynamic adaptation of cell DRX wherein the gNB sleep cycle can be activated/deactivated. Under such a scheme, the gNB sleep cycle can be deactived after ‘x’ ms of data arrival at the UE. We consider ‘x=4ms’ for these evaluations.

Low load

Table 5: cell DRX with 40 ms gNB sleep cycle

| User traffic load: λ = 0.05 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) | Cell DRX |  |  | Dynamic cell DRX |  |  |
| Mean UPT (Mbps) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) |
| 27.36 | 24.89 | -9.05% | +45.10% | 26.79 | -2.09% | +44.83% |

Table 6: cell DRX with 80 ms gNB sleep cycle

| User traffic load: λ = 0.05 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) | Cell DRX |  |  | Dynamic cell DRX |  |  |
| Mean UPT (Mbps) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) |
| 27.36 | 23.85 | -12.84% | +49.33% | 26.61 | -2.74% | +49.23% |

Table 7: cell DRX with 120 ms gNB sleep cycle

| User traffic load: λ = 0.05 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) | Cell DRX |  |  | Dynamic cell DRX |  |  |
| Mean UPT (Mbps) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) |
| 27.36 | 18.47 | -32.52% | +52.80% | 26.50 | -3.14% | +51.87% |

Medium load

Table 8: cell DRX with 40 ms gNB sleep cycle

| User traffic load: λ = 0.40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) | Cell DRX |  |  | Dynamic cell DRX |  |  |
| Mean UPT (Mbps) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) |
| 24.99 | 23.93 | -4.24% | +14.14% | 24.68 | -1.24% | +13.35% |

Table 9: cell DRX with 80 ms gNB sleep cycle

| User traffic load: λ = 0.40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) | Cell DRX |  |  | Dynamic cell DRX |  |  |
| Mean UPT (Mbps) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) |
| 24.99 | 23.06 | -7.72% | +15.93% | 24.47 | -2.08% | +14.88% |

Table 10: cell DRX with 120 ms gNB sleep cycle

| User traffic load: λ = 0.40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline (Always ON) | Cell DRX |  |  | Dynamic cell DRX |  |  |
| Mean UPT (Mbps) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) | Mean UPT (Mbps) | UPT gain/loss (vs Baseline) (%) | ESG (vs Baseline) (%) |
| 24.99 | 21.42 | -14.28% | +17.33% | 24.49 | -2.0% | +16.61% |

Tables 5 - 10 present results for low (5-7) and medium (8-10) loads with 40ms, 80ms, and 120 ms sleep durations, respectively. We utilize two different user traffic arrival rates and a low load RU threshold of (ρl) = 10% for each sleep cycle duration, allowing us to investigate the impact of varying sleep cycles as well as varying cell load on system performance. The results demonstrate that significant energy savings gain can be achieved especially at low loads; ~45 – 53%, with higher gains achievable under longer sleep cycle durations. This can, however, come with an increased penalty for user throughput with longer sleep durations. Utilizing dynamic adaptation wherein the Cell DTX/DRX cycle can be activated/deactivated as needed allows the system to operate with the same energy savings gain as normal DRX operation, while at the same time ensuring minimal impact on user level QoS (throughput). This holds true across all load scenarios.

Observation 4: Utilizing Cell DRX with dynamic activation/deactivation can yield significant energy savings gain, while ensuring marginal negative impact on user level QoS (throughput).

# Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: To maximize network and UE power savings, UE should not monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active periods, even if the UE is in C-DRX Active time. Such can be decided by RAN2.

Observation 2: To maintain HARQ retransmission timely, UE should monitor PDCCH for dynamic grants/assignments for retransmissions during the UE’s C-DRX Active time per legacy behaviour, even during the Cell DTX non-active period. Such can be decided by RAN2.

Observation 3: Utilizing a cell DTX can yield significant energy savings gain, with marginal negative impact on user level QoS (throughput).

Observation 4: Utilizing Cell DRX with dynamic activation/deactivation can yield significant energy savings gain, while ensuring marginal negative impact on user level QoS (throughput).

Proposal 1: UE is not expected to measure periodic/semi-persistent CSI-RS (including PTRS, TRS, BFD, and RLM RS) during non-active periods of cell DTX

Proposal 2: UE is not expected to measure PRS during non-active periods of cell DTX

Proposal 3: UE is not expected to transmit periodic/semi-persistent CSI-RS reports during non-active periods of cell DRX

Proposal 4: UE is not expected to transmit periodic/semi-persistent SRS during non-active periods of cell DRX

Proposal 5: UE transmits HARQ feedback for Dynamic PDSCH assignments if the PUCCH resource is provided in DCI (per legacy), even when the PUCCH overlaps with non-active period of cell DRX

Proposal 6: UE transmits HARQ feedback for SPS-PDSCH if the PUCCH resource is provided in DCI (per legacy), even when the PUCCH overlaps with non-active period of cell DRX

Proposal 7: Support dynamic signaling for indicating the activation/deactivation of a cell DTX/DRX configuration

Proposal 8: Support group common signaling as baseline for indicating the activation/deactivation of a cell DTX/DRX configuration

Proposal 9: DCI is used for group common signaling for indicating the activation/deactivation of a cell DTX/DRX configuration

# Appendix A

Table 3: Urban Macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| System bandwidth | 20 MHz |
| Layout | Single layer: Hex. Grid 7 site, 3 sectors per site |
| ISD | 500m |
| Duplexing | FDD |
| Numerology | 15 KHz, 14 OFDM symbol slot |
| BS antenna config. | 32 Tx/Rx RUs - (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna config | 4 Tx/Rx RUs - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| BS Tx Power | 49dBm |
| UE Tx Power | 23 dBm |
| User distribution and UE speed | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h UL: 2 users per TRxP |
| Traffic model | Use 36.889 Table A.1.1. Uplink: We consider a uniform packet arrival rate across all UEs (and cells). We use two user traffic load values (low and medium), to investigate impact of different user traffic loads on the system. |

# Appendix B

RAN1-110b Agreements Agreement Confirm the previous Working Assumption with the following update - For RAN1 evaluation purpose, for reference configuration set 1/2/3, the values are provided as below. - The transition time is confirmed without update. - FFS: The time unit to be used when calculating the energy consumption Power state; Relative Power P for Category 1; Relative Power P for Category 2. Set 1; Set 2; Set 3; Set 1; Set 2; Set 3. Deep sleep; 1; 1; 1; 1; 1; 1. Light sleep; 25; 23 25; 20 25; 2.1; 2.6 2.1; 1.8 2.1. Micro sleep; 55; 50; 38; 5.5; 5; 3. Active DL; 280; 240 200; 152; 32; 40 26; 8.4 17.6. Active UL; 110; 90; 80; 6.5; 5.8; 4.2. Working Assumption For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1: 50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A..

# References

RAN1 chair notes, RAN1#112, Feb 2023

R2-2303604, Report of [POST121][311][NES] DTX/DRX – gNB and UE behaviours, InterDigital

TR 38.864, Study on network energy savings for NR, v18.0.0, Dec 2022