3GPP TSG RAN WG1 #112bis-e			R1-2303029

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.6.1

Source:	China Telecom

Title:	Discussion on further complexity reduction for eRedCap UEs

Document for:	Discussion

Introduction

In RAN1#112, the following agreements and working assumptions on UE complexity reduction were reached [1]:

Initial BWP Conclusion: There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”. Number of PRBs Agreement: For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number Paging bandwidth Agreement: Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. RAR bandwidth and Msg3 transmission time Agreement: For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH. For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs MsgA bandwidth Agreement: For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Msg4 bandwidth Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Simultaneous reception Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

This contribution provides views on UE complexity reduction.

UE complexity reduction

Random access timeline

In the last meeting we achieve the following agreements:

For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot.

- When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied.

- When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot,

  - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms.

    - FFS: value(s) of X

  - Otherwise, the UE behavior is up to the UE implementation.

- Note: it does not mean early indication is needed

- Note: it will not be used as example for unicast PDSCH

For the “FFS: value(s) of X”

- X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS

- Note: Single Value pair for X is to selected for SCSs

As concluded in the previous RAN1 meeting, Rel-18 Redcap can only a maximum of 5MHz PDSCH per slot, and from the operator's perspective, it is highly likely that RedCap and normal UEs share one network where the normal UEs use 20M RAR PDSCH. To reduce the complexity of network deployment, the network will prefer that RedCap also use 20MHz RAR PDSCH. Based on this scenario, the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot will be common, and the value of X is important.

The X is used to ensure the RAR processing and Msg3 preparation for Rel-18 RedCap for the case that the scheduled RAR PDSCH is larger than 5MHz. A larger number of PDSCH PRBs represents a larger PDSCH processing time, although the number of PDSCH PRBs and the PDSCH processing time are not exactly linear, by reducing the complexity of Redcap we reduce its capability, so we need to choose a suitable X to ensure that the RedCap UE can adequately handle the 20MHz RAR. On this basis, we prefer to set a slightly looser parameter: X = [1/0.5 or 2/1] ms for 15/30kHz SCS, choose either of these two configurations without setting multiple sets of parameters.

Thus, we have the following proposal:

Proposal 1: X = [1/0.5 or 2/1] for 15/30kHz SCS, choose either of these two configurations without setting multiple sets of parameters.

Msg4 and MsgB PDSCH bandwidth

RAN1 has made the following agreement regarding the unicast PDSCH bandwidth:

Agreement: - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e

The work assumption in RAN1 #112:

Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Although we believe that the original agreement already contains a limit on the bandwidth of msg4, the new working assumption is more ready to express what everyone wants to achieve and to avoid inconsistent understanding. At the same time, we believe that additional constraints on UE behavior are unnecessary. The eRedcap UE can reuse the existing UE behavior in the spec, UE transmits PUCCH for HARQ-ACK (only for ACK) only if the PDSCH is decoded and the PDSCH is for this UE, so that whether a UE drop PDSCH or fails to process Msg4 (due to >5 MHz or any other reason), there is no impact on the PDCCH. And after the contention resolution failed, UE may restart RACH procedure, just as legacy behavior.

Therefore, we support this working assumption. In addition, MsgB and Msg4 are more alike, so we also consider the MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

Proposal 2: Adopting the current working assumption:

•	For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

•	The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 3: The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

SRS bandwidth

In the WID, the objective on complexity/cost reduction is described as follows [2]:

- UE BB bandwidth reduction

  - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL

  - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

Therefore, according to the previous agreement, the SRS bandwidth can up to 20 MHz. Considering with Rel-17, Rel-17 and Rel-18 may coexist in one network, so from the network deployment considerations, Rel-18 UE may be flexible scheduling in anywhere within BWP up to 20M bandwidth, 20MHz SRS can help the base station to better estimate the channel, more conducive to the flexible scheduling of Rel-18 UE, and, based on the WID, PUCCH bandwidth can up to 20MHz and limiting the SRS bandwidth to 5MHz would not have a significant gain in reducing UE complexity. We therefore do not recommend that limiting the SRS bandwidth.

Proposal 4: The SRS bandwidth do not need to be limited to 5 MHz as for other uplink transmissions

Conclusion

In this contribution, we provide the following proposals:

Proposal 1: X = [1/0.5 or 2/1] for 15/30kHz SCS, choose either of these two configurations without setting multiple sets of parameters.

Proposal 2: Adopting the current working assumption:

•	For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

•	The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 3: The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

Proposal 4: The SRS bandwidth do not need to be limited to 5 MHz as for other uplink transmissions.

- References

R1-2301889, FL summary #4 on Rel-18 RedCap UE complexity reduction, Ericsson, 3GPP TSG RAN1 Meeting#112, 27th February – 3rd March, 2023.

RP-222675, “New WID on enhanced support of reduced capability NR devices”, Ericsson, 3GPP TSG RAN Meeting #97-e, September 12th – 16th, 2022.