3GPP TSG RAN WG1 #112bis-e			R1-2303036

e-Meeting, Apr. 17th – Apr. 26th, 2023

Agenda item:		9.14.3

Source:	China Telecom

Title:	Discussion on dynamic waveform switching between DFT-s-OFDM and CP-OFDM

Document for:		Discussion

Introduction

In RAN #94 e-meeting, a new Rel-18 work item on further NR coverage enhancements was approved [1] and updated in RAN #96 [2]. The objective of the work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. Detailed objective for dynamic switching between DFT-S-OFDM and CP-OFDM is listed as follows:

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, we focus on the dynamic switching between DFT-S-OFDM and CP-OFDM.

Discussion

  - Applicability of dynamic waveform switching

In RAN1 #112 meeting, the following agreements and conclusions were reached in the RAN1 #112[3] to support dynamic switching of UL waveform.

Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI. Conclusion There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18. Agreement Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant. Conclusion - The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

In the above agreement, the DWS indication can be applied to the UL scheduling DCI, i.e., DCI format 0_1/0_2. However, we think such indication can’t be applied to DCI format 0_0, which is a fall-back format. Considering the purpose of DCI format 0_0, which is used for the basic operations and the robustness of RRC reconfiguration, no optional function should be indicated by this format. When the DCI format 0_0 is adopted, the scheduling of PUSCH should be as simple as possible, and the accuracy of decoding should be as high as possible, thus introduce the DWS for DCI format 0_0 should not be supported.

Proposal 1: Dynamic waveform switching applied to PUSCH scheduled or activated by DCI format 0_0 is not supported in R18.

As for DWS being applicable to the subsequent PUSCH, we don’t think it is really needed. On the one hand, the waveform of PUSCH will be conflict between UE and gNB and won’t be recovered in a while if the DCI is missed. Which means it should also not be supported even the CG type 2 PUSCH is supported. On the other hand, it is no needed to indicate the persistent change of waveform with a DCI indicated, the function is actually the same as reconfigure the PUSCH with RRC since we have agreed that the DWS won’t be applied to CG type 1 PUSCH.

Proposal 2: Dynamic waveform switching applied to subsequence PUSCH is not supported in Rel-18.

For the Msg3 transmission, for the initial transmission, the waveform is indicated with the RRC parameters “msg3-transformPrecoder”, to reuse the 1-bit DCI field for the Msg3 initial waveform switching indication can be quite complex and unnecessary. But the DWS can be considered for the retransmission of Msg3. However, if the retransmission, it means the Msg3 is failed. In such situation, the DCI format 0_0 is adopted for scheduling the Msg transmission. But from our prospective, the new 1-bit DCI field shouldn’t be applied to the fall-back format. Therefore, we think at least the current DWS indication method should not be applied to Msg3. However, we think if the implicit indication method can be applied to the DWS for Msg3 retransmission.

Proposal 3: Dynamic waveform switching indicated with 1-bit new field in UL scheduling DCI applied to Msg3 is not supported.

Proposal 4: Dynamic waveform switching with implicit indication can be considered for Msg3 retransmission in Rel-18.

  - Dynamic switching mechanism

The following suggestion was given by FL as a start point for the next meeting about the DCI size alignment.

RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: For C-RNTI, assume field sizes based on DWS indication. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of C-RNTI and indicated waveform based on existing C-RNTI/CS-RNTI alignment clause. - FFS: If DWS is not supported or not configured for CG Type 2 activation, and DWS field does not match RRC-configured waveform for indicated CG index, and NDI=0, it is an error case. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). [Option 4: Per-field alignment only for DCI addressed to CS-RNTI] - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). Notes: - For Option 1 and [Option 4], DWS field must be positioned before any field that depends on waveform. - For Option 3, DWS field must be positioned before NDI field, or immediately after NDI field

As it is agreed in RAN1 #110b-e and #111, dynamic waveform switching is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 and the DCI size should be aligned between CP-OFDM and DFT-s-OFDM, otherwise the UE may be confused about some fields since some of the fields are related to whether transform precoder enabled or not, i.e., “Precoding information and number of layers”, “Second Precoding information”, “Antenna ports”, “DMRS sequence initialization”.

We think the above agreement means that, the alignment of size between different waveforms must be done. As for adopting which principle to achieve the alignment of DCI size, we think the most convenient mechanism should be taken since the alignment must be done. By aligning the DCI size per field, i.e., the bit width of each field is set to the maximum between the bit width of the field if transform precoding is disabled and the bit width of the field if transform precoding is enabled, it is much easier for UE to detect the corresponding fields and no need to configure the payload of DCI in advance. The procedure of per field can be more simplifier than the per format method. Thus, we support to select option 2, i.e., the per field method for the alignment of DCI size between DFT-s-OFDM and CP-OFDM.

Proposal 5: Support to select the Option 2, i.e. the per-field alignment, to align the DCI size of DFT-s-OFDM and CP-OFDM, the size of each DCI field should be aligned to the maximum size of the corresponding DCI of DFT-s-OFDM and CP-OFDM.

  - PHR report

The following agreement has been reached in the RAN1 #111[4].

Agreement Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching: - Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform - Target waveform can be same or different from waveform of an actual PUSCH transmission - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission - FFS determination of target waveform, target RB allocation, target modulation order - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms - PHR triggering enhancements, e.g. - Network-triggered PHR - PH becomes lower (higher) than a threshold - PHR triggered by waveform switching - Reporting of recommended waveform or request to switch waveform - Other solutions not precluded

When the waveform switching happens, since the cubic metric of different waveform is different, the transmission power of the new waveform should be reported to the gNB for reference, i.e., the assistant information about the PHR should be reported. Which means if the waveform switching is happened, the PHR should be triggered at once, and the assistant information can be reported together. Also, the PHR can be used for triggering the waveform switching with assistant information together to help adjust the power level. To simplify the procedure of PHR, the MAC CE, which is how PHR be transmitted in the current specs, should still be used, the assistance information can be included at the same time. The specific content and scheduling of PHR for waveform switching can be further studied.

Proposal 6: Support to transmit PHR along with the assistant information about the power adjustment for waveform switching via MAC CE.

Conclusions

In this contribution, we discuss dynamic switching between DFT-S-OFDM and CP-OFDM and have following proposals:

Proposal 1:

Dynamic waveform switching applied to PUSCH scheduled or activated by DCI format 0_0 is not supported in Rel-18.

Proposal 2:

Dynamic waveform switching in R18 applied to subsequence PUSCH is not supported in Rel-18.

Proposal 3:

Dynamic waveform switching indicated with 1-bit new field in UL scheduling DCI applied to Msg3 is not supported in Rel-18.

Proposal 4:

Dynamic waveform switching with implicit indication can be considered for Msg3 retransmission in Rel-18.

Proposal 5:

Support to select the Option 2, i.e. the per-field alignment, to align the DCI size of DFT-s-OFDM and CP-OFDM, the size of each DCI field should be aligned to the maximum size of the corresponding DCI of DFT-s-OFDM and CP-OFDM.

Proposal 6:

Support to transmit PHR along with the assistant information about the power adjustment for waveform switching via MAC CE

References

- 3GPP RP-213579, “New WI: Further NR coverage enhancements”, China Telecom, RAN#94e, December 6-17, 2021.

- 3GPP RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom, RAN#96, Budapest, Hungary, June 6-9, 2022.

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #112, Feb 27th – Mar.3rd, 2023.

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #111, December 14th – 18th, 2022.