3GPP TSG RAN WG1 #112bis-e			R1-2303041

e-Meeting, April 17th – 26th, 2023

Source:	Panasonic

Title: 	Discussion on general aspects of AI/ML framework

Agenda Item:		9.2.1

Document for:	Discussion

- Introduction

A study item “Study on Artificial Intelligence and Machine Learning for NR air-interface” has been approved for Rel.18 [1].

This document describes our view on general aspects of AI/ML framework. The agreement in the past meeting is described in Annex.

- Discussion

The usage of the term "model"

The actual characteristics of the "AI/ML model" is different depending on the following. Therefore, we propose to clarify following.

- Physical model

- Binary model

- It is executable model with specific calculation accuracy.

- The input/output relation are fully predictable among the same binary models.

- This is usually proprietary model unless standardization or de-fact is established.

- The metaphor of general processor usage is "exe file", "binary file", machine language, or assembler language.

- Source code model

- It is the model before compilation and often described by a model description language (e.g., ONNX).

- It is usually open-format as mutually recognizable across vendors and allow interoperability, although  vendor specific source code is also possible, which is proprietary-format model.

- The input/output relation are almost predictable among the same source code models. Depending on the quantization or compilation, there can be some difference.

- The metaphor of general processor usage is "high level language like C or Python".

- Logical model

- It is aligned with the current definition of AI/ML model "a data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs". The certain input and output relation are defined as test in order to define the algorithm. The condition to define these tests is applicable condition. The test can be RAN4 test or bi-lateral test among terminal and NW, which is the same meaning of "test" usage in UE capability definition.

- The input/output relation are predictable only within the test as there can be different realization by the physical models among different vendors and/or versions of the same vendor. The result can be different when the input is other than test input but aims to achieve certain function.

- NW identifies logical model at least. There can be the case that NW identifies physical/source code/binary AI/ML models.

- The function of automatically/internally adjust parameters or switch physical model can be included in this model definition.

- The metaphor of general processor usage is "functional definition" or "logical definition".

- If one function equals one logical model, the meaning of logical model would be same as a function. If multiple functions are supported by one logical model, the meaning of the logical model and the function can be different.

Note that although "the function of automatically/internally adjust parameters or switch physical model" are included in logical model but not physical model, the actual boundary between logical and physical model itself can be ambiguous. This is especially when "the function of automatically/internally adjust parameters or switch physical model" itself is carried out by AI/ML. It just mean a bit larger physical AI/ML model which contains the function of automatically/internally adjust parameters or switch sub-physical model. So these model usage difference should be seen as what aspect of the characteristics is focusing.

Note that there were some offline discussion to distinguish "logical model in broad sense", where the model is defined by data sets or test, and "logical model in narrow sense", where to cover various physical model implementations of different version/devices/vendors but the same identification from NW. When the logical model in narrow sense is used, some rule or mechanism to group multiple various physical model implementations is necessary. If not, these are just meaningless group of multiple various physical model implementations. Our view is rule or mechanism to group multiple various physical model implementations is test or input/output relations. Therefore, we think logical model in broad sense and logical model in narrow sense are ultimately same meaning at the end.

Proposal 1: For the discussion purpose, following terminology can be used when necessary. Detailed definition may not be required.

- Physical model consists of binary model and source code model.

- Binary model is executable with specific implementation. Note it can be proprietary or open-format.

- Source code model is before the compilation. Note it can be proprietary or open-format.

- Logical model is the model defined by the input and output relation and test.

The discussion above can be summarized in the following figure.

Figure 1. The relation among physical/source code/binary/logical models

All above models can generate inference output. On the other hand, another usage of AI/ML model is the case only to indicate the AI/ML model structure without parameters. The AI/ML model structure without parameters cannot generate inference output. Although such model usage is important for generic AI/ML realization, its usage is not so meaningful, because 3gpp framework for AI/ML is to use AI/ML model for the specific target use case, which requires the inference output. It is not aligned with the AI/ML model definition "a data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs". Therefore, we propose following.

Proposal 2: In 3gpp RAN1 discussion perspective, the case that does not produce inference output i.e. the AI/ML model structure without parameters, are not called as a AI/ML model. An AI/ML mode is used only when the certain inference output is generated.

Clarification of model identification and model update

The terminology of model update and model parameter update was agreed as working assumption. On the other hand, there is not clear boundary between what part is called as "model structure" and what part is called as "model parameters". Some of model structure itself can be modified by the parameter itself. Therefore, we don't see the specific need to confirm the working assumption of the term "model parameter update". Regardless of the change is whole or some of parameters or structure, these can be categorized as just model update.

The working assumption of the model update is "process of updating the model parameters and/or model structure of a model". The change of the parameters can be the same or different usage/purpose/applicable condition. The same usage/purpose/applicable condition may aim to achieve better performance or reduced power consumption and so on. Our view is one model ID can be assigned with specific usage/purpose/applicable condition. When the usage/purpose/applicable condition is same, it is just the version difference and it is categorized as model update. If the usage/purpose/applicable condition is different, it should be interpreted as new model as usage/purpose/applicable condition itself is different.

Proposal 3: Not to confirm working assumption of model parameter update.

Proposal 4: Model update is restricted to the process updating the model parameters and/or model structure of a model of the same usage/purpose/applicable condition. It is same meaning with version update. When different usage/purpose/applicable condition is applied, it is new model.

By restricting a model update to the same usage/purpose/applicable condition, we propose model identification as the meaning to identify the specific usage/purpose/applicable condition. Current working assumption of model identification without the knowledge of usage/purpose/applicable condition is not so meaningful from NW perspective as NW cannot use AI/ML model without such information.

Proposal 5: Model identification is "a process/method of identifying an AI/ML model for the common understanding of the usage/purpose/applicable condition between the NW and the UE".

With such definition of model identification, we can see following observations.

Observation 1: To have the common understanding on the usage/purpose/applicable condition between NW and UE can be easier via offline i.e. the specification or negotiation between UE side and NW side, although to have the common understanding on the usage/purpose/applicable condition between NW and UE via signaling is possible.

Observation 2: For the model update, i.e. the parameter update of the same usage/purpose/applicable condition, the version number may be added instead of new model ID for the version identification.

Observation 3: The parameter update with new usage/purpose/applicable condition is equal to model identification is realized via signaling. New model ID is assigned.

Note we think "a function" equal to "a specific model ID" and it also equals to "a specific usage/purpose/applicable condition". Therefore, "model identification via offline" is same with "functionality is determined offline" and "model identification via signaling" is same with "functionality is dynamically determined". If a function definition does not contain applicable condition, our description can be interpreted as "function plus applicable condition".

Note that our view on Meta information is meant to describe usage/purpose/applicable condition by human interpretable manner. The usage/purpose/applicable condition from machine interpretable manner is defined by input/output relation and by test in our view.

The discussion above can be summarized in the following figure.

Figure 2. The relation among model identification and model update

The relation between cases and AI/ML models

In RAN1#112, following cases y to z5 were agreed to be studied. Depending on the cases, the actual usage of AI/ML models are different. Our view is following described in "applicable AI/ML models". For case y, the training is carried out by NW side and the others are different usage. Therefore, we distinguish them as y1 and y2. We didn't analyze z5 as it is not realistic case.

Figure 1. The relation between cases and AI/ML models

| Case | Model delivery/transfer | Model storage location | Training location | Applicable AI/ML models |
| --- | --- | --- | --- | --- |
| y1 | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / neutral site | - Storage and training are via binary or source code model - NW indication is via logical model |
| y2 | model delivery (if needed) over-the-top | Outside 3gpp Network | NW-side | - Storage, training and NW indication are via source code model. |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site | - Storage, training and NW indication are via binary or source code model. |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side | - Not sure the standardization feasibility of this option as NW needs to aware proprietary format. It can be bi-lateral agreement between UE and NW. |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site | - Storage, training and NW indication are via binary or source code model. |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side | - Storage, training and NW indication are via source code model. |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side | - Not realistic |

With above table, we observed following points.

Observation 4: The models stored in the specific location are always physical model. It can be binary or source code model.

Observation 5: When the model stored is source code model, some compilation processing is required. It can be within UE or other network entity for the compilation.

Observation 6: When the training location is NW-side, only source code model can be trained as NW doesn't know UE proprietary implementation and NW cannot manage the UE internal switching functions among physical models. The storage location is within 3GPP network does not influence the control of AI/ML models by the NW.

Observation 7: When the training location is UE-side or neutral side but the storage location is 3gpp network, physical model stored in the 3gpp network is uploaded to 3gpp network beforehand from UE, UE-side or neutral site. Then, depending on the usage/purpose/applicable condition, NW can transfer specific physical models to UE.

Some further observation is following.

Observation 8: Site specific model is rather only feasible when the training location is NW side if the network does not disclose the detail of antenna deployment or other network proprietary information.

Observation 9: Basically, the output of the model is responsible by the entity where the model is trained. The network trained model resolves the concern of the difficulty to test of all possible inputs and the difficulty to define the function more detail.

Observation 10: The case y with the training location is UE-side or neutral side requires to resolve the boundary condition of the definition of the function. The boundary condition means what is the boundary between Umi vs. Uma, different UE speeds and indoor vs. outdoor if applicable conditions are different for these conditions.

Online training at the UE

If online training by UE-side or neutral side is used, AI/ML model is updated after online training. One option is the updated AI/ML model can be deployed without test. The other option is the updated AI/ML model needs some of the test before the deployment. The deployment without test can reduce the cycle of the update but the deployment without test has the reliability concern especially UE side model. It can be harmful to the network deployment. Therefore, we propose following. It can be said as pre-commercial deployment.

Proposal 6: The need of test or pre-commercial deployment after online training before the deployment needs some discussion.

- Conclusion

We propose following.

Proposal 1: For the discussion purpose, following terminology can be used when necessary. Detailed definition may not be required.

- Physical model consists of binary model and source code model.

- Binary model is executable with specific implementation. Note it can be proprietary or open-format.

- Source code model is before the compilation. Note it can be proprietary or open-format.

- Logical model is the model defined by the input and output relation and test.

Proposal 2: In 3gpp RAN1 discussion perspective, the case that does not produce inference output i.e. the AI/ML model structure without parameters, are not called as a AI/ML model. An AI/ML mode is used only when the certain inference output is generated.

Proposal 3: Not to confirm working assumption of model parameter update.

Proposal 4: Model update is restricted to the process updating the model parameters and/or model structure of a model of the same usage/purpose/applicable condition. It is same meaning with version update. When different usage/purpose/applicable condition is applied, it is new model.

Proposal 5: Model identification is "a process/method of identifying an AI/ML model for the common understanding of the usage/purpose/applicable condition between the NW and the UE".

Proposal 6: The need of test or pre-commercial deployment after online training before the deployment needs some discussion.

We observed following.

Observation 1: To have the common understanding on the usage/purpose/applicable condition between NW and UE can be easier via offline i.e. the specification or negotiation between UE side and NW side, although to have the common understanding on the usage/purpose/applicable condition between NW and UE via signaling is possible.

Observation 2: For the model update, i.e. the parameter update of the same usage/purpose/applicable condition, the version number may be added instead of new model ID for the version identification.

Observation 3: The parameter update with new usage/purpose/applicable condition is equal to model identification is realized via signaling. New model ID is assigned.

Observation 4: The models stored in the specific location are always physical model. It can be binary or source code model.

Observation 5: When the model stored is source code model, some compilation processing is required. It can be within UE or other network entity for the compilation.

Observation 6: When the training location is NW-side, only source code model can be trained as NW doesn't know UE proprietary implementation and NW cannot manage the UE internal switching functions among physical models. The storage location is within 3GPP network does not influence the control of AI/ML models by the NW.

Observation 7: When the training location is UE-side or neutral side but the storage location is 3gpp network, physical model stored in the 3gpp network is uploaded to 3gpp network beforehand from UE, UE-side or neutral site. Then, depending on the usage/purpose/applicable condition, NW can transfer specific physical models to UE.

Observation 8: Site specific model is rather only feasible when the training location is NW side if the network does not disclose the detail of antenna deployment or other network proprietary information.

Observation 9: Basically, the output of the model is responsible by the entity where the model is trained. The network trained model resolves the concern of the difficulty to test of all possible inputs and the difficulty to define the function more detail.

Observation 10: The case y with the training location is UE-side or neutral side requires to resolve the boundary condition of the definition of the function. The boundary condition means what is the boundary between Umi vs. Uma, different UE speeds and indoor vs. outdoor if applicable conditions are different for these conditions.

- Reference

[1]	RP-213599, “New SI: Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface,”	Qualcomm (Moderator), RAN#94e.

[2] R1- 2301868, "Final Summary of General Aspects of AIML Framework ", Moderator (Qualcomm), RAN1#112

[3] R2-2302268, " Report of Offline 027 model transfer delivery (Huawei)", Huawei (email rapporteur), RAN2#121

- Past agreements

Agreements in RAN1#109:

Agreement

Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Table: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Agreements in RAN1#110:

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIsNote: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note:

Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

Agreements in RAN1#110bis:

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z

- Clarifying note: Level y includes cases without model delivery.

Agreement

Clarify Level x/y boundary as:

- Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE.
(Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

FFS: Detailed discussion of model ID with associated information and/or model functionality.

FFS: usage of model ID with associated information and/or model functionality based LCM procedure

FFS: whether support of model ID

FFS: the detailed applicable AI/ML operations

Agreement

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

Conclusion

Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact.

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

-	Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training).

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case:

    - Monitoring based on inference accuracy, including metrics related to intermediate KPIs

    - Monitoring based on system performance, including metrics related to system peformance KPIs

    - Other monitoring solutions, at least following 2 options.

      - Monitoring based on data distribution

        - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc.

        - Output-based: e.g., drift detection of output data

      - Monitoring based on applicable condition

Note: Model monitoring metric calculation may be done at NW or UE

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

    - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

    - Overhead (e.g., signaling overhead associated with model monitoring)

    - Complexity (e.g., computation and memory cost for model monitoring)

    - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

    - FFS: Power consumption

    - Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

Agreement

Study various approaches for achieving good performance across different scenarios/configurations/sites, including

    - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

    - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

      - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.]

    - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Agreement

The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Clarification on inference complexity

  - Note: Inference complexity includes complexity for pre- and post-processing.

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- FFS: Power consumption, latency (e.g., Inference latency)

Conclusion

This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP.

Agreements in RAN1#111:

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Funtionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

Agreements in RAN1#112:

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Agreement

For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.