3GPP TSG RAN WG1 #112bis-e		R1-2303046

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.3.2

Source:	Google

Title:	On SRS Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #112, the following agreements on SRS enhancement have been achieved.

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity . - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. - FFS: s = 4, s = 8. - m = 2,4,8, 10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), - Option 2: For comb 4, do not support 4 comb offsets. Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index within a radio frame and OFDM symbol index , and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index . - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) . - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame. Conclusion No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols. Conclusion No consensus to support the following for TDD CJT SRS enhancement in Rel-18: - Further enhancements to frequency hopping - Sequence hopping/randomization, per-hop sequence from a long SRS sequence - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment - Precoded SRS for DL CSI acquisition - Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Configuration of v (sequence index within a group) per SRS resource - Multiplying mask sequence to the legacy SRS sequence

In this contribution, we provide some discussions on SRS enhancement.

# SRS for CJT

## Power control enhancement for SRS

In RAN1 #110b, the following agreement on power control enhancement has been achieved.

Agreement For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set: - Option 1: - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha - Each transmission occasion of the SRS resource is towards multiple TRPs - Option 2: - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS - Different transmission occasions of the SRS resource can be towards different TRPs

Option 1 is a subset of option 2. In option 2, if the gNB configures the same P0 and closed-loop index in the more than 1 power control processes and selects the more than 1 power control processes for uplink power control, option 2 can cover the functionality of option 1. Besides, option 2 can provide a more flexible framework for the gNB to dynamically select the target different TRP(s) for power control. Therefore, option 2 should be supported.

Proposal 1: Support option 2 for power control enhancement for SRS

- The gNB can dynamically indicate the power control process(es) to be applied from the configured more than 1 power control processes for an SRS transmission occasion

## SRS interference randomization

In RAN1 #112, the following agreement on SRS interference randomization is achieved.

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity . - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index within a radio frame and OFDM symbol index , and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index . - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) . - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame.

The first issue is whether the combined cyclic shift hopping and comb offset hopping can be supported. Such hopping does not take too much complexity, but the UE only needs to calculate the cyclic shift and comb offset for each symbol. Thus, it is unnecessary to preclude concurrent configuration for both hopping. Therefore, the combined cyclic shift hopping and comb offset hopping should be supported.

Proposal 2: Do not support to introduce constraint to preclude concurrent configuration of cyclic shift hopping and comb offset hopping.

A second issue is whether to define the hopping pattern based on a new ID or the virtual cell ID. Now that the SRS generation is based on virtual cell ID, it is unnecessary to introduce a new ID, which could increase the RRC overhead but without clear benefit. Therefore, the option 1 should be supported.

Proposal 3: Support option 1 (reuse the SRS sequence ID) to determine the cyclic shift and comb offset hopping pattern.

A third issue is how to define the hopping pattern within a slot. To generate different SRS sequences at different time, one way is to determine the hopping pattern based on the symbol index for the SRS. For SRS with repetition operation, the SRS signal does not have to be the same. Instead, different SRS signals can help to achieve the diversity gain. Therefore, the option 2 (hopping based on only the symbol index) should be sufficient for the hopping pattern determination.

Proposal 4: Support option 2 (hopping based on only the symbol index) for the intra-slot cyclic shift and comb offset hopping.

One open issue is how to decide candidate hopping pattern. With regard to the multiplexing with legacy SRS without comb offset/cyclic shift hopping, it is better that the hopping pattern is configured by the gNB. Then the gNB can guarantee the SRS in each hop can always be multiplexed orthogonally with legacy SRS. Therefore, the candidate hopping pattern should be configured by the gNB.

Proposal 5: Support to configure the candidate hopping patterns for comb offset and cyclic shift hopping by RRC signaling.

# SRS for 8Tx

## TDMed SRS with repetitions

In RAN1 #112, the following is agreed on 8Tx SRS in one symbol.

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Option 2-1 produces the SRS repetitions for each port in a cyclic manner and option 2-2 produces the SRS repetitions for each port in a sequential manner. Both options have pros and cons. Option 2-1 is good for partial repetition dropping when a collision happens. Option 2-2 is good for receiving complexity reduction. The gNB can perform joint channel estimation for an SRS port in consecutive symbols. Thus, with regard to different benefits for option 2-1 and option 2-2, both options should be supported.

Proposal 6: Support both option 2-1 (cyclic mapping) and option 2-2 (sequential mapping) for SRS with repetitions.

## UL Full Power Transmission for TDMed SRS

A potential issue for 8Tx SRS is whether the UE is able to support full power transmission for each symbol with partial antenna ports. If the gNB configures the UE to transmit the 8Tx SRS in 4 symbols, the UE may or may not be able to support full power transmission, which could be based on the UE’s PA architecture. Therefore, it should be supported for the UE to report UE capability on whether it supports full power transmission for each symbol for TDM based 8Tx SRS.

Proposal 7: Support the UE to report the UE capability to indicate whether it supports full power transmission for each symbol for TDM based 8Tx SRS.

## Transmission power scaling for TDMed SRS

A UE may be scheduled with a TDM based SRS and another uplink signal overlapped in time domain in the same or different CCs. The total transmission power in the overlapped symbol could exceed the maximum transmission power for the UE as shown in Figure 1. It is necessary to study how to perform the transmission power scaling for the TDM based SRS when it overlaps with another uplink signal.

Figure 1: A potential issue for transmission power scaling when the TDM based SRS overlaps with another uplink signal in time domain

Proposal 8: Study how to perform the transmission power scaling for the TDM based SRS when it overlaps with another uplink signal.

# Conclusion

In this contribution, we provided discussion on SRS enhancement. Based on the discussion, the following proposals have been achieved.

SRS for CJT

Proposal 1: Support option 2 for power control enhancement for SRS

- The gNB can dynamically indicate the power control process(es) to be applied from the configured more than 1 power control processes for an SRS transmission occasion

Proposal 2: Do not support to introduce constraint to preclude concurrent configuration of cyclic shift hopping and comb offset hopping.

Proposal 3: Support option 1 (reuse the SRS sequence ID) to determine the cyclic shift and comb offset hopping pattern.

Proposal 4: Support option 2 (hopping based on only the symbol index) for the intra-slot cyclic shift and comb offset hopping.

Proposal 5: Support to configure the candidate hopping patterns for comb offset and cyclic shift hopping by RRC signaling.

8Tx SRS

Proposal 6: Support both option 2-1 (cyclic mapping) and option 2-2 (sequential mapping) for SRS with repetitions.

Proposal 7: Support the UE to report the UE capability to indicate whether it supports full power transmission for each symbol for TDM based 8Tx SRS.

Proposal 8: Study how to perform the transmission power scaling for the TDM based SRS when it overlaps with another uplink signal.