3GPP TSG RAN WG1 #112bis-e		R1-2303047

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.4.1

Source:	Google

Title:	On Simultaneous Multi-Panel Transmission

Document for:	Discussion/Decision

# Introduction

In RAN1 #112, the following agreements on simultaneous multi-panel transmission have been achieved.

Conclusion There is no consensus to support dynamic switching between STxMP SDM/SFN scheme and Rel-17 mTRP PUSCH TDM scheme Conclusion There is no consensus to support 2 CWs in single-DCI based STxMP SDM scheme Agreement Confirm the working assumption with the following updates: Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with one CW - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e. Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH: Agreement On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN Agreement To support indicating DMRS ports in different CDM groups for layer combination {1+2} in SDM - Add new entry {0, 2, 3} to the DMRS table for the layer combination {1+2}; - This is optional UE capability for UE that supports sDCI based STxMP SDM Agreement For single-DCI based STxMP SFN scheme, - Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB - Each PTRS port is transmitted in SFN manner Agreement Among the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH, the SRS resource set with lower set ID is the first SRS resource set.

In this contribution, we provide some discussions on simultaneous multi-panel transmission.

# General aspects

Several general assumption should be clarified on the enhancement of STxMP:

- Maximum number of active UE panels

- Maximum number of physical UE panels

- Number of ports per UE panel

- Enhancement for connected mode or connected mode + idle mode

Regarding the maximum number of active UE panels, since only up to 2 TRPs are supported. It is reasonable to assume up to 2 active UE panels for a UE, where each panel can be used for uplink signal toward one TRP. On the maximum number of physical UE panels, usually this can be considered an implementation related issue. But it should be clarified that whether the Rel-18 STxMP would introduce any restriction on the number of physical UE panels. In addition, the number of ports per panel should be clarified. Usually the number of ports per panel should be 2. Last, it needs to be clarified whether the enhancement should focus on connected mode only or both connected mode and idle mode. Since the mTRP can be enabled for connected mode UE, it should be more reasonable to consider STxMP for connected mode only. Thus, the PUSCH enhancement for MsgA/Msg3 should be precluded.

In addition, usually UE cannot always support STxMP as a result of overheating issue. When a UE keeps communicating with a gNB from a Tx panel, the overheating issue could happen for this panel. The UE has to turn off the panel. If there is no additional Tx panel, the UE cannot support STxMP when such overheating issue happens. Therefore, it is necessary to study the solution to the overheating issue for STxMP.

Proposal 1: Rel-18 STxMP only considers UE with up to 2 active panels.

Proposal 2: Rel-18 STxMP supports UEs with more physical panels than active panels.

Proposal 3: The number of antenna ports per UE panel should be assumed to be 2.

Proposal 4; Rel-18 STxMP only considers the enhancement for connected mode UE, i.e. enhancement for MsgA/Msg3 PUSCH should be precluded.

Proposal 5: Study the solution to the overheating issue for STxMP, where the UE has to turn off one panel when overheating is detected for that panel.

# STxMP on PUSCH

## Single DCI based STxMP

### SDM scheme

Maximum number of layers for sTRP and SDM

In RAN1 #111, the following working assumption on maximum number of layers for sTRP and mTRP was achieved.

Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels

One open issue is whether to enable the same number of ports for SDM and PUSCH. This could depend on whether such same number of ports could provide the performance benefit or not. Figure 1 illustrates the system level results for single-TRP operation with 2-port per panel and 4-port per panel. The detailed simulation assumption is shown in Table A-1 in appendix. For 4-port per panel operation, the following two cases were studied:

- Case 1: 4-port per panel based on the same transmission power as the 2-port per panel case, i.e., the power control is based on 2 Rx port per panel

- Case 2: 4-port per panel with higher transmission power, where the power control is based on 4 Rx port per panel

Figure 1: Simulation results for sTRP operation with 2-port per panel and 4-port per panel

It can be observed that 4-port per panel with same transmission power can cause performance loss compared to 2-port per panel case, and 4-port per panel with higher transmission power can bring in performance benefit for cell center UEs, but it can cause performance loss for cell edge UEs. Therefore, it is unnecessary to introduce additional spec impact on the same number of ports for sTRP and mTRP operation.

Proposal 6: Do not support to introduce additional spec impact on the same number of ports for sTRP and mTRP operation.

PT-RS for SDM scheme

For SDM operation, the UE transmits the uplink signal from two panels. Usually, two PT-RS ports should be necessary. However, it still depends on the UE architecture on whether the two panels share the same oscillator or not. Therefore, it is better to introduce a UE capability to indicate whether the UE supports 1 or 2 PT-RS ports for SDM scheme.

Another issue is whether the PT-RS from one panel needs to be orthogonal with the PUSCH from the other panel, i.e., whether the REs for PT-RS in one panel can be available for PUSCH form the other panel. As the two panels are with opposite direction and different NW-UE beam pairs can be applied, such mutual interference between PT-RS and PUSCH could not be severe. Further, if the REs for PT-RS in one panel are not available for the PUSCH from the other panel, the UE may not be able to perform power boosting for the PT-RS as it cannot support cross-panel power borrowing. Therefore, it is unnecessary to reserve the resources from one panel to the PT-RS.

Proposal 7: Support the UE to report the UE capability to indicate whether the UE supports 1 or 2 PT-RS ports for SDM scheme.

Proposal 8: The REs for PT-RS corresponding to one TPMI/SRIs are “available” for PUSCH resource mapping corresponding to another TPMI/SRI.

### SFN scheme

For SFN scheme, the following on SRS configuration was agreed in RAN1 #111.

Agreement For single-DCI based STxMP SFN scheme, - Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB - Each PTRS port is transmitted in SFN manner

One remaining issue is how to indicate the PT-RS and DMRS association. For SFN scheme, the gNB can indicate two precoders. However, the strongest layer for each indicated precoder could be different. Then to make sure the PT-RS is associated with the strongest layer, a layer swap indicator can be introduced for the second indicated precoder so as to construct the strongest layer for PT-RS and DMRS port association as shown in Figure 2.

Figure 2: Layer swapping to construct the strongest layer for PT-RS and DMRS port association

Proposal 9: For SFN scheme, support to indicate layer swapping for the second indicated precoder in DCI to construct the strongest layer for both indicated precoders for PT-RS and DMRS port association.

## Multi-DCI based STxMP

Maximum number of layers

In RAN1 #110b, the following agreement on maximum number of layers was achieved.

Agreement Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2.

One open issue is whether the number of layers for each PUSCH should be up to 2. For sDCI based SDM scheme, currently only layer combination {1+2}, {2+1} and {1+1} is supported. Therefore, it is reasonable to define the maximum number of layers for each PUSCH to be up to 2.

Proposal 10: For mDCI based STxMP, support up to 2 layer transmission for a PUSCH.

Power sharing for two scheduled PUSCHs

For mDCI based STxMP, it is possible that the gNB schedules the two PUSCHs overlaps in time domain. Then the total transmission power in overlapped symbols may exceed the maximum transmission power as shown in Figure 3. When determining the transmission power for the PUSCH scheduled by the first DCI, the UE may or may not have decoded the second DCI, then it may or may not be able to determine the transmission power for the first scheduled PUSCH with regard to the second scheduled PUSCH. The UE may not be able to perform the dynamic power sharing for the two scheduled PUSCH, since the UE may not be able to “look ahead” a potential second scheduled PUSCH when determining the transmission power for the first scheduled PUSCH. Therefore, it is necessary to study the power sharing operation for the two scheduled PUSCHs.

Figure 3: A potential issue for power sharing for two scheduled PUSCHs in mDCI mode

Proposal 11: RAN1 shall study the power sharing for the PUSCHs from different panels.

Proposal 12: UE shall not be mandated to perform dynamic power sharing for the PUSCHs from different panels.

UCI multiplexing

For mDCI based STxMP, the gNB may schedule a PUCCH overlapped with two PUSCHs in time domain, as shown in Figure 3. In RAN1 #112, the following on UCI multiplexing was agreed.

Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH

Figure 3: A potential issue for UCI multiplexing for mDCI based PUSCH

The UE may transmit the UCI corresponding to one TRP or both TRPs in the PUCCH. It is better to consider a unified solution for both cases. For UCI corresponding to one TRP, the UE may multiplex the UCI on the PUSCH corresponding to one TRP. For UCI corresponding to more than one TRPs, the UE can transmit the UCI on both PUSCHs. Then both TRPs can receive the same UCI, which can improve the reliability for the UCI. Therefore, with regard to the reliability for the UCI, support to multiplex UCI on both PUSCHs that overlap with the PUCCH in time domain.

Proposal 13: Support to multiplex the UCI on both PUSCHs that overlap with the PUCCH in time domain.

# STxMP on PUCCH

To support STxMP on PUCCH, in general, there can be two options:

- Option 1: To transmit UCI on two PUCCH resources with one beam indicated per resource

- Option 2: To transmit UCI on one PUCCH resource with two beams indicated

In option 1, more flexible PUCCHs multiplexing schemes can be supported. The two PUCCH resources can be multiplexed in FDM/TDM/SDM manner, which is like mDCI based PUSCH. In option 2, the gNB can indicate two TCI states for a PUCCH resource, and the UCI can be transmitted in different repetitions with different beams for the PUCCH resource. The PUCCH multiplexing schemes could not be as flexible as option 1. Since some multiplexing manner is hard to be configured, e.g. partial overlapped based multiplexing scheme. Therefore, compared to option 2, option 1 is preferred.

Proposal 14: Support to transmit UCI on two PUCCH resources, where each PUCCH resource is indicated with one TCI.

# Conclusion

In this contribution, we provided discussion on STxMP enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: Rel-18 STxMP only considers UE with up to 2 active panels.

Proposal 2: Rel-18 STxMP supports UEs with more physical panels than active panels.

Proposal 3: The number of antenna ports per UE panel should be assumed to be 2.

Proposal 4; Rel-18 STxMP only considers the enhancement for connected mode UE, i.e. enhancement for MsgA/Msg3 PUSCH should be precluded.

Proposal 5: Study the solution to the overheating issue for STxMP, where the UE has to turn off one panel when overheating is detected for that panel.

Proposal 6: Do not support to introduce additional spec impact on the same number of ports for sTRP and mTRP operation.

Proposal 7: Support the UE to report the UE capability to indicate whether the UE supports 1 or 2 PT-RS ports for SDM scheme.

Proposal 8: The REs for PT-RS corresponding to one TPMI/SRIs are “available” for PUSCH resource mapping corresponding to another TPMI/SRI.

Proposal 9: For SFN scheme, support to indicate layer swapping for the second indicated precoder in DCI to construct the strongest layer for both indicated precoders for PT-RS and DMRS port association.

Proposal 10: For mDCI based STxMP, support up to 2 layer transmission for a PUSCH.

Proposal 11: RAN1 shall study the power sharing for the PUSCHs from different panels.

Proposal 12: UE shall not be mandated to perform dynamic power sharing for the PUSCHs from different panels.

Proposal 13: Support to multiplex the UCI on both PUSCHs that overlap with the PUCCH in time domain.

Proposal 14: Support to transmit UCI on two PUCCH resources, where each PUCCH resource is indicated with one TCI.

# Appendix

Table A-1: Simulation Assumption

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz, |
| Scenarios | Dense urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per cell), 100% outdoor |
| UE speed | Stationary UEs |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2), dH = 0.5 λ |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8 |
| UE dropping | Random |
| UE and panel orientation | Vertical but random in azimuth |
| Traffic Model | Full buffer |
| Control and RS overhead | 2 symbol overhead per slot |
| Scheduler | PF |
| UE Antenna height | 1.5 m |
| UL MIMO Mode, rank | UL SU-MIMO, rank 2 |
| Transmission scheme | Codebook based |
| MCS | MCS Table 1 |
| HARQ | Up to 3 retransmissions |
| Power control parameters | P0 = -80 dBm, alpha = 0.8 |