3GPP TSG RAN WG1 #112bis-e		R1-2303048

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.4.2

Source:	Google

Title:	On SRI/TPMI Indication for 8Tx Transmission

Document for:	Discussion/Decision

# Introduction

In RAN1 #112, the following agreements on SRI/TPMI indication for 8Tx transmission have been achieved.

Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2)` A pair of (N1, N2) can be configured with subject to UE capability. Agreement Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support - Alt.2: A second MCS field (5 bits) is indicated for the second codeword Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated. - FFS: Details on how to signal Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q{0,1}, where , and are defined similar to the legacy single CW PUSCH transmission. Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), - Study whether/how to support (O1, O2) = (2,1), (2,2) - whether for all rank, or rank 1-2, or rank 3-8 - applicability of different (O1, O2) values per agreed (N1, N2) - companies are encouraged to submit simulation results Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from, - Alt1: First CW - Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW) Agreement For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission. with the scaling factor of . Agreement For NCB-based 8TX PUSCH transmission with , where is the number of configured single-port SRS resources in a resource set, - All SRS port combinations are supported - For SRI indication, down-select from, - Option 1: Use an bit length bitmap - Option 2: Use a legacy-based solution - Consideration of Lmax for SRI indication For , Rel-15 SRI indication is reused Agreement For CB-based 8TX PUSCH transmission, where Mode 2 uplink full power transmission (if supported) is not used, re-use legacy Rel-15 mechanism, that is - when only one SRS resource in a resource set is configured, the SRI field in DCI is absent, - when two SRS resources are configured in a resource set, 1 bit of SRI field in DCI is used to indicate the selected SRS resource in the set. Agreement For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated.

In this contribution, we provide some discussions on SRI/TPMI indication for 8Tx transmission.

# Discussion

## UCI Multiplexing on Two CW

In RAN1 #112, the following is agreed on UCI multiplexing on two CW based uplink transmission.

Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from, - Alt1: First CW - Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

Usually, the CW with higher MCS should have a better channel condition. Thus, it is reasonable to multiplex the UCI on the CW with higher MCS. However, for retransmission, the gNB can indicate the reserved MCS as shown in Table 1, where the indicated MCS is always with a higher index than other unreserved MCS. But such reserved MCS does not always provide higher SE than other MCS. Therefore, the Alt2 could be supported for initial transmission. But for retransmission, as the gNB may indicate reserved MCS, the CW should be selected based on the MCS with highest SE instead of the highest MCS.

Table 1: MCS Table 1 defined in 38.214

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Proposal 1: Support a modified Alt2 for UCI multiplexing as follows:

- Alt2: The CW with the MCS with highest SE (if MCSs are the same, UCI is multiplex on the first CW)

## Spatial Domain Fallback operation

For an 8Tx UE, the gNB may configure it to transmit the uplink signal from 2-port or 4-port as a spatial-domain fallback operation. Currently, UE is only able to report the UE capability indicating one supported full power mode and codebook subset. However, for such fallback operation, the UE may support different operations. For example, the UE may support full power mode 1 or mode 2 for 8Tx, but the UE is able to support the full power mode 0 when it is configured for 4Tx operation. Similarly, the UE may support 4-port partial-coherent transmission for 8Tx, but it is able to support full coherent transmission when it is configured for 4Tx operation. Thus, it is necessary to introduce the UE capability for each number of configured ports for UL transmission.

Proposal 2: Support the 8Tx UE to report the UE capability with regard to spatial domain fallback operation, including:

- Supported UL full power mode when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

- Supported codebook coherency subset when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

## TRI/TPMI indication

In RAN1 #111, it was agreed to further down-select whether to indicate the TRI/TPMI jointly or separately as follows:

Agreement For CB-based 8TX PUSCH transmission, for rank indication, down-select among the following - Separate indication of TRI and TPMI - Joint indication of TRI and TPMI

Currently, TRI and TPMI are jointly coded. Several other features, e.g., mTRP, STxMP and so on, are already designed based on such operation. It is unnecessary to introduce a new TRI/TPMI indication scheme. Therefore, joint indication of TRI and TPMI should be supported.

Proposal 3: Support joint indication of TRI and TPMI for 8Tx PUSCH.

# Conclusion

In this contribution, we provided discussion on 8Tx enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: Support a modified Alt2 for UCI multiplexing as follows:

- Alt2: The CW with the MCS with highest SE (if MCSs are the same, UCI is multiplex on the first CW)

Proposal 2: Support the 8Tx UE to report the UE capability with regard to spatial domain fallback operation, including:

- Supported UL full power mode when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

- Supported codebook coherency subset when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

Proposal 3: Support joint indication of TRI and TPMI for 8Tx PUSCH.