3GPP TSG RAN WG1 #112bis-e		R1-2303050

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.2.1

Source:	Google

Title:	On Evaluation of AI/ML based CSI

Document for:	Discussion/Decision

# Introduction

In RAN1 #112, the following agreements on evaluation of AI/ML based CSI have been achieved.

Conclusion For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method. Agreement Confirm the following working assumption of RAN1#110bis-e: Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance. Conclusion For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory. - It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR) Agreement For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison Agreement Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111: Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction. Agreement The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) is added to the list of scenarios for performing the generalization verification. - FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs) Agreement For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered: - The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training” - A second separate template is introduced to capture the evaluation results for “multi-vendor joint training” - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training. - A third separate template is introduced to capture the evaluation results for “separate training” - FFS: additional KPIs for each template, e.g., overhead, latency, ect. Agreement For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison. - FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case Agreement For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers. - Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly. - FFS on the reported complexity and storage - FFS: input/output type - Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly. - FFS: input/output type - Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference. - FFS on the reported complexity and storage - Note: input/output type is Precoding matrix - Companies to report the setting is - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values) - Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference. - FFS on the reported complexity and storage - Note: input/output type is Precoding matrix - Companies to report whether the setting is - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers) - Other options not precluded. Agreement The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. - For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank. - For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank Working Assumption For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as: - X is <=80bits - Y is 100bits-140bits - Z is >=230bits Working Assumption X, Y and Z are applicable for per layer Working assumption The following initial template is considered to replace the template achieved in the working assumption in the RAN1#111 meeting, for companies to report the evaluation results of AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification - To be collected before 112bis-e meeting - FFS the description and results for generalization/scalability may need a separate table - Note: the values of CSI feedback overhead for the mean UPT and 5% UPT may need to be revisited in the 112bis-e meeting - FFS: training related overhead - FFS: how to capture CSI overhead reduction to the template - Note: It is to be captured to the template after a way is found on how to derive the CSI overhead reduction. Table X. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, [traffic type], [Max rank value], [RU] Source 1; …. CSI generation part; AI/ML model backbone. Pre-processing. Post-processing. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. CSI reconstruction part; AI/ML model backbone. [Pre-processing]. [Post-processing]. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. Common description; Input type. Output type. Quantization /dequantization method. Rank/layer adaptation settings for rank>1. Dataset description; Train/k. Test/k. Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters). Overhead reduction compared to Float32 if high resolution quantization of ground-truth CSI is applied. [Other assumptions/settings agreed to be reported]. Benchmark. Benchmark assumptions, e.g., CSI overhead calculation method (Optional). SGCS of benchmark, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. SGCS of benchmark, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for SGCS, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for SGCS, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. … (other layers). NMSE of benchmark, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. NMSE of benchmark, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for NMSE, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for NMSE, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. … (other layers). Other intermediate KPI (description/value) (optional). Gain for other intermediate KPI (description/value) (optional). Gain for Mean UPT (for a specific CSI feedback overhead); [CSI feedback payload X*Max rank value]. [CSI feedback payload Y*Max rank value]. [CSI feedback payload Z*Max rank value]. Gain for 5% UPT; [CSI feedback payload X*Max rank value]. [CSI feedback payload Y*Max rank value]. [CSI feedback payload Z*Max rank value]. Gain for upper bound without CSI compression over Benchmark –Mean UPT (Optional); [CSI feedback payload X*Max rank value]. [CSI feedback payload Y*Max rank value]. [CSI feedback payload Z*Max rank value]. Gain for upper bound without CSI compression over Benchmark –5% UPT (Optional); [CSI feedback payload X*Max rank value]. [CSI feedback payload Y*Max rank value]. [CSI feedback payload Z*Max rank value]. [CSI feedback reduction (%)]. …. FFS others. - Note: “Benchmark” means the type of Legacy CB used for comparison. - Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc. - Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part. Working assumption A separate table to capture the evaluation results of generalization/scalability verification for AI/ML-based CSI compression is given in the following initial template - To be collected before 112bis-e meeting - FFS whether the intermediate KPI results are gain over benchmark or absolute values - FFS whether the intermediate KPI results are in forms of linear or dB Table X. Evaluation results for CSI compression with model generalization/scalability, [Max rank value], [Scenario/configuration] Source 1; …. CSI generation part; AL/ML model backbone. Pre-processing. Post-processing. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. CSI reconstruction part; AL/ML model backbone. [Pre-processing]. [Post-processing]. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. Common description; Input type. Output type. Quantization /dequantization method. Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc.. Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s). Dataset description; Ground-truth CSI quantization method. [Other assumptions/settings agreed to be reported]. Generalization Case 1; Train (setting#A, size/k). Test (setting#A, size/k). SGCS, layer 1; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. SGCS, layer 2; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. NMSE, layer 1; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. NMSE, layer 2; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. … (other settings for Case 1). …. Generalization Case 2; Train (setting#A, size/k). Test (setting#B, size/k). … (results for Case 2). … (other settings for Case 2). Generalization Case 3; Train (setting#A+#B, size/k). Test (setting#A/#B, size/k). … (results for Case 3). … (other settings for Case 3). Fine-tuning case (optional); Train (setting#A, size/k). Fine-tune (setting#B, size/k). Test (setting#B, size/k). … (results for Fine-tuning). … (other settings for Fine-tuning). FFS others. - Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc. - Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part. Working Assumption The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction with generalization verification - To be collected before 112bis-e meeting - FFS whether the intermediate KPI results are gain over benchmark or absolute values - FFS whether the intermediate KPI results are in forms of linear or dB Table X. Evaluation results for CSI prediction with model generalization, [Max rank value] Source 1; …. AI/ML model description; AL/ML model description (e.g., backbone, structure). [Pre-processing]. [Post-processing]. FLOPs/M. Parameters/M. [Storage /Mbytes]. Input type. Output type. Assumption; CSI feedback periodicity. Observation window (number/distance). Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance). Whether/how to adopt spatial consistency. Generalization Case 1; Train (setting#A, size/k). Test (setting#A, size/k). SGCS (1,…N, N is number of prediction instances). NMSE (1,…N, N is number of prediction instances). … (other settings and results for Case 1). Generalization Case 2; Train (setting#A, size/k). Test (setting#B, size/k). SGCS (1,…N, N is number of prediction instances). NMSE (1,…N, N is number of prediction instances). … (other settings and results for Case 2). Generalization Case 3; Train (setting#A+#B, size/k). Test (setting#A/#B, size/k). SGCS (1,…N, N is number of prediction instances). NMSE (1,…N, N is number of prediction instances). … (other settings and results for Case 3). Fine-tuning case (optional); Train (setting#A, size/k). Fine-tune (setting#B, size/k). Test (setting#B, size/k). SGCS (1,…N, N is number of prediction instances). NMSE (1,…N, N is number of prediction instances). … (other settings and results for Fine-tuning). FFS others. Working Assumption The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for multi-vendor joint training and without generalization/scalability verification - To be collected before 112bis-e meeting - FFS whether the intermediate KPI results are gain over benchmark or absolute values - FFS whether the intermediate KPI results are in forms of linear or dB - FFS case of multiple layers Table X. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, [Max rank value] Source 1; …. Common description; Input type. Output type. [Training method]. Quantization /dequantization method. Dataset description; Train/k. Test/k. Ground-truth CSI quantization method. Case 1 (baseline): NW#1-UE#1; UE part AI/ML model backbone/structure. Network part AI/ML model backbone/structure. ... (other NW-UE combinations for Case 1). Case 2 (1 NW part to M>1 UE parts); NW part model backbone/structure. UE#1 part model backbone/structure. UE#1 part training dataset description and size. …. UE#M part model backbone/structure. UE#M part training dataset description and size. Case 3 (N>1 NW parts to 1 UE part); UE part model backbone/structure. NW#1 part model backbone/structure. NW#1 part training dataset description and size. …. NW#N part model backbone/structure. NW#N part training dataset description and size. Intermediate KPI type (SGCS/NMSE). FFS other cases. Case 1: NW#1-UE#1: Intermediate KPI; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. … (results for other NW-UE combinations for Case 1). Case 2: Intermediate KPI; CSI feedback payload X, NW-UE#1. …. CSI feedback payload X, NW-UE#M. CSI feedback payload Y …. CSI feedback payload Z …. Case 3: Intermediate KPI; CSI feedback payload X, NW#1-UE. …. CSI feedback payload X, NW#N-UE. CSI feedback payload Y …. CSI feedback payload Z …. FFS other cases. FFS others. - Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc. - Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction par Working Assumption The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for sequentially separate training and without generalization/scalability verification - To be collected before 112bis-e meeting - FFS whether the intermediate KPI results are gain over benchmark or absolute values - FFS whether the intermediate KPI results are in forms of linear or dB - FFS case of multiple layers Table X. Evaluation results for CSI compression of separate training without model generalization/scalability, [Max rank value] Source 1; …. Common description; Input type. Output type. Quantization /dequantization method. Shared output of CSI generation part/input of reconstruction part is before or after quantization. Dataset description; Test/k. Ground-truth CSI quantization method. [Benchmark: NW#1-UE#1 joint training]; UE part AI/ML model backbone/structure. Network part AI/ML model backbone/structure. Training dataset size. ... (other NW-UE combinations for benchmark). Case 1-NW first training; NW part AI/ML model backbone/structure. UE#1 part model backbone/structure. UE#1 part training dataset description and size. …. UE#M part model backbone/structure. UE#M part training dataset description and size. [air-interface overhead of information (e.g., dataset) sharing]. Case 1-UE first training; NW#1 part model backbone/structure. NW#1 part training dataset description and size. …. NW#N part model backbone/structure. NW#N part training dataset description and size. UE part model backbone/structure. [air-interface overhead of information (e.g., dataset) sharing]. Case 2-UE first training; UE#1 part model backbone/structure. …. UE#M part model backbone/structure. UE part AI/ML model backbone/structure. NW part training dataset description and size (e.g., description/size of dataset from M UEs and how to merge). Case 3-NW first training; NW#1 part model backbone/structure. …. NW#N part model backbone/structure. UE part model backbone/structure. UE part training dataset description and size (e.g., description/size of dataset from N NWs and how to merge). Intermediate KPI type (SGCS/NMSE). FFS other cases. NW#1-UE#1 joint training: Intermediate KPI; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. … (results for other 1-on-1 NW-UE joint training combinations). Case 1-NW first training: Intermediate KPI; CSI feedback payload X, NW-UE#1. …. CSI feedback payload X, NW-UE#M. CSI feedback payload Y …. CSI feedback payload Z …. Case 1-UE first training: Intermediate KPI; CSI feedback payload X, NW#1-UE. …. CSI feedback payload X, NW#N-UE. CSI feedback payload Y …. CSI feedback payload Z …. Case 2-NW first training: Intermediate KPI; CSI feedback payload X, NW#1-UE. …. CSI feedback payload X, NW#N-UE. CSI feedback payload Y …. CSI feedback payload Z …. Case 3-NW first training: Intermediate KPI; CSI feedback payload X, NW-UE#1. …. CSI feedback payload X, NW-UE#M. CSI feedback payload Y …. CSI feedback payload Z …. FFS other cases. FFS others. - Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc. - Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

In this contribution, we provide some discussion on evaluation of AI/ML based CSI.

# CSI compression

Currently, the SGCS has been agreed as the intermediate KPI. However, SGCS cannot actually reflect the performance gap between two CSIs. Figure 1 illustrates one simulation result to compare the SE offset with two precoders with a certain SCS. It can be observed that low SCS does not always produce large performance gap. Therefore, although the SGCS can be considered as an intermediate KPI, it should not be used for model performance monitoring.

Figure 1: Simulation results on SGCS vs SE offset for two precoders

Observation 1: SGCS cannot indicate the status of performance gap for two CSIs.

Proposal 1: Model monitoring should not be based on SCS.

In addition, for throughput evaluation, the rank adaptation and CQI measurement scheme should be clarified. There can be the following measurement schemes:

- Scheme 1: RI/CQI is always measured based on the ideal precoders

- Scheme 2: RI/CQI is measured based on the decompressed precoders

- Scheme 3: RI is measured based on the ideal precoders and CQI is measured based on the decompressed precoders

- Scheme 4: RI is measured based on the decompressed precoders and CQI is measured based on the ideal precoders

Figure 2 and Figure 3 illustrates the system level performance for the schemes above. Table A-1 in appendix illustrates the details simulation assumption for the system level simulation. It can be observed that different schemes can lead to different performance.

Figure 2: Average user throughput gain for different RI/CQI measurement schemes

Figure 3: Cell edge user throughput gain for different RI/CQI measurement schemes

Observation 2: Different RI/CQI measurement scheme could cause different performance.

Proposal 2: The RI/CQI measurement scheme should be clarified for the system performance evaluation.

We evaluate the ML based CSI compression with 100K UEs in dense urban macro with 80% UEs for training, 10% UEs for validation and 10% UEs for testing. The input of the ML is the channel eigenvector for each subband. For eType2 CSI, for simple comparison, it is assumed the payload size for each layer is the same. The detailed simulation assumption is shown in Table A-1. Table 1 illustrates the simulation results.

Table 1: Simulation results for ML based CSI compression based on channel eigenvector

|  |  | Google |
| --- | --- | --- |
| CSI generation part | AL/ML model backbone | CNN |
|  | Pre-processing | Channel eigenvector calculation for each subband |
|  | Post-processing | Quantization (4 bits per coefficient) |
|  | FLOPs/M | 1M |
|  | Number of parameters/M | 0.1M |
|  | [Storage /Mbytes] | 0.1M |
| CSI reconstruction part | AL/ML model backbone | CNN |
|  | [Pre-processing] | De-quantization (4 bits per coefficient) |
|  | [Post-processing] | N/A. |
|  | FLOPs/M | 1M |
|  | Number of parameters/M | 0.1M |
|  | [Storage /Mbytes] | 0.1M |
| Common description | Input type | Rank 1 channel eigenvector for each subband |
|  | Output type | Decompressed rank 1 channel eigenvector for each subband |
|  | Quantization /dequantization method | Linear quantization per coefficient (4-bits per coefficient) |
|  | Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc. |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) |  |
| Dataset description | Ground-truth CSI quantization method | Channel eigenvector calculation |
| SGCS, layer 1 | CSI feedback payload 60 bits | 0.7762 (-0.6%) |
|  | CSI feedback payload 168 bits | 0.8250 (+0.5%) |
|  | CSI feedback payload 280 bits | 0.8776 (+4.0%) |
| SGCS, layer 2 | CSI feedback payload 60 bits | 0.6349 (+10.3%) |
|  | CSI feedback payload 168 bits | 0.6820 (+13.0%) |
|  | CSI feedback payload 280 bits | 0.7650 (+22.4%) |
| SGCS, layer 3 | CSI feedback payload 60 bits | 0.4647 (+24.9%) |
|  | CSI feedback payload 168 bits | 0.5364 (+12.0%) |
|  | CSI feedback payload 280 bits | 0.6200 (+16.3%) |
| SGCS, layer 4 | CSI feedback payload 60 bits | 0.3804 (+36.1%) |
|  | CSI feedback payload 168 bits | 0.4599 (+15.6%) |
|  | CSI feedback payload 280 bits | 0.5359 (+24.8%) |

Observation 3: For rank 1 case, the ML based CSI compression cannot provide significant gain on SGCS.

Observation 4: For rank >1 case, the ML based CSI compression can provide significant gain on SGCS.

# CSI prediction

In RAN1 #110b, the following agreement on the input of CSI prediction was achieved.

Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations: - Raw channel matrixes - Eigenvector(s)

As discussed in CSI compression, a more practical way is to use the eigenvectors of the wideband precoded channel as the input instead of the eigenvectors for each subband channel. Compared to calculating the eigenvectors for each subband channel, calculating the eigenvectors of the wideband precoded channel could require less UE complexity. Therefore, the further study of the CSI prediction should focus on the input based on the eigenvectors of the wideband precoded channel.

In addition, currently the Type1 codebook is widely implemented, which is a mandatory feature for Rel-15 UE. The CSI prediction should also consider the Type1 codebook as the starting point. The output for the CSI prediction could the predicted PMI based on a Type1 codebook. Then the CSI prediction could become a classification issue instead of a regression issue. Moreover, with the help of the CSI prediction, the UE can also predict the CSI dwelling time, which could be much helpful for the network to determine when to trigger the CSI feedback. Thus, it is necessary to study the CSI prediction with CSI dwelling time as the output.

Proposal 3: The study of the input of CSI prediction should prioritize the input based on the eigenvectors of the raw channel with a wideband precoder selected as SD basis, e.g. HW1.

Proposal 4: Study the following output of CSI prediction:

- Predicted RI/PMI based on Type1 codebook

- Predicted CSI dwelling time

# Conclusion

In this contribution, we provided discussion on evaluation of AI/ML based CSI compression. Based on the discussion, the following observations and proposals have been achieved.

Observation 1: SGCS cannot indicate the status of performance gap for two CSIs.

Observation 2: Different RI/CQI measurement scheme could cause different performance.

Observation 3: For rank 1 case, the ML based CSI compression cannot provide significant gain on SGCS.

Observation 4: For rank >1 case, the ML based CSI compression can provide significant gain on SGCS.

Proposal 1: Model monitoring should not be based on SCS.

Proposal 2: The RI/CQI measurement scheme should be clarified for the system performance evaluation.

Proposal 3: The study of the input of CSI prediction should prioritize the input based on the eigenvectors of the raw channel with a wideband precoder selected as SD basis, e.g. HW1.

Proposal 4: Study the following output of CSI prediction:

- Predicted RI/PMI based on Type1 codebook

- Predicted CSI dwelling time

# Appendix

Table A-1: System level simulation results

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 64QAM |
| Scheduler |  | PF with open-loop link adaptation |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 30kHz |
| MIMO scheme |  | SU-MIMO |
| CSI feedback periodicity |  | 5 ms |
| Overhead |  | 2 symbol overhead |
| Traffic model |  | Full buffer |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |