3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303059

# e-meeting, April 17th – 26th, 2023

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on candidate slots for aperiodic transmission in partial sensing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For aperiodic transmission in a resource pool enabling periodical reservation, if the Y’ candidate slots selected by the UE is less than Ymin’, it is up to UE implementation to include other candidate slots. The other candidate slots selected by the UE do not have enough corresponding PBPS sensing results, i.e. the UE does not monitor all the PBPS sensing occasions associated with such candidate slots. In determination of the sensing occasions for PBPS, the UE shall monitor all the sensing occasions of ty-k×Preserve''SL associated with a candidate slot ty'SL and ty'SL is any slot of all the selected candidate slots including the other candidate slots as mentioned in the above case, which contradicts with the case that the included other candidate slots do not have enough PBPS sensing results. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that ty'SL is a slot of the selected candidate slots except for the other candidate slots included when Y’<Ymin’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The periodical sensing occasions include slots that the UE does not monitor. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged text omitted>

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource for UE performing periodic-based partial sensing together with contiguous partial sensing and resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), or in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing at least contiguous partial sensing and resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0), where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate slots Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots except for the other candidate slots when Y' is smaller than Ymin' and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors sensing occasion(s) determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31.

When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots.

<Unchanged text omitted>