3GPP TSG RAN WG1 Meeting #112bis-e	R1-2303063

e-Meeting, April 17th - 26th, 2023

Source:	Sharp

Title:	SL positioning reference signal

Agenda Item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

A new work item on expanded and improved NR Positioning was approved in RAN#98-e meeting [1], with one of the objectives to “specify solutions for support of sidelink positioning (including ranging) in NR systems”, where one of the detailed objectives is as follows:

- Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

In this document, we share our views on a few aspects of SL positioning reference signal.

Discussion

SL-PRS sequence design

In RAN1#112, the following on SL-PRS sequence generation and initialization were agreed,

Agreement SL PRS sequence is generated based on Gold sequence: rn=121−2⋅c2n+j121−2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095.

And the following on SL-PRS multiplexing was also agreed,

Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs.

With the support for comb-based multiplexing of SL-PRS from different UEs in a slot, it is desirable to add RE offset (of the first SL-PRS symbol) as an additional parameter for SL-PRS sequence initialization, such that multiple SL-PRS transmissions from different UEs multiplexed in a same SL-PRS comb-based pattern use different scrambled SL-PRS sequences.

Proposal 1: In addition to the agreed slot number, symbol number, and nID,seqSL-PRS, RE offset is used as a parameter for SL-PRS sequence initialization.

SL-PRS resources and resource sets

In RAN1#112, SL-PRS transmissions with and without periodic reservations were agreed, as follows,

Agreement For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering

With the above agreements we don’t think the definition of “DL-PRS resource” (which is always periodic) can be reused. Instead, a “SL-PRS resource” should be defined such that it corresponds to a single SL-PRS transmission with resources configured within a comb.

Proposal 2: A SL-PRS resource corresponds to one RE offset of the first SL-PRS symbol within a comb-based SL-PRS pattern.

And since it has been agreed that periodic reservation with a similar mechanism as in legacy SL communication should be used, we don’t think there is a need to define/configure any “SL-PRS resource set” by reusing the concept of DL-PRS resource set. A UE can indicate periodic reservation starting from whichever slot a corresponding SCI is transmitted.

Proposal 3: “SL-PRS resource set” is not supported.

Proposal 4: For periodic reservation of SL-PRS resources, the periodicity values supported in legacy SL communications are considered as a baseline.

Comb-based SL-PRS patterns

The following was agreed in RAN1#112,

Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering.

Regarding the FFS on (M,N)=(6,6) for full staggering, it can be useful for some SL configurations where only partial symbols in a slot are configured for SL, hence we propose to support it.

Proposal 5: The SL-PRS pattern with (M,N)=(6,6) is supported.

For simplicity, a dedicated resource pool or a shared resource pool should be configured with a single pair of (M, N). Support for multiple pairs of (M, N) can be realized by configuring multiple dedicated resource pool(s) and/or multiple shared resource pool(s).

Proposal 6: A single pair of (M, N) is configured in a dedicated resource pool or a shared resource pool.

Symbols other than for SL-PRS transmissions

AGC training and TX/RX turnaround

The design of AGC and TX/RX turnaround for a legacy sidelink slot structure (for PSCCH/PSSCH/PSFCH transmissions) can be reused here, i.e. one symbol is used for AGC training, and one symbol is used for RX/TX turnaround.

Proposal 7: Regarding AGC training and TX/RX turnaround, one symbol is used for AGC training, and one symbol is used for RX/TX turnaround.

Associated PSCCH transmissions

The following was agreed in RAN1#112,

Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

With either “Option 2” i.e. PSCCH which carries SCI associated with SL-PRS transmission(s) is included, or “Option 3” i.e. PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included, one remaining issue is which symbols are used for the PSCCH carrying SCI associated with SL-PRS transmission(s). In order to simplify the design it is proposed that the first few starting SL symbols in a slot are used for PSCCH (similarly to legacy SL communications). Mapping of a PSCCH resource to a SL-PRS resource in a slot can be based on index of the SL-PRS transmission occasion and the RE offset of the SL-PRS resource in the SL-PRS transmission occasion.

Proposal 8: For a dedicated resource pool, the first few SL symbols in a slot are used for PSCCH.

- The number of PSCCH symbols in a slot is (pre-) configured.

Proposal 9: Mapping of a PSCCH resource to a SL-PRS resource in a slot can be based on index of the SL-PRS transmission occasion and the RE offset of the SL-PRS resource in the SL-PRS transmission occasion.

SL-PRS bandwidth

The following was agreed in the study item phase,

Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

Considering resource configuration flexibility, at least for dedicated resource pools we think it makes sense to allow the total bandwidth of a resource pool being larger than that of the SL-PRS transmission bandwidth. Therefore, we propose to adopt Alt. 1. Further study can be conducted for shared resource pools, when the framework for multiplexing SL-PRS resources with legacy PSCCH/PSSCH resources is agreed.

Proposal 10: With regards to the bandwidth of SL-PRS transmission, at least for a dedicated resource pool, adopt Alt. 1 (i.e. the bandwidth of SL-PRS can be same or smaller than that of the resource pool).

Conclusion

In this contribution, we discuss a few aspects relating to SL positioning reference signal, and make the following proposals.

Proposal 1: In addition to the agreed slot number, symbol number, and nID,seqSL-PRS, RE offset is used as a parameter for SL-PRS sequence initialization.

Proposal 2: A SL-PRS resource corresponds to one RE offset of the first SL-PRS symbol within a comb-based SL-PRS pattern.

Proposal 3: “SL-PRS resource set” is not supported.

Proposal 4: For periodic reservation of SL-PRS resources, the periodicity values supported in legacy SL communications are considered as a baseline.

Proposal 5: The SL-PRS pattern with (M,N)=(6,6) is supported.

Proposal 6: A single pair of (M, N) is configured in a dedicated resource pool or a shared resource pool.

Proposal 7: Regarding AGC training and TX/RX turnaround, one symbol is used for AGC training, and one symbol is used for RX/TX turnaround.

Proposal 8: For a dedicated resource pool, the first few SL symbols in a slot are used for PSCCH.

- The number of PSCCH symbols in a slot is (pre-) configured.

Proposal 9: Mapping of a PSCCH resource to a SL-PRS resource in a slot can be based on index of the SL-PRS transmission occasion and the RE offset of the SL-PRS resource in the SL-PRS transmission occasion.

Proposal 10: With regards to the bandwidth of SL-PRS transmission, at least for a dedicated resource pool, adopt Alt. 1 (i.e. the bandwidth of SL-PRS can be same or smaller than that of the resource pool).

References

RP-223549, “New WI: Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson, RAN#98e.