3GPP TSG RAN WG1 #112bis-e	R1-2303066

e-Meeting, April 17th – 26th, 2023

Source:	Sharp

Title:	Views on UL multi-panel transmission

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

Background

In RAN plenary #94e, the work item on MIMO Evolution for Downlink and Uplink was approved [1]. According to the WID, the following work item needs to be studied, and if justified, specified.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

In this contribution, we share our views on UL multi-panel transmission.

Discussions

  - Single-DCI based STxMP PUSCH

For the supporting 2CW in single-DCI based STxMP SDM scheme, the following conclusion and agreement were made.

Conclusion There is no consensus to support 2 CWs in single-DCI based STxMP SDM scheme Agreement Confirm the working assumption with the following updates: Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with one CW - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

There has been much discussion on 2CW at previous meetings and it was agreed at the last meeting that 2CW transmission is not supported in the SDM scheme. For the other discussion, we should not spend any more time.

Proposal 1: No more discussion for 2CW in SDM scheme.

For dynamic switching between single DCI based SDM scheme/SFN scheme and single TRP transmission, the following working assumption and agreement were made.

Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels Agreement On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN Agreement (RAN1#111) For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission: - Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission. - FFS: how to interpret each codepoint of “SRS resource set indicator”. - For the maximal number of layers for sTRP transmission, down-select: - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec - Option2: Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec. - Down-select one from the following for maximal number of layers of SDM transmission: - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately. - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately. - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission. - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately. FFS: To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

Regarding to the dynamic switching, a number of options were proposed at the RAN1#111 meeting. The main issues are the maxRank (or Lmax) settings and how they apply to mTRP and sTRP transmissions. In order to form a smooth consensus at this meeting, we should first unify our understanding of the Option1 and Alt1 and confirm the working assumption at the previous meeting.

Proposal 2: We should unify the understanding of the Option1 and Alt1.

Proposal 3: Confirm the working assumption for the dynamic switching between SDM scheme and sTRP transmission.

For maxRank, we recognize that the working assumption with the combination of Option1 and Alt1, sets two maxRanks for UEs that support the SDM scheme. One is the maxRank as in the current specification and another is the maxRank_SDM (as we call it in the contribution) separated from the existing maxRank. When sTRP transmission, the maximal number of layers is configured by the maxRank, i.e. current procedure is applied. When SDM scheme transmission, the maximal number of layers for each panel (1st SRS resource set and 2nd SRS resource set) is configured by the maxRank_SDM, i.e. two panels follow the procedure with same maxRank configuration. A temporary name should be given to the maxRank for the SDM scheme as maxRank_SDM to avoid misunderstandings.

Observation 1: Working assumption indicate the two maxRank configuration corresponding to the SDM scheme and sTRP transmission respectively.

- One is the maxRank in the current specification

- Another is the maxRank for SDM scheme separated from existing configuration

Proposal 4: A temporary name should be given to the maxRank for the SDM scheme as maxRank_SDM to avoid misunderstandings.

FFS and Note show the two cases that digital ports are shared between the panels and can’t be shared. As described in the second Note, RAN1 should support the both implementations. In the case that digital ports are shared between the panels, maxRank can be configured a larger value than the maxRank_SDM. On the other hand, if UE can’t share the digital ports, maxRank must be configured to be up to the same value as the maxRank_SDM. This means that if maxRank_SDM is 2, maxRank can be set to 4 only for UEs that have capability of digital ports sharing. Therefore, we should support a way to explicitly or implicitly notify whether digital port sharing is possible. For example, introducing the associated capability.

Observation 2: If UE can’t share the digital ports between the panels, maxRank must be configured to be up to same value as maxRank_SDM.

Proposal 5: Support the explicitly or implicitly indication whether digital port sharing is possible. E.g. introducing the associated capability.

As described the agreement in RAN1#111, we should discuss how to interpret each codepoint of SRS resource set indicator for dynamic switching between sTRP and SDM scheme. To achieve agreement earlier, we should consider the specification based on the following latest proposal discussed in the previous meeting.

FL Proposal for Round 2 discussion (RAN1#112) Proposal 1-1b: The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For DCI field interpretation, down-select one from the following Alts: - Alt-a1: For CB PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB, the 1st SRI field indicates the precoding/rank. FFS: 2nd TPMI/SRI field are reserved/unused or absent. - Alt-a2: For CB PUSCH, concatenating the 1st and 2nd TPMI fields indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB, concatenating the 1st and 2nd SRI fields indicate the precoding/rank. - Alt-a3: - When codepoint is 00: for CB PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB, the 1st SRI field indicates the precoding/rank. - When codepoint is 01: for CB PUSCH, the 2nd TPMI field indicates the precoding/rank and the 2nd SRI field indicates the SRS resource. For NCB, the 2nd SRI field indicates the precoding/rank. - The other TPMI/SRI fields are unused. - The codepoints 10 indicate SDM transmission with the first and second SRS resource set. - The 1st TPMI/SRI fields indicate the precoding/rank/SRI for the first SRS resource set and the 2nd TPMI/SRI fields indicate the precoding/rank/SRI for the second SRS resource set. - For the codepoint 11, consider and down-select one from the following Alts: - Alt-b1: this codepoint is reserved/unused - Alt-b2: it indicates SDM scheme with two SRS resource set. The 1st TPMI/SRI fields indicate the precoding/rank/SRI for the second SRS resource set and the 2nd TPMI/SRI fields indicate the precoding/rank/SRI for the first SRS resource set - Alt-b3: the refined Option 2 in Proposal 1-6:

As in the current specification for the Rel-17 mTRP PUSCH TDM scheme, SRS resource set indicator is configured as the following table 1.

Table 1: SRS resource set indicator for Rel-17 mTRP PUSCH TDM

| Codepoint | scheme | SRS resource set indication |
| --- | --- | --- |
| 00 | sTRP | 1st SRI/TPMI fields are associated with 1st SRS resource set 2nd SRI/TPMI fields are reserved |
| 01 | sTRP | 1st SRI/TPMI fields are associated with 2nd SRS resource set 2nd SRI/TPMI fields are reserved |
| 10 | mTRP | 1st SRI/TPMI fields are associated with 1st SRS resource set 2nd SRI/TPMI fields are associated with 2nd SRS resource set |
| 11 | mTRP | 1st SRI/TPMI fields are associated with 1st SRS resource set 2nd SRI/TPMI fields are associated with 2nd SRS resource set |

It is a common understanding that the Codepoint 00 and 01 should be used for sTRP and 10 and 11 for mTRP according to the Rel-17 principles. At each code point, each SRI/TPMI filed needs to determine the corresponding SRS resource set. For dynamic switching, the DCI sizes must be aligned, i.e. the largest DCI size required for switching should be used as the reference. If an SRS resource set indicator is present, a 2nd SRI/TPMI field is also present. Therefore, when STxMP is supported, a DCI bitwidth corresponding to two fields is always required, regardless of sTRP and mTRP. Regarding to the SRS resource sets, it is expected that each SRS resource set is associated with each panel, and that switching between transmitting panels in sTRP transmission is particularly useful in cases such as back-to-back panel configurations. 
From the above, as an essential feature, Codepoint 00 and 01 related to sTRP transmission must be related to the 1st SRS resource set and 2nd SRS resource set. In addition, although the bitwidth for the 2nd SRI/TPMI field must always be kept, the Rel-17 specification shown in Table 1 indicates that the 2nd SRI/TPMI field is reserved in sTRP mode. Therefore, the interpretation of the SRS resource set indicator should be set as shown in Table 2 below.

Table 2: SRS resource set indicator for Rel-18 STxMP PUSCH SDM

| Codepoint | scheme | SRS resource set indication |
| --- | --- | --- |
| 00 | sTRP | 1st SRI/TPMI fields are associated with 1st SRS resource set 2nd SRI/TPMI fields are reserved |
| 01 | sTRP | 1st SRI/TPMI fields are associated with 2nd SRS resource set 2nd SRI/TPMI fields are reserved |
| 10 | SDM | 1st SRI/TPMI fields are associated with 1st SRS resource set 2nd SRI/TPMI fields are associated with 2nd SRS resource set |
| 11 | SDM | 1st SRI/TPMI fields are associated with 2nd SRS resource set 2nd SRI/TPMI fields are associated with 1st SRS resource set |

Proposal 6: Support the following interpretation of SRS resource set indicator field for Rel-18 STxMP PUSCH SDM.

- Codepoint 00:

  - 1st SRI/TPMI fields are associated with 1st SRS resource set

  - 2nd SRI/TPMI fields are reserved

- Codepoint 01:

  - 1st SRI/TPMI fields are associated with 2nd SRS resource set

  - 2nd SRI/TPMI fields are reserved

- Codepoint 10:

  - 1st SRI/TPMI fields are associated with 1st SRS resource set

  - 2nd SRI/TPMI fields are associated with 2nd SRS resource set

- Codepoint 11:

  - 1st SRI/TPMI fields are associated with 2nd SRS resource set

  - 2nd SRI/TPMI fields are associated with 1st SRS resource set

For max number of PTRS ports in single DCI based SDM scheme/SFN scheme, the following agreements were made.

Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission Agreement For single-DCI based STxMP SFN scheme, - Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB - Each PTRS port is transmitted in SFN manner

The maximum number of PTRS ports is currently given by maxNrofPorts, and the agreement at the last meeting indicates that the number of PTRS ports for SDM is 2. Although, it is not clear how maxNrofPorts is set and how the actual number of PTRS ports is determined in the case of UEs supporting STxMP with one or two oscillators connected to each panel or two oscillators connected to only one panel. Therefore, the UE should report the maximum number of PTRS ports corresponding to each panel.

Proposal 7: The maximum number of PTRS ports corresponding to each panel.

  - Power control for STxMP

STxMP is considered for transmission to multiple TRPs simultaneously. For example, if each panel transmits based on conventional PCMAX, at any given moment the regulated transmission power may be exceeded. Thus, it is necessary to specify the appropriate power control for each transmission occasion. In addition, the dynamic switching between sTRP and SDM requires to obtain information for power control per panels. Therefore, for proper power control, at least per-panel open-loop and closed-loop power control and power limits for multi-panel PUSCH transmission and panel-specific PHR reporting should be considered.

Proposal 8: We should discuss the following at least to support proper power control for STxMP.

- Power limits for multi-panel transmission

- Per-panel open/closed-loop power control

- Panel-specific PHR reporting

If PCMAX,1 and PCMAX,2 are maximum transmit power for panel#1 and panel#2 respectively, two cases can be assumed under the assumption that sum of PCMAX,1 and PCMAX,2 is less than PCMAX. Case 1: PCMAX,1 = PCMAX,2 = PCMAX/2, and Case 2: PCMAX,1 ≠ PCMAX,2. In case 2, PCMAX,1 and PCMAX,1 would be changed according to MAC CE or RRC parameters. Case 1 is a simple way and Case 2 have more flexibility, therefore we should support case 1 as a default function and case 2 based on UE capability.

Proposal 9: Support the power limitation for multi-panel transmission as follows.

- (Default): PCMAX,1 = PCMAX,2 = PCMAX/2

- (Capability): PCMAX,1 ≠ PCMAX,2, (PCMAX,1 + PCMAX,2 ≤ PCMAX)

Conclusion

In this contribution, we have the following proposal:

Proposal 1: No more discussion for 2CW in SDM scheme.

Proposal 2: We should unify the understanding of the Option1 and Alt1.

Proposal 3: Confirm the working assumption for the dynamic switching between SDM scheme and sTRP transmission.

Proposal 4: A temporary name should be given to the maxRank for the SDM scheme as maxRank_SDM to avoid misunderstandings.

Proposal 5: Support the explicitly or implicitly indication whether digital port sharing is possible. E.g. introducing the associated capability.

Proposal 6: Support the following interpretation of SRS resource set indicator field for Rel-18 STxMP PUSCH SDM.

- Codepoint 00:

  - 1st SRI/TPMI fields are associated with 1st SRS resource set

  - 2nd SRI/TPMI fields are reserved

- Codepoint 01:

  - 1st SRI/TPMI fields are associated with 2nd SRS resource set

  - 2nd SRI/TPMI fields are reserved

- Codepoint 10:

  - 1st SRI/TPMI fields are associated with 1st SRS resource set

  - 2nd SRI/TPMI fields are associated with 2nd SRS resource set

- Codepoint 11:

  - 1st SRI/TPMI fields are associated with 2nd SRS resource set

  - 2nd SRI/TPMI fields are associated with 1st SRS resource set

Proposal 7: The maximum number of PTRS ports corresponding to each panel.

Proposal 8: We should discuss the following at least to support proper power control for STxMP.

- Power limits for multi-panel transmission

- Per-panel open/closed-loop power control

- Panel-specific PHR reporting

Proposal 9: Support the power limitation for multi-panel transmission as follows.

- (Default): PCMAX,1 = PCMAX,2 = PCMAX/2

- (Capability): PCMAX,1 ≠ PCMAX,2, (PCMAX,1 + PCMAX,2 ≤ PCMAX)

Observation 1: Working assumption indicate the two maxRank configuration corresponding to the SDM scheme and sTRP transmission respectively.

- One is the maxRank in the current specification

- Another is the maxRank for SDM scheme separated from existing configuration

- Observation 2: If UE can’t share the digital ports between the panels, maxRank must be configured to be up to same value as maxRank_SDM.

References

RAN Meeting#94, RP-213598, New WID: MIMO Evolution for Downlink and Uplink

RAN1 Meeting#112, Chair's notes RAN1#112.