3GPP TSG RAN WG1 #112bis-e	R1- 2303067

e-Meeting, April 17th – 26th, 2023

Source:	Sharp

Title:	Views on 8 TX UL transmission

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

Background

In RAN plenary #94e, the work item on MIMO Evolution for Downlink and Uplink was approved [1]. According to the WID, the following work item needs to be studied, and if justified, specified.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we share our views on 8TX UL transmission.

Discussions

  - Codebook design for fully-coherent

For codebook design of fully-coherent UE, the following agreements were achieved.

Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2) A pair of (N1, N2) can be configured with subject to UE capability. Agreement Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), - Study whether/how to support (O1, O2) = (2,1), (2,2) - whether for all rank, or rank 1-2, or rank 3-8 - applicability of different (O1, O2) values per agreed (N1, N2) - companies are encouraged to submit simulation results

For supported antenna layout, we need to determine pairs of (N1, N2) values. So far, (N1, N2) = (4,1) and (2,2) have been agreed. The values are reasonable because that are specified for conventional specification of DL Type I codebook. In addition, the information should be based on UE capability. There seems to be no reason why other values should be supported. Therefore, no other value is required for 8TX UL transmission.

Observation 1: It is sufficient to support the pairs of (N1, N2) = {(4,1), (2,2)} with capability for 8TX UL transmission.

Proposal 1: Rel-18 8TX does not support the pairs of values other than (N1, N2) = {(4,1), (2,2)}.

No particular change is necessary when DL Type I codebook is used for UL 8TX codebook, but the granularity of the beam should be discussed. In the RAN1#110 meeting, some combination of oversampling value (O1, O2) = {(1,1), (2,1), (2,2)} were proposed. For the oversampling ratio O1, O2, a very large value should not be used to prevent an increase in TPMI overhead. Also, too high value is not feasible for UL transmission due to UE’s capability. Companies were encouraged to submit simulation results at the last meeting, and multiple results submitted by RAN1#112 have shown that performance improves as (O1, O2) increases. We agree that performance comparisons should be made with increased (O1, O2). Table 1 summarizes the simulation results of performance increasing with (O1, O2) and Table 2 shows the assumption. As Table 1 shows, (2,2) deserve to be supported because of the expected increase in throughput with increased (O1, O2). On the other hand, to support high O1O2, the UE needs to have high phase calibration capability. Therefore, the UE should report the capability of supported (O1, O2).

Proposal 2: Support the pairs of oversampling value (O1, O2) = {(1,1), (2,1), (2,2)}.

Proposal 3: UE should report the capability of supported (O1, O2).

Table 1: Performance gain with increasing (O1, O2)

| (O1, O2) = (1,1) (2,2) | 50%ile UE Throughput | 95%ile UE Throughput |
| --- | --- | --- |
| maxRank = 2 | +3.6% | +11% |
| maxRank = 8 | +0.2% | +6.1% |

Table 2: Simulation parameters

| Parameters | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | SCS, 30 KHz |
| Scenario | Industrial (38.901): Indoor Office (Inh.), 3Km/h |
| System bandwidth | 20 MHz |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (2,2,2,1,1) with (dH, dV) = (0.5,0.5) λ |
| UE antenna configuration | (M, N, P, Mg, Ng) = (2,2,2,1,1) with (dH, dV) = (0.5,0.5) λ |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| Traffic model | Full buffer |
| Precoder granularity | Wideband |
| Power control | Open loop, alpha = 0.8, P0 = -80 dBm |
| UE power rating | 23 dBm (UE, 38.101) |

  - Codebook design for partial/non-coherent

For codebook design of partial/non-coherent UE, the following agreements were achieved.

Agreement For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated.

In the past meeting, various structures have been proposed for partial-coherent precoding matrix, each structure seem to have the different advantage. However, we should determine the codebook according to the two points; support of the full power mode described as the agreement, and as simple as possible structure not to increase the TPMI overhead. We recognize that the previous agreement implies the following codebook design, at least for Ng=2.

Proposal 4: Support the following precoding structure of codebook design for partial-coherent with Ng = 2 according to the agreement in the previous meeting.

W= A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are precoding matrices taken from Rel-15 4TX UL codebook, the rank = rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8.

In the case of Ng=4, the following two designs are candidates. These Alts indicate the precoding structures described in the following Table 3. Only one codebook design should be supported to reduce standardization effort. The performance of the codebook by each Alt should at least be clear for down-selecting.

Table 3: Codebook design for partial coherent with Ng = 4

| Alt1: Four 2TX codebooks | Alt2: Two 4TX codebooks |
| --- | --- |
| W=A104×rank(A2)04×rank(A3)04×rank(A4)04×rank(A1)A204×rank(A3)04×rank(A4)04×rank(A1)04×rank(A2)A304×rank(A4)04×rank(A1)04×rank(A2)04×rank(A3)A4 | W=A104×rank(A2)04×rank(A1)A2 |
| A1, A2, A3 and A4 are FC precoders | A1 and A2 are FC precoders |

Proposal 5: The performance difference between Alt1 and Alt2 of codebook design for partial coherent with Ng = 4 should be clarified.

Table 4 summarizes the simulation results of performance difference between Alt1 and Alt2, and Table 5 shows the assumption (same as Table 2). Taking the throughput of Alt1 as 100%, Alt2 showed a 111% for 50%ile UE throughput and 109% for 95%ile UE throughput.

Observation 2: Alt2 of codebook design for partial coherent with Ng = 4 have higher performance than Alt1.

Table 4: Performance difference between Alt1 and Alt2

| Codebook design | 50%ile UE Throughput | 95%ile UE Throughput |
| --- | --- | --- |
| Alt1 | 100% | 100% |
| Alt2 | 111% | 109% |

Table 5: Simulation parameters

| Parameters | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | SCS, 30 KHz |
| Scenario | Industrial (38.901): Indoor Office (Inh.), 3Km/h |
| System bandwidth | 20 MHz |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (2,2,2,1,1) with (dH, dV) = (0.5,0.5) λ |
| UE antenna configuration | (M, N, P, Mg, Ng) = (2,2,2,1,1) with (dH, dV) = (0.5,0.5) λ |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| Traffic model | Full buffer |
| Precoder granularity | Wideband |
| Power control | Open loop, alpha = 0.8, P0 = -80 dBm |
| UE power rating | 23 dBm (UE, 38.101) |

Besides performance, we need to consider the TPMI overhead to determine the codebook design for partial coherent. Through the previous meeting, there is common understanding that need to reduce the TPMI overhead. The overhead become an issue especially for the partial-coherent codebook. In the case as the partial-coherent precoding structure consist of two precoding matrices taken from Rel-15 4TX UL codebook (A1, A2 described as proposal 4), it needs as two times as the conventional TPMI overhead. As the codebook Subset is same as existing configuration (E.g. codebookSubset=fullyAndPartialAndNonCoherent), the number of PMI candidates become 62 for each 4TX UL codebook. Considered when the rank(A1) or the rank(A2) is 0, the total number of TPMI is 63*63-1 = 3968 for 8TX partial-coherent codebook. This TPMI indication needs 12 bits.

Table 1: Number of PMI candidate of 4TX UL codebook for each coherence type

|  | FC | PC | NC | FCPCNC |
| --- | --- | --- | --- | --- |
| Rank 1 | 16 | 8 | 4 | 28 |
| Rank 2 | 8 | 8 | 6 | 22 |
| Rank 3 | 4 | 2 | 1 | 7 |
| Rank 4 | 2 | 2 | 1 | 5 |
| sum | 30 | 20 | 12 | 62 |

Table 2: Number of PMI candidate of 2TX UL codebook for each coherence type

|  | FC | NC(PC) | FCPCNC |
| --- | --- | --- | --- |
| Rank 1 | 4 | 2 | 6 |
| Rank 2 | 2 | 1 | 3 |
| sum | 6 | 3 | 9 |

On the other hand, if codebookSubset is separated by one coherent capability, the total number of TPMI is 31*31-1 = 960, which needs 10 bits. In other words, 2 bits of overhead can be reduced. Therefore, the codebookSubset should be separated so that the codebookSubset contains only one coherent capability.

Proposal 6: The codebookSubset should be separated by one coherent capability.

Also, if an 8TX UL codebook consists of four 2TX UL codebooks for Ng=4 (Alt1), the total number of TPMI is 10*10*10*10-1 = 9999, which needs 14 bits. Even if only TPMIs corresponding to fully-coherent were included, the total number of TPMI would be as large as 7*7*7*7-1 = 2400 which needs 13bits. Thus, even with Ng=4, the precoding structure better to be same as Ng=2; consist of two precoding matrices taken from Rel-15 4TX UL codebook (Alt2).

Proposal 7: Support the following precoding structure of codebook design for partial-coherent with Ng = 2 and 4.

W= A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are precoding matrices taken from Rel-15 4TX UL codebook, the rank = rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8.

- Ng = 2: A1 and A2 correspond to only FC precoder.

- Ng = 4: A1 and A2 correspond to only PC precoder.

For rank(A1) and rank(A2) described in Proposal 4, the PMI candidates for each rank combination is shown in the table below. And assuming only TPMI corresponding to PC for Ng=4, the total number of PMI candidate is 21*21-1 = 440 as shown in Table 5. Since the number of candidates becomes large when all rank combinations are considered, it is expected to reduce the number of PMI candidates by supporting only some combinations.

Table 3: Number of PMI candidates of 4TX UL codebook for each rank combination

| maxRank | rank(A1) | rank(A2) |  | N(A1) | N(A2) | N(A1)*N(A2) | sum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 |  | 16 | 1 | 16 | 32 |
|  | 0 | 1 |  | 1 | 16 | 16 |  |
| 2 | 2 | 0 |  | 8 | 1 | 8 | 272 |
|  | 1 | 1 |  | 16 | 16 | 256 |  |
|  | 0 | 2 |  | 1 | 8 | 8 |  |
| 3 | 3 | 0 |  | 4 | 1 | 4 | 264 |
|  | 2 | 1 |  | 8 | 16 | 128 |  |
|  | 1 | 2 |  | 16 | 8 | 128 |  |
|  | 0 | 3 |  | 1 | 4 | 4 |  |
| 4 | 4 | 0 |  | 2 | 1 | 2 | 196 |
|  | 3 | 1 |  | 4 | 16 | 64 |  |
|  | 2 | 2 |  | 8 | 8 | 64 |  |
|  | 1 | 3 |  | 16 | 4 | 64 |  |
|  | 0 | 4 |  | 1 | 2 | 2 |  |
| 5 | 4 | 1 |  | 2 | 16 | 32 | 128 |
|  | 3 | 2 |  | 4 | 8 | 32 |  |
|  | 2 | 3 |  | 8 | 4 | 32 |  |
|  | 1 | 4 |  | 16 | 2 | 32 |  |
| 6 | 4 | 2 |  | 2 | 8 | 16 | 48 |
|  | 3 | 3 |  | 4 | 4 | 16 |  |
|  | 2 | 4 |  | 8 | 2 | 16 |  |
| 7 | 4 | 3 |  | 2 | 4 | 8 | 16 |
|  | 3 | 4 |  | 4 | 2 | 8 |  |
| 8 | 4 | 4 |  | 2 | 2 | 4 | 4 |
|  |  |  |  |  |  | total | 960 |

Table 3: Number of PMI candidates of 2TX UL codebook for each rank combination

| maxRank | rank(A1) | rank(A2) |  | N(A1) | N(A2) | N(A1)*N(A2) | sum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 |  | 8 | 1 | 8 | 16 |
|  | 0 | 1 |  | 1 | 8 | 8 |  |
| 2 | 2 | 0 |  | 8 | 1 | 8 | 80 |
|  | 1 | 1 |  | 8 | 8 | 64 |  |
|  | 0 | 2 |  | 1 | 8 | 8 |  |
| 3 | 3 | 0 |  | 2 | 1 | 2 | 132 |
|  | 2 | 1 |  | 8 | 8 | 64 |  |
|  | 1 | 2 |  | 8 | 8 | 64 |  |
|  | 0 | 3 |  | 1 | 2 | 2 |  |
| 4 | 4 | 0 |  | 2 | 1 | 2 | 100 |
|  | 3 | 1 |  | 2 | 8 | 16 |  |
|  | 2 | 2 |  | 8 | 8 | 64 |  |
|  | 1 | 3 |  | 8 | 2 | 16 |  |
|  | 0 | 4 |  | 1 | 2 | 2 |  |
| 5 | 4 | 1 |  | 2 | 8 | 16 | 64 |
|  | 3 | 2 |  | 2 | 8 | 16 |  |
|  | 2 | 3 |  | 8 | 2 | 16 |  |
|  | 1 | 4 |  | 8 | 2 | 16 |  |
| 6 | 4 | 2 |  | 2 | 8 | 16 | 36 |
|  | 3 | 3 |  | 2 | 2 | 4 |  |
|  | 2 | 4 |  | 8 | 2 | 16 |  |
| 7 | 4 | 3 |  | 2 | 2 | 4 | 8 |
|  | 3 | 4 |  | 2 | 2 | 4 |  |
| 8 | 4 | 4 |  | 2 | 2 | 4 | 4 |
|  |  |  |  |  |  | total | 440 |

Observation 3: It is expected to reduce the number of PMI candidates by supporting only some combinations.

  - Support of two Codewords

For supporting two codewords, the following working assumption and agreements were achieved in previous meeting.

Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support - Alt.2: A second MCS field (5 bits) is indicated for the second codeword Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated. - FFS: Details on how to signal Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q∈{0,1}, cinit=nRNTI∙215+q∙214+nID where nRNTI, and nID are defined similar to the legacy single CW PUSCH transmission. Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from, - Alt1: First CW - Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

Currently, dual codeword transmission is specified for DL when rank>4, and the codeword to layer mapping for DL transmission can be reused for UL to reduce the specification effort. On the other hand, it is undesirable to make dual codeword transmission a mandatory feature because various use cases such as FWA/CPE are expected for 8TX UL. Therefore, it should be a UE capability for rank>4 to support dual codeword transmission.

Proposal 8: It should be a UE capability for rank>4 to support dual codeword transmission.

To reduce standardization effort, we support disabling of the second CW method same as existing the DL specification, i.e. the combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable transmission of its corresponding TB.

Proposal 9: Support the disabling the second CW following method, used for DL specification.

- The combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable transmission of its corresponding TB.

Regarding multiplexing of UCI, we support Alt2, which was proposed at the last meeting, because it is a superset of Alt1.

Proposal 10: Support the Alt2 for UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE.

- Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

  - Full power operation

For full power operation, the following agreement was achieved.

Agreement For CB-based 8TX PUSCH transmission, where Mode 2 uplink full power transmission (if supported) is not used, re-use legacy Rel-15 mechanism, that is - when only one SRS resource in a resource set is configured, the SRI field in DCI is absent, - when two SRS resources are configured in a resource set, 1 bit of SRI field in DCI is used to indicate the selected SRS resource in the set. Agreement (RAN1#112) Study full TX power uplink codebook-based transmission by a partially/non-coherent 8TX precoder, - Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3 - For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN#112. - This can be used for other UE Power Classes as well. 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA. Regular UE; P1=P2= …=P8=14 dBm (Full power supported by Mode1). Full-power capable UE; Full power capability with any PA comb. (CAP1) Example: P1=P2= …=P8= 23 dBm. Full power capability with 1 PA (CAP3) Example: P1=P2= …=P7= 14 dBm P8= 23 dBm. (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2= …=P6= 14 dBm, P7=P8 ≥ 20 dBm Example 2b: P1=P2= …= P8= 20 dBm. (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2= …=P4= 14 dBm, P5=P6= …=P8 ≥ 17 dBm Example 3b: P1=P2= …= P8 = 17 dBm. (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2= 14 dBm, P3=P4= …=P8 ≥ 15.3 dBm Example 4b: P1=P2= … = P8≥ 15.3 dBm.

It was agreed to study several architectures at the last meeting, but more capabilities are being discussed; it is not desirable to divide capability into too many types as it will complicate the specification. It is sufficient to know at least how many antennas can be used to achieve full power transmission.

Proposal 11: UE capability should indicate only the number of antennas needed to achieve full power transmission.

In addition to Mode 2 agreed upon at the last meeting, Mode 0 and Mode 1 should be supported. These Modes could be applied to 8TX UL transmission without major specification changes.

Proposal 12: Both of Mode 0 and Mode 1 for uplink full power transmission should be supported.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: Rel-18 8TX does not support the pairs of values other than (N1, N2) = {(4,1), (2,2)}.

Proposal 2: Support the pairs of oversampling value (O1, O2) = {(1,1), (2,1), (2,2)}.

Proposal 3: UE should report the capability of supported (O1, O2).

Proposal 4: Support the following precoding structure of codebook design for partial-coherent with Ng = 2 according to the agreement in the previous meeting.

W= A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are precoding matrices taken from Rel-15 4TX UL codebook, the rank = rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8.

Proposal 5: The performance difference between Alt1 and Alt2 of codebook design for partial coherent with Ng = 4 should be clarified.

Proposal 6: The codebookSubset should be separated by one coherent capability.

Proposal 7: Support the following precoding structure of codebook design for partial-coherent with Ng = 2 and 4.

W= A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are precoding matrices taken from Rel-15 4TX UL codebook, the rank = rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8.

- Ng = 2: A1 and A2 correspond to only FC precoder.

- Ng = 4: A1 and A2 correspond to only PC precoder.

Proposal 8: It should be a UE capability for rank>4 to support dual codeword transmission.

Proposal 9: Support the disabling the second CW following method, used for DL specification.

- The combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable transmission of its corresponding TB.

Proposal 10: Support the Alt2 for UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE.

- Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

Proposal 11: UE capability should indicate only the number of antennas needed to achieve full power transmission.

Proposal 12: Both of Mode 0 and Mode 1 for uplink full power transmission should be supported.

Observation 1: It is sufficient to support the pairs of (N1, N2) = {(4,1), (2,2)} with capability for 8TX UL transmission.

Observation 2: Alt2 of codebook design for partial coherent with Ng = 4 have higher performance than Alt1.

Observation 3: It is expected to reduce the number of PMI candidates by supporting only some combinations.

References

RAN Meeting#94, RP-213598, New WID: MIMO Evolution for Downlink and Uplink

RAN1 Meeting#112, Chair's notes RAN1#112.