3GPP TSG RAN WG1 #112bis-e				                            R1-2303068

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.1.1

Source:	LG Electronics

Title:	Unified TCI framework extension for multi-TRP/panel

Document for:	Discussion and Decision

Introduction

In RAN#94 meeting, the objectives for further enhancements on MIMO for NR were approved [1], which includes the following objectives related to multi-beam (MB) operations for multi-TRP/panel operation:

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss the enhancements on MB operations for multi-TRP/panel based on the Rel-17 TCI framework for Rel-18.

- Discussion

TCI state update and activation

In RAN1#111, TCI state indication method for S-DCI based MTRP in one beam indication instance was agreed as shown below.

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

Based on this, the combinations of joint/DL/UL TCI states for TCI codepoint mapping was also discussed including how to support dynamic switching between STRP and MTRP. There was discussion on whether the switching between STRP and MTRP can depend on the number of TCI states indicated in a beam indication DCI. In our view, it is unnecessary to tie the number of indicated TCI states in one beam indication instance with the transmission mode because NW can indicate same TCI state even with M/N=2 for STRP operation. Instead, as discussed in the last meeting, a full-set or any sub-set of TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2. Then, when the sub-set of TCI states is indicated to a UE, the UE can update the corresponding TCI state in the subset and keep the current TCI state for the other sub-set of TCI states. This leads flexible TCI applicability and also supports dynamic STRP/MTRP switching, e.g. via indicating same TCI state for all subsets. In addition, especially for non-ideal backhaul, each TRP can manage its beam with separate beam indication command so that it is not preferred to overwrite one TCI indication from TRP#0 into the other TCI for TRP#1.

Proposal #1: For S-DCI based MTRP, if a sub-set of TCI states for one TRP is indicated in a beam indication instance, the TCI states not in the sub-set for other TRP shall be maintained.

Mapping the TCI state(s) to target channels/RSs

In RAN1#112, there were agreements on mapping/association between indicated/configured TCI states and target channels/RSs for M-DCI/S-DCI based MTRP as follows.

Agreement (S-DCI PDSCH) On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement (S-DCI PDSCH) On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed) Agreement (S-DCI PUSCH) On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission Agreement (M-DCI PUCCH) On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

Flexible application of STRP and MTRP schemes to different DL/UL channels/RSs can be done by configuring/indicating the linkage between the TCI states and the corresponding channels/RSs for each STRP/MTRP scheme with the various configuration of M/N.

For S-DCI MTRP PDSCH, a new TCI selection field configured by RRC is introduced to apply which indicated TCI states to STRP/MTRP PDSCH reception. The remaining issues are the detailed design of the TCI selection field with codepoint ‘11’ and of the applicability with indicated joint/DL TCI state(s) when the new indication field is not present.

For the first issue of using the codepoint ‘11’, it can be used for swapping the order of TCI state mapping, e.g., {2nd indicated TCI, 1st indicated TCI} for MTRP TDM PDSCH reception. For the second issue for when the new indication field is not present, several alternatives were agreed. From Alt 1 to Alt3, it restricts PDSCH for a certain transmission mode permanently, i.e. STRP or MTRP mode only. Also for Alt3A, it does not work if TRP#1 serves as a primary TRP that manage common control channels/beams and TRP#2 serves as a secondary TRP for data transmission. In this case, all CORESETs are likely be configured to follow TCI state from TRP#1 so that a PDSCH cannot be sent from TRP#2.

In our view, Alt4 supports dynamic STRP/MTRP switching and has more benefit compared to Alt3A. Figure 1 illustrates one exemplary scenario.

Figure 1. Example of the PDSCH beam applicability with DCI format 1_1/1_2

In this figure, we assume that the indicated TCI states {TCI#1, TCI#2} are available. In PDSCH scheduling DCI1 from TRP#1, it contains TCI field with a codepoint mapped to sub-set of TCI states {TCI#3, -}. Based on this, the TCI state update via DCI1 is done after BAT1. Then, after BAT1, the indicated TCI states are updated from {TCI#1, TCI#2} to {TCI#3, TCI#2} according to DCI1. Assume that PDSCH2 is scheduled by DCI2 from TRP#2 later. In this case, if pre-defined/configured rule using 1st indicated TCI state is applied for PDSCH2 reception (Alt1 or Alt2), it leads that both PDSCH1 and PDSCH2 needs to be received from TRP#1. If Alt3 is applied in this example, both PDSCH1 and PDSCH2 needs to be received from both TRPs. If Alt3A is applied, which TRP is applied for PDSCH1 and PDSCH2 depends on the TCI configured for the CORESET(s) for DCI1 and DCI2, respectively, where all CORESETs may be associated to one TRP in the above mentioned scenario. If Alt4 is applied, PDSCH1 is received by TRP#1 with TCI#1, PDSCH2 is received by TRP#2 with TCI#2. From NW perspective, it is natural that a TRP that wants to update its serving beam is more likely to send DL traffic because it is not very urgent/critical to update a beam for the UE having no data to send or receive from NW perspective. With this principle, we think that STRP/MTRP scheme for the scheduled PDSCH can be determined by the TCI field in DCI. If the first subset of TCI states is indicated by the DCI, the scheduling PDSCH is for STRP with the indicated TCI state(s). If the second subset of TCI states is indicated by the DCI, the scheduling PDSCH is for STRP with the indicated TCI state(s). If full set of TCI states is indicated by the DCI, the scheduling PDSCH is for MTRP with the indicated TCI state(s). It means that the beam update and STRP/MTRP switching is coupled. Compared with the option of configuring the new field, Alt4 slightly reduce the flexibility of beam update due to the coupling of unified TCI update with STRP/MTRP switching, however, it supports STRP/MTRP dynamic switching without increasing DCI payload so that NW can have better option for BM operation. In the case that PDSCH is scheduled before timeDurationForQCL, Alt4 is still applicable when the UE supports two default beams. For the UEs supporting only one default beam, i.e., one buffer beam, the beam for the PDSCH needs to be determined by the 1st indicated TCI state as similar to legacy default beam determination methods for MTRP.

Proposal #2: For S-DCI based PDSCH reception, the indicated TCI states can be applied for STRP/MTRP operation depending on new indication field in scheduling DCI

- For codepoint ‘11’ on TCI selection field, the UE shall apply the both indicated TCI states with swapping the order of the TCI states

- When new indication field is not present, support Alt4 for dynamic switching between STRP and MTRP for the scheduled PDSCH

For M-DCI based MTRP, TCI field in a DCI transmitted from one CORESET pool index can indicate joint/DL/UL TCI state(s) specific to the same CORESET pool index value and the indicated TCI state(s) can be applied for those DL/UL channels sharing the same TCI state. For the channels/signals other than PDCCH and PDSCH, those channels triggered by DCI can follow the same principle as PDSCH, i.e. TCI follows the CORESET pool of the DCI such as PUCCH transmission for HARQ-ACK can follow the indicated TCI when corresponding DL grant is scheduled via a CORESET pool. For other channels that does not triggered by DCI, explicit indication would be necessary. For PUCCH, for example, same solution as for S-DCI MTRP PUCCH can be used, i.e. PUCCH resource grouping by RRC. SRS resource grouping can also be supported in this regard. To be specific, network can configure which subset of TCI states to be applied for each SRS resource group or resource set regardless of S-DCI or M-DCI and also regardless of the number of SRS resource sets. In addition, it is beneficial to consider unified TCI applicability per SRS resource, e.g. to support different beam with an SRS resource set, e.g. BM SRS, CB SRS. In addition, legacy beam indication via spatial relation configured for some SRS resources should also be allowed even when N=2 unified TCI. For UL TCI association, we propose not directly to use coresetPoolIndex in order to align design for both S-DCI MTRP and M-DCI MTRP. For example, UL TCI state index can be introduced and tagged into PUCCH/SRS resource group.

Proposal #3: For M-DCI MTRP,

- Implicit association with coresetPoolIndex can be used for those channels/RSs triggered by DCI.

- Explicit association is used for other channels/RSs

  - For UL channels/RSs, do not directly use coresetPoolIndex for the explicit indicator in order to align design for both S-DCI and M-DCI and for heterogeneous DL/UL configurations(e.g. M=2, N=1)

  - For SRS, TCI applicability can be configured per SRS resource group/set or per SRS resource where certain SRS resource(s) can still be configured with legacy spatial relation.

For channel(s)/RS(s) that doesn’t have association with a coresetPoolIndex value, it can be handled by defining pool index determination rule(s). For example, Type-1 CG PUSCH scheduled by RRC can follow the beam applicability through the CORESET pool index indicating DCI for retransmission on the corresponding PUSCH.

Proposal #4: For M-DCI MTRP, support pre-defined coresetPoolIndex value association for channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value

For CSI-RS, unified TCI applicability can be configured per resource or per resource set. To align with Rel-17 unified TCI design, per-set configuration may be considered as baseline. However, it may not be applicable to NCJT CSI introduced in Rel-17 because two TRPs use resource pairing within one set. For this case, each indicated TCI state can be mapped to each CSI-RS in resource pair for NCJT CSI measurement. Alternatively, it may also considerable to extend TCI applicability as per-resource configuration.

Proposal #5: For TCI state applicability on CSI-RS, consider to extend to per-resource configuration. If per-set configuration is adopted as Rel-17, special handling for NCJT CSI needs to be considered.

In Rel-17, common TCI state update for multiple CCs based on unified TCI framework was introduced. In Rel-18 unified TCI framework considering MTRP extension, common TCI state update for multiple CCs needs to be enhanced. For this, we need to consider that the number of indicated TCI states(i.e. M/N values) can be different among CCs. If so, the issue is how to update TCI state(s) of other CCs utilizing the indicated TCI state(s) in a specific CC with different number of TCI states. For a simple approach, we can introduce a restriction that a CC list for common TCI state update should include the CC(s) with the same M/N configuration only. This approach may cause an increased control signaling overhead by increasing the number of CC lists to cover the various M/N configuration of different CCs. To address this demerit, another potential solution is to determine the number of indicated TCI states based on the largest M/N value among CCs in a CC list. For example, when the M/N configuration for CC1 is {M=2, N=1} and CC2 is {M=1, N=2}, the number of indicated TCI states applying common TCI state update can be {M_common=2, N_common=2}. Then, common TCI state update can be operated via the association between indicated TCI state(s) on a certain CC and the indicated TCI states applying to all CCs.

Proposal #6: For common TCI update for multi-CC with M/N=2, determine the number of indicated TCI states based on the largest M/N value among CCs in a CC list

Power control for UL S-DCI based MTRP

In RAN1#112, the following agreement was made as:

Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

For power control (PC) aspects, the PC parameters including P0, alpha, closed-loop index, and path-loss reference RS (PL-RS) can be included in the TCI state where the PC parameters excluding PL-RS are channel/RS-specific and PL-RS is channel/RS-common. Considering N>1 in Rel-18, TRP/panel-specific UL PC parameters can be indicated by the common beam which can include PC parameters. As shown above, UL PC parameters included in the UL or joint TCI states can be applied for S-DCI MTRP PUSCH/PUCCH transmission case. When N=2, the first and second TCI state in S-DCI can be associated with the specific PUSCH/PUCCH repetition transmission occasion, respectively. One case for further consideration is when the UL PC setting is not included in one or both indicated joint/UL TCI state(s) for PUSCH/PUCCH repetition. In this case, it can be possible to extend Rel-17 mechanism where UE determines the UL PC parameter setting configured in the corresponding UL BWP. Hence, in unified TCI extension for UL MTRP, two default UL PC parameter settings can be configured in the UL BWP and the UE can apply the one of or both UL PC parameter settings to the corresponding TCI state.

Proposal #7: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter setting(s) configured in the corresponding UL BWP.

In addition, it was agreed in RAN1#109e to send LS to RAN4 regarding UE power limitation for STxMP as follows.

Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

Regarding the above, RAN4 has replied the LS in the last meeting [2], where Assumption 1 and Assumption are feasible from RAN4 perspective and both assumptions can be applied to a same UE. For STxMP transmission, each TCI state including PC parameters can be associated with a certain UE Tx panel and UE transmits PUSCH/PUCCH by using two panels at the same time. As a result, the total transmit power through multiple UE panels can exceed UE Pc,max value according to the reply LS from RAN4. Hence, it should be further studied with transmit power management for STxMP transmission. Consider per-panel power control for STxMP transmission by taking maximum Tx power of UE and panel-wise maximum power into account. Note that the maximum Tx power may be different depending on power amplifier configuration of each panel or maximum permissible exposure (MPE) issue on human body. Hence, power head room report (PHR) to gNB for the current power budget of UE seems to be more preferable by including virtual PH value and per panel Pc,max together. In addition, as RAN4 mentioned, the concept of ‘panel’ has not been defined and then it needs to be clarified on the panel and the panel association for handling each assumption of UE power limitation. To this end, extending the usage of UE capability value set as mentioned in the following sub-section 2.4 can be the one of a promising approaches.

Proposal #8: Consider per panel power control for STxMP transmission by taking maximum Tx power of UE and panel-wise maximum power into account.

Beam management for simultaneous UL multi-panel transmission

For the STxMP, UL beam should be determined on per panel basis. Rel-17 beam management introduced an enhanced beam reporting mode in which UE reports CRI/SSBRI with UE capability value set ID (i.e. C-ID) but it cannot distinguish panels with the same number of antenna ports, i.e. homogeneous panels. Thus, gNB cannot acquire two UL spatial relation RSs where each spatial relation RS correspond to different UE STxMP panel having the same number of antenna ports. Hence, it is required to extend/enhance the Rel-17 beam report by indicating different C-ID(s) with the same number of maximal SRS ports. In addition, UE can report STxMP capable CRI/SSBRI pair with corresponding C-ID pair where the C-ID pair corresponds to STxMP capable panels. Another aspect for enhancement is the beam indication where only one TCI/TPMI/TRI/SRI is supported for PUSCH/PUCCH/SRS transmission currently. The design for N>1 TCI states discussed in the previous section could also be used for simultaneous UL multi-panel transmission where each UL/joint TCI state corresponds to each UE panel.

Proposal #9: Enhanced beam management for simultaneous UL multi-panel transmission should be supported.

- Enhanced beam report via extending the usage of UE capability value set to homogeneous panels and to report multiple CRI/SSBRI pair with multiple UE capability indices among STxMP capable pairs

- Enhanced beam indication based on N>1 UL/joint TCI states, where each UL/joint TCI state corresponds to each UE panel

TRP-specific beam failure recovery

In the last meeting, the following is agreed for TRP-specific BFR issue.

Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request

With M=2, it would be beneficial to support TRP-specific BFR introduced in Rel-17. For BFD RS determination for M-DCI MTRP, it seems natural to include currently maintained two DL TCI states, one for each BFD RS set since these two TCI are being used by different TRP. Considering that there can be CORESET(s) that does not follow unified TCI, each BFD RS set may include two or more BFD RSs based on Rel-17 BFD-RS determination rule where which set to be included needs to be clarified further, e.g. via CORESET pool index for M-DCI. In addition for implicit BFD RS determination, we need to check whether current BFD RS selection rule can still work since it is unclear how to interpret and compare monitoring periodicity and CORESET index for those CORESETs following unified TCI.

Proposal #10: For TRP-specific BFD with M=2, the following needs to be considered.

- How to comprise each BFD RS set when some CORESETs do not follow unified TCI for both S-DCI MTRP and M-DCI MTRP

- Whether/how to modify BFD-RS selection rule for implicit BFD RS determination

Conclusion

In this contribution, we have discussed the potential enhancements on MTRP operation based on the extension of unified TCI framework. Based on the discussions above, following proposals are given as:

Proposal #1: For S-DCI based MTRP, if a sub-set of TCI states for one TRP is indicated in a beam indication instance, the TCI states not in the sub-set for other TRP shall be maintained.

Proposal #2: For S-DCI based PDSCH reception, the indicated TCI states can be applied for STRP/MTRP operation depending on new indication field in scheduling DCI

- For codepoint ‘11’ on TCI selection field, the UE shall apply the both indicated TCI states with swapping the order of the TCI states

- When new indication field is not present, support Alt4 for dynamic switching between STRP and MTRP for the scheduled PDSCH

Proposal #3: For M-DCI MTRP,

- Implicit association with coresetPoolIndex can be used for those channels/RSs triggered by DCI.

- Explicit association is used for other channels/RSs

  - For UL channels/RSs, do not directly use coresetPoolIndex for the explicit indicator in order to align design for both S-DCI and M-DCI and for heterogeneous DL/UL configurations(e.g. M=2, N=1)

  - For SRS, TCI applicability can be configured per SRS resource group/set or per SRS resource where certain SRS resource(s) can still be configured with legacy spatial relation.

Proposal #4: For M-DCI MTRP, support pre-defined coresetPoolIndex value association for channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value

Proposal #5: For TCI state applicability on CSI-RS, consider to extend to per-resource configuration. If per-set configuration is adopted as Rel-17, special handling for NCJT CSI needs to be considered.

Proposal #6: For common TCI update for multi-CC with M/N=2, determine the number of indicated TCI states based on the largest M/N value among CCs in a CC list

Proposal #7: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter setting(s) configured in the corresponding UL BWP.

Proposal #8: Consider per panel power control for STxMP transmission by taking maximum Tx power of UE and panel-wise maximum power into account.

Proposal #9: Enhanced beam management for simultaneous UL multi-panel transmission should be supported.

- Enhanced beam report via extending the usage of UE capability value set to homogeneous panels and to report multiple CRI/SSBRI pair with multiple UE capability indices among STxMP capable pairs

- Enhanced beam indication based on N>1 UL/joint TCI states, where each UL/joint TCI state corresponds to each UE panel

Proposal #10: For TRP-specific BFD with M=2, the following needs to be considered.

- How to comprise each BFD RS set when some CORESETs do not follow unified TCI for both S-DCI MTRP and M-DCI MTRP

- Whether/how to modify BFD-RS selection rule for implicit BFD RS determination

- References

[1] RP-213598	New WID: MIMO Evolution for Downlink and Uplink		Samsung

[2] R4-2303494	Reply LS on UE power limitation for STxMP in FR2			vivo