3GPP TSG RAN WG1 #112bis-e	   	 	  	                	      R1-2303069

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.1.2

Source:	LG Electronics

Title:	Two TAs for multi-TRP/panel

Document for:	Discussion and Decision

Introduction

The approved WID [1] includes enhancements on MIMO for Rel-18, including multi-TRP enhancement, CSI enhancement, reference signal enhancement, and uplink transmission enhancement. Specifically for multi-TRP enhancement, there are for sub-topics, regarding unified TCI framework extension, simultaneous multi-panel UL transmission, power control for UL single DCI, and two TAs for multi-DCI, respectively.

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we discuss the enhancement on two TAs for multi-DCI based M-TRP operation.

Discussion

The following agreements were made in previous meetings.

Agreement Support two TA enhancement for both intra-cell and inter-cell multi-DCI multi-TRP scenarios in Rel-18. Agreement Enhancements on two TAs for UL multi-DCI for multi-TRP operation are applicable to both FR1 and FR2. Agreement Two TA enhancement for uplink multi-DCI based multi-TRP operation are applicable to at least: - TDM based multi-DCI uplink transmission - simultaneous multi-DCI uplink transmission (if simultaneous uplink multi-DCI uplink transmission is supported in Agenda 9.1.4.1) - Note: Whether two TA enhancement is applicable to other schemes is a separate discussion, which is not in the scope of AI 9.1.1.2. Agreement For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes: - whether to introduce scheduling restriction in overlapping part - whether to introduce dropping rules - whether specification impact is needed, or if the issue can be handled via implementation - whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18) Agreement For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell. Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported) Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to support enhancements for CBRA triggered by PDCCH order. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure. Agreement: Send LS to RAN2 regarding the above conclusion. Final LS in R1-2213004. Conclusion For inter-cell multi-DCI based multi-TRP operation with two TA enhancement, there is no consensus to introduce additional type 1 CSS configuration per additional PCI. Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS Agreement Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, down-select at least one of the following in RAN1#112bis-e: - Alt 1: Introducing a time gap X between two UL transmissions associated with two different TA values - E.g., X symbols in the slot(s) corresponding to the two UL transmission remain unused - FFS: How X is determined - Alt 2: Reduce the overlapping duration of one of the two UL transmissions - Alt 3: Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap - Other alternatives are not precluded TBD: how to capture the downselected alternative(s) in the specifications in case specification impact is deemed needed.

Use cases of two TAs

When it comes to different TA per TRP/panel, there are three factors to be considered.

- Difference of propagation delay between a UE and each TRP

- DL reference timing offset between TRPs

- Inter-UE-panel delay

Especially, UE distributed antenna system (DAS) implementation is being attractive in automotive industry. As being requested by LS from 5GAA [2], two TA feature would be useful for vehicle DAS UE. For vehicle DAS UE, each TA value could be managed by each distributed antenna group or panel, and this could be used for S-TRP deployment scenario as well as M-TRP since ‘TRP’ can be transparent to UE.

Proposal #1. Enhancements for two TAs should take vehicle DAS UE implementation into account, where each TA value is managed by each distributed antenna group or panel.

Analysis on TRP/panel-specific UL timing synchronization

In numerous results, it has been shown that UL performance degradation can be significant if UL timing error exceeds 0.1us (i.e., about 22% of the CP length) [4][5]. It is obvious that if it exceeds the CP length, UL reception will not be completed successfully. Based on the fact, we show simulation results related to Rx timing offset between two TRPs for MTRP scheme in this contribution.

The following Table 1 shows a generalized formulation of Rx timing offset between two TRPs for the case that the same TA value is used for both TRPs, where τasync represents relative timing offset of TRP2 and τIPD represents relative inter-panel timing offset of the UE where each UE panel is assumed to be associated to each TRP. Based on RAN4 requirements, the possible largest value of τasync is 3us and that of τIPD is 130ns [6][7].

Table 1. Rx timing at each TRP with Tx timing asynchronization between TRPs and inter-panel delay of MPUE

|  | TRP1 | TRP2 |
| --- | --- | --- |
| TRP reference timing | T1 | T1+τasync |
| Propagation delay | RTT12 | RTT22 |
| UE Rx timing & TA reference timing | T1+RTT12 | T1+τasync+RTT22+τIPD |
| TRP Rx timing | TRx,TRP1=T1+RTT1-TA | TRx,TRP2=T1+τasync+RTT2+τIPD-TA |
| Rx timing offset between TRPs | Toffset=TARx,TRP1-TARx,TRP2=RTT1-RTT2-τasync-τIPD=2cd1-d2-c2τasync+τISD |  |

In Table 1, d1, d1 and c represent distance between UE and TRP1/2, and the speed of light, respectively. As described in Table 1, Rx timing offset between TRPs is determined by the difference in the distance between UE and each TRP, Tx timing offset between TRPs, and inter-panel delay of MPUE. If UE has to transmit UL signal based on one TA value, performance would be degraded due to such Rx timing offset between TRPs. Based on this formulation, we observed the ratio of UEs of which Rx timing offset Toffset exceeds the CP length via system level simulation. Simulation parameters are shown in Table 2. The parameters are set based on [8][9].

Table 2. Simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario | UMa, InH-office |
| Carrier frequency | 2GHz, 30GHz, 60GHz |
| Channel model | TR38.901 |
| TRP antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2), (dH, dV) = (0.5, 0.8)λ for UMa 2GHz 2 ports: (8,8,2,1,1), (dH, dV) = (0.5, 0.8)λ for UMa 30GHz2 ports: (4,8,2,1,1), (dH, dV) = (0.5, 0.5)λ for InH 60GHz |
| UE antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for 2GHz 2 ports: (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ, Ω0,1=Ω0,0+180 for 30GHz 2 ports: (4, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ, Ω0,1=Ω0,0+180 for 60GHz |

Table 3-1. Portion of MTRP UEs exceeding CP length (UMa, 2GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 kHz | 4.68 us | 1.8 % (3.7 %) | 1.9 % (3.9 %) | 2.9 % (5.9 %) |
| 1 | 30 kHz | 2.34 us | 7.8 % (16.1 %) | 7.8 % (16.1 %) | 15.7 % (32.4 %) |
| 2 | 60 kHz | 1.17 us | 13.2 % (27.2 %) | 13.2 % (27.2 %) | 31.7 % (65.4 %) |

Table 3-2. Portion of MTRP UEs exceeding CP length (UMa, 30GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 2 | 60 kHz | 1.17 us | 13.0 % (38.4 %) | 13.0 % (38.4 %) | 22.4 % (66.2 %) |
| 3 | 120 kHz | 585.93 ns | 18.8 % (55.6 %) | 18.9 % (55.9%) | 28.1 % (83.1 %) |

Table 3-3. Portion of MTRP UEs exceeding CP length (UMa, 30GHz, with blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 2 | 60 kHz | 1.17 us | 20.5 % (46.3 %) | 20.5 % (46.3 %) | 29.3 % (66.2%) |
| 3 | 120 kHz | 585.93 ns | 27.3 % (61.7 %) | 27.4 % (61.9 %) | 36.7 % (83.0%) |

Table 4-1. Portion of MTRP UEs exceeding CP length (InH, 60GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 5 | 480 kHz | 146.48 ns | 5.8 % (19.7 %) | 8.6 % (29.2 %) | 28.1 % (95.5 %) |
| 6 | 960 kHz | 73.24 ns | 14.9 % (50.6 %) | 17.9 % (60.8 %) | 28.8 % (97.9%) |

Table 4-2. Portion of MTRP UEs exceeding CP length (InH, 60GHz, with blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 5 | 480 kHz | 146.48 ns | 9.3 % (23.9 %) | 12.6 % (32.3 %) | 37.2 % (95.6 %) |
| 6 | 960 kHz | 73.24 ns | 21.1 % (54.2 %) | 24.0 % (61.6 %) | 38.0 % (97.6 %) |

Table 3-1, 3-2, and 3-3 show simulation results in UMa scenario at 2GHz and 30GHz, Table 4-1 and 4-2 show simulation results InH scenario at 60GHz. In the simulation, we assume that MTRP transmission is done only for the UEs that the RSRP difference between two TRPs having the best RSRP is smaller than 6dB for the case without blockage or 10dB for the case with blockage, where blockage model A in TR38.901 is used, respectively. For other UEs, STRP transmission is assumed. We observed the portion of MTRP UEs of which timing offset exceeds the CP length among all UEs (x%) or among all MTRP UEs (y%) in the deployment, where both x and y are present in the tables as ‘x % (y %)’. In the tables, the impact of timing offset/error of TRP or UE side is also observed, where the term τasync + τIPD in Table 1 is randomly generated with uniform distribution in [-τMAX, τMAX]. Three values of τMAX were tested, i.e. ideal timing offset (τMAX =0), inter-panel delay at UE side (τMAX =130ns), and inter-TRP asynch scenario (τMAX =3us).

Based on the above simulation results, the ratio of UEs of which Rx timing offset Toffset exceeds the CP length is not negligible in various network deployment scenarios. The ratio becomes larger for higher carrier frequency, higher inter-TRP backhaul delay, and/or higher inter-UE-panel delay. In Table 3-1/3-2/3-3, we can observe that dominant factors causing delay difference larger than CP are carrier frequency, sub-carrier spacing, difference on distance to each TRP, and inter-TRP reference timing offset. In Table 4-1/4-2, we can also observe that inter-UE-panel delay causes delay difference larger than CP in addition to the above factors.

Observation #1: In simulation results, carrier frequency, subcarrier spacing, difference on distance to each TRP, inter-TRP reference timing offset and inter-UE-panel delay are dominant factors causing significant timing offset between TRPs.

In this regard, TRP-specific two TAs should be managed/maintained not only TRP specifically but also UE-panel specifically, when we consider unified design of two TAs configuration across different frequency ranges.

Proposal #2: Enhancements for two TAs should take the case that each TA value is managed by each UE-panel into account.

Configuration of two TAs for multi-DCI mTRP

- Two reference timings for two TAGs

In TS 38.133[6], it is described that the reference point to apply timing advance is the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in PTAG, UE shall use the SpCell as the reference cell, and for serving cell(s) in STAG, UE shall use any of the activated SCells as the reference cell.

# 7 Timing ## 7.1 UE transmit timing ### 7.1.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in connected state or when transmiting PUSCH on CG resources for SDT in RRC_Inactive. The uplink frame transmission takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements. In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available during at least one discovery burst transmission window, at the UE due to DL CCA failures at gNB during the last 1280

In previous meeting, it was agreed to support two reference timings for two TAGs in a serving cell. There are remaining issues regarding the definition of per-TRP/TAG reference timing and association between TAGs and reference timings. For these issues, it should be firstly investigated whether the reference cell for two TAGs is TAG-common or TAG-specific. If TAG-common reference cell is supported, two different reference point/timing for a common reference cell should be defined for two TA operation. In this case, for serving cell(s) in PTAG, although UE shall use the SpCell as the reference cell, it is not guaranteed that two CORESET pool indexes are configured in the SpCell in order to derive TAG-specific reference timing. Else if TAG-specific reference cell is supported, the reference cell for each TAG could be different within a same set of CCs, and UE can easily derive TAG-specific reference timing for each TAG-specific reference cell.

Proposal #3: Support TAG-specific reference cell for two reference timings.

- Overlapping between two UL transmissions associated with two TAs

In RAN1 #109-e meeting, it was agreed to consider both cases of TDM based multi-DCI uplink transmission and simultaneous multi-DCI uplink transmission. Difference between two cases is whether different channels/RSs associated with two TAs can be simultaneously transmitted(the latter) or not(the former). For TDM based multi-DCI uplink transmission, different channels/RSs associated with different TAs can still be overlapped when the channels/RSs are scheduled in consecutive symbols. For this case, 3 solutions were listed in the last meeting. In our view, for non-STxMP-capable UEs, either last symbol of the first channel/RS or first symbol of the second channel/RS needs to be dropped (i.e., Alt 2: Reduce the overlapping duration of one of the two UL transmissions) since the UE cannot transmit these two channels/RSs simultaneously in the overlapped symbol. In order to protect front-loaded DMRS symbol(s) of the second channel/RS, last symbol of the first channel/RS could be always reduced for simplicity. For SRS or PUSCH, symbol-level dropping for overlapped part between two UL transmissions associated with different TAs can be applied, since symbol-level dropping of SRS and PUSCH(e.g., rate matching procedure for actual PUSCH in case of PUSCH repetition type B) is already supported in the current specification.

Regarding Alt 1(Introducing a time gap X), UE behavior is not clear when the time gap is configured, e.g., whether the UE is not expected to be scheduled for the time gap or the UE shall drop the allocated symbol within the time gap. And, for Alt 3(scheduling restriction), only semi-static level scheduling restriction can be applied since it was concluded in RAN1 #110bis-e that it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs, considering non-ideal backhaul scenario. The semi-static scheduling restriction will introduce a permanent waste of UL resource(symbols) between two consecutive UL slots associated with different TAs. In addition, the overlapped region between two UL transmissions associated with two TAs can be more than one symbol depending on SCS.

Proposal #4: Support symbol-level dropping for overlapped part between two consecutive UL transmissions associated with different TAs(i.e., Alt 2: Reduce the overlapping duration of one of the two UL transmissions).

- Association of TA to UL channels/signal

In the last meeting, it was agreed to use unified UL/joint TCI state for TA association to UL channels/RSs. There are two remaining issues. One is the TA association method for UL channel/RS which doesn’t follow the indicated TCI in Rel-17/18 unified TCI framework and the other is the TA association methods for UL channel/RS when Rel-15/16 spatial relation framework is used. Regarding these two cases, association between TA and spatialRelation can be one candidate solution. However, since UL spatialRelation is not applicable in FR1, this cannot be applicable to all cases. In our view, one unified solution for these two cases is to directly associate TAG to UL channel/RS, which can be also applicable to FR1 without UL spatialRelation configured/indicated.

Proposal #5: For TA association for UL channel/RS which doesn’t follow the indicated TCI in Rel-17/18 unified TCI framework and for UL channel/RS when Rel-15/16 spatial relation framework is used, associate TAG ID to target UL channels/RSs directly for semi-static UL channels/RSs (e.g., P CSI PUCCH, P SRS, CG PUSCH).

- For dynamic UL channels/RSs, support association of TAG ID to CORESETPoolIndex.

- RACH enhancement for two TA acquisition

If two TAs for M-DCI are supported, it should be investigated on how to measure the two TAs for M-TRP uplink transmission. In legacy behavior, PRACH is generally used for UL TA measurement and RAR MAC CE and/or TA command MAC CE can be used to indicate/update UL TA per TAG. First, PRACH could be enhanced to support TRP/panel-specific PRACH transmission. Also, RAR MAC CE and/or TA command MAC CE can be enhanced to indicate TRP/panel-specific TA.

In RAN1 #110bis-e meeting, 7 alternatives were listed in order to achieve TRP-specific PRACH and/or TRP-specific RAR. It should be discussed separately for the case of PDCCH ordered RACH and UE triggered RACH. However, in RAN1#111 meeting, it was concluded that discussion on UE triggered RACH for two TAs is up to RAN2 regarding whether there is a necessity of enhancement or not and there is related RAN1 work, if supported. So, we can firstly focus on PDCCH ordered RACH for two TAs acquisition.

On the other hand, cross-TRP RACH triggering was agreed to be supported for inter-cell M-DCI based M-TRP case. There are further study points including whether cross-TRP RACH triggering is an optional feature or basic feature. Target use case of M-DCI based M-TRP operation includes both ideal backhaul scenario and non-ideal backhaul scenario. In our view, if TRP #1 triggers UE PRACH transmission (by PDCCH order) targeting TRP #2, dynamic coordination between TRPs would be needed for TRP #2 to correctly receive UE PRACH transmission and send RAR on time with consideration of limited RAR window for the UE. However, dynamic coordination between TRPs cannot always be assumed considering non-ideal backhaul case, so cross-TRP RACH triggering should not be the basic feature. In other words, cross-TRP RACH triggering should be an optional feature from network perspective, i.e., whether or not to support cross-TRP RACH should be configurable by network. In that sense, For PDCCH ordered RACH, at least Alt 6 (TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order) needs to be supported for when cross-TRP RACH triggering is not supported, which is natural enhancement with less specification impact. In this case, subsequent RAR also can be associated with CORESETPoolIndex of the PDCCH order, so TRP/TAG-specific RAR can be naturally supported implicitly. For when cross-TRP RACH triggering is configured to be supported by network for ideal backhaul case, Alt 2 (indicate TAG ID as part of PDCCH order) needs to be supported since the implicit mapping between CORESET pool index of PDCCH order and actual target TRP/TAG of PRACH transmission cannot be assumed, i.e., ordering TRP cannot always be the same with target TRP. So, explicit indication of TAG ID in PDCCH order is needed.

Proposal #6: For PDCCH ordered RACH,

- Support Alt 6, i.e., TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order, when cross-TRP RACH triggering is disabled by network.

- Support Alt 2, i.e., indicate TAG ID as part of PDCCH order, when cross-TRP RACH triggering is enabled by network.

In the last meeting, it was concluded that there is no consensus to introduce additional type 1 CSS configuration per additional PCI. Then, RAR from additional PCI can be forwarded to serving cell by network implementation so two TA still work well without enhancement. In this case, RAR window timing needs to be considered for inter-cell case considering the network delay on forwarding RAR, e.g., additional delay on starting point of RAR window or additional RAR window length can be considered for receiving RAR. RAR forwarding delay may exist in intra-cell M-TRP case with non-ideal backhaul.

Proposal #7: Study RAR window by considering RAR forwarding delay.

- TA adjustment signaling (RAR/TAC MAC CE enhancement)

Since two reference timings with two TAGs were agreed to be supported, it would be better to introduce TRP/panel-specific timing advance command(TAC) MAC CE in order to indicate different TA values for TRPs with different reference timings. From our perspective, for when cross-TRP RACH triggering is enabled for ideal backhaul case, TRP-common TAC MAC CE should be fine since dynamic coordination between TRPs can be assumed. On the contrary, considering non-ideal backhaul case, it may be better to have TRP-specific(CORESET pool-specific) TAC MAC CE when backhaul delay between TRPs is considered which causes TA indication/update delay from a TRP to another TRP.

- TRP-specific timing alignment timer(TAT)

In the previous meeting, two TAs in different TAGs for a serving cell is agreed to be supported. From our understanding, since timing alignment timer(TAT) is managed per TAG in legacy behavior, so two TATs for a serving cell are naturally supported. The remaining issue is UE behavior when either/both TAT in a serving cell is/are expired. In the legacy MAC procedure regarding TA maintenance, when TAT for a TAG is expired semi-static DL/UL resources and SRS/PUCCH belonging to the TAG are cleared/released(uplink Out-of-Synch behavior). However, for two TAGs case in this agenda, two TAT of TAGs are concurrently managed for a serving cell so the legacy MAC procedure could be modified. In our view, if either TAT of two TATs is expired for a serving cell, UE doesn’t need to do uplink Out-of-Synch behavior because CORESET pool associated with the other unexpired TAG can be used for DL/UL transmission. Furthermore, it should be investigated for the SpCell(i.e., PCell or PSCell) that which combination of two TAGs can be considered, e.g., PTAG + PTAG, PTAG + STAG.

Proposal #8: Uplink Out-of-Synch behavior when TAT for TAG is expired should be studied for a serving cell with two TAGs.

Conclusion

In this contribution, we have discussed the enhancement on two TAs for multi-DCI based M-TRP operation, and provided the following observations and proposals.

Proposal #1. Enhancements for two TAs should take vehicle DAS UE implementation into account, where each TA value is managed by each distributed antenna group or panel.

Observation #1: In simulation results, carrier frequency, subcarrier spacing, difference on distance to each TRP, inter-TRP reference timing offset and inter-UE-panel delay are dominant factors causing significant timing offset between TRPs.

Proposal #2: Enhancements for two TAs should take the case that each TA value is managed by each UE-panel into account.

Proposal #3: Support TAG-specific reference cell for two reference timings.

Proposal #4: Support symbol-level dropping for overlapped part between two consecutive UL transmissions associated with different TAs(i.e., Alt 2: Reduce the overlapping duration of one of the two UL transmissions).

Proposal #5: For TA association for UL channel/RS which doesn’t follow the indicated TCI in Rel-17/18 unified TCI framework and for UL channel/RS when Rel-15/16 spatial relation framework is used, associate TAG ID to target UL channels/RSs directly for semi-static UL channels/RSs (e.g., P CSI PUCCH, P SRS, CG PUSCH).

- For dynamic UL channels/RSs, support association of TAG ID to CORESETPoolIndex.

Proposal #6: For PDCCH ordered RACH,

- Support Alt 6, i.e., TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order, when cross-TRP RACH triggering is disabled by network.

- Support Alt 2, i.e., indicate TAG ID as part of PDCCH order, when cross-TRP RACH triggering is enabled by network.

Proposal #7: Study RAR window by considering RAR forwarding delay.

Proposal #8: Uplink Out-of-Synch behavior when TAT for TAG is expired should be studied for a serving cell with two TAGs.

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink

[2] R1-2205116(S-220026), LS on Rel-18 WI related to vehicular distributed antenna systems, 5GAA WG4, LG Electronics

[3] R1-2103511, Analysis on TRP- and/or UE panel-specific UL timing synchronization, LG Electronics

[4] R1-062834, UL Timing Control Accuracy and Update Rate, Nokia

[5] R1-071479, Simulation of Uplink Timing Error Impact on PUSCH, Texas Instruments

[6] TS 38.133, Requirements for support of radio resource management

[7] TS 38.101-1, User Equipment (UE) radio transmission and reception

[8] “3GPP RAN1 #94bis Chairman’s Notes”, October 8th – 12th, 2018

[9] TR 38.808, Study on supporting NR from 52.6 GHz to 71 GHz