3GPP TSG RAN WG1 #112bis-e	   	 	  	                	      R1- 2303070

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.2

Source:	LG Electronics

Title:	Potential CSI enhancement for high/medium UE velocities and coherent JT

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss potential CSI enhancement for high/medium UE velocities and for coherent JT. Since specification impact and functionality are different for high speed UE and for CJT, we first discuss Type II codebook enhancement for high/medium UE velocities in Section 2.1, TDCP reporting in Section 2.2, and potential CSI enhancement for CJT in Section 2.3

Discussion

Type II codebook enhancement for high speed UE

In this section, we focus the type II codebook enhancement for high speed UE and list up potential issues in the following order: channel measurement, CSI reporting window, TD (time domain) basis.

In order to support burst channel measurement for aperiodic CSI-RS, the following agreement was made in the last meeting.

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR, support the following:

- (Alt1) Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s)

Since the K AP NZP CSI-RS resources are burst CMRs for the same channel, UE should assume the same antenna port for the same antenna port index across the CSI-RS resources like TRS. Since it was agreed that the K CSI-RS resources are configured in NZP-CSI-RS-ResourceSet without CRI reporting, UE can assume the same antenna port for the same antenna port index across the CSI-RS resources if CRI reporting is not configured with CMR of multiple CSI-RS resources. Furthermore, powerControlOffset of the K AP CSI-RS resources, which is power offset of PDSCH RE to NZP CSI-RS RE, should be the same since the K AP CSI-RS resources are configured for the same channel measurement, in this case.

Proposal 1: UE assumes the same antenna port for the same antenna port index across the K AP CSI-RS resources if CRI reporting is not configured with CMR of multiple CSI-RS resources in a same CSI-RS resource set.

Proposal 2: UE expects the same power offset of PDSCH RE to NZP CSI-RS RE for the K AP CSI-RS resources.

Also, since the K AP NZP CSI-RS resources are burst CMRs for the same channel, CPU relaxation is needed. According to current specification, L CPUs are occupied when L AP CSI-RS are configured for AP CSI reporting but given that the K CSI-RSs show the same antenna port, one CPU may be enough. In addition, CSI processing time relaxation is needed because UE should predict channel for N4 future time instances and calculate corresponding future PMIs. Legacy Z and Z’ is not sufficiently large enough considering the additional complexity.

Proposal 3: one CPU is occupied for an AP CSI report configured with K AP NZP CSI-RS resources for burst CMR.

Proposal 4: Study CSI processing time relaxation with consideration of additional complexity to predict future channel and calculate multiple future PMIs.

Regarding UCI omission, the following agreement was made in the last meeting.

Agreement

On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, down-select between the following three alternatives (by RAN1#112bis-e where q denotes the q-th DD basis vector):

- Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.q

  - Note: This implies that DD basis is designated the highest priority

- Alt2. Prio(,l,m,q)=2L.S(q).RI.N3+2L.RI. P(m)+RI.l+

  - Note: This implies that DD basis is designated the lower priority (after FD basis)

  - FFS: S(q) maps the index q according to a rule

- Alt3. Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l + 

  - Note: This implies that DD basis is designated the least priority

- Alt4. Prio(,l,m,q)=2L.P(m).RI.Q+2L.RI.S(q)+RI.l+

  - Note: This implies that DD basis is designated with lower priority (after SD basis) and higher priority (before FD basis)

  - FFS: S(q) maps the index q according to a rule

FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m

q=0,…,Q-1

In Alt 2 and 3, the same PMI is reported for the entire CSI reporting window when UCI omission occurs since only one DD basis, which is all one vector, is survived. In our view, it is reasonable to apply Doppler domain compression with Q=2 only if CSI container size is sufficient. Between Alt 2 and Alt 3, Alt 3 should be supported because Q>2 is not supported.

Proposal 5: Support Alt 3 (i.e., Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l + ) as a priority rule for UCI omission.

Regarding the constraint on the maximum number of non-zero coefficients, we agreed to reuse K0 in legacy way but how K0 is calculated remains FFS. Since DD basis vector is introduced, not only L and M but also Q should be reflected in K0. Therefore, K0 should be ⌈β×2LMQ⌉.

Proposal 6: Support K0 = ⌈β×2LMQ⌉.

One additional remaining issue is whether to support different CBSR for different time during CSI reporting window. Since UE is moving fast, it is desirable to support different SD beam restriction in different time considering time-varying UE location.

Proposal 7: Support different CBSR in different time during CSI reporting window considering time-varying UE location.

In the last meeting, reporting X=2 CQIs is agreed as optional UE feature and how to reduce CQI overhead remains FFS. For the 1st TD CQI, WB CQI and SB CQI can be calculated in legacy way. Meanwhile, for the 2nd TD CQI, WB CQI is not needed and only SB CQI can be reported given that it is unlikely to have different WB CQI for the 1st TD and 2nd TD since WB CQI is already averaged in wide frequency domain. 2nd TD SB CQI can be differential CQI from 1st TD WB CQI and encoded in part 2 CSI.

Proposal 8: For X=2, 1st TD WB CQI is reported, and 1st TD SB CQI and 2nd TD SB CQI are reported as differential CQI from 1st TD WB CQI.

TDCP reporting for high speed UE

In the last meeting the priority of the CSI report associated with TDCP reporting was discussed and the following agreement was made.

Agreement

For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is down-selected from the following alternatives:

- Alt1. Lower than other CSI reports

- Alt2. Same as CSI report(s) not carrying L1-RSRP or L1-SINR

- Alt3. Higher than other CSI reports

- Other alternatives are not precluded

Considering all possibility, we list up the following 5 alternatives assuming the same time domain reporting behaviour, the same cell ID and the same CSI report ID:

  - TDCP < CSI report < beam report

  - TDCP = CSI report < beam report

  - CSI report < TDCP < beam report

  - CSI report < TDCP = beam report

  - CSI report < beam report < TDCP

Since TDCP is used to determine CSI configuration such as selecting between type I/II codebook and periodicity of CMR, it make sense for TDCP to have higher priority than CSI report. So, Alt A and B are not good options. We are open for the other Alts.

Proposal 9: TDCP should have higher priority than CSI report not carrying L1-RSRP or L1-SINR.

Potential CSI enhancement for coherent JT

In the last meeting, the following CSI-RS configuration for CJT was agreed.

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported:

- The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP)

  - Each of the CSI-RS resources has a same number of CSI-RS ports

- Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

Regarding multiple CSI-RS configuration for CJT, one remaining issue is how to aggregate CSI-RS ports of K NZP CSI-RS. One simple approach is to index ports of K NZP CSI-RS resources in the order of CSI-RS resource ID.

Proposal 10: For aggregated K NZP CSI-RS resources, index ports of the CSI-RS resources in the order of CSI-RS resource ID.

Another remaining issue is time domain restriction for K NZP CSI-RS resources. If those CSI-RS are distributed in different slot, there is possibility that UE measures each CSI-RS with random phase rotation and it is not able to report correct phase coherence information among K CSI-RS resources, which is essential part of coherent JT. Also, there is UE side buffering issue when K CSI-RS resources are far distributed in time. A similar issue was discussed in Rel-17 NCJT CSI-RS configuration and the restriction of the same slot or 1 slot difference between two CSI-RS for NCJT was agreed.

Proposal 11: Support to configure K NZP CSI-RS resources for CJT CMR in the same slot only.

In the last meeting, the following was discussed and agreed.

Agreement

On the Type-II codebook refinement for CJT mTRP, the selection of N CSI-RS resources is performed by UE and reported as a part of CSI report where N∈{1,..., NTRP}

- N is the number of cooperating CSI-RS resources, while NTRP is the maximum number of cooperating CSI-RS resources configured by gNB via higher-layer signaling

- The selection of N out of NTRP CSI-RS resources is reported via NTRP-bit bitmap in CSI part 1

  - Note: The value of N is inferred from the selection

- A restricted configuration (gNB-configured via higher-layer signaling) where N=NTRP is supported

  - NTRP-bit bitmap is not reported when the restriction is configured

  - FFS: Whether other RRC-configured TRP selection restriction including configuring the value of N is supported

- This feature is UE optional

Note: This agreement does not impact the decision on Ln being configured by gNB or selected by UE

Note: per WID and previous agreement, the candidate values for NTRP of are 1, 2, 3, and 4.

Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses.

Since selecting N TRPs among NTRP increases UE complexity, UE needs more CPUs. Even though UE is not mandated to calculate CSI for multiple transmission hypotheses, it does not preclude full search UE implementation. For given N and NTRP, there are NTRPCN hypotheses and if one CPU is used for each hypothesis NTRPCN CPUs are needed. Alternatively, instead of increasing CPU, we can consider to increase CSI processing time Z/Z’. In addition, to reduce complexity for TRP selection, some restriction for candidate combination for N TRPs can be considered. For example, gNB can configure N value and UE selects N out of NTRP for a given N.

Proposal 12: For UE side TRP selection, study CPU/CSI processing time relaxation with consideration of additional complexity to select CJT transmission hypothesis.

Proposal 13: For UE side TRP selection, introduce additional restriction such as configuring N value by gNB.

Additionally, we see codebook normalization issue for CJT codebook. In legacy codebook, selected precoding matrix is normalized to make sum of layer power to be 1. If the same normalization method is applied for agreed CJT codebook, UE selects lower CQI than what UE can actually achieve because transmission power gain from multiple TRPs is not reflected in codebook due to the conventional normalization.

Observation 1: UE selects lower CQI than what UE can actually achieve, if CJT codebook is normalized in conventional way.

Proposal 14: Study how to normalize CJT codebook considering transmission power gain from multiple TRP transmission.

Regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, the following was agreed in the last meeting.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer:

- Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2)

  - FFS: Amplitude quantization table enhancement

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

- Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111)

  - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N)

    - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

- If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

In our view, Alt 3 should be supported. Alt 1 is not sufficient to consider RSRP difference among TRPs. As a result, performance may not be guaranteed in case of distributed TRPs especially for larger ISD scenario.

Proposal 15: Confirm working assumption with supporting Alt 3 for W2 quantization group.

Regarding linkage between Ln and {p,beta} combination, we should support at least legacy combination for NTRP=1. For NTRP>1, the sum of Ln can be used to make linkage to FD combination since UE complexity and overhead for PMI depends on the sum of Ln, rather than specific Ln combination. For example, if the sum of Ln = 4, then legacy FD combination for L=4 should be supported. Based on this background, supported combinations are summarized in Table 1, where other combinations may also be added on top of this table.

Proposal 16: For NTRP=1, support at least legacy combination for NTRP=1 and for NTRP>1, legacy FD combination for L= the sum of Ln should be supported.

Table 1 Type-II CJT: parameter combination

| NTRP | SD combo | FD combo {pv}, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | {1/8, 1/8, 1/16, 1/16}, ¼ | {1/8, 1/8, 1/16, 1/16}, ½ | {1/4, 1/4, 1/8, 1/8}, ¼ | {1/4, 1/4, 1/8, 1/8}, ½ | {1/4, 1/4, 1/4, 1/4}, ¾ | {1/2, 1/2, 1/2, 1/2}, ½ |
| 1 | 2 |  |  | X | X |  |  |
|  | 4 |  |  | X | X | X |  |
|  | 6 w/ restriction |  |  |  | X | X |  |
| 2 | {2,2} |  |  | X | X | X |  |
|  | {2,4} |  |  |  | X | X |  |
|  | {4,2} |  |  |  | X | X |  |
|  | {4,4} |  |  |  |  |  |  |
| 3 | {2,2,2} |  |  |  | X | X |  |
|  | {2,2,4} |  |  |  |  |  |  |
|  | {2,4,2} |  |  |  |  |  |  |
|  | {4,2,2} |  |  |  |  |  |  |
|  | {4,4,4} |  |  |  |  |  |  |
| 4 | {2,2,2,2} |  |  |  |  |  | N/A |
|  | {2,2,2,4} |  |  |  |  |  | N/A |
|  | {2,2,4,4} |  |  |  |  |  | N/A |
|  | {4,4,4,4} |  |  |  |  |  | N/A |

X= supported

--= not supported

Regarding UCI omission, the following was agreed in the last meeting.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource):

- Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n

  - Note: This implies that CSI-RS resource is designated the highest priority

- Alt2. Prio(,l,m,n)=2L’.Q(n).RI.N3+2L’.RI. P(m)+RI.l(n)+

  - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis)

  - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources

  - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI.

- Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+

FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m

In Alt2, it can drop all the information for a specific or a group of TRP(s). As a result, it reduces the benefit of mTRP transmission. Also, we don’t expect meaningful performance difference between Alt 1 and Alt 3, so we prefer Alt 3, which is simple extension of legacy.

Proposal 17: Support Alt 3, i.e., replace SD basis index l in legacy Priority calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+

Conclusion

In this contribution, we discuss potential CSI enhancement for high/medium UE velocities and for coherent JT and propose the followings.

Type II codebook enhancement for high speed UE:

Proposal 1: UE assumes the same antenna port for the same antenna port index across the K AP CSI-RS resources if CRI reporting is not configured with CMR of multiple CSI-RS resources in a same CSI-RS resource set.

Proposal 2: UE expects the same power offset of PDSCH RE to NZP CSI-RS RE for the K AP CSI-RS resources.

Proposal 3: one CPU is occupied for an AP CSI report configured with K AP NZP CSI-RS resources for burst CMR.

Proposal 4: Study CSI processing time relaxation with consideration of additional complexity to predict future channel and calculate multiple future PMIs.

Proposal 5: Support Alt 3 (i.e., Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l + ) as a priority rule for UCI omission.

Proposal 6: Support K0 = ⌈β×2LMQ⌉.

Proposal 7: Support different CBSR in different time during CSI reporting window considering time-varying UE location.

Proposal 8: For X=2, 1st TD WB CQI is reported, and 1st TD SB CQI and 2nd TD SB CQI are reported as differential CQI from 1st TD WB CQI.

TDCP reporting for high speed UE:

Proposal 9: TDCP should have higher priority than CSI report not carrying L1-RSRP or L1-SINR.

Potential CSI enhancement for coherent JT:

Proposal 10: For aggregated K NZP CSI-RS resources, index ports of the CSI-RS resources in the order of CSI-RS resource ID.

Proposal 11: Support to configure K NZP CSI-RS resources for CJT CMR in the same slot only.

Proposal 12: For UE side TRP selection, study CPU/CSI processing time relaxation with consideration of additional complexity to select CJT transmission hypothesis.

Proposal 13: For UE side TRP selection, introduce additional restriction such as configuring N value by gNB.

Observation 1: UE selects lower CQI than what UE can actually achieve, if CJT codebook is normalized in conventional way.

Proposal 14: Study how to normalize CJT codebook considering transmission power gain from multiple TRP transmission.

Proposal 15: Confirm working assumption with supporting Alt 3 for W2 quantization group.

Proposal 16: For NTRP=1, support at least legacy combination for NTRP=1 and for NTRP>1, legacy FD combination for L= the sum of Ln should be supported.

Proposal 17: Support Alt 3, i.e., replace SD basis index l in legacy Priority calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+

- References

[1] RP-213598	New WID: MIMO Evolution for Downlink and Uplink		Samsung

[2] R1-2210523	Summary of TDCP Alternatives for Comparison			Samsung